3GPP TSG-RAN WG4 Meeting #113	R4-2418996

Orlando, Florida, USA, 18th to 22nd  November 2024

Agenda Item:	7.8.2.1

Source:	Ericsson

Title: 	On Co-existence simulation assumption considerations for NR-NTN and NB-IoT-NTN HP UE

Document for:	Discussion

# 1	Introduction

In the last RAN plenary meeting RAN#104, there was an agreement for a new Rel-19 RAN4 led WI Enhanced requirements and test methodology for NR and IoT NTN [1]. The baseline assumptions agreed in RAN4#112bis meeting have been captured in WF [2].

This contribution intends to trigger further discussion on co-existence studies assumptions focusing on adjacent channel interference between NTN and TN networks based on the Release – 17 NTN co-existence studies [3].

# 2	Co-existence simulation assumptions

### 2.1	TN network topology – with/ without wrap around functionality

The wraparound of TN is used to derive the SINR of victim TN. When wrap around is not implemented in simulation studies, there is a possibility to see unrealistically higher coverage and better SINR pointing to over optimistic results. When wrap around is implemented, the effect of interference is statistically equivalent across the simulation area. It is viable to say that the effect of SINR may be more prominent for the cell edge UEs since they will operate in low SINR region.

Figure 2.1: With and without Wrap around modelling [4]

The bold figure depicts the entire network region relevant for simulation in a cluster of 19 sites. In the left-hand figure, the network region is without wrap around. Site 0 in the central cluster (in red), experiences interference from all sides. If you look at Site 15 considered as cell edge experiences less interference as compared to Site 0. In the right-hand figure, enabling wrap around repeats the cluster to uniformly surround the central cluster.

When wrap around is implemented, the effect of interference is statistically equivalent across the simulation area.

In our companion contribution [5], we have presented our simulation results to study the performance impact with and without wrap around functionality. As can be seen from the results, with the wrap around functionality the ACIR requirements are more stringent – a few dBs higher as compared to without wrap around functionality.

ACIR requirements are more stringent – a few dBs higher when wrap around functionality is enabled.

It is important to consider the worst case when studying impact on victim TN network, to ensure the legacy networks are protected.

RAN4 to consider wrap around functionality as it impacts the derivation of SINR for victim TN BS.

### 2.2	NR NTN UE ACLR

The Table 6.5.2.4.1-2 from TS 38.101-1 [6] below depicts the ACLR values for NR UE for Power Classes 3, 2,1.5 and 1.

Table 6.5.2.4.1-2: NR ACLR requirement

|  | Power class 1 | Power class 1.5 | Power class 2 | Power class 3 |
| --- | --- | --- | --- | --- |
| NR ACLR | 37 dB | 31 dB | 31 dB | 30 dB |
| NOTE 1: Void |  |  |  |  |

The NR ACLR value is specified as 31 dB for Power Class 2 and 1.5 and 37 dB for Power Class 1.

In our companion contribution [5], we presented our simulation results for Case 4 when NR-NTN/NB-IoT NTN are interfering interfering TN UL with Power class 2,1.5 and 1. The ACIR requirements for 5%-tile have been summarized below -

Table 2.2-1: Summary of ACIR requirements for PC2, 1.5 and 1

| ACIR (dB) |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Case 4 |  | PC 2 | PC 1.5 | PC 1 |
| With wrap-around | GEO | NR NTN and TN BS with AAS | 11 | 13 | 15 |
|  |  | NR NTN and TN BS with Non-AAS | 3 | 6 | 8 |
|  |  | NB-IoT NTN | NA | 1 | 3 |
|  | LEO1200 | NR NTN and TN BS with AAS | 9 | 13 | 15 |
|  |  | NR NTN and TN BS with Non-AAS | 3 | 5 | 7 |
|  |  | NB-IoT NTN | 1 | 4 | 5 |
| Without wrap-around | GEO | NR NTN and TN BS with AAS | 3 | 7 | 9 |
|  |  | NR NTN and TN BS with Non-AAS | NA | 2 | 4 |
|  |  | NB-IoT NTN | NA | 2 | 4 |
|  | LEO1200 | NR NTN and TN BS with AAS | 6 | 9 | 11 |
|  |  | NR NTN and TN BS with Non-AAS | NA | 2 | 4 |
|  |  | NB-IoT NTN | NA | 0 | 2 |

The ACIR requirements are lower when isolation distance is considered.

RAN4 to re-use TN ACLR requirements for PC 2, PC 1.5 and PC 1 even if lower ACIR requirements are observed from co-existence studies.

# Conclusion

In the previous sections we made the following observations:

Observation 1	When wrap around is implemented, the effect of interference is statistically equivalent across the simulation area.

Observation 2	ACIR requirements are more stringent – a few dBs higher when wrap around functionality is enabled.

Observation 3	It is important to consider the worst case when studying impact on victim TN network, to ensure the legacy networks are protected.

Observation 4	The NR ACLR value is specified as 31 dB for Power Class 2 and 1.5 and 37 dB for Power Class 1.

Observation 5	The ACIR requirements are lower when isolation distance is considered.

Based on the discussion in the previous sections we propose the following:

Proposal 1	RAN4 to consider wrap around functionality as it impacts the derivation of SINR for victim TN BS.

Proposal 2	RAN4 to re-use TN ACLR requirements for PC 2, PC 1.5 and PC 1 even if lower ACIR requirements are observed from co-existence studies.

# References

- RP-240857, “New WID: Enhanced requirements and test methodology for NR and IoT NTN”, RAN4 chair (Huawei)

- R4-2417122,”WF on co/existence study and HPUE for NR-NTN”, 3GPP RAN4#112bis, October 2024

- 3GPP TR 38.863 v18.3.0: “Solutions for NR to support non-terrestrial networks (NTN): Non-terrestrial networks (NTN) related RF and co-existence aspects”

- NR Interference Modeling with Toroidal Wrap-Around - MATLAB & Simulink - MathWorks Nordic, accessed November 5, 2020

- R4-2418995, “On Co-existence simulation assumption considerations for NR-NTN and NB-IoT-NTN HP UE”, 3GPP RAN4#113, Ericsson

- 3GPP TS 38.101-1 V18.7.0, “NR; User Equipment (UE) radio transmission and reception; Part 1: Range 1 Standalone”