3GPP TSG-RAN WG4 Meeting # 113					R4-2418998

Orlando, US, 18th – 22nd November, 2024

Source:		LG Electronics, OPPO

Title:			Draft TP to TR 38.769 for A-IoT intermediate node RX requirements

Agenda item:		7.21.3.3

Document for:		Approval

Introduction

In this paper, we present our thoughts on Intermediate node(Rx) for A-IoT SI.

- References

- RP-240826, Revised SID: Study on solutions for Ambient IoT (Internet of Things) in NR, CMCC, Huawei, T-Mobile USA, RAN #103, Mar. ,2024

- R4-2417088, WF on impacts of A-IoT on RF requirements, Moderator (Huawei), RAN4#112bis, Oct.,2024

- TP for TR 38.769

<Start of Text Proposal>

- 6.7.3	Intermediate node (UE)

- The intermediate UE, under NW control, can send Ambient IoT data and/or signalling to the device, and receive the backscatter signal from the device. The intermediate UE can also operate as a legacy NR UE. For simplicity, inside topology is assumed(D2T2-A2).

- 6.7.3.1 Rx requirement

- 6.7.3.1.1 Reference sensitivity

Reference sensitivity could be derived based on the following equation:

PREFSENSdBm=-174dBm+10×log10BW+NF+IM+SNR+[desens]

-	BW is the transmission bandwidth for D2R.

-	NF is the noise figure of Intermediate node.

-	IM is the implementation margin.

-	SNR is the SNR value for D2R decoding.

-    desens is considered for the residual CW interference signal after cancellation.

- 6.7.3.1.2 Maximum input power

- Maximum input power is defined to ensure the intermediate node will not be damaged from power coming from outside, and mainly depending on saturation region of active component(e.g. LNA, Mixer) for intermediate node. The intermediate UE is expected to reuse the existing maximum input power.

- 6.7.3.1.3 ACS

ACS requirement shall be determined by RAN4 co-existence study.

- 6.7.3.1.4 In-band blocking

In-band blocking requirement shall be determined by RAN4 co-existence study.

- 6.7.3.1.5 Out of band blocking

Out-of-band blocking is defined for an unwanted CW interfering signal falling outside a frequency range 15MHz below or above the UE receive band. the existing OOBB requirement is still applicable for intermediate node.

- 6.7.3.1.6 Narrow band blocking

TBD

- 6.7.3.1.7 Spurious response

TBD

- 6.7.3.1.8 Receiver intermodulation

Intermodulation response rejection is a measure of the capability of the receiver to receive a wanted signal on its assigned channel frequency in the presence of two or more interfering signals which have a specific frequency relationship to the wanted signal. The intermodulation requirement can be defined to consider D2R operation assumption e.g. single tone or multiple tone D2R assumption

- 6.7.3.1.9 Rx spurious emission.

The spurious emissions power is the power of emissions generated or amplified in a receiver that appear at the UE antenna connector. the existing Rx spurious emission requirement is still applicable for intermediate node.

A summary for RX part requirement is captured in the Table X.

| A-IoT intermediate UE- RX RF Requirement | Potentially needed or not |
| --- | --- |
| Reference sensitivity | YES |
| Maximum input power | YES |
| ACS | YES |
| In-band blocking | YES |
| Out-of-band blocking | YES |
| Narrow band blocking | TBD |
| Spurious response | TBD |
| Receiver intermodulation | YES |
| RX spurious emissions | YES |

<End of Text Proposal>