# 3GPP TSG-RAN WG4 Meeting #113	R4-2419004

# Orlando, US, 18th – 22nd November, 2024

Agenda Item:	4.5.1

Source: 	Ericsson

Title:	Clarification on Rel-16 mandatory gap patterns capability

Document for:	Discussion

# Clarification

In Rel-16, RAN4 introduced mandatory gap pattern capability supportedGapPattern-NRonly-r16 in NR SA on top of Rel-15 supportedGapPatterns.

In our understanding, Rel-16 mandatory gap pattern is the gap pattern introduced for NR SA without E-UTRA measurement objects. When UE reports to support such gap pattern, it means UE supports NR only measurement but not support LTE&NR measurement. On the contrary, if UE reports to support a gap pattern in Rel-15 supportedGapPatterns, it means UE supports both NR and LTE measurement wo restriction.

However, it will be a mismatch based on today’s capability design when UE reports to support a pattern in Rel-15 supportedGapPatterns but reports not support the pattern in Rel-16 supportedGapPattern-NRonly-r16. From NW side, it’s difficult to understand the UE reporting capability with such mismatch.

|  |  | Support for GP2-GP11 |  |  |
| --- | --- | --- | --- | --- |
| supportedGapPatterns | supportedGapPatterns-NRonly-r16 | LTE measurements only | NR FR1 + LTE measurements | NR FR1 measurements only |
| 0 | 0 | No | No | No |
| 0 | 1 | No | No | Yes |
| 1 | 0 | Yes | Yes | Yes |
| 1 | 1 | Yes | Yes | Yes |

Proposal 1: RAN4 to send LS to RAN2 to clarify that UE is not allowed to report ‘0’ for one pattern in supportedGapPatterns-NRonly-r16 if UE reports ‘1’ to support the same pattern in Rel-15 supportedGapPatterns.

- Summary

In this contribution, we discussed the mandatory gap capability mismatch. Based on the discussions, we have made following proposals and observations:

Proposal 1: RAN4 to send LS to RAN2 to clarify that UE is not allowed to report ‘0’ for one pattern in supportedGapPatterns-NRonly-r16 if UE reports ‘1’ to support the same pattern in Rel-15 supportedGapPatterns.