3GPP TSG-RAN WG4 Meeting #113                                         R4-2419026

Orlando, US, November 18 - 22, 2024

Source: 	Huawei, HiSilicon

Title: 	On MPR applicability for FR1 intra-band UL CA

Agenda Item:	7.1.1.2.2

Document for:	Approval

Introduction

During RAN4#112bis, MPR applicability for FR1 intra-band contiguous/non-contiguous UL CA has been discussed. Following conclusions can be found in [1].

- Intra-band contiguous UL CA - Applicable MPR for intra-band contiguous CA with single activated cell - WF - For PC3/PC2 intra-band contiguous carrier aggregation with single CC with activated cell, the following MPR requirements are applied - MPR defined in Table 6.2.2-1 applies for UE power class 3 CA bandwidth classes B and C; - MPR defined in Table 6.2D.2-1 applies for power class 2 CA bandwidth classes B and C when TxD and/or UL-MIMO capability are indicated - MPR defined in Table 6.2.2-2 applies for power class 2 CA bandwidth classes B and C when TxD and/or UL-MIMO capability are absent. - The Rel-18 power boosting feature can be supported depending on UE capability - Feature group description shall clarify the interaction between Rel-18 power boosting capabilities and Rel-19 MPR applicability enhancement capability. - FFS PC1.5 MPR enhancement as intra-band CA MPR requirements for PC1.5 are not available yet - Single CC CBW or aggregated CBW for applying requirements of ACLR/SEM/SE - Proposals - Proposal 1: activated CC CBW or aggregated CBW adopted for integral region and boundary of spurious emissions/ACLR/SEM depending on dualPA-Architecture IE indication. - Proposal 2: Emission requirements are based on aggregated channel bandwidth also in case that only one CC is activated. - WF - FFS in next meeting - Single CC or CA requirements of ACLR/SEM/SE applied for single activated cell - Proposals - Proposal 1: CA requirements of ACLR/SEM/SE should be applied for intra-band contiguous CA with single activated cell. - Proposal 2: - For UE supporting contiguous ULCA with the dualPA IE: - The applicable SEM, ACLR and spurious emissions are the single CC emissions defined in Table 6.5.2.2-1, Table 6.5.2.4.1-1 and Table 6.5.3.1-2 respectively. - For UE supporting contiguous ULCA without signalling the dualPA IE or signalling PC1.5 or TxD or UL MIMO support: - The applicable SEM, ACLR and spurious emissions are the configured contiguous ULCA - Proposal 1: After the initial configuration of contiguous ULCA, upon SCELL deactivation with only 1CC active, the applied SEM mask should be the aggregated CA BW SEM as stated in sub-clause 6.5A.2.2.1 in TS38.101-1. Further discuss SEM mask for single CC fall back only if dualPA-architecture is supported - Proposal 3: Apply single carrier spurious emission/ACLR/SEM requirements for contiguous UL CA with only 1 CC transmitted. - WF - FFS in next meeting - Whether UL interruption is allowed for intra-band contiguous CA with single activated cell - Proposals - Proposal 1: No interruptions are allowed for enabling single carrier MPR for the case when only one CC is activated in intra-band contiguous UL CA. - Agreement in AH - Proposal 1 - Intra-band non-contiguous CA - Applicable MPR for FR1 intra-band non-contiguous UL CA - Proposals Option 1: No spec impact or minimum spec impact with some clarification but no changes of requirements - Proposal 1: for PC3 and PC2 intra-band non-contiguous CA as the standard already accounts for the use of the single CC MPR tables when only 1 CC is scheduled no further changes to the standard are required. - Proposal 2: adding 1CC activation as a supplementary scenario in the spec but no changes of the requirements - Proposal 3: single CC MPR applied with single CC emission requirements: Option 2: Single CC MPR/SEM applied on indication of dualPA IE - Proposal 4: To apply non-CA (single CC) MPR based on indication of dualPA IE - Proposal 5: After the initial configuration of non-contiguous ULCA, upon SCELL deactivation with only 1CC active, the applied SEM mask should be the composite SEM of the 2CCs as stated in sub-clause 6.5A.2.2.2 in TS38.101-1. Further discuss to support the single CC SEM only if dualPA-architecture is supported. Option 3: Possible spec impact on LO retuning - Proposal 6: Discuss impact on requirements if LO needs to be re-configured to support single CC MPR. This discussion would be required for intra-band contiguous and non-contiguous carrier aggregation - WF - FFS on whether for PC2 and PC3, no MPR requirements to be changed. - FFS PC1.5 when NC CA requirements are available - FFS clarification for the applicable emission requirements whether they are single CC based or composite based Agreement in main session - Drop the discussion for LO retuning since it is out of WI scope - FFS for FR2

In this contribution, we would like to share our views regarding this topic.

Discussion

In previous discussion, companies proposed whether MPR improvement along with the OOB application could be related to the UE indication on dualPA-architecture, which is a per BC capability. Thus we think it is more appropriate to introduce the UE capability for Rel-19 FR1 intra-band UL CA as per BC as well, whether to consider separate capability for contiguous and non-contiguous case can be FFS.

Proposal 1: The granularity of Rel-19 UE capability on FR1 intra-band UL CA MPR improvement should be per BC. Whether separate capabilities for contiguous and non-contiguous case can be FFS.

One remaining issue is how to handle the spurious emissions/ACLR/SEM requirements for the new MPR applicability for contiguous case. Intuitively, reuse the emission requirements defined for intra-band contiguous UL CA would be the most UE implementation friendly way, which is also very simple to move forward comparing to excessive discussion on whether the UE could support dualPA-architecture or not.

Proposal 2: CA requirements of ACLR/SEM/SE should be applied for FR1 intra-band contiguous UL CA with single activated cell.

Proposal 3: For Rel-19 MPR applicability for FR1 intra-band contiguous UL CA, emission requirements are based on aggregated channel bandwidth also in case that only one CC is activated.

Conclusion

In this contribution we discussed on the remaining issue for Rel-19 MPR applicability for FR1 intra-band UL CA. We have the following proposals:

Proposal 1: The granularity of Rel-19 UE capability on FR1 intra-band UL CA MPR improvement should be per BC. Whether separate capabilities for contiguous and non-contiguous case can be FFS.

Proposal 2: CA requirements of ACLR/SEM/SE should be applied for FR1 intra-band contiguous UL CA with single activated cell.

Proposal 3: For Rel-19 MPR applicability for FR1 intra-band contiguous UL CA, emission requirements are based on aggregated channel bandwidth also in case that only one CC is activated.

- References

- R4-2417204, “WF on power domain enhancements”, Huawei, HiSilicon, 3GPP TSG-RAN4 Meeting #112bis, Hefei, China, October 14-18, 2024.