3GPP TSG-RAN WG4 Meeting #113	R4-2419029

# Orlando, US, 18th – 22nd November, 2024

Title: 	On MIMO layer evaluation for 6Rx UE

Source: 	Huawei, HiSilicon

Agenda item:	7.1.1.3.2

Document for:	Approval

Background

In last meeting, WF [1] was approved, one objective is MIMO layers for 6Rx. This contribution provides our views.

- Simulation results

Simulation assumptions are captured in Table 2.1-1:

Table 2.1-1: Simulation assumptions for MIMO layers evaluation for 6Rx

| Assumptions |  | Values |
| --- | --- | --- |
| Duplex mode |  | TDD |
| Bandwidth |  | 40MHZ |
| SCS |  | 30kHz |
| Antenna configuration |  | 8T6R |
| Propagation channel |  | TDLA30-10 |
| Rank |  | Fixed rank {4,6} |
| MCS |  | Adaptive MCS |
| PDSCH configuration | Mapping type | Type A |
|  | Starting symbol | 2 |
|  | Length | 12 |
|  | PRB bundling size | 2 |
|  | VRB-to-PRB mapping type | Non-interleaved |
|  | Precoding | Random PMI (Rel-15 Type 1 codebook) |
| PDSCH DMRS configuration | DMRS Type | Rel-15 Type 1 |
|  | DMRS Configurations | Option 1: Rel-15 1+1(4L), Rel-15 2+0(6L) Option 2: Rel-15 1+1(4L), Rel-15 2+2(6L) |
|  | Precoding | Random PMI (Rel-15 Type 1 codebook) |
| UE Correlation matrix |  | Option 2: 1.000.100.101.000.300.10 0.300.150.100.141.000.12 0.130.120.300.200.200.120.150.140.130.300.120.20 0.121.000.200.110.120.30 0.110.301.000.100.101.00 Option 3: 1.000.220.221.000.300.580.600.150.150.100.050.150.300.600.580.151.000.150.151.000.150.150.150.100.150.050.100.150.150.150.150.101.000.300.301.00 |
| BS correlation matrix |  | Option 1: 3GPP MIMO correlation matrix with Alpha = 0.0 Option 2: 3GPP MIMO correlation matrix with Alpha = 0.005 |

The simulation results for case of  DMRS1+1 for Rank4 and DMRS2+0 for Rank6, alpha=0.0 are captured in Figure 2.1-1:

Figure 2.1-1: Simulation results for case of DMRS 1+1 for Rank4 and DMRS 2+0 for Rank6, alpha=0.0

We have following observations for case of DMRS 1+1 for Rank4 and DMRS 2+0 for Rank6, alpha=0.0:

Observation 1: 	For case of DMRS 1+1 for Rank4 and DMRS 2+0 for Rank6, alpha=0.0:

For 1% TxEVM, 6 layers outperforms 4 layers when SNR is higher than 31dB

For 3% TxEVM, 4 layers always outperforms 6 layers

The simulation results for case of DMRS1+1 for Rank4 and DMRS2+2 for Rank6, alpha=0.0 are captured in Figure 2.1-2:

Figure 2.1-2: Simulation results for case of DMRS 1+1 for Rank4 and DMRS 2+2 for Rank6, alpha=0.0

We have following observations for case of DMRS 1+1 for Rank4 and DMRS 2+2 for Rank6, alpha=0.0:

Observation 2: For case of DMRS 1+1 for Rank4 and DMRS 2+2 for Rank6, alpha=0.0:  4 layers always outperforms 6 layers.

The simulation results for case of DMRS1+1 for Rank4 and DMRS2+2 for Rank6, alpha=0.005 are captured in Figure 2.1-2:

Figure 2.1-3: Simulation results for case of DMRS 1+1 for Rank4 and DMRS 2+2 for Rank6, alpha=0.005

We have following observations for case of DMRS 1+1 for Rank4 and DMRS 2+2 for Rank6, alpha=0.005:

Observation 3: For case of DMRS 1+1 for Rank4 and DMRS 2+2 for Rank6, alpha=0.005: 4 layers outperforms 6 layers.

- Discussions

UE type

According to WI[1], the requirements should target at handled UE and the specified requirements can be applicable to both UE and FWA devices.

6Rx for handheld and FWA UE - Specify the core requirements to enable 6Rx for higher frequency bands (>2.5GHz) targeting at support of handheld UE for NR FR1 single carrier scenario - Example bands: n41, n77/n78, n79, n104 - Support 4 MIMO layers at least, and study the gain and feasibility and if feasible, support 6 MIMO layers - Specify the Rx requirements including reference sensitivity requirements for support 6Rx - Note: the specified requirements can be applicable to both handheld UE and FWA devices - Specify the requirements to support SRS antenna switching including t1r6, t2r6, t3r6, t4r6 depending on UE capability - Study the issue of insertion loss imbalance across SRS ports, and if justified, specify the corresponding solution.

Therefore, FWA should be excluded when evaluating the feasibility of 6 MIMO layers. FWA already support 8Rx with 8 layers and are available on market, it’s very meaningless to discuss the feasibility of 6 layers. Hence we propose the following: Whether to introduce the 6 layers capability depends on the outcome of 6 MIMO layers evaluation on handled UE.

Observation 4: WI specifies that the target UE type for 6 MIMO layers evaluation is handled UE.

Proposal 1: RAN4 shall conclude that whether to introduce 6 layers for both FWA and handheld UE depends on the outcome of 6 MIMO layers evaluation on handled UE.

Rank

It is noted that Rank 5 is one candidate option, however, according to the WI, rank5 is out of the scope, we propose to only focus on 6layers and 4layers.

Proposal 2: RAN4 shall only focus on Rank4 and 6, not consider Rank5.

Number of Tx ports

We don’t support 32 ports+ SVD precoder. As we discussed in last meeting, the correlation at BS side is quite low, under the assumption of almost low correlation at BS side, the channel beam is omnidirectional, which means there is no chance to benefit from beamforming gain. Furthermore, 8Tx and 32Tx has similar diversity gain since the number of Tx is enough. So we don’t think there is performance gap between 8Tx and 32Tx. Regarding the SVD based precoder, , 6Rx SRS antenna switching will cause big performance degradation compared to 4Rx due to larger insertion loss and reporting delay, even with limited UE pre-compensation  also is hard to be modelled in the simulation. So RAN4 shall use 8Tx+Single panel I codebook feedback.

Observation 5: With the assumption of almost low correlation at BS side, the channel beam is omnidirectional, there is no chance to benefit from beamforming gain.  8Tx and 32Tx should perform similar.

Observation 6: 6Rx SRS antenna switching will cause big performance degradation compared to 4Rx due to larger insertion loss and reporting delay, even with limited UE pre-compensation also is hard to be modelled in the simulation.

Proposal 3: RAN4 shall use 8Tx+Single panel I codebook feedback for 6Rx 6layers evaluation.

DMRS configuration

DMRS overhead is a key factor that affects evaluation of number of MIMO layers. It’s mentioned that Rel-15 DMRS type 1 is the most common configuration in the real deployment. Rel-18 eDMRS is introduced in Rel-18 and has not been supported by UE and BS vendors currently.  It is uncertain whether it will be widely used in the future.. Feasibility evaluation should be based on the simulation assumptions which has been widely used in the real deployment. Rel-18 eDMRS can be used for evaluation in the future release if commonly implemented by most UE vendors. Also, additional DMRS is needed to guarantee the channel estimation accuracy.

Proposal 4: In Rel-19, RAN4 to use Rel-15 DMRS type 1 for 6Rx MIMO layers evaluation, DMRS 1+1 for Rank4 and DMRS 2+2 for Rank6.

TxEVM

TxEVM is one open issue, following options were made in the last meeting:

Issue 3-1-3: Tx EVM for feasibility study Way Forward: FFS on Tx EVM values to be used for feasibility evaluation. RAN4 to further discuss the following: - Option 1: Use Tx EVM = 1% - Option 2: Use Tx EVM = 3% for all of modulation order in MCS Table 2 for performance evaluation - Option 3: Consider TxEVM of 6% for 64QAM and TxEVM 3% for 256QAM - Option 4: use the existing BS Tx EVM core requirement - Option 5: The representative BS Tx EVM values provided by the BS vendor.

We support Option3, which is always used for demodulation requirements definition and is already relaxed compared to core requirement. Different BS vendor may have different Tx EVM, which is hard to be unified.

Proposal 5: RAN4 shall use 6% for 64QAM and 3% for 256QAM

RxEVM

It should be mentioned that RF impairment is not considered in the simulation and the SNR range where 6 layers outperforms 4layers is very high (higher than 30dB), but RxEVM plays an important role at such high SNR. For demodulation requirements definition, RAN4 always adds additional margin to the ideal SNR to capture the RxEVM impact. RAN4 shall also consider RxEVM impact for 6 MIMO layers evaluation with 2~3dB additional margin or certain number of RxEVM, such as 2% Rx EVM

Observation 7: RF impairment (RxEVM) is not negligible for 6 MIMO feasibility evaluation.

Observation 8: For demodulation requirements definition, RAN4 always adds margin on top of ideal results to capture RxEVM impact.

Proposal 6: 2~3dB additional margin or 2% Rx EVM shall be considered for 6 MIMO layers evaluation.

Concerns on foldable UE

People may think that the foldable UE could have more space to accommodate 6Rx , but we have different views on this with the following concerns.:

- One should note that the foldable phone is not always unfolded, which could depend on unpredictable factors e.g. user preference and hand blockage. When the phone is folded, the spatially overlapped antenna could be impacted which would lead to degraded receiving performance.

- From receiving power perspective, if the diversity branches are not on the main board, there could be power imbalance issue since the insertion loss would be high for the path across the hinge.

- Actually the hinge would partially occupy the edge of each screen in order to guarantee the structural stability, hence unlike phablet at least the occupied space cannot be used for antenna placement anymore. Then adding more antenna could make the antenna correlation worse.

In conclusion, we have the following proposals for the foldable UE assumed for the 6 layers feasibility study.

Proposal 7: For the foldable UE assumed for the 6 layers feasibility study, the following aspects should be considered for the 6 MIMO evaluation with additional 1~2dB margin:

- When the phone is folded, the spatially overlapped antenna could be impacted which would lead to degraded receiving performance.

- If the diversity branches are not on the main board, there could be power imbalance issue since the insertion loss would be high for the path across the hinge.

- The hinge would partially occupy the edge of each screen in order to guarantee the structural stability, hence unlike phablet at least the occupied space cannot be used for antenna placement anymore. Then adding more antenna could make the antenna correlation worse.

Complexity analysis

Besides performance gain, baseband complexity analysis and power consumption should also be considered. To support 2CWs, , which means complexity of decoder is doubled. Besides, dimension of channel matrix is increased from 4x6 to 6x6, the complexity of MIMO detector is also much increased, which is the module with the highest complexity increase. Besides, complexity of channel estimation and DMRS based interference estimation is also increased by 1.5 times when number DMRS ports is increased from 4 to 6. So, we strongly encourage to take baseband processing into account in addition to consider performance gains. It is not worthwhile to implement 6Layers after trading off the complexity and performance gain.

Proposal 8: RAN4 shall analyse the increased baseband complexity and power consumption for 6Layers compared to 4Layers including LDPC decoder, channel estimation, DMRS based interference estimation and MIMO detector.

Performance analysis

Based on our simulation results and Observations 1~3 shown in clause 2, in most normal cases, 4layers always outperforms 6layers, 6 layers outperforms 4layers only in corner case: DMRS 2+0 configuration with extreme high SNR, however, additional DMRS symbol is configured in the real network to guarantee the channel estimation performance and against high speed. 31dB SNR is also is not common even when UE is close to the Base station. It is mentioned that RxEVM and impacted factors for foldable phone as stated in Proposal 7 are not considered in the simulation, we believe that the SNR switching point where 6layers outperforms 4 layers will be much higher.

Proposal 9: Don’t introduce 6 layers for 6Rx in Rel-19.

- Conclusion

This contribution provides our views on spatial channel model for demodulation requirements definition, the observations and proposals are:

Observation 1: 	For case of DMRS 1+1 for Rank4 and DMRS 2+0 for Rank6, alpha=0.0:

For 1% TxEVM, 6 layers outperforms 4 layers when SNR is higher than 31dB

For 3% TxEVM, 4 layers always outperforms 6 layers

Observation 2: For case of DMRS 1+1 for Rank4 and DMRS 2+2 for Rank6, alpha=0.0:  4 layers always outperforms 6 layers.

Observation 3: For case of DMRS 1+1 for Rank4 and DMRS 2+2 for Rank6, alpha=0.005: 4 layers outperforms 6 layers.

Observation 4: WI specifies that the target UE type for 6 MIMO layers evaluation is handled UE.

Proposal 1: RAN4 shall conclude that whether to introduce 6 layers for both FWA and handheld UE depends on the outcome of 6 MIMO layers evaluation on handled UE.

Proposal 2: RAN4 shall only focus on Rank4 and 6, not consider Rank5.

Observation 5: With the assumption of almost low correlation at BS side, the channel beam is omnidirectional, there is no chance to benefit from beamforming gain.  8Tx and 32Tx should perform similar.

Observation 6: 6Rx SRS antenna switching will cause big performance degradation compared to 4Rx due to larger insertion loss and reporting delay, even with limited UE pre-compensation  also is hard to be modelled in the simulation.

Proposal 3: RAN4 shall use 8Tx+Single panel I codebook feedback for 6Rx 6layers evaluation.

Proposal 4: In Rel-19, RAN4 to use Rel-15 DMRS type 1 for 6Rx MIMO layers evaluation, DMRS 1+1 for Rank4 and DMRS 2+2 for Rank6.

Proposal 5: RAN4 shall use 6% for 64QAM and 3% for 256QAM

Observation 7: RF impairment (RxEVM) is not negligible for 6 MIMO feasibility evaluation.

Observation 8: For demodulation requirements definition, RAN4 always adds margin on top of ideal results to capture RxEVM impact.

Proposal 6: 2~3dB additional margin or 2% Rx EVM shall be considered for 6 MIMO layers evaluation.

Proposal 7: For the foldable UE assumed for the 6 layers feasibility study, the following aspects should be considered for the 6 MIMO evaluation with additional 1~2dB margin:

- When the phone is folded, the spatially overlapped antenna could be impacted which would lead to degraded receiving performance.

- If the diversity branches are not on the main board, there could be power imbalance issue since the insertion loss would be high for the path across the hinge.

- The hinge would partially occupy the edge of each screen in order to guarantee the structural stability, hence unlike phablet at least the occupied space cannot be used for antenna placement anymore. Then adding more antenna could make the antenna correlation worse.

Proposal 8: RAN4 shall analyse the increased baseband complexity and power consumption for 6Layers compared to 4Layers including LDPC decoder, channel estimation, DMRS based interference estimation and MIMO detector.

Proposal 9: Don’t introduce 6 layers for 6Rx in Rel-19.

- Reference

[1]  R4-2417209 WF on 6Rx UE requirements