3GPP TSG-RAN WG4 Meeting #113                                         R4-2419030

Orlando, US, November 18 - 22, 2024

Source: 	Huawei, HiSilicon

Title: 	On 6Rx SRS antenna switching requirements

Agenda Item:	7.1.1.3.3

Document for:	Approval

Introduction

During RAN4#112bis, FR1 6Rx UE RF requirements have been discussed. Following conclusion regarding ΔTRxSRS can be found in [1].

- Sub-topic 2-1: General considerations for SRS antenna switching and ΔTRxSRS Issue 2-1-1: Whether to consider an additional breakpoint for bands whose FUL_high is higher than the FUL_low of n104 Agreement: RAN4 agrees to consider additional breakpoint for bands whose FUL_high is higher than the FUL_low of n104. - Sub-topic 2-2: ΔTRxSRS values Issue 2-2-1: Proposed ∆TRxSRS values for t1r6, t2r6, and t1r6-t2r6 Agreement: RAN4 agrees to adopt the values for Bands n79 and n104 as specified below. Table: ΔTRxSRS values Operating Bands; ΔTRxSRS t1r6 (dB); ΔTRxSRS t2r6 (dB); ΔTRxSRS t1r6-t2r6 (dB). Band n79; 5.5; 5.0; 6.0. Band n104; 6.5; 5.5; 7.0. Issue 2-2-2: Proposed ∆TRxSRS values for t3r6, t1r6-t3r6, t2r6-t3r6, and t1r6-t2r6-t3r6 Agreement: As this is the first RAN4 meeting since the confirmation that RAN1 will consider t3r6, companies are encouraged to provide ∆TRxSRS values for t3r6, t1r6-t3r6, t2r6-t3r6, and t1r6-t2r6-t3r6 at RAN4#113 for further discussion with the Apple and Qualcomm proposals in R4-2415324 and R4-2416264, respectively.

In this contribution, we would like to share our views regarding this topic.

Discussion

As captured in RAN1 FL summary [2], RAN1 agreed to reuse the principles used for 4T8R AS-SRS pattern definition. Thus RAN4 can derive the ΔTRxSRS requirements for 3T6R AS-SRS accordingly. Obviously, we can focus on the following cases since they are all allowed by the current signalling design:

- 3T6R

- 1T6R-3T6R

- 2T6R-3T6R

- 1T6R-2T6R-3T6R

One observation from previous discussion is as the indicated number of AS-SRS patterns becomes more, the required number of RF components on a diversity branch would increase. Consequently, more relaxed requirement can be considered for 1T6R-3T6R or 1T6R-2T6R-3T6R. But for 3T6R, such relaxation won’t be too high since no extra RF switch would be needed, which normally contributes the most insertion loss relative to other RF components. Besides, no matter which power class the 3Tx UE is capable of, since there are three Tx chains available, 4.77dB (10*log3) margin is reserved per RAN1 specification for equal power split among SRS ports within one transmission occasion. Hence RAN4 could simplify the set of requirements starting from omitting ΔTRxSRS for the type of 3Tx UE that indicates 3T6R only. In conclusion, we have the following proposal for RAN4 consideration.

Proposal: For 3T6R AS-SRS, only define the ΔTRxSRS requirements for a subset of available patterns i.e. 1T6R-3T6R, 2T6R-3T6R and 1T6R-2T6R-3T6R.

- Further narrowing on the above subset can be considered in conjunction with the proposed values and the 4.77dB margin from 3Tx.

Conclusion

In this contribution we discussed on the 6Rx SRS antenna switching requirements. We have the following observations and proposal:

Proposal: For 3T6R AS-SRS, only define the ΔTRxSRS requirements for a subset of available patterns i.e. 1T6R-3T6R, 2T6R-3T6R and 1T6R-2T6R-3T6R.

- Further narrowing on the above subset can be considered in conjunction with the proposed values and the 4.77dB margin from 3Tx.

- References

- R4-2417209, “WF on requirements for 6Rx UE requirements”, AT&T, 3GPP TSG-RAN4 Meeting #112bis, Hefei, China, October 14-18, 2024.

- R1-2408191, “FL Summary Support for 3TX CB-based Uplink”, Moderator (InterDigital), 3GPP TSG-RAN1 Meeting #118bis, Hefei, China, October 14-18, 2024.