3GPP TSG-RAN WG4 Meeting #113                                         R4-2419031

Orlando, US, November 18 - 22, 2024

Source: 	Huawei, HiSilicon

Title: 	On SRS IL imbalance issue

Agenda Item:	7.1.1.3.4

Document for:	Approval

Introduction

During RAN4#112bis, SRS IL imbalance issue has been discussed. Following conclusions are reproduced from [1].

- Sub-topic 4-1: General considerations for SRS IL imbalance issue Issue 4-1-1: Whether to solve SRS IL imbalance issue in Rel-19 Way Forward: RAN4 to further discuss the following options. - Option 1: RAN4 should not continue the discussion on how to solve the SRS IL imbalance issue. - Option 2: Continue to pursue a solution to the SRS IL imbalance issue based on compromised solutions for consideration with minimal impact to the specification and to indicate the specific impacts to RAN1, RAN2, and RAN4 specifications and performance gain based on the outcome of Issue 4-2-1. Issue 4-1-2: Initial Considerations for SRS IL imbalance issue Way forward: RAN4 to focus on the compromised solution proposals in Issue 4-2-1. - Sub-topic 4-2: SRS IL imbalance issue solutions Issue 4-2-1: Candidate solutions for the SRS IL imbalance issue Way forward: Companies are encouraged to provide comments and indication of support for the solution proposals listed below by RAN4#113. Proponent companies (in parentheses) for the candidate solution and solution proposals below are encouraged to provide expected benefit/performance gains and draftCRs so that the impacts to RAN1, RAN2, and RAN4 specifications (for RAN1 and RAN2, a description of the necessary changes is sufficient if draftCRs are not possible) can be further discussed. Other proposals including expected performance gains and draftCRs are not precluded. - Candidate solutions - Option 1: - UE performs self-compensation of SRS IL up to its maximum Tx power capabilities - UE provides assistance to the network on SRS IL compensation (semi-static and/or dynamic), e.g. - Per SRS resource power headroom reporting - Configured maximum output power per SRS resource reporting - SRS insertion loss value reporting (per SRS resource or per UE) - Other - Details of SRS IL self-compensation and assistance framework are FFS - Other options are not precluded - Proposals - Proposal 1: Only introduce UE compensation and new accurate SRS power headroom reporting for non-CA/DC scenarios can be a compromised solution. Further study the granularity of new SRS power headroom report (China Telecom) - Proposal 2: IL imbalance reporting mechanism for SRS AS should include the configured maximum output power per SRS resource, the power headroom per SRS resource and ΔPPowerClass . The PH used for the SRS resource can be a Type 3 but used for a new MAC-CE “SRS resource power report” and can be used also for a carrier configured for PUSCH transmission (Ericsson). - Proposal 3: Another way to resolve the SRS IL imbalance reporting issue would be to introduce two types of reporting for a UE: a “baseline” and an “advanced” reporting. The baseline reporting would not require any calibration at the UE and it could include e.g. the PHR and ΔPPowerClass information. The advanced reporting could in addition include the information on e.g. the configured maximum output power (Ericsson). - Proposal 4: Solution for IL imbalance issue as identified below. Reporting power threshold needs to be considered for static and dynamic reporting. The granularity of reporting could be per band (Spreadtrum). - If UE reports statically, report the actual SRS insertion loss with no UE self-compensation - If UE reports dynamically, UE report the difference value of each diversity branch output power to NW according to the SRS period (including periodic, semi-persistent and aperiodic) in real time - Proposal 5: Introduce semi-static UE assistance or capability signalling on the information on the UE SRS IL (∆TRxSRS,UE) (Intel). Adjust the existing equations on PCMAX_L,f,c so that UE applies compensation up to UE-specific SRS IL PCMAX_L,f,c = MIN {PEMAX,c– ∆TC,c, (PPowerClass – ΔPPowerClass + ΔPPowerBoost) – MAX(MAX(MPRc+∆MPRc, A-MPRc)+ ΔTIB,c + ∆TC,c + ∆TRxSRS,UE, P-MPRc) } - Proposal 6: Given that specification defines the Tx power should be equally distributed across SRS ports for each SRS transmission, per SRS resource IL reporting would be sufficient (Huawei, HiSilicon). - Proposal 7: If dynamic reporting can be considered for SRS IL reporting, network configurable threshold related to e.g. historical change of PSRS can be considered in order to give the network authority for handling SRS IL reporting frequency (Huawei, HiSilicon). - Proposal 8: To keep the current UE implementation untouched and avoid specification impact to most extent, following solution can be considered in Rel-19 (Huawei, Hi Silicon): - UE is allowed to indicate whether it enables self-compensation on the SRS IL once the network requests such information, which would benefit the network by adjusting expectation on the antenna switching SRS based PMI estimation. - Proposal 9: Dynamic reporting for actual SRS IL reporting for each SRS-TxSwitch pattern, and several thresholds associated with capability class for the actual SRS IL reporting can be considered. (ZTE, Sanechips) - Proposal 10: For Rel-19 UE, at least, optionally indicates self-compensation of SRS IL up to its maximum Tx power capabilities.

In this contribution, we would like to share our views regarding this topic.

Discussion

During last meeting, SRS IL issue has been discussed but with limited convergence. Given that this issue itself has been justified, we tend to provide our thinking with further clarifications.

- Specification examination and target scenario identification

For a start, below figure can be used for better illustration on the SRS IL issue.

Figure 1. Free space channel targeted PMI estimation error due to imbalanced power delivery at UE antenna connector

As clearly defined in TS 38.213, the actual power delivered per SRS occasion is the minimum value between PCMAX and another calculated one, and it should be clarified that the PCMAX utilized here is a configured value according to the definition in TS 38.101-1 instead of the range [PCMAX, L, PCMAX, H].

If a UE transmits SRS based on a configuration by SRS-ResourceSet on active UL BWP b of carrier f of serving cell c using SRS power control adjustment state with index l, the UE determines the SRS transmission power PSRS,b,f,c(i,qs,l) in SRS transmission occasion i as [dBm] where, - PCMAX,f,c(i) is the UE configured maximum output power defined in [8, TS 38.101-1], [8-2, TS 38.101-2] and [TS 38.101-3] for carrier f of serving cell c in SRS transmission occasion i - PO_SRS,b,f,c(qs) is provided by p0 for active UL BWP b of carrier f of serving cell c and SRS resource set qs provided by SRS-ResourceSet and SRS-ResourceSetId --- Omit the rest part for simplicity ---

Proposal 1: Clarify that for PSRS calculation as defined in TS 38.213, PCMAX is a value determined by UE according to the definition in TS 38.101-1, where ∆TRxSRS has already been counted respective to each SRS occasion, but not the range [PCMAX, L, PCMAX, H] itself.

Look closely at the PSRS definition, the network can do power control based on SRS resource set level configuration along with SRS power control adjustment state for each SRS occasion:

- Target power PO_SRS,b,f,c

- Scaling factor αSRS,b,f,c for the estimated path loss of each SRS resource set

- Power up or down controlled by hb,f,c for each SRS occasion

Moreover, PSRS equals to PCMAX as the UE determined for each SRS occasion if it is the smaller one. Thus we think SRS transmission power determination can be categorised as following cases.

| Case #1 | For all SRS resources within the set, none of PSRS equals to PCMAX | The estimated path loss could be relatively small, e.g. cell centre |
| --- | --- | --- |
| Case #2 | For all SRS resources within the set, some of PSRS equals to PCMAX | The estimated path loss could still be not so high while some of the diversity branches becomes MOP limited due to ∆TRxSRS, or P-MPR dominated scenario (not in the scope), e.g. mildly away from cell centre |
| Case #3 | For all SRS resources within the set, all of PSRS equals to PCMAX | The estimated path loss could be high and all diversity branches becomes MOP limited due to ∆TRxSRS, or P-MPR dominated scenario (not in the scope), e.g. cell middle/edge |

In our understanding, at least Case #3 is the target scenario to consider SRS IL reporting because it is not the network duty to consider compensation for the path loss.

Observation 1: SRS transmission power determination can be categorised as following cases.

| Case #1 | For all SRS resources within the set, none of PSRS equals to PCMAX | The estimated path loss could be relatively small, e.g. cell centre |
| --- | --- | --- |
| Case #2 | For all SRS resources within the set, some of PSRS equals to PCMAX | The estimated path loss could still be not so high while some of the diversity branches becomes MOP limited due to ∆TRxSRS, or P-MPR dominated scenario (not in the scope), e.g. mildly away from cell centre |
| Case #3 | For all SRS resources within the set, all of PSRS equals to PCMAX | The estimated path loss could be high and all diversity branches becomes MOP limited due to ∆TRxSRS, or P-MPR dominated scenario (not in the scope), e.g. cell middle/edge |

Proposal 2: The following scenario can be considered as the target for SRS IL reporting:

- For all SRS resources within the set, all of PSRS equals to PCMAX

  - In this scenario, the estimated path loss could be high and all diversity branches becomes MOP limited due to ∆TRxSRS, or P-MPR dominated scenario (not in the scope), e.g. cell middle/edge

- Whether or not to consider effect of loss imbalance across Rx paths

During Rel-18 discussion, concern regarding Rx IL imbalance was raised. Unlike antenna switching SRS transmission, DL reception would not require Rx switching. Consequently, IL link budget for each Rx path should be within the same level given that it can be realized by similar PCB trace pattern and RF component selection.

Observation 2: Unlike antenna switching SRS transmission, DL reception would not require Rx switching. Consequently, IL of each Rx path should be within the same level given that it can be realized by similar PCB trace pattern and RF component selection.

For better illustration, we can take one proposed architecture in [4] e.g., Figure 1 for analysis. It can be calculated that the IL imbalance among Rx branches is about 0.3~0.5dB @3.5GHz which is negligible. Unfortunately, there is no example has been provided for us to better understand if there could be any exceptional but reasonable implementation. Besides, we believe it is rational that per branch REFSENS has never been studied by RAN4.

Observation 3: RAN4 has never discussed about per branch REFSENS since no exceptional but reasonable RF implementation can be provided to prove the necessity of it.

Proposal 3: Tx/Rx-Rx imbalance should not be considered for the discussion on SRS IL reporting.

- Other aspects if SRS IL reporting can be considered

- Reporting granularity

In our understanding, the reporting granularity should be SRS resource, because RAN1 spec defines a UE behaviour as below.

- 7.3 Sounding reference signals For SRS, - if a UE is provided tdm for an SRS resource with 8 ports in an SRS resource set with usage ‘codebook’ or ‘antennaSwitching’, the UE splits a linear value PSRS,b,f,c(i,qs,l) of the transmit power PSRS,b,f,c(i,qs,l) on active UL BWP b of carrier f of serving cell c equally across the configured antenna ports on each symbol for SRS transmission. - else, a UE splits a linear value PSRS,b,f,c(i,qs,l) of the transmit power PSRS,b,f,c(i,qs,l) on active UL BWP b of carrier f of serving cell c equally across the configured antenna ports for SRS.

In this sense, the transmitted power should be divided equally between SRS ports for one SRS resource.

Proposal 4: Given that specification defines the Tx power should be equally distributed across SRS ports for each SRS transmission, per SRS resource IL reporting would be sufficient.

- Threshold for reporting frequency control

Similar as the threshold defined for legacy PHR reporting, if dynamic reporting can be considered for SRS IL reporting, then a network configurable threshold related to e.g. historical change of PSRS can be considered in order to give the network authority for handling SRS IL reporting frequency.

Proposal 5: If dynamic reporting can be considered for SRS IL reporting, network configurable threshold related to e.g. historical change of PSRS can be considered in order to give the network authority for handling SRS IL reporting frequency.

By hearing all the arguments and concerns from opponent, we can understand that companies may think the dynamic reporting will leave the UE implementation overexposed, while static reporting cannot match the real UE implementation (e.g. TAS mechanism) then fail to improve the network performance degradation. Consequently, a way that can keep the current UE implementation untouched and avoid specification impact to most extent can be pursued.

Since it is up to implementation regarding self-compensation, the very thing that the UE can indicate to the network would be whether insertion loss will be counted into the PCMAX, L calculated for any AS-SRS occasions. In that case, the network would acquire an additional information, which would benefit the network by adjusting expectation on the antenna switching SRS based PMI estimation.

Proposal 6: To keep the current UE implementation untouched and avoid specification impact to most extent, following solution can be considered in Rel-19:

- UE is allowed to indicate whether it enables self-compensation on the SRS IL once the network requests such information, which would benefit the network by adjusting expectation on the antenna switching SRS based PMI estimation.

Conclusion

In this contribution we discussed on the SRS IL reporting. We have the following observations and proposals:

Observation 1: SRS transmission power determination can be categorised as following cases.

| Case #1 | For all SRS resources within the set, none of PSRS equals to PCMAX | The estimated path loss could be relatively small, e.g. cell centre |
| --- | --- | --- |
| Case #2 | For all SRS resources within the set, some of PSRS equals to PCMAX | The estimated path loss could still be not so high while some of the diversity branches becomes MOP limited due to ∆TRxSRS, or P-MPR dominated scenario (not in the scope), e.g. mildly away from cell centre |
| Case #3 | For all SRS resources within the set, all of PSRS equals to PCMAX | The estimated path loss could be high and all diversity branches becomes MOP limited due to ∆TRxSRS, or P-MPR dominated scenario (not in the scope), e.g. cell middle/edge |

Observation 2: Unlike antenna switching SRS transmission, DL reception would not require Rx switching. Consequently, IL of each Rx path should be within the same level given that it can be realized by similar PCB trace pattern and RF component selection.

Observation 3: RAN4 has never discussed about per branch REFSENS since no exceptional but reasonable RF implementation can be provided to prove the necessity of it.

Proposal 1: Clarify that for PSRS calculation as defined in TS 38.213, PCMAX is a value determined by UE according to the definition in TS 38.101-1, where ∆TRxSRS has already been counted respective to each SRS occasion, but not the range [PCMAX, L, PCMAX, H] itself.

Proposal 2: The following scenario can be considered as the target for SRS IL reporting:

- For all SRS resources within the set, all of PSRS equals to PCMAX

  - In this scenario, the estimated path loss could be high and all diversity branches becomes MOP limited due to ∆TRxSRS, or P-MPR dominated scenario (not in the scope), e.g. cell middle/edge

Proposal 3: Tx/Rx-Rx imbalance should not be considered for the discussion on SRS IL reporting.

Proposal 4: Given that specification defines the Tx power should be equally distributed across SRS ports for each SRS transmission, per SRS resource IL reporting would be sufficient.

Proposal 5: If dynamic reporting can be considered for SRS IL reporting, network configurable threshold related to e.g. historical change of PSRS can be considered in order to give the network authority for handling SRS IL reporting frequency.

Proposal 6: To keep the current UE implementation untouched and avoid specification impact to most extent, following solution can be considered in Rel-19:

- UE is allowed to indicate whether it enables self-compensation on the SRS IL once the network requests such information, which would benefit the network by adjusting expectation on the antenna switching SRS based PMI estimation.

- References

- R4-2417209, “WF on requirements for 6Rx UE requirements”, AT&T, 3GPP TSG-RAN4 Meeting #112bis, Hefei, China, October 14-18, 2024.