# 3GPP TSG-RAN WG4 Meeting # 113	R4-2419038

Maastricht, Netherland, August 19 – 23, 2024

Source: 	Huawei, HiSilicon

Title: 	Discussion on MSD for CA_n41C-n79A with intra-band UL CA_n41C

Agenda Item:	6.3.3

Document for:	Approval

Introduction

In the RAN4#112bis meeting, RAN4 reached the way forward [1] on how to specify the MSD for CA_n41C-n79A with intra-band UL CA_n41C with some options.

Way forward on MSD for DL_n41C-n79A_UL_n41C: Proposal 4: For the MSD value DL_n41C-n79A_UL_n41C, the following inputs from companies can be further checked considering the assumption of filter performance and PA interference. - 4.1) 1RB+1RB test case 1: UL band; DL band; UL BW; SCS of UL band; UL RB Allocation; DL BW; MSD; UL/DL fc condition; UL/DL harmonic order. (MHz); (kHz); LCRB; (MHz); (dB). CA_n41-n79; n41; 2555; 80; 1 (RBSTART=0); 2555; N/A; TDD; N/A. 2635; 80; 1 (RBSTART=0); 2635; N/A; TDD; N/A. n79; N/A; 10; N/A; 4950; 8.4; TDD; IMD4x. Note x: this DL band is also subject to IMD6 that is not specified Note y: this second test point is optional. - 4.2) For fully UL RB allocation test case 2, RAN4 further discuss whether MSD due to IMD4 or >ACLR2 cross band isolation is considered for DL_n41C-n79A_UL_n41C. - Option 1: UL band; DL band; UL BW; SCS of UL band; UL RB Allocation; DL BW; MSD; UL/DL fc condition; UL/DL harmonic order. (MHz); (kHz); LCRB; (MHz); (dB). CA_n41-n79; n41; 2555; 80; 216 (RBSTART=1); 2555; N/A; TDD; N/A. 2635; 80; 216 (RBSTART=0); 2635; N/A; TDD; N/A. n79; N/A; 10; N/A; 4950; 4.6; TDD; IMD4xy. Note x: this DL band is also subject to IMD6 that is not specified Note y: this second test point is optional. - Option 2: UL band; DL band; UL Fc; UL BW; SCS of UL band; UL RB Allocation; DL Fc; DL BW; MSD; Cross-band Interference source. (MHz); (MHz); (kHz); LCRB; (MHz); (MHz); (dB). CA_n41C; n79; 2595; 190; 30; 270 (RBstart=0) + 243 ((RBstart=0); 4420; 40; 0.5; >ACLR2. - Option 3: There is no need to define additional cross band isolation MSD requirements caused by fully allocated maximum aggregated BW of UL CA_n41C falls into n79. - Other proposal and value are also welcome. NOTE: In the next meeting, RAN4 can further evaluate the MSD values based on the following channel bandwidth configurations: Band n41: one 100MHz BW CC at 2565MHz centre frequency and another one 60MHz BW CC at 2645MHz centre frequency with 30kHz SCS. Band n79: one 10 or 40MHz BW CC at 4405 or 4420MHz centre frequency. Companies may provide input for both channel bandwidths.

In the previous meeting, we provided the contribution [2] to evaluate the MSD value for CA_n41C-n79A with fully allocated RBs. In this paper, we’d like to share our views on the MSD test definition with the fully allocated maximum aggregated BW.

Discussion

From technical perspective, the MSD with fully allocated maximum aggregated BW for intra-band UL CA is similar to the traditional MSD due to the cross band isolation for single UL carrier. Thus, it’s inappropriate to introduce such MSD test configuration into the IMD MSD table.

In order to verify this important scenario, if necessary, RAN4 can specify the MSD test case for DL_ n41C-n79A_UL_n41C with fully allocated maximum aggregated BW.

Proposal 1: The following MSD test configuration due to cross band isolation for DL_n41C-n79A_UL_n41C can be introduced into the spec.

| UL band | DL band | UL Fc | UL BW | SCS of UL band | UL RB Allocation | DL Fc | DL BW | MSD | Cross-band Interference source |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (MHz) | (MHz) | (kHz) | LCRB | (MHz) | (MHz) | (dB) |  |
| CA_n41C | n40A | 2565 | 100 | 30 | 270 (RBstart=0) | 4420 | 40 | 1 | >ACLR2 |
|  |  | 2645 | 60 | 30 | 160 (RBstart=0) |  |  |  |  |

Summary

Proposal 1: The following MSD test configuration due to cross band isolation for DL_n41C-n79A_UL_n41C can be introduced into the spec.

| UL band | DL band | UL Fc | UL BW | SCS of UL band | UL RB Allocation | DL Fc | DL BW | MSD | Cross-band Interference source |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (MHz) | (MHz) | (kHz) | LCRB | (MHz) | (MHz) | (dB) |  |
| CA_n41C | n40A | 2565 | 100 | 30 | 270 (RBstart=0) | 4420 | 40 | 1 | >ACLR2 |
|  |  | 2645 | 60 | 30 | 160 (RBstart=0) |  |  |  |  |

- References

[1] R4-2417181, WF on MSD requirements with intra-band contiguous UL CA_n40A-n41C and CA_n41C-n79A, Huawei, HiSilicon

[2] R4-2416030, Discussion on MSD for CA_n41C-n79A with intra-band UL CA_n41C, Huawei, HiSilicon

- Annex

For CA_n41C-n79A with UL intra-band CA_n41C, if RAN4 choose the fully allocated maximum aggregated BW, the DL band n79 will not be affected by the 1st to 5th adjacent channel leakage of UL intra-band CA_n41C. Since the PA non-linearity noise (-130dBm/Hz) is lower enough and band n41 Tx filter performance at DL frequency range of band n79 can provide good rejection 40 dB above with antenna isolation. Thus, there is no need to specify MSD with fully allocated maximum aggregated BW for CA_n41C-n79A with UL intra-band CA_n41C.

Generally, separate antenna RF architecture is used for DL_n41C-n79A_UL_n41C as shown below.

Figure 1 separate antenna RF architecture for DL_n41C-n79A_UL_n41C

And the MSD calculation for CA_n41C-n79A cross band isolation is shown below. The PA noise PSD at PA output port is assumed as -130dBm/Hz. Band n79 filter rejection at 2515~2675MHz is assumed as 40dB. And n41 Tx filter rejection at 4400~5000MHz is assumed as 29 dB. 10dB antenna isolation is assumed. It can be found that there is no MSD for full RB allocation of UL n41C.

|  | parameters | n79 main path | n79 diversity path |
| --- | --- | --- | --- |
| transmit power for n41C, dBm |  | 23 | 23 |
| Tx band n41C BW(MHz, Lcrb) | 190 |  |  |
| RFFE loss, dB | 4 |  |  |
| Diplexer isolation at n41C uplink freq，dB | 10 |  |  |
| antenna isolation, dB | 10 |  |  |
| n79 receival singal at ANT port, dBm |  | 13 | 13 |
| n79 filter rejection at 2515~2675MHz, dB | 40 |  |  |
| signal After n79 filter, dBm |  | -31 | -31 |
| Typical receiver IIP2, dB | 50 |  |  |
| TX IM2 noise level refer to RX LNA input, dBm |  | -112 | -112 |
| front-end loss | 4 |  |  |
| TX IM2 noise level at ANT port, dBm |  | -108 | -108 |
| noise figure dB | 9 |  |  |
| Thermal noise at RX ant port(dBm/Hz) | -165 |  |  |
| Rx band n79 BW(MHz, NRB) | 38.16 |  |  |
| Thermal noise, dBm |  | -89.18 | -89.18 |
| n41 PA noise PSD at PA output port, dBm/Hz | -130 |  |  |
| n41 PA noise PSD at PA output port, dBm/RxBW |  | -54.18 |  |
| n41 Tx filter rejection at 4400~5000MHz, dB | 29 |  |  |
| n41 PA noise power at Rx n79 ant port at 4400~5000MHz, dBm |  | -93.18 | -93.18 |
| Total noise level at ANT port |  | -87.69 | -87.69 |
| SNR requirement for QPSK | -1 |  |  |
| REFSENSE (referred to antenna)(40MHz BW) |  | -88.69 | -88.69 |
| Implementation Margin, dB | 2 |  |  |
| combined REFSENS(40MHz BW), dBm | -89.70 |  |  |
| MSD | -0.05 |  |  |

If the PA noise PSD at PA output port is assumed as -125dBm/Hz due to the near-miss impacts of 2nd harmonic interference, the MSD calculation is shown below. The maximum MSD value is about 2dB.

|  | parameters | n79 main path | n79 diversity path |
| --- | --- | --- | --- |
| transmit power for n41C, dBm |  | 23 | 23 |
| Tx band n41C BW(MHz, Lcrb) | 200 |  |  |
| RFFE loss, dB | 4 |  |  |
| Diplexer isolation at n41C uplink freq，dB | 10 |  |  |
| antenna isolation, dB | 10 |  |  |
| n79 receival singal at ANT port, dBm |  | 13 | 13 |
| n79 filter rejection at 2515~2675MHz, dB | 40 |  |  |
| signal After n79 filter, dBm |  | -31 | -31 |
| Typical receiver IIP2, dB | 50 |  |  |
| TX IM2 noise level refer to RX LNA input, dBm |  | -112 | -112 |
| front-end loss | 4 |  |  |
| TX IM2 noise level at ANT port, dBm |  | -108 | -108 |
| noise figure dB | 9 |  |  |
| Thermal noise at RX ant port(dBm/Hz) | -165 |  |  |
| Rx band n79 BW(MHz, NRB) | 38.16 |  |  |
| Thermal noise, dBm |  | -89.18 | -89.18 |
| n41 PA noise PSD at PA output port, dBm/Hz | -125 |  |  |
| n41 PA noise PSD at PA output port, dBm/RxBW |  | -49.18 |  |
| n41 Tx filter rejection at 4400~5000MHz, dB | 29 |  |  |
| n41 PA noise power at Rx n79 ant port at 4400~5000MHz, dBm |  | -88.18 | -88.18 |
| Total noise level at ANT port |  | -85.62 | -85.62 |
| SNR requirement for QPSK | -1 |  |  |
| REFSENSE (referred to antenna)(40MHz BW) |  | -86.62 | -86.62 |
| Implementation Margin, dB | 2 |  |  |
| combined REFSENS(40MHz BW), dBm | -87.63 |  |  |
| MSD | 2.02 |  |  |