# 3GPP TSG-RAN WG4 Meeting # 113	R4-2419050

# Orlando, US, 18th – 22nd November, 2024

Source: 	Huawei, HiSilicon

Title: 	Discussion on UL CA_n5-n8

Agenda Item:	6.3.3

Document for:	Approval

Introduction

In last RAN4 meeting, WF [1] for 2UL n5-n8 was agreed as following,

- RAN4 agrees to specify the support of the following implementation architecture in Rel-19 for CA_n5-n8 (i.e., Option 2)

  - 2UL/2DL CA is supported using non-concurrent n5 DL and n8 UL

  - Send LS to RAN2 to complete necessary signalling designs

- Stop discussion on Option 3 in Rel-19. If option 3 is discussed in Rel-20 or later, new band numbers are considered.

This contribution provides the views on the remaining issues to define the requirements.

RF requirements for UL CA_n5A-n8A

In Rel-18, RAN4 concluded that there is no IMD issue for CA_n5-n8 with the following frequency range restriction.

n5	UL: 824 MHz – 835 MHz	DL: 869 MHz – 880 MHz;

n8	UL: 904 MHz – 915 MHz	DL: 949 MHz – 960 MHz.

In theory, IMD3 may fall into the DL band n8 if both band n5 and n8 transmit the signal without frequency restriction. Referring to the IMD3 MSD value from CA_n8-n20, 25dB MSD can be specified for UL CA_n5-n8 if necessary.

| Band / Channel bandwidth / NRB / Duplex mode |  |  |  |  |  |  |  | Source of IMD |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NR CA band combination | NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | Duplex mode |  |
| CA_n5-n8 | n5 | 837.5 | 5 | 25 | 882.5 | N/A | FDD | N/A |
|  | n8 | 882.5 | 5 | 25 | 927.5 | 25 | FDD | IMD3 |

Observation 1: In theory, IMD3 may fall into the DL band n8 if both band n5 and n8 transmit the signal without frequency restriction. Referring to the IMD3 MSD value from CA_n8-n20, 25dB MSD can be specified for UL CA_n5-n8 if necessary.

LS contents

In order to support UL CA_n5A-n8A feature, RAN4 agrees to specify the support of the following implementation architecture in Rel-19 for CA_n5-n8 (i.e., Option 2) o 2UL/2DL CA is supported using non-concurrent n5 DL and n8 UL. RAN4 would like kindly request RAN2 to design the corresponding UE capability for option 2 referring to the following characteristics: 1) It’s FDD-FDD non-simultaneous Rx/Tx capability due to the frequency overlapping between DL frequency range of band n5 and UL frequency range of band n8. 2) For a UE, the band pairs (DL n5, UL n8) and (DL n8, UL n5) have different capabilities about whether to support non-simultaneous Rx/Tx. The UE capabilities should distinguish these two cases. 3) Considering the forward compatibility, the UE capabilities can support the indication of band pair (DL n5, UL n8) in a higher order band combination, e.g. DL_n5A-n8A-n78A_UL_n5A-n8A and DL_n5A-n8A-n78A_UL_n8A-n78A. Thus, the UE capability should be per band pair per BC and conditionally mandatory capability.

Summary

Observation 1: In theory, IMD3 may fall into the DL band n8 if both band n5 and n8 transmit the signal without frequency restriction. Referring to the IMD3 MSD value from CA_n8-n20, 25dB MSD can be specified for UL CA_n5-n8 if necessary.

| Band / Channel bandwidth / NRB / Duplex mode |  |  |  |  |  |  |  | Source of IMD |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NR CA band combination | NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | Duplex mode |  |
| CA_n5-n8 | n5 | 837.5 | 5 | 25 | 882.5 | N/A | FDD | N/A |
|  | n8 | 882.5 | 5 | 25 | 927.5 | 25 | FDD | IMD3 |

Observation 2: the following LS contents can be considered.

In order to support UL CA_n5A-n8A feature, RAN4 agreed to specify the support of the following implementation architecture in Rel-19 for CA_n5-n8 (i.e., Option 2)

o	2UL/2DL CA is supported using non-concurrent n5 DL and n8 UL.

RAN4 would like kindly request RAN2 to design the corresponding UE capability for option 2 referring to the following characteristics:

1) It’s FDD-FDD non-simultaneous Rx/Tx capability due to the frequency overlapping between DL frequency range of band n5 and UL frequency range of band n8.

2) For a UE, the band pairs (DL n5, UL n8) and (DL n8, UL n5) have different capabilities about whether to support non-simultaneous Rx/Tx. The UE capabilities should distinguish these two cases.

3) Considering the forward compatibility, the UE capabilities can support the indication of band pair (DL n5, UL n8) in a higher order band combination, e.g. DL_n5A-n8A-n78A_UL_n5A-n8A and DL_n5A-n8A-n78A_UL_n8A-n78A. Thus, the UE capability should be per band pair per BC and conditionally mandatory capability.

- References

[1] R4-2417125, “WF on UL CA_n5-n8”, CATT, China Telecom, Huawei, HiSilicon, RAN4#112bis