3GPP TSG-RAN WG4 Meeting #113                                   R4-2419051

Orlando, US, 18th – 22nd November, 2024

Source: 	MediaTek Inc.

Title: 	Discussion on IMT UE parameters for 14800 to 15350 MHz

Agenda Item:	7.2.4.2

Document for:	Approval

- Introduction

In RAN#103, a new SI [1] was agreed upon to study the IMT parameters for the frequency ranges of 4400 to 4800 MHz, 7125 to 8400 MHz, and 14800 to 15350 MHz. In the RAN4#112bis meeting, a UE WF for 14800 to 15350 MHz was agreed upon in [2] to capture the agreements regarding the radio parameters of the UE, and there are still some UE parameters that need to be discussed. In this paper, we provide our views on the radio parameters for the UE for 14800 to 15350 MHz.

- Discussion

Several UE IMT parameters for the 14800 to 15350 MHz frequency range remain to be discussed. In this paper, we present our views on these remaining parameters.

- Noise figure

The following was agreed at RAN4#112bis:

- For LS to ITU, noise figure is [13] dB

- The common understanding is that the current co-existence simulation work (based on 11 dB UE NF) is not changed due to above noise figure value.

A noise figure of 12 dB was assumed during the band n104 discussions. Besides, a noise figure of 13 dB for the LS reply was agreed upon in the previous RAN4 meeting for the 7125-8400 MHz frequency range. Based on our analysis, considering the higher implementation loss for the frequency range of 14800 to 15350 MHz, the noise figure would be larger than that for the 7125-8400 MHz frequency range. As a compromise, we could consider a 13 dB NF for the LS reply and mention that the actual noise figure to be used to define RF requirements should be further studied in the WI phase in the TR.

Proposal 1: RAN4 to consider 13 dB NF for the LS reply and mention that the actual noise figure to be used to define RF requirements should be further studied in the WI phase in the TR.

- Power dynamic range

The following was agreed at RAN4#112bis:

- Proposals

  - Option 1: 56dB for 100MHz assuming 23dBm

  - Option 2: 56dB for 200MHz assuming 23dBm

The UE power dynamic range can be calculated as maximum output power minus minimum output power, where the minimum output power for 100 MHz is -33 dBm. For a PC3 UE operating at a 100 MHz channel bandwidth, the power dynamic range is 23 - (-33) = 56 dB. Therefore, we propose 56 dB for 100 MHz for PC3 UE.

Proposal 2: RAN4 to consider 56dB for 100MHz for PC3 UE.

- Emissions mask

The following was agreed at RAN4#112bis:

- Option 1: UE SEM given in Section 7.1.2, TR 38.921.

- Option 2: Other options.

The following proposes a possible option for “Option 2: Other options” based on FR2 Emissions Mask (TS 38.101-2 Table 6.5.2.1-1).

FCC imposes regulatory limits for FR1 bands emissions in 0-1MHz and 1-5MHz offset such as for PCS and AWS bands [3][4][5]. However the 15GHz band does not require these regulatory restrictions. Also, the channel bandwidths for 15GHz band are expected to be larger, so the small offsets in the range of 0-5MHz will increase the linearity requirements of the PA which will then increase the overall implementation costs. Furthermore, the ACLR for 15GHz band is expected to be relaxed, so the SEM requirements should be relaxed with ACLR. Therefore, the 15GHz band SEM spec should be the same as FR2 SEM Spec (TS 38.101-2 Table 6.5.2.1-1) with SEM spec = -5dBm/MHz up to 10% of channel BW, and -13dBm/MHz up to FOOB.

FOOB should follow the ITU-R SM.1539 Table 2 [6] (as referenced in TR 38.803) for 10-15GHz of min(250%,150%+250MHz). Note ITU-R SM.1539 FOOB is defined from channel center while TS 38.101-1 and TS 38.101-2 FOOB are defined from channel edge.

These requirements would satisfy both ITU-R and FCC. These are the same requirements for FCC emissions for UMFUS band [7] (for FR2 bands) while still meeting the ITU-R SM.329 Table 3 Category B devices (land mobile services) emissions spec [8] outside of FOOB (FOOB as defined by ITU-R SM.1539). The FCC has not yet finalized any emissions requirements for 12.7-13.25GHz emissions limits and requests for comment [9].

General Spectrum Emission Mask for FR3 15GHz band

| ΔfOOB (MHz) | Spectrum emission limit (dBm) | Measurement bandwidth | Comment |
| --- | --- | --- | --- |
| ± 0.1 BWChannel | -5 | 1 MHz |  |
| ± 0.1 BWChannel – FOOB1 | -13 |  |  |
| > ± FOOB1 | -30 |  | Spurious Emissions Region |
| NOTE 1: FOOB = min(2 BWChannel , 250MHz + BWChannel) |  |  |  |

Boundary between NR out of band and spurious emission domain for FR3 15GHz band

| Channel bandwidth | 10 MHz | 20 MHz | 50 MHz | 100 MHz | 200 MHz | 400 MHz |
| --- | --- | --- | --- | --- | --- | --- |
| OOB boundary FOOB (MHz) | 20 | 40 | 100 | 200 | 400 | 650 |
| NOTE 1: formula for FOOB = min(2 BWChannel , 250MHz + BWChannel) based on ITU-R SM.1539 Table 2 for 10-15GHz. |  |  |  |  |  |  |

Proposal 3: RAN4 to consider UE SEM given in TS 38.101-2 Table 6.5.2.1-1, with FOOB = min(2 BWChannel , 250 MHz + BWChannel) based on ITU-R SM.1539 Table 2.

- Spurious emission

The following was agreed at RAN4#112bis:

- FFS if RAN4 to adopt the UE general spurious emissions defined in TS 38.101-1 clause 6.5.3.1

The UE general spurious emissions defined in TS 38.101-1 clause 6.5.3.1 can apply to the frequency range of 14800 to 15350 MHz. This is in agreement with ITU-R SM.329 Table 3 Category B devices (land mobile services) emissions spec [8].

Proposal 4: RAN4 to consider TS 38.101-1 clause 6.5.3.1 as the UE general spurious emission.

- Blocking and spurious response

The following was agreed at RAN4#112bis:

- FFS if RAN4 to consider the blocking characteristic specified in clause 7.6 of TS 38.101-1 for frequency larger than 3300 MHz could be applied.

See Appendix 1 for text proposals for FR3 15GHz band RX blocking requirements. Use the blocking characteristics of clause 7.6, 7.7, 7.8 of TS 38.101-1 for frequency larger than 3300MHz for all other RX blocker requirements.

In-band blocking:

- Desired signal power: RESENS + 14dB (add note to TS 38.101-1 Table 7.6.2-3)

  - Explanation: More relaxation is needed for higher frequency 15GHz, and to align with ACS in-band power. 15GHz band will have higher channel bandwidths so ADC oversampling ratios will be lower. Therefore more ADC alias rejection will be needed, so a relaxed in-band blocker requirement is needed to lower implementation costs and UE power consumption.

Out-of-band blocking:

- Desired signal power: RESENS + 14dB (add note to TS 38.101-1 Table 7.6.3-3)

  - Explanation: from a feasibility perspective, relaxing out of band blocker requirement will reduce front-end filtering requirement which will allow for lower in-band loss. Lower in-band front-end filter loss will improve UE sensitivity enabling co-located FR3 15GHz base-station with FR1.

- Blocker power and frequency: same as n104 range 2 and range 3 (TS 38.101-1 Table 7.6.3-4), but for range 3 extend the max frequency from 12.75 GHz to 43.5 GHz or 2 f0.

  - Explanation: The extended frequency range is to cover FR2 frequency range as aggressor. Testability of this large frequency range should also be considered, and number of tested frequency points can be FFS for RAN5.

Proposal 5: For in-band blocking and out-of-band blocking, desired signal power = REFSENS + 14dB which is relaxed compared to n104 and same as ACS case 1.

Proposal 6: Out of band blocker power and frequency offset is same as n104 range 2 and range 3 (TS 38.101-1 Table 7.6.3-4), but for range 3 extend the max frequency from 12.75 GHz to 43.5 GHz or 2 f0.

Proposal 7: Use the blocking characteristics of clause 7.6, 7.7, 7.8 of TS 38.101-1 for frequency larger than 3300MHz for other RX blocking requirements.

- ACLR/ACS

Our simulation results [3] for the coexistence study for 14800 to 15350 MHz, submitted in this meeting, show that the maximum required DL ACIR value does not exceed 24 dB and the maximum required UL ACIR value does not exceed 17dB. Therefore, we propose an ACLR of 18 dB and an ACS of 25dBc.

Proposal 8: RAN4 to consider 18dB as the ACLR value and 25dBc as the ACS value.

- 3	Conclusion

In this contribution, we have presented our perspectives on the IMT parameters for 14800 to 15350 MHz for UEs, and have provided the following proposals:

Proposal 1: RAN4 to consider 13 dB NF for the LS reply and mention that the actual noise figure to be used to define RF requirements should be further studied in the WI phase in the TR.

Proposal 2: RAN4 to consider 56dB for 100MHz for PC3 UE.

Proposal 3: RAN4 to consider UE SEM given in TS 38.101-2 Table 6.5.2.1-1, with FOOB = min(2 BWChannel , 250 MHz + BWChannel) based on ITU-R SM.1539 Table 2.

Proposal 4: RAN4 to consider TS 38.101-1 clause 6.5.3.1 as the UE general spurious emission.

Proposal 5: For in-band blocking and out-of-band blocking, desired signal power = REFSENS + 14dB which is relaxed compared to n104 and same as ACS case 1.

Proposal 6: Out of band blocker power and frequency offset is same as n104 range 2 and range 3 (TS 38.101-1 Table 7.6.3-4), but for range 3 extend the max frequency from 12.75 GHz to 43.5 GHz or 2 f0.

Proposal 7: Use the blocking characteristics of clause 7.6, 7.7, 7.8 of TS 38.101-1 for frequency larger than 3300MHz for other RX blocking requirements.

Proposal 8: RAN4 to consider 18dB as the ACLR value and 25dBc as the ACS value.

- 4	References

- RP-240787, “Study on IMT parameters for 4400 to 4800 MHz, 7125 to 8400 MHz and 14800 to 15350 MHz”, Ericsson.

- R4-2417108, WF on 15GHz UE parameters, Qualcomm

- FCC 47 CFR Part 22 Public Mobile Service https://www.ecfr.gov/current/title-47/chapter-I/subchapter-B/part-22

- FCC 47 CFR Part 24 Personal Communication Services https://www.ecfr.gov/current/title-47/chapter-I/subchapter-B/part-24

- FCC 47 CFR Part 27 MISCELLANEOUS WIRELESS COMMUNICATIONS SERVICES https://www.ecfr.gov/current/title-47/chapter-I/subchapter-B/part-27

- ITU-R SM.1539 https://www.itu.int/rec/R-REC-SM.1539/en

- FCC 47 CFR Part 30 UPPER MICROWAVE FLEXIBLE USE SERVICE https://www.ecfr.gov/current/title-47/chapter-I/subchapter-B/part-30

- ITU-R SM.329 Unwanted emissions in the spurious domain https://www.itu.int/rec/R-REC-SM.329

- FCC: Expanding Use of the 12.7-13.25 GHz Band for Mobile Broadband or Other Expanded Use https://www.federalregister.gov/documents/2023/07/10/2023-13500/expanding-use-of-the-127-1325-ghz-band-for-mobile-broadband-or-other-expanded-use

Appendix 1: Text Proposals for TS 38.101-1 Tables with additions for FR3 15GHz

Table 7.6.2-3: In-band blocking parameters for NR bands with FDL_low ≥ 3300 MHz and FUL_low ≥ 3300 MHz

| RX parameter | Units | Channel bandwidth (MHz) |
| --- | --- | --- |
|  |  | 10, 15, 20, 25, 30, 35, 40, 45, 50, 60, 70, 80, 90, 100, 200, 400 |
| Power in transmission bandwidth configuration | dBm | REFSENS + 6 dB3,4 |
| BWinterferer | MHz | BWChannel |
| FIoffset, case 1 | MHz | (3/2)*BWChannel |
| FIoffset, case 2 | MHz | (5/2)*BWChannel |
| NOTE 1: The transmitter shall be set to 4 dB below PCMAX_L,f,c at the minimum UL configuration specified in Table 7.3.2-3 with PCMAX_L,f,c defined in clause 6.2.4. NOTE 2: The interferer consists of the RMC specified in Annexes A.3.2.2 and A.3.3.2 with one sided dynamic OCNG Pattern OP.1 FDD/TDD for the DL-signal as described in Annex A.5.1.1/A.5.2.1 NOTE 3: For Band n104, the power in transmission bandwidth configuration is REFSENS + 9 dB NOTE 4: For FR3 15GHz Band, the power in transmission bandwidth configuration is REFSENS + 14 dB |  |  |

Table 7.6.3-3: Out-of-band blocking parameters for NR bands with FDL_low ≥ 3300 MHz and FUL_low ≥ 3300 MHz

| RX parameter | Units | Channel bandwidth (MHz) |  |  |
| --- | --- | --- | --- | --- |
|  |  | 10 | 15 | 20, 25, 30, 35, 40, 45, 50, 60, 70, 80, 90, 100, 200, 400 |
| Power in transmission bandwidth configuration | dBm | REFSENS + 6 dB | REFSENS + 7 dB | REFSENS + 9 dB2 |
| NOTE 1: The transmitter shall be set to 4 dB below PCMAX_L,f,c at the minimum UL configuration specified in Table 7.3.2-3 with PCMAX_L,f,c defined in clause 6.2.4. NOTE 2: For FR3 15GHz Band, the power in transmission bandwidth configuration is REFSENS + 14 dB |  |  |  |  |

Table 7.6.3-4: Out of-band blocking for NR bands with FDL_low ≥ 3300 MHz and FUL_low ≥ 3300 MHz

| NR band | Parameter | Unit | Range1 | Range 2 | Range 3 |
| --- | --- | --- | --- | --- | --- |
| n77, n78 (NOTE 3) | Pinterferer | dBm | -44 | -30 | -15 |
|  | Finterferer (CW) | MHz | -60 < f – FDL_low ≤ -3*BWChannel or 3*BWChannel ≤ f – FDL_high < 60 | -200 < f – FDL_low ≤ -MAX(60,3*BWChannel) or MAX(60,3*BWChannel) ≤ f – FDL_high < 200 | 1 ≤ f ≤ FDL_low – MAX(200,3*BWChannel) or FDL_high + MAX(200,3*BWChannel) ≤ f ≤ 12750 |
| n79 (NOTE 4) | Finterferer (CW) | MHz | N/A | -150 < f – FDL_low ≤ -MAX(60,3*BWChannel) or MAX(60,3*BWChannel) ≤ f – FDL_high < 150 | 1 ≤ f ≤ FDL_low – MAX(150,3*BWChannel) or FDL_high + MAX(150,3*BWChannel) ≤ f ≤ 12750 |
| n104 (NOTE 5) | Finterferer (CW) | MHz | N/A | -150 < f – FDL_low ≤ -MAX(60,3*BWChannel) or MAX(60,3*BWChannel) ≤ f – FDL_high < 150 | 1 ≤ f ≤ FDL_low – MAX(150,3*BWChannel) or FDL_high + MAX(150,3*BWChannel) ≤ f ≤ 12750 |
| FR3 15GHz band (NOTE 6) | Finterferer (CW) | MHz | N/A | -150 < f – FDL_low ≤ -MAX(60,3*BWChannel) or MAX(60,3*BWChannel) ≤ f – FDL_high < 150 | 1 ≤ f ≤ FDL_low – MAX(150,3*BWChannel) or FDL_high + MAX(150,3*BWChannel) ≤ f ≤ 43500 |
| NOTE 1: The power level of the interferer (PInterferer) for Range 3 shall be modified to -20 dBm for FInterferer > 6000 MHz. NOTE 2: BWChannel denotes the channel bandwidth of the wanted signal NOTE 3: The power level of the interferer (PInterferer) for Range 3 shall be modified to -20 dBm, for FInterferer > 2700 MHz and FInterferer < 4800 MHz. For BWChannel > 15 MHz, the requirement for Range 1 is not applicable and Range 2 applies from the frequency offset of 3*BWChannel from the band edge. For BWChannel larger than 60 MHz, the requirement for Range 2 is not applicable and Range 3 applies from the frequency offset of 3*BWChannel from the band edge. NOTE 4: The power level of the interferer (PInterferer) for Range 3 shall be modified to -20 dBm, for FInterferer > 3650 MHz and FInterferer < 5750 MHz. For BWChannel ≥ 40 MHz, the requirement for Range 2 is not applicable and Range 3 applies from the frequency offset of 3*BWChannel from the band edge. NOTE 5: The power level of the interferer (PInterferer) for Range 3 shall be modified to -20 dBm, for FInterferer > 5175 MHz. For BWChannel > 60 MHz, the requirement for Range 2 is not applicable and Range 3 applies from the frequency offset of 3*BWChannel from the band edge. The power level of the interferer (PInterferer) for Range 2 shall be modified to -33 dBm for the range 5925 – MAX(60,3*CBW) ≤ f < FDL_low - MAX(60,3*CBW). NOTE 6: The power level of the interferer (PInterferer) for Range 3 shall be modified to -20 dBm, for FInterferer > 5175 MHz. For BWChannel > 60 MHz, the requirement for Range 2 is not applicable and Range 3 applies from the frequency offset of 3*BWChannel from the band edge. |  |  |  |  |  |

Appendix 2: FCC and ITU-R Reference Tables for Emission Mask Proposal

FCC 47 CFR Part 30 UMFUS:

ITU-R SM.1539:

ITU-R SM.329: