# 3GPP TSG-RAN WG4 Meeting # 113	R4-2419065

# Orlando, US, 18th – 22nd November, 2024

Source: 	Huawei, HiSilicon

Title: 	Discussion on Rel-19 ATG UE RF with DL inter-band CA_n3-n39

Agenda Item:	7.10.2.2

Document for:	Approval

Introduction

In the RAN4#110bis meeting, RAN4 started to discuss the Rel-19 ATG enhancement. One high level way forward [1] were approved. In our contribution [2], we shared some initial considerations for this topic.

In the RAN4#112 meeting, RAN4 reached a lot of consensus on this topic below, which are captured into the WF [3].

Issue 3-2-1: whether to allow n39 UL for CA_n3A-n39A Agreement: - Whether to allow n39 UL for CA_n3A-n39A depend on companies’ input. - The filter with [X] MHz frequency gap can be considered. Issue 3-2-2: BCS for DL CA_n3_n39 Agreement: - Table 1: BCS for CA_n3A-n39A NR CA configuration; Uplink CA configuration or single uplink carrier; NR Band; Channel bandwidth (MHz); Bandwidth combination set. CA_n3A-n39A; -; n3; 5, 10, 15, 20, 25, 30; 0. n39; 5, 10, 15, 20, 25, 30, 35, 40. Issue 3-4-1: delta RIB for NR CA_n3A-n39A could be used as the baseline Agreement: Table 3: ΔRIB,c Inter-band CA combination; ΔRIB,c for NR bands (dB)*. Component band in order of bands in configuration**. CA_n3-n39; -; -. NOTE *: “-” denotes ΔRIB,c = 0. NOTE **: The component band order in the configuration should be listed by the order of NR bands, such as for CA_n1-n77 the band order from left to right is n1 and n77.. Issue 3-4-2: MSD for CA_n3A-n39A Agreement: - There is no MSD issue for n3 impact n39. Issue 3-4-3: OOB exception for CA_n3A-n39A Agreement: - No need to specify OOB blocking exception for CA_n3-n39 since the second order intermodulation product of the n3 UL carrier and the CW interfering signal will not overlaps with the n39 DL carrier.

In the RAN4#112bis meeting, RAN4 reached some consensus in the approved WF [4].

Issue 2-1: whether to allow n39 UL for CA_n3A-n39A Agreement: - The conclusions about allowing n39 UL for CA_n3A-n39A if any should only apply to ATG and will not have any impact on existing NR specification for legacy NR UE. - Further check if the performance of these parameters in the following table can be achieved in ATG UE with 5MHz frequency gap in n39 filter. - Parameters; Values. Antenna Isolation; 20dB. The rejection for band n3 Rx filter at 1885~1915MHz; 50dB. n39 Tx filter rejection at 1805~1880MHz; 50dB. - Other RF architectures or parameters are not precluded. Issue 2-2: BCS for DL CA_n3_n39 WF: Further consider the following table, if the feasibility is confirmed for uplink of n39. Table 1. Supported bandwidths per CA band combination of band n3+n39 NR CA configuration; Uplink CA configuration or single uplink carrier; NR Band; Channel bandwidth (MHz); Bandwidth combination set. CA_n3A-n39A; n3, [n39]; n3; 5, 10, 15, 20, 25, 30; 0. n39; 5, 10, 15, 20, 25, 30, 35, 40. Issue 2-3: MSD for CA_n3A-n39A with n39 UL WF: - Further check the MSD value with 5MHz frequency gap in 1880~1885MHz, if the feasibility is confirmed for uplink of n39. UL band; DL band; UL Fc; UL BW; SCS of UL band; UL RB Allocation; DL Fc; DL BW; MSD; Cross-band Interference source. (MHz); (MHz); (kHz); LCRB; (MHz); (MHz); (dB). n39; n3; 1900; 30; 15; 160 (RBstart=0); 1827.5; 5; [TBD]; ACLR2. - Other test points are not precluded.

In this paper, we’d like to our views on how to make progress for DL CA_n3-n39 of ATG UE.

Discussion

We’d like to provide two exemplary RF architectures below to allow the transition guard band between DL band n3 and band n39.

figure 1 Common antenna RF architecture for DL CA_n3-n39 with UL in either band n3 or n39

Figure 2 separate antenna RF architecture for DL CA_n3-n39 with UL in either band n3 or n39

For the RF architectures in figures, only partial frequency range can be implemented for the filter(s) in DL band n3 or band n39 or both DL band n3 and n39.

Based on the way forward [3], companies are encouraged to provide the inputs for the frequency gap in the filters.

In China, the spectrum allocation in band n3 and n39 is shown below.

table 1 Band 3 UL spectrum allocation in China

| Band 3 UL |  |  |
| --- | --- | --- |
| CMCC | CU | CTC |
| 1710~1735MHz | 1735~1765MHz | 1765~1785MHz |

table 2 Band 3 DL and band 39 UL/DL spectrum allocation in China

| Band 3 DL |  |  | Band 39 UL/DL |  |
| --- | --- | --- | --- | --- |
| CMCC | CU | CTC | No allocation | CMCC |
| 1805~1830MHz | 1830~1860MHz | 1860~1880MHz | 1880~1885MHz | 1885~1915MHz |

Observation 1: 5MHz frequency gap was reserved for the band 3 and 39 spectrum allocation in China.

Observation 2: From ATG operator’s perspective, the frequency separation between band 3 DL and band 39 UL is about 55MHz.

Thus, we checked the MSD based on 30MHz aggressor located in 1885~1915MHz and 5MHz victim located in 1825~1830MHz, which is the MSD case due to ACLR2 cross band isolation. The evaluation is shown in the table 3. The antenna isolation is assumed as 20dB. The rejection for band n3 Rx filter at 1885~1915MHz and n39 Tx filter rejection at 1805~1880MHz are assumed as 50dB. Since there is no volumn limitation and power consumption limitation in the airplane, the assumed performance for these parameters are better than the handheld UE. In addition, based on CMCC spectrum allocation, the victim 5MHz in 1825~1830MHz falls into the second adjacent channel of band n39 30MHz.

table 3 The MSD evaluation for band n39 UL => band n3 DL for common antenna architecture

|  | parameters | n3 main path | n3 diversity path |
| --- | --- | --- | --- |
| transmit power for n39, dBm |  | 23 | 23 |
| Tx band n39 BW(MHz, Lcrb@15kHz) | 30 |  |  |
| RFFE loss, dB | 4 |  |  |
| Diplexer isolation at n3 uplink freq，dB | 0 |  |  |
| antenna isolation, dB | 20 |  |  |
| n3 receival singal at ANT port, dBm |  | 23 | 3 |
| n3 filter rejection at 1885~1915MHz, dB | 50 |  |  |
| signal After n3 filter, dBm |  | -31 | -51 |
| Typical receiver IIP2, dB | 50 |  |  |
| TX IM2 noise level refer to RX LNA input, dBm |  | -112 | -152 |
| front-end loss | 4 |  |  |
| TX IM2 noise level at ANT port, dBm |  | -108 | -148 |
| noise figure dB | 12 |  |  |
| Thermal noise at RX ant port(dBm/Hz) | -162 |  |  |
| Rx band n3 BW(MHz, NRB) | 4.5 |  |  |
| Thermal noise, dBm |  | -95.47 | -95.47 |
| ACLR1, dB | 30 |  |  |
| ACLR2, dB | 43 |  |  |
| n39 PA leakage PSD at PA output port at 2nd adjacent channel, dBm/MHz |  | -34.77 |  |
| n39 PA noise PSD at PA output port, dBm/RxBW |  | -28.24 |  |
| n39 Tx filter rejection at 1805~1880MHz, dB | 50 |  |  |
| n39 PA noise power at Rx n3 ant port at 1825~1830MHz, dBm |  | -78.24 | -98.24 |
| Total noise level at ANT port |  | -78.15 | -93.63 |
| SNR requirement for QPSK | -1 |  |  |
| REFSENSE (referred to antenna)(20MHz BW) |  | -79.15 | -94.63 |
| Implementation Margin, dB | 2.5 |  |  |
| combined REFSENS(5MHz BW), dBm | -92.25 |  |  |
| MSD | 4.75 |  |  |

table 4 The MSD evaluation for band n39 UL => band n3 DL for separate antenna architecture

|  | parameters | n3 main path | n3 diversity path |
| --- | --- | --- | --- |
| transmit power for n39, dBm |  | 23 | 23 |
| Tx band n39 BW(MHz, Lcrb@15kHz) | 30 |  |  |
| RFFE loss, dB | 4 |  |  |
| Diplexer isolation at n3 uplink freq，dB | 0 |  |  |
| antenna isolation, dB | 20 |  |  |
| n3 receival singal at ANT port, dBm |  | 23 | 3 |
| n3 filter rejection at 1885~1915MHz, dB | 50 |  |  |
| signal After n3 filter, dBm |  | -31 | -51 |
| Typical receiver IIP2, dB | 50 |  |  |
| TX IM2 noise level refer to RX LNA input, dBm |  | -112 | -152 |
| front-end loss | 4 |  |  |
| TX IM2 noise level at ANT port, dBm |  | -108 | -148 |
| noise figure dB | 12 |  |  |
| Thermal noise at RX ant port(dBm/Hz) | -162 |  |  |
| Rx band n3 BW(MHz, NRB) | 4.5 |  |  |
| Thermal noise, dBm |  | -95.47 | -95.47 |
| ACLR1, dB | 30 |  |  |
| ACLR2, dB | 43 |  |  |
| n39 PA leakage PSD at PA output port at 1st adjacent channel, dBm/MHz |  | -21.77 |  |
| n39 PA leakage PSD at PA output port at 2nd adjacent channel, dBm/MHz |  | -34.77 |  |
| n39 PA noise PSD at PA output port, dBm/Hz | -130 |  |  |
| n39 PA noise PSD at PA output port, dBm/RxBW |  | -28.24 |  |
| n39 Tx filter rejection at 1805~1880MHz, dB | 50 |  |  |
| n39 PA noise power at Rx n3 ant port at 1825~1830MHz, dBm |  | -98.24 | -98.24 |
| Total noise level at ANT port |  | -93.47 | -93.63 |
| SNR requirement for QPSK | -1 |  |  |
| REFSENSE (referred to antenna)(20MHz BW) |  | -94.47 | -94.63 |
| Implementation Margin, dB | 2.5 |  |  |
| combined REFSENS(5MHz BW), dBm | -95.06 |  |  |
| MSD | 1.94 |  |  |

Observation 3: If the following performance of these parameters can be achieved in ATG UE, the MSD for band n3 DL located in 1825~1830MHz is less than 1dB.

| Parameters | Values |
| --- | --- |
| Antenna Isolation | 20dB |
| The rejection for band n3 Rx filter at 1885~1915MHz | 50dB |
| n39 Tx filter rejection at 1805~1880MHz | 50dB |

Generally, 10dB antenna isolation is assumed for handheld UE and the 30dB coupling loss for co-location studies is assumed for BS. Thus, it’s reasonable for ATG UE to assume 20dB antenna isolation in the airplane with large space.

For handheld UE, 50~55dB filter performance can be assumed in 3GPP. For BS co-location coexistence between band n3 DL and n39 UL, 5MHz frequency gap is enough to guarantee the coexistence requirements. However, ATG UE has better capabilities and performance than handheld UE, so we believe it’s feasible and achievable to assume 50dB filter rejection for ATG UE. For ATG UE, the traditional SAW may not be used in order to achieve such high filter performance.

In addition, ATG UE is allowed to indicate the maximum output power with the range 23dBm to 40dBm. However, we only assumed ATG UE transmit 23dBm when testing this kind of MSD. This case can cover all different ATG UE capabilities of maximum output power. If we assume some value higher than 23dBm, the MSD test case can’t be verified for some ATG UE with lower output power.

Observation 4: if 23dBm ATG UE output power can be assumed, the test case can cover all different ATG UE capabilities of maximum output power.

Proposal 1: To allow n39 UL for CA_n3A-n39A with 5MHz frequency gap in 1880~1885MHz of band n39 filter.

Proposal 2: For ATG UE, RAN4 can specify the following MSD requirements for DL band n3 with band n39 UL interreference under 23dBm output power considering operator’s spectrum allocation and 5MHz frequency gap in 1880~1885MHz.

| UL band | DL band | UL Fc | UL BW | SCS of UL band | UL RB Allocation | DL Fc | DL BW | MSD | Cross-band Interference source |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (MHz) | (MHz) | (kHz) | LCRB | (MHz) | (MHz) | (dB) |  |
| n39 | n3 | 1900 | 30 | 15 | 160 (RBstart=0) | 1827.5 | 5 | 5 | ACLR2 |

Summary

Observation 1: 5MHz frequency gap was reserved for the band 3 and 39 spectrum allocation in China.

Observation 2: From ATG operator’s perspective, the frequency separation between band 3 DL and band 39 UL is about 55MHz.

Observation 3: If the following performance of these parameters can be achieved in ATG UE, the MSD for band n3 DL located in 1825~1830MHz is less than 1dB.

| Parameters | Values |
| --- | --- |
| Antenna Isolation | 20dB |
| The rejection for band n3 Rx filter at 1885~1915MHz | 50dB |
| n39 Tx filter rejection at 1805~1880MHz | 50dB |

Observation 4: if 23dBm ATG UE output power can be assumed, the test case can cover all different ATG UE capabilities of maximum output power.

Proposal 1: To allow n39 UL for CA_n3A-n39A with 5MHz frequency gap in 1880~1885MHz of band n39 filter.

Proposal 2: For ATG UE, RAN4 can specify the following MSD requirements for DL band n3 with band n39 UL interreference under 23dBm output power considering operator’s spectrum allocation and 5MHz frequency gap in 1880~1885MHz.

| UL band | DL band | UL Fc | UL BW | SCS of UL band | UL RB Allocation | DL Fc | DL BW | MSD | Cross-band Interference source |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (MHz) | (MHz) | (kHz) | LCRB | (MHz) | (MHz) | (dB) |  |
| n39 | n3 | 1900 | 30 | 15 | 160 (RBstart=0) | 1827.5 | 5 | 5 | ACLR2 |

- References

[1] R4-2406594	WF on Rel-19 ATG UE requirements, CMCC

[2] R4-2405344	General discussion on Rel-19 ATG UE RF, Huawei, HiSilicon

[3] R4-2414322 WF on Rel-19 ATG UE RF requirements, CMCC

[4] R4-2417121	WF on UE RF for ATG enhancement, CMCC