# 3GPP TSG-RAN WG4 Meeting # 113	R4-2419072

# Orlando, US, 18th – 22nd November, 2024

Source: 	Huawei, HiSilicon

Title: 	Discussion on Tx requirements for NR NTN HPUE

Agenda Item:	7.8.2.2.1

Document for:	Approval

Introduction

In the RAN4#112 meeting, the WF [1] on UE RF requirements for NTN HPUE was approved with the following agreements.

Issue 3-1-1: General considerations - Agreement - The study of NTN HPUE Tx requirements generally would have to consider co-existence results, UE implementation feasibility, and existing HPUE requirements. Issue 3-1-5: Consideration of HPUE Architecture - Agreement: - For PC2 (NR NTN), 1 Tx and 2 Tx, are to be considered for requirements. - For PC2 IoT NTN, at least 1Tx shall be considered - For other PCs (both IoT NTN and NR NTN), FFS Issue 3-2-1: NR-NTN HPUE feasibility for different power classes Issue 3-2-2: NB-IoT based IoT-NTN HPUE feasibility for different power classes Issue 3-2-3: eMTC based IoT-NTN HPUE feasibility for different power classes - Agreement of Issue 3-2-1 to Issue 3-2-3: - For PC2, it is feasible to support both handheld and non-handheld devices. - FFS other power classes for applicable device type. Issue 3-3-1: Maximum output power - Agreement - Re-use NR TN TS 38.101-1 nominal MOP and +2/-3 tolerance can be considered as starting point for PC2/1.5/1 for NTN HPUE including NR-NTN and IoT-NTN. Issue 3-3-2: MPR - Agreement: - FFS the MPR requirements. Following potential factors to be further discussed: - ACLR from co-existence study - Current TN MPR requirements - MPR simulation assumptions - Other factors not precluded if identified. Issue 3-3-3: A-MPR - Agreement - FFS A-MPR requirements. Following potential factors to be further discussed: - Simulation reduction by not considering large RB allocations, CP-OFDM, higher-order modulations - A-MPR simulation assumption including ACLR from co-existence study - Applicable bands - Regional requirements for A-MPR - Other factors Issue 3-3-4: SAR for handheld Agreement: - Using P-MPR as the starting point - The other solutions are not precluded - E.g., the solution similar to TN duty cycle based on UE capability and network scheduling Issue 3-3-5: SEM - Agreement: - For NR-NTN: further discuss whether to reuse existing SEM requirements from PC3 to other PCs - For IoT-NTN: further discuss whether to reuse existing SEM requirements from PC3 to other PCs Issue 3-3-6: ACLR - Agreement: - The ACLR requirements will depend on co-existence study outcomes.

In the RAN4#112bis meeting, we have the following way forward.

Issue 2-2: UE architecture/type considerations for NR-NTN PC2 UE Agreement: - Follow previous agreement Issue 3-1-5: Consideration of HPUE Architecture - Agreement: - For PC2 (NR NTN), 1 Tx and 2 Tx, are to be considered for requirements. - .... Issue 2-3: UE architecture/type considerations for NR-NTN PC1.5 UE Agreement: - Both 1Tx and 2Tx for PC1.5 non-handheld UE can be considered as starting point for requirements discussions: - FFS whether PC1.5 non-handheld UE need to be specified if PC1 non-handheld UE be specified - FFS whether PC1.5 handheld UE can be specified or not Issue 2-4: UE architecture/type considerations for NR-NTN PC1 UE Agreement: - Confirm feasibility of PC1 1Tx for non-handheld UE. - For PC1, at least 1Tx to be considered for requirements. - Further discuss consideration of PC1 1Tx, 2Tx, 3Tx for handheld UE. - Further discuss consideration of PC1 2Tx, 3Tx for non-handheld UE. Issue 2-5: ACLR Agreement: - Co-existence study results and conclusion is an important factor for deriving out-band emission requirements - Check the derived ACLR for A-MPR and additional spurious emission against regulatory information. Issue 2-8: SEM Agreement: - To re-use existing SEM requirements from PC3 to other PCs as starting point. Issue 2-9: MOP tolerance Agreement: - Re-use NR TN TS 38.101-1 nominal MOP and +2/[-3] tolerance for PC2 for NR-NTN Non-Redcap HPUE

In this paper, we’d like to share our views on parts of Tx RF requirements, feasibility and RF architecture.

Feasibility and RF architecture for different power classes

In last meeting, RAN4 reached the consensus that both handheld and non-handheld devices can support PC2. However, there is no consensus on whether the other power classes can be applicable for handheld device. We’d like to share the following aspects to discuss the feasibility for different UE power classes.

Demands: Based on the rough evaluation, there are around 10~15dB gaps for handheld UE coupling loss between LEO scenario and GEO scenario. Due to large pathloss, it’s very hard to apply wideband communication technology for GEO scenario. Thus, GEO scenario, is not friendly to wideband communication technology, i.e. NR NTN. It’s not technically reasonable to force handheld UE to support other higher power classes for NR NTN as PC2 can meet the demands of LEO scenario.

UE implementation/cost: currently, PC3 is widely supported for handheld UE in TN FDD bands. Although PC2 for TN FDD bands was introduced in Rel-17 with both 1Tx and 2Tx RF architecture, this PC2 feature in FDD bands is not widely supported or implemented in handheld UE due to the difficulties of some RF components, e.g. FDD band duplexer and so on. If we assume handheld UE to support PC1.5/PC1 by using 1 Tx chain, all of RF components have to be redesign in order to tolerate the higher output power, which are not realistic. And this RF architecture is not assumed for TN PC1.5 TDD bands. If we want to reuse the current industry ecosystem as much as possible, the only way to implement PC1.5 is to use 2Tx architecture with two PC2 PAs or 4Tx architecture with four PC3 PAs. However, since the handheld UE has the limited form factor and can’t tolerate the large power consumption, currently it’s infeasible to implement PC1.5/PC1 for FDD bands in handheld UE.

Standard perspective: In TN UE specification, only TDD bands can support PC1.5 with two Tx RF architecture assumption as it is very mature that current industry can support 26dBm with one Tx chain in TDD bands. However, in FDD bands, the situation seems different that current industry can’t widely support 26dBm with one Tx chain in FDD bands. Thus, current TN UE specification doesn’t support PC1.5 in FDD bands. Generally, NTN UE need to reuse the ecosystem of TN RF components. From standard perspective, it is premature to specify PC1.5/PC1 for FDD bands in handheld UE.

Proposal 1: For NR NTN, only non-handheld UE can support PC1 in Rel-19. It’s unnecessary to further discuss the handheld UE supports for PC1.

Proposal 2: For NR NTN, it’s proposed not to support / specify PC1.5 for handheld UE in Rel-19.

As RAN4 has agreed to consider the 1Tx PC1 RF requirements for non-handheld UE, logically it’s unnecessary to specify a lower power class for non-handheld UE to fragment the non-handheld UE market.

Proposal 3: As RAN4 has agreed to consider the 1Tx PC1 RF requirements for non-handheld UE, it’s unnecessary to specify a lower power class for non-handheld UE to fragment the non-handheld UE market.

In the last meeting, RAN4 reached the way forward to further discuss the 2Tx and 3Tx architecture for PC1 non-handheld UE.

Issue 2-4: UE architecture/type considerations for NR-NTN PC1 UE Agreement: - Confirm feasibility of PC1 1Tx for non-handheld UE. - For PC1, at least 1Tx to be considered for requirements. - Further discuss consideration of PC1 1Tx, 2Tx, 3Tx for handheld UE. - Further discuss consideration of PC1 2Tx, 3Tx for non-handheld UE.

We think it’s also feasible to achieve 4Tx for PC1 non-handheld UE. PC1 non-handheld UE can utilize four 26dBm PAs to implement such architecture to reduce the costs comparing to the single 31dBm PA.

Proposal 4: 4Tx RF architecture for PC1 non-handheld UE can be specified.

Tx requirements

1) Antenna gain assumption for handheld UE

In the replied LS [2], RAN4 thinks that a realistic UE antenna gain value would be -5.5 dBi. Thus, 26dBm NR NTN UE is only applicable to the LEO scenario. In the ETSI EN 301 681, it’s only applicable to the Geostationary satellite scenario.

Observation 1: for some regulatory discussion (e.g. ETSI EN 301 681), it’s very hard to consider 3GPP handheld UE with antenna gain larger than -5.5dBi, even if 15dBw maximum EIRP is allowed.

2) SAR

Issue 3-3-4: SAR for handheld Agreement: - Using P-MPR as the starting point - The other solutions are not precluded - E.g., the solution similar to TN duty cycle based on UE capability and network scheduling

Issue 1-3-1: Duty cycle based approach Agreement: Continue the effort to study other potential solutions in addition to P-MRP solution Issue 1-3-2: Network related o FFS

In last two meetings, RAN4 has agreed to use P-MPR as starting point, but other solutions similar to TN duty cycle are not precluded. After further investigation, the following options can be considered to move forward.

Option 1: P-MPR solution is introduced as baseline.

Option 2: leave UE implementation to achieve the goal without network scheduling restriction.

Option 3: To restrict the network scheduling to achieve the goal.

Note: the goal is to guarantee the SAR compliance without NTN UE output power reduction

From system performance perspective, option 2 seems better without network scheduling restriction.

Proposal 5: RAN4 can discuss whether to leave UE implementation to achieve the SAR compliance without NTN UE output power reduction and network scheduling restriction.

Summary

Proposal 1: For NR NTN, only non-handheld UE can support PC1 in Rel-19. It’s unnecessary to further discuss the handheld UE supports for PC1.

Proposal 2: For NR NTN, it’s proposed not to support / specify PC1.5 for handheld UE in Rel-19.

Proposal 3: As RAN4 has agreed to consider the 1Tx PC1 RF requirements for non-handheld UE, it’s unnecessary to specify a lower power class for non-handheld UE to fragment the non-handheld UE market.

Proposal 4: 4Tx RF architecture for PC1 non-handheld UE can be specified.

Proposal 5: RAN4 can discuss whether to leave UE implementation to achieve the SAR compliance without NTN UE output power reduction and network scheduling restriction.

Observation 1: for some regulatory discussion (e.g. ETSI EN 301 681), it’s very hard to consider 3GPP handheld UE with antenna gain larger than -5.5dBi, even if 15dBw maximum EIRP is allowed.

- References

[1] R4-2414452, WF on UE RF requirements for NTN HPUE, Samsung

[2] R4-2214968, LS Reply on UE antenna gain for NR NTN coverage enhancement, RAN WG4