3GPP TSG-RAN WG4#113	R4-2419076

Orlando, USA, November 18th – 22nd, 2024

Agenda item:	6.3.3

Source: 	Qualcomm France

Title: 	Considerations on CA_n40A-n41C and CA_n41C_n79A

Document for:	Approval

# Introduction

Considerations on CA_n40A-n41C and CA_n41C-n79A are provided in this contribution.

- Background

WF was agreed in RAN4#112-bis [1]:

# Way forward on intra-band ULCA within an inter-band DL CA

Proposal 1: In Rel-19, to keep as-is in spec as most of cases are covered and some history is built since Rel-17.

Proposal 2: For the specific cases DL_n40A-n41C_UL_n41C and DL_n41C-n79A_UL_n41C, the following test configurations for MSD requirements can be introduced in next meeting.

- Test case 1: Discrete UL RB allocation, e.g. 1RB + 1RB UL RB allocation in UL intra-band CA.

- Test case 2: fully UL RB allocation with maximum aggregated BW.

NOTE: In the next meeting, RAN4 need to discuss how to capture these two test cases into the spec considering some consistencies.

# Way forward on MSD for DL_n40A-n41C_UL_n41C

Proposal 3: For the MSD value DL_n40A-n41C_UL_n41C, the following inputs from companies can be further checked considering the assumption of filter performance and PA interference.

- 3.1) 1RB+1RB test case 1:

| UL band | DL band | UL BW | SCS of UL band | UL RB Allocation | DL BW | MSD | UL/DL fc condition | UL/DL harmonic order |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (MHz) | (kHz) | LCRB | (MHz) | (dB) |  |  |
| CA_n40-n41 | n40 | N/A | 5 | N/A | 2397.5 | 42.5 | TDD | IMD3x |
|  | n41 | 2555 | 80 | 1 (RBSTART=0) | 2555 | N/A | TDD | N/A |
|  |  | 2635 | 80 | 1 (RBSTART=107) | 2635 | N/A | TDD | N/A |
| Note x: this DL band is also subject to IMD5 that is not specified Note y: this second test point is optional |  |  |  |  |  |  |  |  |

- 3.2) For fully UL RB allocation test case 2, RAN4 further discuss whether MSD due to IMD3 or ACLR1 cross band isolation is considered for DL_n40A-n41C_UL_n41C.

  - Option 1:

| UL band | DL band | UL BW | SCS of UL band | UL RB Allocation | DL BW | MSD | UL/DL fc condition | UL/DL harmonic order |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (MHz) | (kHz) | LCRB | (MHz) | (dB) |  |  |
| CA_n40-n41 | n40 | N/A | 5 | N/A | 2397.5 | 34.9 | TDD | IMD3xy |
|  | n41 | 2555 | 80 | 216 (RBSTART=1) | 2555 | N/A | TDD | N/A |
|  |  | 2635 | 80 | 216 (RBSTART=0) | 2635 | N/A | TDD | N/A |
| Note x: this DL band is also subject to IMD5 that is not specified Note y: this second test point is optional |  |  |  |  |  |  |  |  |

  - Option 2:

| UL band | DL band | UL Fc | UL BW | SCS of UL band | UL RB Allocation | DL Fc | DL BW | MSD | Cross-band Interference source |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (MHz) | (MHz) | (kHz) | LCRB | (MHz) | (MHz) | (dB) |  |
| CA_n41C | n40 | 2595 | 190 | 30 | 270 (RBstart=0) + 243 ((RBstart=0) | 2397.5 | 5 | 34 | ACLR1 |

  - Option 3: The cross band isolation MSD caused by fully allocated maximum aggregated BW of UL CA_n41C falls into n40 is 31.5dB and 21.7dB for CA ACLR1 and CA ACLR2, respectively.

  - Other proposal and value are also welcome.

NOTE: In the next meeting, RAN4 can further evaluate the MSD values based on the following channel bandwidth configurations:

Band n41: one 100MHz BW CC at 2565MHz centre frequency and another one 60MHz BW CC at 2645MHz centre frequency with 30kHz SCS.

Band n40: one 5MHz BW CC at 2397.5MHz centre frequency.

# Way forward on MSD for DL_n41C-n79A_UL_n41C

Proposal 4: For the MSD value DL_n41C-n79A_UL_n41C, the following inputs from companies can be further checked considering the assumption of filter performance and PA interference.

- 4.1) 1RB+1RB test case 1:

| UL band | DL band | UL BW | SCS of UL band | UL RB Allocation | DL BW | MSD | UL/DL fc condition | UL/DL harmonic order |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (MHz) | (kHz) | LCRB | (MHz) | (dB) |  |  |
| CA_n41-n79 | n41 | 2555 | 80 | 1 (RBSTART=0) | 2555 | N/A | TDD | N/A |
|  |  | 2635 | 80 | 1 (RBSTART=0) | 2635 | N/A | TDD | N/A |
|  | n79 | N/A | 10 | N/A | 4950 | 8.4 | TDD | IMD4x |
| Note x: this DL band is also subject to IMD6 that is not specified Note y: this second test point is optional |  |  |  |  |  |  |  |  |

- 4.2) For fully UL RB allocation test case 2, RAN4 further discuss whether MSD due to IMD4 or >ACLR2 cross band isolation is considered for DL_n41C-n79A_UL_n41C.

  - Option 1:

| UL band | DL band | UL BW | SCS of UL band | UL RB Allocation | DL BW | MSD | UL/DL fc condition | UL/DL harmonic order |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (MHz) | (kHz) | LCRB | (MHz) | (dB) |  |  |
| CA_n41-n79 | n41 | 2555 | 80 | 216 (RBSTART=1) | 2555 | N/A | TDD | N/A |
|  |  | 2635 | 80 | 216 (RBSTART=0) | 2635 | N/A | TDD | N/A |
|  | n79 | N/A | 10 | N/A | 4950 | 4.6 | TDD | IMD4xy |
| Note x: this DL band is also subject to IMD6 that is not specified Note y: this second test point is optional |  |  |  |  |  |  |  |  |

  - Option 2:

| UL band | DL band | UL Fc | UL BW | SCS of UL band | UL RB Allocation | DL Fc | DL BW | MSD | Cross-band Interference source |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (MHz) | (MHz) | (kHz) | LCRB | (MHz) | (MHz) | (dB) |  |
| CA_n41C | n79 | 2595 | 190 | 30 | 270 (RBstart=0) + 243 ((RBstart=0) | 4420 | 40 | 0.5 | >ACLR2 |

  - Option 3: There is no need to define additional cross band isolation MSD requirements caused by fully allocated maximum aggregated BW of UL CA_n41C falls into n79.

  - Other proposal and value are also welcome.

NOTE: In the next meeting, RAN4 can further evaluate the MSD values based on the following channel bandwidth configurations:

Band n41: one 100MHz BW CC at 2565MHz centre frequency and another one 60MHz BW CC at 2645MHz centre frequency with 30kHz SCS.

Band n79: one 10 or 40MHz BW CC at 4405 or 4420MHz centre frequency. Companies may provide input for both channel bandwidths

- Discussion

To finalize the long-discussed specifications for both CA_n40A-n41C and CA_n41C-n79A we did further measurements and analysis. Please note that we did measure different cases with expected operator spectrum holdings; we are fine to modify the analysis during the meeting as long as any operator specific test point, let it be full RB allocations with max DL BW or anything else is specified in addition to 1+1 RB case.

First case is 1+1 RB, the with assumption that MPR is applied just to meet -13dBm/MHz at antenna.

| Band / Channel bandwidth / NRB / Duplex mode |  |  |  |  |  |  |  | Source of IMD |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NR CA band combination | NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | Duplex mode |  |
| CA_n40-n41 | n40 | N/A | 5 | N/A | 2358 | 41.3 | TDD | IMD3 |
|  | n41 | 2565 | 100 | 1 (RBSTART= 0) | 2565 | N/A | TDD | N/A |
|  |  | 2644.8 | 60 | 1 (RBSTART= 161) | 2644.8 |  |  |  |

We note that the test case above is fitted into operator spectrum. The MSD number would be the same if tested with 80+80MHz case with victim DL centered around the IMD3 peak. The number is well aligned with the example number in WF.

Proposal 1: Use the following evaluation when deciding 1+1 RB test point.

| Band / Channel bandwidth / NRB / Duplex mode |  |  |  |  |  |  |  | Source of IMD |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NR CA band combination | NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | Duplex mode |  |
| CA_n40-n41 | n40 | N/A | 5 | N/A | 2358 | 41.3 | TDD | IMD3 |
|  | n41 | 2565 | 100 | 1 (RBSTART= 0) | 2565 | N/A | TDD | N/A |
|  |  | 2644.8 | 60 | 1 (RBSTART= 161) | 2644.8 |  |  |  |

In addition, we did evaluate another DL case; 50MHz n40 DL at 2320…2370MHz

| Band / Channel bandwidth / NRB / Duplex mode |  |  |  |  |  |  |  | Source of Interference |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NR CA band combination | NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | Duplex mode |  |
| CA_n40-n41 | n40 | N/A | 50 | N/A | 2345 | 28.8 | TDD | ACLR1 |
|  | n41 | 2565 | 100 | 270 (RBSTART= 0) | 2565 | N/A | TDD | N/A |
|  |  | 2644.8 | 60 | 162 (RBSTART= 0) | 2644.8 |  |  |  |

The above is fitted, in our understanding, into operator spectrum holding. We would like to discuss if our understanding is different than proponent operator, because before the meeting it was proposed to use 100MHz n40 as victim.

Proposal 2: Use the following evaluation when deciding additional full RB allocation test point.

| Band / Channel bandwidth / NRB / Duplex mode |  |  |  |  |  |  |  | Source of Interference |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NR CA band combination | NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | Duplex mode |  |
| CA_n40-n41 | n40 | N/A | 50 | N/A | 2345 | 28.8 | TDD | ACLR1 |
|  | n41 | 2565 | 100 | 270 (RBSTART= 0) | 2565 | N/A | TDD | N/A |
|  |  | 2644.8 | 60 | 162 (RBSTART= 0) | 2644.8 |  |  |  |

For CA_n41C-n79A we did only 1+1 RB case, assuming MPR needed to meet -13dBm/MHz for IMD3.

| Band / Channel bandwidth / NRB / Duplex mode |  |  |  |  |  |  |  | Source of IMD |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NR CA band combination | NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | Duplex mode |  |
| CA_n41-n79 | n79 | N/A | 10 | N/A | 4874 | 12.4 | TDD | IMD4 |
|  | n41 | 2565 | 100 | 1 (RBSTART= 0) | 2565 | N/A | TDD | N/A |
|  |  | 2644.8 | 60 | 1 (RBSTART= 161) | 2644.8 |  |  |  |

Proposal 3: Use the following evaluation when deciding 1+1 RB test point

| Band / Channel bandwidth / NRB / Duplex mode |  |  |  |  |  |  |  | Source of IMD |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NR CA band combination | NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | Duplex mode |  |
| CA_n41-n79 | n79 | N/A | 10 | N/A | 4874 | 12.4 | TDD | IMD4 |
|  | n41 | 2565 | 100 | 1 (RBSTART= 0) | 2565 | N/A | TDD | N/A |
|  |  | 2644.8 | 60 | 1 (RBSTART= 161) | 2644.8 |  |  |  |

- Conclusion

Considerations on CA_n40A-n41C and CA_n41C-n79A were provided with the following proposals.

Proposal 1: Use the following evaluation when deciding 1+1 RB test point.

| Band / Channel bandwidth / NRB / Duplex mode |  |  |  |  |  |  |  | Source of IMD |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NR CA band combination | NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | Duplex mode |  |
| CA_n40-n41 | n40 | N/A | 5 | N/A | 2358 | 41.3 | TDD | IMD3 |
|  | n41 | 2565 | 100 | 1 (RBSTART= 0) | 2565 | N/A | TDD | N/A |
|  |  | 2644.8 | 60 | 1 (RBSTART= 161) | 2644.8 |  |  |  |

Proposal 2: Use the following evaluation when deciding additional full RB allocation test point.

| Band / Channel bandwidth / NRB / Duplex mode |  |  |  |  |  |  |  | Source of Interference |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NR CA band combination | NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | Duplex mode |  |
| CA_n40-n41 | n40 | N/A | 50 | N/A | 2345 | 28.8 | TDD | ACLR1 |
|  | n41 | 2565 | 100 | 270 (RBSTART= 0) | 2565 | N/A | TDD | N/A |
|  |  | 2644.8 | 60 | 162 (RBSTART= 0) | 2644.8 |  |  |  |

Proposal 3: Use the following evaluation when deciding 1+1 RB test point.

| Band / Channel bandwidth / NRB / Duplex mode |  |  |  |  |  |  |  | Source of IMD |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NR CA band combination | NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | Duplex mode |  |
| CA_n41-n79 | n79 | N/A | 10 | N/A | 4874 | 12.4 | TDD | IMD4 |
|  | n41 | 2565 | 100 | 1 (RBSTART= 0) | 2565 | N/A | TDD | N/A |
|  |  | 2644.8 | 60 | 1 (RBSTART= 161) | 2644.8 |  |  |  |

- Reference

[1] R4-2417181, “WF on MSD requriemenets with intra-band contiguous UL CA_n40A-n41C and CA_n41C-n79A”, Huawei