# 3GPP TSG-RAN WG4 Meeting #113  		R4-2419086

# Orlando, US, 18th – 22nd November, 2024

Source: 	vivo

Title: 	Discussion on Maintenance for R18 NES

Agenda Item:	5.27.1

Document for:	Discussion

Introduction

In RAN4 #112 meeting, some issues left in the maintenance part were discussed. Accordingly, the latest progress was captured in WF [1]. In this paper, we would like to provide our thinking on the remaining issues on inter-band SSB-less SCell and NES-based CHO.

Discussion on maintenance of inter-band SSB-less SCell operation

In RAN4 #112, the follow issue was discussed and the agreements are achieved as follows.

Issue 1-1-4: Neighbour cells on carrier of SSB-less SCell FFS:  Intra-frequency/Inter-frequency definition: o Inter-frequency based on existing definition. No impact to spec about the intra-f and inter-f definition.  No spec impact in terms of inter- or intra- frequency, and with or without gap.

Note that in RAN2 #127bis, similar issue is discussed, and RAN2 conclusion is as follows.

Agreements 1 From RAN2 perspective, no benefit is identified for configuring servingCellMO for inter-band SSB-less SCell. 2 No LS is sent to R4. 3 Discuss in the next meeting on whether to capture “For inter-band SSB-less SCell(s), this field is not present” in the conditional presence of servingCellMO, depending on R4 progress. 4 Postpone the discussion on intra-band SSB-less SCell.

The servingCellMO configuration in RRC is defined for the purpose to indicate serving cell L3 measurement to UE. At least for SSB-based L3 measurement, there is no such scenario. However, for CSI-RS based L3 measurement, it is worth to check whether configuration of servingCellMO in SSB-less SCell is feasible or not.

In existing TS 38.331, the signalling only supports the scenario when the CSI-RS and the corresponding associated SSB are in the same band. The following can be found in the field descriptions of MeasObjectNR.

TS 38.331 v18.3.0

freqBandIndicatorNR

The frequency band in which the SSB and/or CSI-RS indicated in this MeasObjectNR are located and according to which the UE shall perform the RRM measurements. This field is always provided when the network configures measurements with this MeasObjectNR.

Since the core part has already been closed, it is difficult to further support CSI-RS based L3 measurement on SSB-less SCell, no matter how much benefit RAN4 can see. On the other hand, RAN4 also agreed the following for L3 measurements in RAN4 #108bis.

- Sub-topic 1-5 L3 measurement for SSB-less SCell

Issue 1-5-1: L3 RRM requirements

Agreement:

- No RRM impacts on L3 measurement for inter-band SSB-less operation.

Above agreement means RAN4 agreed to follow legacy procedure without further enhancement on SSB-less SCell, implying no big benefit from RAN4 perspective also.

Based on above analysis, we have the following proposal.

Proposal 1: From RAN4 perspective, configuring CSI-RS based servingCellMO for inter-band SSB-less SCell is an additional optimization, but it is not supported in R18, since the WI is closed.

If this is agreeable in RAN4, it is up to RAN2 whether to capture “For inter-band SSB-less SCell(s), this field is not present” in the conditional presence of servingCellMO.

Since anyway servingCellMO cannot be configured to the carrier of SSB-less SCell, the above FFS points shall be agreeable.

Proposal 2: RAN4 to agree the following:

- Intra-frequency/Inter-frequency definition:

  - Inter-frequency based on existing definition. No impact to spec about the intra-f and inter-f definition.

- No spec impact in terms of inter- or intra- frequency, and with or without gap.

Discussion on maintenance of NES-based CHO

Fig.1 the timeline of delay requirement on NES-based conditional handover

For NES-based conditional handover, regarding the case that if the CHO condition has been met but is not any more fulfilled when receiving the DCI 2-X command, we will provide our thinking on this issue with the help of the above Fig.1.

RAN2 agreement in RAN2#125bis meeting - Understanding is that “The fulfilment of the event includes the case that the NES mode indication has been received [Cond 1] and the entry condition is now fulfilled [Cond 2]”. No need to update the specifications - No further clarification to continue measurement and CHO evaluation is necessary for the case UE now receives NES mode-enabled indication while NES CHO event is not yet fulfilled, and for the case NES CHO event is now fulfilled and UE has received a NES mode-enabled indication.

After receiving RRC message implying NES-based handover, UE performs measurements periodically to check if entry condition is fulfilled. The checking points are shown as measurement Point 1/2/3/4 in Fig.1. Due to the existence of sufficient good channel conditions at Point 5 and Point 6, UE realizes that entry condition is met at measurement Point 1 and Point 3, respectively. Based on the agreement reached in RAN2 as above, NES-based CHO will be triggered only when both Cond1 and Cond2 are fulfilled. That is to say, Point 5 which the condition exists is not the actual ending point of TEvent_DU referred in spec description. After Point 5, the channel condition gets worse until Point 6 comes. Since UE realizes the entry condition is met at measurement Point 3 and subsequently receives DCI 2-9, Point 6 becomes the actual time point mentioned in definition of TEvent_DU.

Regarding how to implement CR for reflecting such case in the spec, our preference is to keep the current definition of TEvent_DU and measurement delay Tmeasure unchanged. The reasons mainly include:

Firstly, based on the RAN2 agreement provided, either Con1 or Con2 is fulfilled will not trigger the execution of NES-based CHO. Therefore, the current spec anyway is applicable for the case that the CHO condition has been met but is not any more fulfilled when receiving the DCI 2-X command. To avoid making too much to the NES CHO requirements (e.g., the definition of TEvent_DU and Tmeasure) that differ from the existing CHO, we prefer to use the existing definition, even though it may not be perfect.

The current spec description: a condition exists at the measurement reference point which will trigger the NES-based conditional handover RAN2 conclusion: The fulfilment of the event includes the case that the NES mode indication has been received(con1) and the entry condition is now fulfilled(con2).

Another point is that the same case may also arise with legacy conditional handover procedure: UE starts to execute CHO procedure once UE realizes the condition is met at measurement point 1, while the channel condition of target cell gets bad at the measurement point 2 after HO. Actually, UE cannot foresee further signal quality changes and the decision of HO execution shall only base on the current measurements. It is difficult to specify further enhancement to avoid wrong HO execution due to signal changes since UE has already tried its best to evaluate the measurement event continuously based on measurement period.

Proposal 3: For the delay requirement on NES-based CHO, RAN4 to keep the current definition of TEvent_DU and measurement delay Tmeasure unchanged

##### 6.1.4.2.1 Handover delay Procedure delays for all procedures that can command a conditional handover are specified in TS 38.331 [2]. When the UE receives a RRC message implying conditional handover the UE shall be ready to start the transmission of the new uplink PRACH channel within DCHO seconds from the end of the last TTI containing the RRC command. DCHO = TRRC + TEvent_DU + Tmeasure + Tinterrupt + TCHO_execution Where: TRRC is the RRC procedure delay defined in clause 12 in TS 38.331 [2]. TEvent_DU is the delay uncertainty which is the time from when the UE successfully decodes a conditional handover command until - a condition exists at the measurement reference point which will trigger the conditional handover, or - a condition exists at the measurement reference point which will trigger the NES-based conditional handover Tmeasure is the measurements time stated in clause 6.1.4.2.2. TCHO_execution is the conditional execution preparation time in clause 6.1.4.2.3. Tinterrupt is the interruption time stated in clause 6.1.4.2.4. The NES-based conditional handover delay requirements shall apply if UE receives a RRC message implying conditional handover before receiving the NES indication in DCI 2-9 command. When UE receives a RRC message implying NES-based conditional handover but no NES indication in DCI 2-9 command, or when UE only receives a NES indication in DCI 2-9 command before receiving a RRC message implying NES-based conditional handover, no NES-based conditional handover requirement is applied. For NES-based conditional handover, UE shall start RRM measurement before receiving the NES indication in DCI 2-9 command. The NES indication is specified in clause [5.5.4] in TS 38.331[2]. - 6.1.4.2.2 Measurement time The measurement time delay is defined from the end of TEvent_DU until UE executes a handover to a target cell and interruption time starts. For conditional intra-frequency handover, the measurement time delay measured without Time To Trigger (TTT) and L3 filtering shall be less than Tidentify intra with index or Tidentify_intra_without_index defined in clause 9.2.5.1 or clause 9.2.6.2. For conditional inter-frequency handover, the measurement time delay measured without Time To Trigger (TTT) and L3 filtering shall be less than Tidentify_inter_with_index or Tidentify_inter_without_index defined in clause 9.3.4 or clause 9.3.9. For NES-based conditional intra-frequency handover: - If UE successfully decodes DCI 2-9 command earlier than the time at the end of TEvent_DU + Tidentify_intra_with_index or TEvent_DU + Tidentify_intra_without_index, then the measurement time delay equal to Tidentify_intra_with_index or Tidentify_intra_without_index - If UE successfully decodes DCI 2-9 command later than the time at the end of TEvent_DU + Tidentify_intra_with_index or TEvent_DU + Tidentify_intra_without_index, then the measurement time delay equals to the time from the end of Tevent_DU until UE successfully decodes DCI 2-9 command. For NES-based conditional inter-frequency handover: - If UE successfully decodes DCI 2-9 command earlier than the time at the end of TEvent_DU + Tidentify_inter_with_index or TEvent_DU + Tidentify_inter_without_index, then the measurement time delay equal to Tidentify_inter_with_index or Tidentify_inter_without_index - If UE successfully decodes DCI 2-9 command later than the time at the end of TEvent_DU + Tidentify_inter_with_index or TEvent_DU + Tidentify_inter_without_index, then the measurement time delay equals to the time from the end of Tevent_DU until UE successfully decodes DCI 2-9 command. Editor Notes: The measurement time delay for NES-based conditional handover is FFS. Editor Notes: TEvent_DU for NES-based conditional handover is FFS. When TTT or L3 filtering is used an additional delay can be expected. A cell is detectable only if at least one SSB measured from the cell being configured remains detectable during the time period Tidentify_intra_without_index or Tidentify_intra_with_index for intra-frequency handover or Tidentify_inter_without_index for inter-frequency handover. If a cell which has been detectable at least for the time period Tidentify_intra_without_index or Tidentify_intra_with_index for intra-frequency handover or Tidentify_inter_without_index for inter-frequency handover becomes undetectable for a period and then the cell becomes detectable again and triggers a handover, the measurement time delay shall be less than TSSB_measurement_period_intra or TSSB_measurement_period_inter provided the timing to that cell has not changed more than ± 3200/2µ Tc while the measurement gap has not been available and the L3 filter has not been used, where µ is the SCS configuration as defined in clause 4.2 of TS 38.211 [3]. When L3 filtering is used, an additional delay can be expected. ##### 6.1.4.2.3 Preparation time TCHO_execution is the UE execution preparation time for conditional handover, and starts after UE realizes the condition of CHO is met and identity of the target cell is determined. TCHO_execution can be up to 10ms.

Moreover, based on the highlighted description, the measurement time delay (i.e., Tmeasure) includes two parts: one is the actual measure delay from UE, and another is the time duration from UE finished measurement to UE successfully decodes DCI 2-9. Correspondingly, for NES-based CHO, the starting point of TCHO_execution specified in the highlighted part shall correspondingly adapt as the time point that UE successfully decodes DCI 2-9 in some cases. For reference, we provide the example as follows:

##### 6.1.4.2.3 Preparation time TCHO_execution is the UE execution preparation time for conditional handover. For conditional handover, and it starts after UE realizes the condition of CHO is met and identity of the target cell is determined. For NES-based conditional handover, - It starts after UE realizes the condition of CHO is met and identity of the target cell is determined provided that UE has already successfully decoded DCI 2-9. - Otherwise, it starts after UE successfully decodes DCI 2-9. And TCHO_execution can be up to 10ms.

Proposal 4: RAN4 to revise the starting point of TCHO_execution as follows:

TCHO_execution is the UE execution preparation time for conditional handover.

For NES-based conditional handover,

- It starts after UE realizes the condition of CHO is met and identity of the target cell is determined provided that UE has already successfully decoded DCI 2-9.

- Otherwise, it starts after UE successfully decodes DCI 2-9.

Conclusions

Based on above analysis, we have following observations and proposals.

Proposal 1: From RAN4 perspective, configuring CSI-RS based servingCellMO for inter-band SSB-less SCell is an additional optimization, but it is not supported in R18, since the WI is closed.

Proposal 2: RAN4 to agree the following:

- Intra-frequency/Inter-frequency definition:

  - Inter-frequency based on existing definition. No impact to spec about the intra-f and inter-f definition.

- No spec impact in terms of inter- or intra- frequency, and with or without gap.

Proposal 3: For the delay requirement on NES-based CHO, RAN4 to keep the current definition of TEvent_DU and measurement delay Tmeasure unchanged

Proposal 4: RAN4 to revise the starting point of TCHO_execution as follows:

TCHO_execution is the UE execution preparation time for conditional handover.

For NES-based conditional handover,

- It starts after UE realizes the condition of CHO is met and identity of the target cell is determined provided that UE has already successfully decoded DCI 2-9.

- Otherwise, it starts after UE successfully decodes DCI 2-9.

- References

- R4-2414102, WF on RRM requirements for NR network energy saving, Huawei