# 3GPP TSG-RAN WG4 Meeting #113  		R4-2419096

# Orlando, US, 18th – 22nd November, 2024

Source: 	vivo

Title: 	Discussion on maintenance of RRM requirements for R18 NR_Mob_enh2

Agenda Item:	5.23.1

Document for:	Discussion

Introduction

In RAN4 #112, the WF [1] is approved. Moreover, lots of CRs agreed in RAN4 #112.

In RAN4 #112bis and RAN4 #113, RAN4 have received 3 LSs from RAN2. [2][3][4]

Based on all above information, we provide our views on maintenance issue for R18 NR_Mob_enh.

Discussion on core part maintenance

<On clarification on serving cell L1 measurement in LTM>

So far, in existing TS 38.133, the serving cell L1 measurement, including L1 measurement configured for SSB with additional PCI as introduced in R17 inter-cell beam management, are captured in 9.5 and 9.13. These types of L1 measurement are configured under csi-MeasConfig of ServingCellConfig. Under csi-MeasConfig, csi-ReportConfigToAddModList and csi-ReportConfigToReleaseList maintain a set of report configuration CSI-ReportConfig, which may include SSBRI or CRI based L1-RSRP reporting, and each report configuration is associated to one of the resource config CSI-ResourceConfig that maintained by csi-ResourceConfigToAddModList and csi-ResourceConfigToReleaseList.

CSI-MeasConfig ::=                  SEQUENCE {

nzp-CSI-RS-ResourceToAddModList     SEQUENCE (SIZE (1..maxNrofNZP-CSI-RS-Resources)) OF NZP-CSI-RS-Resource   OPTIONAL, -- Need N

nzp-CSI-RS-ResourceToReleaseList    SEQUENCE (SIZE (1..maxNrofNZP-CSI-RS-Resources)) OF NZP-CSI-RS-ResourceId OPTIONAL, -- Need N

nzp-CSI-RS-ResourceSetToAddModList  SEQUENCE (SIZE (1..maxNrofNZP-CSI-RS-ResourceSets)) OF NZP-CSI-RS-ResourceSet

OPTIONAL, -- Need N

nzp-CSI-RS-ResourceSetToReleaseList SEQUENCE (SIZE (1..maxNrofNZP-CSI-RS-ResourceSets)) OF NZP-CSI-RS-ResourceSetId

OPTIONAL, -- Need N

csi-IM-ResourceToAddModList         SEQUENCE (SIZE (1..maxNrofCSI-IM-Resources)) OF CSI-IM-Resource           OPTIONAL, -- Need N

csi-IM-ResourceToReleaseList        SEQUENCE (SIZE (1..maxNrofCSI-IM-Resources)) OF CSI-IM-ResourceId         OPTIONAL, -- Need N

csi-IM-ResourceSetToAddModList      SEQUENCE (SIZE (1..maxNrofCSI-IM-ResourceSets)) OF CSI-IM-ResourceSet     OPTIONAL, -- Need N

csi-IM-ResourceSetToReleaseList     SEQUENCE (SIZE (1..maxNrofCSI-IM-ResourceSets)) OF CSI-IM-ResourceSetId   OPTIONAL, -- Need N

csi-SSB-ResourceSetToAddModList     SEQUENCE (SIZE (1..maxNrofCSI-SSB-ResourceSets)) OF CSI-SSB-ResourceSet   OPTIONAL, -- Need N

csi-SSB-ResourceSetToReleaseList    SEQUENCE (SIZE (1..maxNrofCSI-SSB-ResourceSets)) OF CSI-SSB-ResourceSetId OPTIONAL, -- Need N

csi-ResourceConfigToAddModList      SEQUENCE (SIZE (1..maxNrofCSI-ResourceConfigurations)) OF CSI-ResourceConfig

OPTIONAL, -- Need N

csi-ResourceConfigToReleaseList     SEQUENCE (SIZE (1..maxNrofCSI-ResourceConfigurations)) OF CSI-ResourceConfigId

OPTIONAL, -- Need N

csi-ReportConfigToAddModList        SEQUENCE (SIZE (1..maxNrofCSI-ReportConfigurations)) OF CSI-ReportConfig  OPTIONAL, -- Need N

csi-ReportConfigToReleaseList       SEQUENCE (SIZE (1..maxNrofCSI-ReportConfigurations)) OF CSI-ReportConfigId

OPTIONAL, -- Need N

reportTriggerSize                   INTEGER (0..6)                                                            OPTIONAL, -- Need M

aperiodicTriggerStateList           SetupRelease { CSI-AperiodicTriggerStateList }                            OPTIONAL, -- Need M

semiPersistentOnPUSCH-TriggerStateList    SetupRelease { CSI-SemiPersistentOnPUSCH-TriggerStateList }         OPTIONAL, -- Need M

...,

[[

reportTriggerSizeDCI-0-2-r16        INTEGER (0..6)                                                            OPTIONAL -- Need R

]],

[[

sCellActivationRS-ConfigToAddModList-r17  SEQUENCE (SIZE (1..maxNrofSCellActRS-r17)) OF SCellActivationRS-Config-r17   OPTIONAL, -- Need N

sCellActivationRS-ConfigToReleaseList-r17 SEQUENCE (SIZE (1..maxNrofSCellActRS-r17)) OF SCellActivationRS-ConfigId-r17 OPTIONAL  -- Need N

]],

[[

ltm-CSI-ReportConfigToAddModList-r18   SEQUENCE (SIZE (1..maxNrofLTM-CSI-ReportConfigurations-r18)) OF LTM-CSI-ReportConfig-r18

OPTIONAL, -- Need N

ltm-CSI-ReportConfigToReleaseList-r18  SEQUENCE (SIZE (1..maxNrofLTM-CSI-ReportConfigurations-r18)) OF LTM-CSI-ReportConfigId-r18

OPTIONAL  -- Need N

]]

}

LTM-Config-r18 ::=   SEQUENCE {

ltm-ReferenceConfiguration-r18        SetupRelease {ReferenceConfiguration-r18}                             OPTIONAL,   -- Need M

ltm-CandidateToReleaseList-r18        SEQUENCE (SIZE (1..maxNrofLTM-Configs-r18)) OF LTM-CandidateId-r18    OPTIONAL,   -- Need N

ltm-CandidateToAddModList-r18         SEQUENCE (SIZE (1..maxNrofLTM-Configs-r18)) OF LTM-Candidate-r18      OPTIONAL,   -- Need N

ltm-ServingCellNoResetID-r18          INTEGER (1..maxNrofLTM-Configs-plus1-r18)                             OPTIONAL,   -- Need N

ltm-CSI-ResourceConfigToAddModList-r18  SEQUENCE (SIZE (1..maxNrofLTM-CSI-ResourceConfigurations-r18)) OF LTM-CSI-ResourceConfig-r18

OPTIONAL,   -- Need N

ltm-CSI-ResourceConfigToReleaseList-r18 SEQUENCE (SIZE (1..maxNrofLTM-CSI-ResourceConfigurations-r18)) OF LTM-CSI-ResourceConfigId-r18

OPTIONAL,   -- Need N

attemptLTM-Switch-r18                 ENUMERATED {true}                                                    OPTIONAL, -- Cond LTM-MCG

ltm-ServingCellUE-MeasuredTA-ID-r18   INTEGER (1..maxNrofLTM-Configs-plus1-r18)                             OPTIONAL,   -- Need N

...

}

However, a set of LTM-CSI-ReportConfig-r18 for LTM L1-RSRP report config is maintained by ltm-CSI-ReportConfigToAddModList-r18, and ltm-CSI-ReportConfigToReleaseList-r18. They are associated to the one of resource config LTM-CSI-ResourceConfig-r18 maintained by ltm-CSI-ResourceConfigToAddModList-r18 and ltm-CSI-ResourceConfigToReleaseList-r18. The later two are provided under ltm-Config-r18, which is under RRCReconfiguration, but not under csi-MeasConfig, which is part of ServingCellConfig of any serving cell.

Figure 1 is an illustration for such configuration structure.

Figure 1 RRC configuration structure for serving cell L1 measurement and LTM candidate cell measurement

Note that for LTM-CSI-ReportConfig-r18, in existing TS 38.331, spCellInclusion-r18 can be configured. The explanation for it is as follows.

spCellInclusion

This field indicates whether the UE shall include a L1 measurement report associated to the current SpCell. This field can only be configured if the current SpCell is configured as an SpCell of an LTM candidate configuration and the LTM-CSI-ResourceConfig IE associated to the LTM-CSI-ReportConfig IE includes resources for the current SpCell.

Based on TS 38.331, UE can only report L1-RSRP of current SpCell if it is configured as SpCell of one of candidate CGs. If LTM-config does not include SpCell SSB(s) as part of the candidate cell SSB(s) in the set of LTM-CSI-ResourceConfig-r18, UE may not even need to perform LTM L1 measurement on current SpCell SSB(s). Whether UE needs to perform legacy L1 RSRP measurement on SpCell SSB(s) depends on the configuration of CSI-ReportConfig and CSI-ResourceConfig.

Observation 1  From the perspective of RAN1/2 spec, LTM L1 measurements on current serving cell SSBs are configured by LTM-CSI-ResourceConfig-r18 under ltm-Config-r18, while legacy L1 measurements on any current serving cell are configured by CSI-ResourceConfig under csi-MeasConfig.

However, for R18 LTM, in existing TS 38.133, delay requirements for LTM L1 measurement of serving cell is captured in clause 9.5.4.1. The clause 9.5.4.1 of TS 38.133 is cited as follows.

TS 38.133 v18.7.0 9.5.4.1 SSB based L1-RSRP Reporting … When there is no intra-frequency L1-RSRP measurement on LTM neighbor cell(s) to measure, the value of TL1-RSRP_Measurement_Period_SSB is defined in Table 9.5.4.1-1 for FR1, the value of TL1-RSRP_Measurement_Period_SSB is defined in Table 9.5.4.1-2 for FR2 when highSpeedMeasFlagFR2-r17 is not configured and if groupBasedBeamReporting-r17 is not configured, and defined in Table 9.5.4.1-2A for FR2-1 when highSpeedMeasFlagFR2-r17 is not configured and if groupBasedBeamReporting-r17 is configured, and defined in Table 9.5.4.1-3 for FR2 power class 6 UE when highSpeedMeasFlagFR2-r17 is configured, where - M=1 if higher layer parameter timeRestrictionForChannelMeasurement is configured, and M=3 otherwise - The value of N in Table 9.5.4.1-2 is [reducedRxBeamNum] in FR2-1 for UE supporting [faster beam switching capability], according to the conditions described in clause 3.6.x, and activated with multi-Rx operation, otherwise the value of N in Table 9.5.4.1-2 is 8. The UE is activated with multi-Rx operation when the following conditions are met: - UE is configured with group-based beam reporting (GBBR) report - The value of N in Table 9.5.4.1-2A is 8. - The value of N in Table 9.5.4.1-3 is 8. … The value of TL1-RSRP_Measurement_Period_SSB is defined in Table 9.5.4.1-4 for UE incapable of [capability of measurement with RTD>CP] in FR1, Table 9.5.4.1-5 for UE capable of [capability of measurement with RTD>CP] in FR1, Table 9.5.4.1-6 for UE incapable of [capability of measurement with RTD>CP] in FR2 and Table 9.5.4.1-7 for UE capable of [capability of measurement with RTD>CP] in FR2 when there is intra-frequency L1-RSRP measurement on neighbor cell(s) to measure, where - M=1 if higher layer parameter timeRestrictionForChannelMeasurement is configured, and M=3 otherwise - N= 8 in Table 9.5.4.1-6 and Table 9.5.4.1-7. … Table 9.5.4.1-1: Measurement period TL1-RSRP_Measurement_Period_SSB for FR1 Configuration; TL1-RSRP_Measurement_Period_SSB (ms). non-DRX; max(TReport, ceil(M*P)*TSSB). DRX cycle ≤ 320ms; max(TReport, ceil(K *M*P)*max(TDRX,TSSB)). DRX cycle > 320ms; ceil(M*P)*TDRX. Note 1: TSSB = ssb-periodicityServingCell is the periodicity of the SSB-Index configured for L1-RSRP measurement. TDRX is the DRX cycle length. TReport is configured periodicity for reporting. Note 2: K = 1 when TSSB ≤ 40 ms and highSpeedMeasFlag-r16 or highSpeedMeasCA-Scell-r17 are configured; otherwise K = 1.5. Note 3: When highSpeedMeasFlag-r16 is configured, the requirements apply only to UE supporting either measurementEnhancement-r16 or intraNR-MeasurementEnhancement-r16. or measurementEnhancementCA-r17. … Table 9.5.4.1-4: Measurement period TL1-RSRP_Measurement_Period_SSB in FR1 for UE incapable of [capability of measurement with RTD>CP] Configuration; TL1-RSRP_Measurement_Period_SSB_intra (ms). non-DRX; max(TReport, ceil(M*P)*TSSB). DRX cycle ≤ 320ms; max(TReport, ceil(K *M*P)*max(TDRX,TSSB)). DRX cycle > 320ms; ceil(M*P)*TDRX. Note 1: TSSB = ssb-periodicityServingCell is the periodicity of the SSB-Index configured for L1-RSRP measurement. TDRX is the DRX cycle length. TReport is configured periodicity for reporting. Note 2: K = 1.5. Note 3: The requirements apply if the actual RTD between serving cell and neighbor cell is not larger than CP.. Table 9.5.4.1-5: Measurement period TL1-RSRP_Measurement_Period_SSB in FR1 for UE capable of [capability of measurement with RTD>CP] Configuration; TL1-RSRP_Measurement_Period_SSB_intra (ms). non-DRX; max(TReport, ceil(M*P)*TSSB). DRX cycle ≤ 320ms; max(TReport, ceil(K *M*P)*max(TDRX,TSSB)). DRX cycle > 320ms; ceil(M*P)*TDRX. Note 1: TSSB = ssb-periodicityServingCell is the periodicity of the SSB-Index configured for L1-RSRP measurement. TDRX is the DRX cycle length. TReport is configured periodicity for reporting. Note 2: K = 1.5..

Based on discussion so far, our understanding to the motivation behind this design is that, from RRM requirements perspective, RAN4 assumes that UE always performs LTM L1 measurement on current serving cell(s), if configured, with exact the same behaviour as legacy L1 measurement configured by CSI-ResourceConfig under csi-MeasConfig. Based on this assumption, for FR2-1, it is easier to specify Rx beam sharing factor between serving cell and neighbour cell, since UE is assumed to perform LTM L1 measurement at exactly the same time as legacy serving cell L1 measurements configured by CSI-ResourceConfig under csi-MeasConfig.

However, there are multiple technical issues for this kind of approach. Firstly, all requirements captured under 9.5 and 9.13 shall follows the requirement applicability rules defined in 9.5.3 and 9.13.3, respectively. There are two main requirement applicability rules. One is the RTD. The other is the max number of layer/cells/resources UE can measure.

For R18 MIMO, RAN1 has introduced the following UE feature.

| Features | Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Consequence if the feature is not supported by the UE | Type | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 40. NR_MIMO_evo_DL_UL | 40-2-6 | Rx timing difference larger than CP length | 1. Support of the Rx timing difference between the two DL reference timings is larger than CP length |  | yes | Rx timing difference larger than CP is not supported | Per FSPC | n/a | n/a | n/a |  | Optional with capability signaling |

The motivation behind this FG 40-2-6 rxTimingDiff-r18 is to allow UE to indicate the support of L1 measurement for RTD>CP case, before 2-TA based m-TRP is configured by gNB. This L1 measurement is supposed to be configured by CSI-ResourceConfig under csi-MeasConfig. Note that this FG is per-FSPC, which means it only applies to one CC of a given band in a BC. RAN4 have also introduced maximum RTD UE may assume for this case, under clause 7.6.8 of TS 38.133. From RRM requirement perspective, although there is no yet pre-requisite UE capability defined for this FG in RAN feature list, UE supporting either multiDCI-IntraCellMultiTRP-TwoTA-r18 or multiDCI-InterCellMultiTRP-TwoTA-r18 is the pre-requisite for MRTD and MTTD requirements, as defined in 7.6.8 an 7.5.7.

Observation 2  For R18 MIMO, rxTimingDiff-r18 is introduced as a UE capability for indicating support of RTD>CP between any two downlink timing reference signals within a CC, based on which UE will be able to perform L1-RSRP measurement so as to enable 2-TA m-TRP operation. The capability is per-FSPC, and needs pre-requisite of either multiDCI-IntraCellMultiTRP-TwoTA-r18 or multiDCI-InterCellMultiTRP-TwoTA-r18 from RRM requirement perspective.

On the other hand, for R18 Mobility, RAN4 has introduced the following UE FG.

| Features | Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Consequence if the feature is not supported by the UE | Type | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 39. NR_Mob_enh2 | 39-1 | SSB based L1-RSRP measurements for multiple cells with RTD > CP | Capability of simultaneous L1-RSRP measurements for more than one cell when the max RTD among the cells on the same frequency layer or in the same active BWP is larger than CP length of the cell on the frequency layer or in the same active BWP. | 45-1 from RAN1 Rel-18 feature list or 39-2 or 39-2a | Yes | The corresponding RAN4 requirements may not be satisfied when the max RTD among the cells on the same frequency layer or in the same active BWP is larger than CP length of the cell on the frequency layer or in the same active BWP. | Per BC | No | No | N/A |  | Optional with capability signaling |

The motivation of this UE feature multiCellL1-measRTD-greaterThan-CP-r18 is to indicate the support of LTM L1 measurement on candidate cells when the RTD>CP between cells, including serving cell if possible, are in the same frequency layer or in the same active BWP. Actually, the RTD here can be either for the cell on serving cell CC, or for cells that not covered by any active BWP at all. UE supporting the capability shall support one of intra-frequency LTM L1 measurement, inter-frequency LTM L1 measurement within gaps, or inter-frequency LTM L1 measurement outside gaps as pre-requisite.

Observation 3 For R18 Mobility, multiCellL1-measRTD-greaterThan-CP-r18 is introduced as a UE capability for indicating support of RTD>CP between any two cells on the same frequency layer or within the same active BWP, on which UE will be able to perform L1-RSRP measurement so as to enable LTE operations including early DL/UL sync. The capability is per-BC, and needs pre-requisite of one of intraFreqL1-MeasConfig-r18, ltm-InterFreqMeasGap-r18 or interFreqSSB-L1-MeasWithoutGaps-r18.

Observation 4 Both rxTimingDiff-r18 and multiCellL1-measRTD-greaterThan-CP-r18 are for the scenario of RTD>CP, and are for L1-RSRP measurement. However, pre-requisites and scenario details for these two features are different. Hence, UE may not always support both of them at the same time.

Based on above observations, we have the following proposal.

Proposal 1  Although same UE behaviour on L1-RSRP measurement for current serving cell is assumed from RRM requirements perspective, no matter it is configured by either CSI-ResourceConfig under csi-MeasConfig or LTM-CSI-ResourceConfig-r18 under ltm-Config-r18, it is still proposed to capture LTM L1 measurement on serving cell in clause 9.14 of TS 38.133, so as to better differentiate UE support of either rxTimingDiff-r18 or multiCellL1-measRTD-greaterThan-CP-r18.

Another issue is the number of layers/cells/resources UE can measure. For legacy serving cell L1 measurements, only the number of resources is limited, as cited below.

TS 38.133 v18.7.0 9.5 L1-RSRP measurements for Reporting 9.5.1 Introduction … The UE shall be able to measure all CSI-RS resources and/or SSB resources of the nzp-CSI-RS-ResourceSet and/or csi-SSB-ResourceSet within the CSI-ResourceConfig settings configured for L1-RSRP for the active BWP, provided that the number of resources, including the number of SSB resources of the cell with PCI different from serving cell configured for L1-RSRP measurements in 9.13, does not exceed the UE capability indicated by beamManagementSSB-CSI-RS. If the UE supports bwpOperationMeasWithoutInterrupt-r18, [UE shall be able to measure the SSB reources of csi-SSB-ResourceSet within the CSI-ResourceConfig configured outside active BWP], provided that the SSB is within the configured UE-specific CBW. … 9.13 L1-RSRP measurements for a cell with different PCI from serving cell 9.13.1 Introduction … The UE shall be able to measure all SSB resources of the cell with different PCI in csi-SSB-ResourceSet within the CSI-ResourceConfig settings for the active BWP, while the additionalPCI-r17 of the SSB resources are different from serving cell PCI. The number of resources, including the number of resources configured for serving cell L1-RSRP measurement in 9.5, does not exceed the UE capability indicated by beamManagementSSB-CSI-RS. …

Observation 5  UE needs to meet the legacy serving L1 measurement requirements, including L1 measurements on cell with different PCI, as long as the total number of resources to measure does not exceed the UE capability beamManagementSSB-CSI-RS. There is no restriction on the number of layers and/or cells for legacy L1 measurements for serving cell(s).

On the other hand, for LTM, RAN2 have sent two LSs regarding the UE capability design for L1 measurement in recent meetings. [2][3] Based on [3], it seems basically RAN2 have finished the design of UE features for LTM L1 measurements. Here are some of the UE capabilities shown in [3], which is aligned with TS 38.306

| maxFreqLayersL1-Meas-r18 (R4 39-3-1) Indicates the number of frequency layers for L1-RSRP measurement. This capability signalling comprises of the following parameters: - supportedMaxIntraInterFreqLayersWithoutGaps-r18 indicates the maximum number of frequency layers UE can measure for intra- and inter-frequency without measurement gaps L1-RSRP measurement. A UE indicating support for this component shall also indicate support for intraFreqL1-MeasConfig-r18 and/or interFreqSSB-L1-MeasWithoutGaps-r18. - supportedMaxIntraInterFreqLayersWithGaps-r18 indicates the maximum number of frequency layers UE can measure for inter-frequency L1-RSRP measurement with measurement gaps. A UE indicating support for this component shall also indicate support for ltm-InterFreqMeasGap-r18. | BC | No | N/A | N/A |
| --- | --- | --- | --- | --- |
| maxNeighCellsPerFreqLayerL1-Meas-r18 (R4 39-3-2) Indicates the number of neighbouring cells per frequency layer for L1-RSRP measurement. This capability signalling comprises of the following parameters: - supportedMaxNeighCellsPerFreqLayersWithoutGaps-r18 indicates the max number of neighbour cells UE can measure for L1-RSRP per frequency layer for intra-frequency or inter-frequency without measurement gaps. A UE indicating support for this component shall also indicate support for intraFreqL1-MeasConfig-r18 or interFreqSSB-L1-MeasWithoutGaps-r18. - supportedMaxNeighCellsPerFreqLayersWithGaps-r18 indicates the max number of neighbour cells UE can measure for L1-RSRP per frequency layer for inter-frequency with measurement gaps. A UE indicating support for this component shall also indicate support for ltm-InterFreqMeasGap-r18. | BC | No | N/A | N/A |
| supportedMaxCellsWithoutGapsL1-Meas-r18 (R4 39-3-3) Indicates the max number of total cells of serving cells and neighbouring cells across all frequency layers of intra-frequency and inter-frequency without measurement gaps for L1 measurement. A UE indicating support for this feature shall also indicate support for intraFreqL1-MeasConfig-r18 or interFreqSSB-L1-MeasWithoutGaps-r18. | BC | No | N/A | N/A |
| supportedMaxSSB-WithinSlotL1-Meas-r18 (R4 39-3-4) Indicates the max number of SSB resources for L1-RSRP measurement that UE can measure within a slot across candidate cells for intra- and inter-frequency without gap L1-RSRP measurement. A UE indicating support for this feature shall also indicate support for intraFreqL1-MeasConfig-r18 or interFreqSSB-L1-MeasWithoutGaps-r18. | BC | No | N/A | N/A |
| maxSSB-PerFreqLayerL1-Meas-r18 (R4 39-3-5) Indicates the maximum number of SSB resources for L1-RSRP measurement per frequency layer UE can measure. This capability signalling comprises of the following parameters: - supportedMaxSSB-PerFreqLayersWithoutGaps-r18 indicates the max number of SSB resources UE can measure for L1-RSRP per frequency layer for intra-frequency or inter-frequency without measurement gaps. A UE indicating support for this component shall also indicate support for intraFreqL1-MeasConfig-r18 or interFreqSSB-L1-MeasWithoutGaps-r18. - supportedMaxSSB-PerFreqLayersWithGaps-r18 indicates the max number of SSB resources UE can measure for L1-RSRP per frequency layer for inter-frequency with measurement gaps. A UE indicating support for this component shall also indicate support for ltm-InterFreqMeasGap-r18. | BC | No | N/A | N/A |
| supportedMaxSSB-L1-Meas-r18 (R4 39-3-6) Indicates the max number of total SSB resources of serving cells and neighbouring cells across all frequency layers of intra-frequency and inter-frequency without measurement gaps for L1 measurement. A UE indicating support for this feature shall also indicate support for intraFreqL1-MeasConfig-r18 or interFreqSSB-L1-MeasWithoutGaps-r18. | BC | No | N/A | N/A |

For RAN4 FG 39-3-1, regarding intra-frequency L1 measurements, our understanding is that SSB frequency Layers only counts the frequency layers where LTM L1 measurement are configured. However, if RAN4 assumes UE always performs LTM L1 measurements for current serving cell at the same time UE performs serving cell L1 measurements for beam managements, it is not clear that whether all serving cell SSB frequency layers, including SSB frequency layers that are not configured with any LTM L1 measurements, shall be counted.

Proposal 2  RAN4 to clarify that in RAN4 FG 39-3-1 maxFreqLayersL1-Meas-r18, only serving SSB frequency layers configured with LTM L1 measurements are counted. It does not include serving SSB frequency layers on which only legacy L1 measurements configured by CSI-ResourceConfig under csi-MeasConfig, are to be performed.

If above proposal is agreeable to the group, then we have the following observation.

Observation 6  If Proposal 2 is agreeable, UE performs legacy serving cell L1 measurements on any SSB frequency layer that is configured by CSI-ResourceConfig under csi-MeasConfig, but only performs LTM L1 measurement on frequency layers that are configured with LTM L1 measurements on candidate cells.

For RAN4 FG 39-3-2, it is clear that only neighbour cells are counted. Our understanding is that even if a current serving cell, either SpCell or SCell, is configured as an LTM candidate cell, FG 39-3-2 does not count this cell.

For RAN4 FG 39-3-3, it considers both serving cell and neighbour cells. It is not clear that if only an intra-frequency neighbour cell of one current SCell is configured as LTM candidate cell, whether it counts only the LTM candidate cell, or also counts the serving cell which provides the timing reference for LTM L1 measurements if UE does not support RTD>CP.

Proposal 3  RAN4 to clarify that in RAN4 FG 39-3-3 supportedMaxCellsWithoutGapsL1-Meas-r18, which option below is the common understanding:

- Option 1: ‘total cells of serving cells and neighbouring cells’ includes only the LTM candidate cell(s), any of which can be one current serving cell or not.

- Option 2: ‘total cells of serving cells and neighbouring cells’ includes both LTM candidate cell(s) and all serving cells, including the serving cells that are not configured with intra-f LTM L1 measurements or inter-f LTM L1 measurements without gaps.

- Option 3: ‘total cells of serving cells and neighbouring cells’ includes both LTM candidate cell(s) and some serving cells, while only serving cells with either intra-f LTM L1 measurements and/or inter-f LTM L1 measurements without gaps are counted.

For RAN4 FG 39-3-4, it is clear that only LTM candidate cells are counted here.

For RAN4 FG 39-3-5, it is not clear whether ‘max number of SSB resources UE can measure for L1-RSRP per frequency layer’ only counts SSB resources configured by LTM-CSI-ResourceConfig-r18 under ltm-Config-r18, or also counts the SSBs from serving cell which provides the timing reference for the neighbour cells if UE does not support RTD>CP.

Proposal 4  RAN4 to clarify that in RAN4 FG 39-3-5 maxSSB-PerFreqLayerL1-Meas-r18, which option below is the common understanding:

- Option 1: ‘number of SSB resources UE can measure for L1-RSRP per frequency layer’ includes only the SSB resources configured for LTM candidate cell(s), any of which can be one current serving cell or not.

- Option 2: ‘number of SSB resources UE can measure for L1-RSRP per frequency layer’ includes both LTM candidate cell(s) and some SSBs from serving cells, which provides the timing reference for the neighbour cells if UE does not support RTD>CP.

For RAN4 FG 39-3-6, similar issue can be found. It is not clear whether ‘SSB resources of serving cells and neighbouring cells’ includes only the SSBs configured by LTM-CSI-ResourceConfig-r18 under ltm-Config-r18, or also counts the SSBs from serving cell which provides the timing reference for the neighbour cells if UE does not support RTD>CP.

Proposal 5  RAN4 to clarify that in RAN4 FG 39-3-6 supportedMaxSSB-L1-Meas-r18, which option below is the common understanding:

- Option 1: ‘SSB resources of serving cells and neighbouring cells’ includes only the SSB resources configured for LTM candidate cell(s), any of which can be one current serving cell or not.

- Option 2: ‘SSB resources of serving cells and neighbouring cells’ includes both LTM candidate cell(s) and all SSBs from serving cells, including the serving cells that are not configured with intra-f LTM L1 measurements or inter-f LTM L1 measurements without gaps.

- Option 3: ‘SSB resources of serving cells and neighbouring cells’ includes both LTM candidate cell(s) and some SSBs from serving cells, which provides the timing reference for the neighbour cells if UE does not support RTD>CP.

Given Proposal 2 to Proposal 5, no matter which option RAN4 can agree on, in our view, it would be better if LTM related restrictions, including restrictions to related serving cells, are captured under 9.14. Most of UE capabilities discussed above are not related to whether serving cell L1 measurement is configured or not.

Proposal 6  To better capture UE capabilities related to the max number of layers/cells/resource UE can measure under LTM configurations, LTM related serving cell L1 measurement shall be captured under 9.14.

Another point is that, in existing 9.5.4.1 of TS 38.133, as cited above, it is not clear how to understand ‘when there is intra-frequency L1-RSRP measurement on neighbor cell(s) to measure’. For example, if serving cell L1 measurement is configured on more than one CCs, but there is only one CC, e.g. the SpCell CC, is configured with LTM L1 measurements, does it mean UE needs to consider to follow LTM L1 measurement requirement under existing 9.5.4.1, for all CCs. Following current TS 38.133, it seems so.

Observation 7  In existing TS 38.133, in clause 9.5.4.1, the description of ‘when there is intra-frequency L1-RSRP measurement on neighbor cell(s) to measure’ has not restricted whether it only applies the to the corresponding frequency layer where only intra-f LTM L1 measurement happens, or to all frequency layer configured for serving cell L1 measurements across all CCs.

Therefore, RAN4 may further discuss proposal 2 to proposal 6, and based on discussion, further capture the corresponding restrictions in requirement applicability for 9.14 and 9.15. Based on proposal 1 and proposal 6, it is also preferred to move LTM related serving cell L1 measurement to 9.14. If needed, RAN4 may reply LS [3] or send updated UE feature list to RAN2, so as to also align the descriptions in TS 38.306.

Regarding the concern on UE Rx beam sharing between serving cell and LTM neighbor cells, at least in R18, sharing factor is defined only between serving cell and neighbour cell. That means, no RRM requirements is specified if R18 LTM and R17 inter-cell beam management is configured simultaneously in FR2-1.

Observation 8  No R18 RRM requirements are specified for the case R18 LTM and R17 inter-cell L1 measurement for ICBM is configured simultaneously on the same SSB frequency in FR2-1.

Observation 9  For FR2-1, RAN4 has agreed to define sharing factor between all serving cell L1 measurements (including LTM L1 and legacy L1) and neighbour cell(s) LTM L1 measurement, considering different UE Rx beams could be used. It shall be understood as measurement relaxation to legacy serving cell L1 measurements if LTM L1 measurements on neighbour cell(s) are configured in this frequency layer, and PL1_sharing would be enough to capture this impact.

Proposal 7  Capture all intra-frequency LTM L1 measurement requirements, including serving cell L1 measurement requirements if counted as LTM L1 measurement, in 9.14. The impact to serving cell L1 measurement, which is configured by CSI-ResourceConfig, due to Rx beam sharing with neighbour cell LTM L1 measurement is captured by PL1_sharing in 9.5.4.1.

<On L1 report for unmeasured candidate cells >

In last meeting, the following is agreed.

Issue 1-3-3: L1 report for unmeasured candidate cells < Agreement> - Use DIFFRSRP_15 in Table 10.1.6.1-2 for measurement reporting of differential values for unmeasured LTM L1-RSRP resources.

This also impact the known condition of a candidate cell and/or a candidate SSB/TCI. In case the reported resource ID defined in TS 38.212 is associated to DIFFRSRP_15, the cell/SSB/TCI it reflects cannot be regarded as known based on this reporting.

Proposal 8  Update the known condition for candidate cell SSB, candidate cell TCI and LTM target cell with the following:

- A L1-RSRP report is valid if the reported L1-RSRP value corresponding to one of the valid codepoints for L1-RSRP in Table 10.1.6.1-1, or any codepoints other than DIFFRSRP_15 in Table 10.1.6.1-2.

< On ‘time gap’ for TCI activation and PDCCH-ordered RACH>

RAN4#111 < Agreement>: - In early candidate cell’s TCI state activation delay for known TCI state case: Tfirst-SSB is the time to the first SSB occasion overlapped with MGL after slot n + THARQ +3Nslotsubframe,µ + [2ms] if the SSB needs to be measured with MG. - FFS: this is only applicable to UE supporting inter-frequency L1 measurement with MG. - Same agreement applies to the definition of Tfirst-SSB in PDCCH order RACH delay requirement.. < Way Forward > FFS the following options: - Option 1 (Apple, MTK, Ericsson): - remove “FFS: this is only applicable to UE supporting inter-frequency L1 measurement with MG” in definition of TSSB in PDCCH ordered RACH and early TCI activation requirements. - Option 2 (ZTE, vivo): - Confirm the FFS.

In our view, time-frequency tracking for the early TCI activation is actually L1 measurement behaviour. It is already agreed that UE can only perform L1 measurement within gap if UE supports the corresponding capability. In previous meeting, RAN4 has agreed that L3 measurement and reporting can also be considered as one applicable condition for RRM requirements in FR1. Therefore, RRM requirements are applicable if the SSB of the target cell (for either PDCCH-order-RACH or early TCI activation) is within active BWP and L3 MR with beam index is reported. For the case SSB of target cell is not within active BWP, no matter L3 or L1 reporting has been done, UE needs to perform time-frequency tracking again on this SSB with similar behaviour as L1 measurement. UE not supporting L1 measurement with gaps may not do this, at least in FR2. It has no relation on whether L1 measurement or L3 measurement has been used to trigger this.

Proposal 9  Remove the FFS and confirm the following

- In early candidate cell’s TCI state activation delay for known TCI state case: For UE supporting inter-frequency L1 measurement with gap, Tfirst-SSB is the time to the first SSB occasion overlapped with MGL after slot n + THARQ +3Nslotsubframe,µ + [2ms] if the SSB needs to be measured with MG.

- In PDCCH ordered RACH delay, when additional time for T/F tracking is needed: For UE supporting inter-frequency L1 measurement with gap, TSSB is the time to the first SSB occasion overlapped with MGL after [2]ms and [1slot from] the end of the slot of the PDCCH, plus 2ms (SSB processing time), if the SSB needs to be measured with MG.

Discussion on perf part maintenance

<On TCI related test case coverage and Test configurations>

In existing TS 38.133, the following is captured for LTM cell switch test cases including FR1 and FR2.

The test consists of 4 tests, and UE is required to pass one among Test 1A, Test 1B, Test 2A and Test 2B. - Test 1: for a UE supporting ltm-MAC-CE-JointTCI-r18 and/or ltm-MAC-CE-SeparateTCI-r18 - Test 1A: for a UE supporting ltm-MAC-CE-JointTCI-r18. - Test 1B: for a UE supporting ltm-MAC-CE-SeparateTCI-r18 and does not support ltm-MAC-CE-JointTCI-r18. - Test 2: for a UE not supporting ltm-MAC-CE-JointTCI-r18 and ltm-MAC-CE-SeparateTCI-r18 - Test 2A: for a UE supporting ltm-BeamIndicationJointTCI-r18. - Test 2B: for a UE supporting ltm-BeamIndicationSeparateTCI-r18 and does not support ltm-BeamIndicationJointTCI-r18.

As in RAN1 feature list, 45-3a and 45-4a are generally not correlated to each other. However, the R17 pre-requisite for 45-3a is FG 23-1-1, while 45-4a is FG 23-10-1. In R17 RAN1 feature list, it is clearly state that FG 23-1-1 is the pre-requisite of FG 23-10-1.

Observation 10  Since R17, supporting joint TCI configuration is the pre-requisite for supporting separate TCI.

Another issue is for FR1, how joint TCI or separate TCI would be configured. Actually, there was no ever test cases in FR1 that refers to R17 joint/separate TCI configuration. In TS 38.214, we see the following restrictions for UL TCI.

TS 38.214 v18.1.0 6.1 UE procedure for transmitting the physical uplink shared channel … The UE can be configured with a list of up to 64 TCI-UL-State configurations within the higher layer parameter BWP-UplinkDedicated. Each TCI-UL-State configuration contains a parameter for configuring one reference signal, if applicable, for determining UL TX spatial filter for dynamic-grant and configured-grant based PUSCH and PUCCH resource in a CC, and SRS. … When the UE is configured dl-OrJointTCI-StateList or ul-TCI-StateList, the UE shall perform PUSCH transmission corresponding to a Type 1 configured grant or a Type 2 configured grant or a dynamic grant according to the spatial relation, if applicable, with a reference to the RS for determining UL Tx spatial filter. The RS is determined based on an RS configured with qcl-Type set to 'typeD' of the indicated TCI-State or an RS in the indicated TCI-UL-State. The reference RS in the indicated TCI-State can be a CSI-RS resource in a NZP-CSI-RS-ResourceSet configured with higher layer parameter repetition, or a CSI-RS resource in an NZP-CSI-RS-ResourceSet configured with higher layer parameter trs-Info. …

In RAN1 spec, the reference RS configured in UL TCI would be effective only if it is applicable. Normally, the common understanding is that in FR2 the ‘QCL-D’ and UL TCI would be applicable, since the spatial Rx/Tx information, i.e. UE beam information, are necessary. For FR1, the meaning of UL TCI and QCL-D is ambiguous. Different UE implementation might have different interpretation. From test case design perspective, they should not be configured in FR1 test cases.

Observation 11  According to RAN1 spec, the meaning of configuring ‘QCL-D’ and UL TCI to UE would be ambiguous in FR1.

Note that ltm-MAC-CE-JointTCI-r18 and ltm-MAC-CE-SeparateTCI-r18 are per-band UE capabilities and normally UE report supporting of ltm-MAC-CE-SeparateTCI-r18 in a FR1 band does not expect network to configure UL TCI state. Hence allow test coverage to this case would be redundant.

Proposal 10  In RRM test case design for LTM, RAN4 avoid TCI configuration with ‘QCL-D’ or ‘UL TCI’ to UE in FR1, which means only DL or Joint TCI with QCL-A/C configuration, and pathloss RS configuration if necessary, shall be provided to UE in FR1, even if UE supports ltm-MAC-CE-SeparateTCI-r18.

Conclusions

Based on above analysis, we have following observations and proposals.

Observation 1  From the perspective of RAN1/2 spec, LTM L1 measurements on current serving cell SSBs are configured by LTM-CSI-ResourceConfig-r18 under ltm-Config-r18, while legacy L1 measurements on any current serving cell are configured by CSI-ResourceConfig under csi-MeasConfig.

Observation 2  For R18 MIMO, rxTimingDiff-r18 is introduced as a UE capability for indicating support of RTD>CP between any two downlink timing reference signals within a CC, based on which UE will be able to perform L1-RSRP measurement so as to enable 2-TA m-TRP operation. The capability is per-FSPC, and needs pre-requisite of either multiDCI-IntraCellMultiTRP-TwoTA-r18 or multiDCI-InterCellMultiTRP-TwoTA-r18 from RRM requirement perspective.

Observation 3 For R18 Mobility, multiCellL1-measRTD-greaterThan-CP-r18 is introduced as a UE capability for indicating support of RTD>CP between any two cells on the same frequency layer or within the same active BWP, on which UE will be able to perform L1-RSRP measurement so as to enable LTE operations including early DL/UL sync. The capability is per-BC, and needs pre-requisite of one of intraFreqL1-MeasConfig-r18, ltm-InterFreqMeasGap-r18 or interFreqSSB-L1-MeasWithoutGaps-r18.

Observation 4 Both rxTimingDiff-r18 and multiCellL1-measRTD-greaterThan-CP-r18 are for the scenario of RTD>CP, and are for L1-RSRP measurement. However, pre-requisites and scenario details for these two features are different. Hence, UE may not always support both of them at the same time.

Proposal 1  Although same UE behaviour on L1-RSRP measurement for current serving cell is assumed from RRM requirements perspective, no matter it is configured by either CSI-ResourceConfig under csi-MeasConfig or LTM-CSI-ResourceConfig-r18 under ltm-Config-r18, it is still proposed to capture LTM L1 measurement on serving cell in clause 9.14 of TS 38.133, so as to better differentiate UE support of either rxTimingDiff-r18 or multiCellL1-measRTD-greaterThan-CP-r18.

Observation 5  UE needs to meet the legacy serving L1 measurement requirements, including L1 measurements on cell with different PCI, as long as the total number of resources to measure does not exceed the UE capability beamManagementSSB-CSI-RS. There is no restriction on the number of layers and/or cells for legacy L1 measurements for serving cell(s).

Proposal 2  RAN4 to clarify that in RAN4 FG 39-3-1 maxFreqLayersL1-Meas-r18, only serving SSB frequency layers configured with LTM L1 measurements are counted. It does not include serving SSB frequency layers on which only legacy L1 measurements configured by CSI-ResourceConfig under csi-MeasConfig, are to be performed.

Observation 6  If Proposal 2 is agreeable, UE performs legacy serving cell L1 measurements on any SSB frequency layer that is configured by CSI-ResourceConfig under csi-MeasConfig, but only performs LTM L1 measurement on frequency layers that are configured with LTM L1 measurements on candidate cells.

Proposal 3  RAN4 to clarify that in RAN4 FG 39-3-3 supportedMaxCellsWithoutGapsL1-Meas-r18, which option below is the common understanding:

- Option 1: ‘total cells of serving cells and neighbouring cells’ includes only the LTM candidate cell(s), any of which can be one current serving cell or not.

- Option 2: ‘total cells of serving cells and neighbouring cells’ includes both LTM candidate cell(s) and all serving cells, including the serving cells that are not configured with intra-f LTM L1 measurements or inter-f LTM L1 measurements without gaps.

- Option 3: ‘total cells of serving cells and neighbouring cells’ includes both LTM candidate cell(s) and some serving cells, while only serving cells with either intra-f LTM L1 measurements and/or inter-f LTM L1 measurements without gaps are counted.

Proposal 4  RAN4 to clarify that in RAN4 FG 39-3-5 maxSSB-PerFreqLayerL1-Meas-r18, which option below is the common understanding:

- Option 1: ‘number of SSB resources UE can measure for L1-RSRP per frequency layer’ includes only the SSB resources configured for LTM candidate cell(s), any of which can be one current serving cell or not.

- Option 2: ‘number of SSB resources UE can measure for L1-RSRP per frequency layer’ includes both LTM candidate cell(s) and some SSBs from serving cells, which provides the timing reference for the neighbour cells if UE does not support RTD>CP.

Proposal 5  RAN4 to clarify that in RAN4 FG 39-3-6 supportedMaxSSB-L1-Meas-r18, which option below is the common understanding:

- Option 1: ‘SSB resources of serving cells and neighbouring cells’ includes only the SSB resources configured for LTM candidate cell(s), any of which can be one current serving cell or not.

- Option 2: ‘SSB resources of serving cells and neighbouring cells’ includes both LTM candidate cell(s) and all SSBs from serving cells, including the serving cells that are not configured with intra-f LTM L1 measurements or inter-f LTM L1 measurements without gaps.

- Option 3: ‘SSB resources of serving cells and neighbouring cells’ includes both LTM candidate cell(s) and some SSBs from serving cells, which provides the timing reference for the neighbour cells if UE does not support RTD>CP.

Proposal 6  To better capture UE capabilities related to the max number of layers/cells/resource UE can measure under LTM configurations, LTM related serving cell L1 measurement shall be captured under 9.14.

Observation 7  In existing TS 38.133, in clause 9.5.4.1, the description of ‘when there is intra-frequency L1-RSRP measurement on neighbor cell(s) to measure’ has not restricted whether it only applies the to the corresponding frequency layer where only intra-f LTM L1 measurement happens, or to all frequency layer configured for serving cell L1 measurements across all CCs.

Observation 8  No R18 RRM requirements are specified for the case R18 LTM and R17 inter-cell L1 measurement for ICBM is configured simultaneously on the same SSB frequency in FR2-1.

Observation 9  For FR2-1, RAN4 has agreed to define sharing factor between all serving cell L1 measurements (including LTM L1 and legacy L1) and neighbour cell(s) LTM L1 measurement, considering different UE Rx beams could be used. It shall be understood as measurement relaxation to legacy serving cell L1 measurements if LTM L1 measurements on neighbour cell(s) are configured, and PL1_sharing would be enough to capture this impact.

Proposal 7  Capture all intra-frequency LTM L1 measurement requirements, including serving cell L1 measurement requirements if counted as LTM L1 measurement, in 9.14. The impact to serving cell L1 measurement, which is configured by CSI-ResourceConfig, due to Rx beam sharing with neighbour cell LTM L1 measurement is captured by PL1_sharing in 9.5.4.1.

Proposal 8  Update the known condition for candidate cell SSB, candidate cell TCI and LTM target cell with the following:

- A L1-RSRP report is valid if the reported L1-RSRP value corresponding to one of the valid codepoints for L1-RSRP in Table 10.1.6.1-1, or any codepoints other than DIFFRSRP_15 in Table 10.1.6.1-2.

Proposal 9  Remove the FFS and confirm the following

- In early candidate cell’s TCI state activation delay for known TCI state case: For UE supporting inter-frequency L1 measurement with gap, Tfirst-SSB is the time to the first SSB occasion overlapped with MGL after slot n + THARQ +3Nslotsubframe,µ + [2ms] if the SSB needs to be measured with MG.

- In PDCCH ordered RACH delay, when additional time for T/F tracking is needed: For UE supporting inter-frequency L1 measurement with gap, TSSB is the time to the first SSB occasion overlapped with MGL after [2]ms and [1slot from] the end of the slot of the PDCCH, plus 2ms (SSB processing time), if the SSB needs to be measured with MG.

Observation 10  Since R17, supporting joint TCI configuration is the pre-requisite for supporting separate TCI.

Observation 11  According to RAN1 spec, the meaning of configuring ‘QCL-D’ and UL TCI to UE would be ambiguous in FR1.

Proposal 10  In RRM test case design for LTM, RAN4 avoid TCI configuration with ‘QCL-D’ or ‘UL TCI’ to UE in FR1, which means only DL or Joint TCI with QCL-A/C configuration, and pathloss RS configuration if necessary, shall be provided to UE in FR1, even if UE supports ltm-MAC-CE-SeparateTCI-r18.

- References

[1]  R4-2413960	WF on Further NR mobility enhancements, MediaTek.  RAN4 #112

[2]  R4-2414913	LS on LTM L1 intra and inter-frequency measurements capabilities, RAN2.  RAN4 #112bis

[3]  R4-2417508	LS on UE capabilities for inter-frequency L1 measurements for LTM, RAN2.  RAN4 #113

[4]  R4-2417509	LS on the fast RRC processing for LTM, RAN2.  RAN4 #113