# 3GPP TSG-RAN WG4 Meeting #113  		R4-2419101

# Orlando, US, 18th – 22nd November, 2024

Source: 	vivo

Title: 	Discussion on RRM requirements for UE initiated Beam reporting in R19 MIMO

Agenda Item:	7.19.3.1

Document for:	Discussion

Introduction

In RAN4 #112bis, the WF [1] is approved.

Based on all above information, we provide our views on RRM requirements for UE initiated beam reporting.

Discussion

<On Endpoint of L1 RSRP reporting requirements>

In RAN4 #112bis, the follow issue was discussed and the agreements are achieved as follows.

Issue 1-1: How to define Event triggered measurement reporting delay? < Agreement> - Start point of Event triggered measurement reporting delay: reuse the same definition in L3 event trigger report which is the event is met over the air. - Ending point is the point when the UE transmits a [first PUCCH] triggered by the event. - “[first PUCCH]” can be updated based on further RAN1/2 agreement.

In last RAN1 meeting, the following is agreed.

Agreement On beam report transmission procedure for UE-initiated/event-driven beam reporting, regarding the triggering procedure in Step-2 of Mode-A - Reuse CSI request field in DCI format 0_1/0_2 to trigger the transmission of the UEI beam report - If a CSI trigger state associated with UEI beam report configuration(s) is indicated by the CSI request field in DCI format 0_1/0_2, the UE transmits the corresponding UEI beam report(s) in the second PUSCH scheduled by the DCI format 0_1/0_2 - FFS: DCI format 0_3 - FFS: Whether a CSI trigger state should be dedicated to UE-initiated/event-driven beam reporting, i.e., not associated with legacy AP-CSI report configuration.

Therefore, UE may not be able to directly pack its result in the DG-PUSCH without network triggering. Hence, first PUCCH is necessary. The square bracket can be removed.

Proposal 1  Remove square bracket based on RAN1 latest agreements:

- For event triggered measurement reporting delay, ending point is the point when the UE transmits a first PUCCH triggered by the event.

<On how to define requirements for event-triggered L1 reporting>

In last meeting, RAN4 has also discussed whether UE shall update the L1 measurement results during the time UE wait for uplink resources. According to RAN1 agreements above, UE may not know when gNB will send the CSI request field in DCI, after first PUCCH is reported. RAN1 is also discussing first PUCCH retransmission. It is possible from UE perspective that, at the time when actual L1 measurement report shall be sent, the event that triggers the reporting is already exited. In our view, this is related to the report cancelling mechanism, which shall be discussed in RAN1/2 first. On the other hand, RAN4 may proceed without considering further updates to the results reported by UE after first PUCCH is sent. For simplicity, it is proposed to consider a testable scenario, i.e. the condition over the air is fixed after during the time UE performs the evaluation based on L1 measurements.

Proposal 2  To discuss the requirements for event-triggered L1 reporting, it is proposed to firstly discuss the scenario where UE needs not to consider updating the L1 reporting content based on latest results after first PUCCH is sent.

- That means to firstly consider the case that, the condition which will trigger the event is remained fixed during the time UE performs L1 measurements.

- Whether UE shall update the L1 report content based on latest L1 measurement results, or shall even cancel the report in some cases, is up to RAN1/2 discussion.

Moreover, RAN1 has also agreed at least Event-2. Some details about agreements Event-2 is cited as follows.

Agreement On UE-initiated/event-driven beam reporting, regarding trigger-event detection for beam reporting, at least support Event-2: Quality of at least one new beam, such as L1-RSRP, becomes a threshold value better than the current beam. - At least L1-RSRP is supported as quality metrics used for Event-2 - Note-1: ‘New/current beam’ is for discussion purpose. - Note-2: Other trigger events/quality metrics (e.g., L1-SINR) are not precluded. - Note-3: For above implicit manner(s), if there are two QCL RSs in a TCI state, the measurement RS is derived from RS w.r.t. QCL-TypeD, if applicable. Agreement On UE-initiated/event-driven beam reporting, regarding Event-2, ‘current beam’ is a beam corresponding to the indicated TCI state. - Regarding RS measurement for the current beam for Event-2, Option-2a is supported: - Option-2a (implicit manner): The RS for current beam is implicitly derived from a QCL RS of indicated TCI state. Agreement Regarding RS measurement for the current beam for Event 2, for Option-2a, support the both schemes as follows. - Scheme-1: RS for current beam is the QCL RS in the indicated TCI state - Scheme-2: the RS for current beam is the SSB which is QCLed with the QCL RS in the indicated TCI state. - Enabling one of either Scheme-1 or Scheme-2 is selected by NW. - (Working Assumption) Enabling of either Scheme-1 or Scheme-2 should ensure the same RS type for RS measurement for current beam and new beam. - The above QCL RS is the RS w.r.t. QCL-TypeD, if there are two QCL RSs in the indicated TCI state. Agreement Regarding RS measurement for the new beam for Event 2, at least Option-3a is supported - Option-3a (explicit manner): The RS(s) for new beam(s) are explicitly configured Agreement Regarding explicit RS configuration for new beam measurement for Event 2, at least Option-1 is supported - Option-1: The RS(s) for new beam(s) are explicitly configured in one RS resource set associated with an CSI reporting configuration - If legacy UE capability signaling cannot be reused, introduce a UE capability signaling of indicating the maximum number of the configured RS(s) in the RS resource set. Agreement Regarding RS measurement for the current beam for Event 2, for Option-2a, besides for scheme-1 and scheme-2, there is no RAN1 consensus on the following enhancement for handling the case that only one TRS is configured in the indicated TCI state in RAN1#118bis - Option-1: Introducing additional scheme: the RS for current beam can be a CSI-RS for beam management derived from the QCL RS in the indicated TCI state; - Option-2: Further support TRS as measurement RS of current beam for determining L1-RSRP - Option-3: Introducing additional scheme: The RS for current beam is explicitly configured by RRC or MAC-CE (Option-2C in RAN1 116b agreement). Note 3: When only one TRS is configured in the indicated TCI state, either Scheme-1(working assumption) or Scheme-2 is used where enabling one of either Scheme-1 or Scheme-2 is selected by NW. When the Scheme-1 is used, the UE assumes that the CSI-RS resource in the indicated TCI state is configured in a CSI-RS resource set configured with repetition.

RAN1 has agreed to use a serving cell RS, which is derived based on indicated TCI state, as the RS for current beam. This RS can be either SSB or CSI-RS, and RAN1 has already completed the rule for determine the RS from indicated TCI state. Based on definition in TS 38.214, indicated TCI state may change due to either MAC CE based TCI activation, or DCI based TCI switching. Therefore, the TCI used for current beam measurement can be dynamically changed.

On the other hand, the RS for new beam is configured in a resource set. In our understanding this resource set can be either SSB set or CSI-RS set. At least for the case of CSI-RS, UE may not be always able to perform CSI-RS based L1 measurement on the corresponding CSI-RS, if the SNR of QCL-source SSB is not kept high enough, e.g. over -2dB. Therefore, theoretically, the new beam where UE will measure may also change dynamically. Note that RAN1 is still FFS on the semi-persistent CSI-RS for this case.

Based on above, it is also proposed to focus on the simplest case in RAN4 first, i.e. RAN4 firstly discuss the RRM requirements for event triggered L1 reporting considering no change to the indicated TCI state, and no change on the RS for the new beam measurements.

Proposal 3  To discuss the requirements for event-triggered L1 reporting, it is proposed to firstly discuss the scenario where, during the time UE performs L1 measurements for evaluating the event,

- No change on the indicated TCI state

- No change on the to-be-measured RS of the new beam.

<On known/unknown condition>

In legacy L3 measurement and reporting, the measurement can deal with much lower RS SINR, and L1 and L3 filtering are performed, as illustrated in TS 38.300. However, for L1-RSRP reporting, so far TimeRestrictionForChannelMeasurement can be configured as True and in that case neither L1 nor L3 filtering is done. For this event triggered L1 reporting, at least based on following RAN1 conclusion, we think event-triggered L1 reporting shall follow legacy L1 measurement behaviour.

Agreement Regarding the triggering event determination for Event 2: - If within a time window (which is configurable), the number of Event-2 instance(s) for at least one same new beam is greater than or equal to a configurable number M, UE initiated beam report occurs. - Note: Event-2 instance for a new beam is determined if the L1-RSRP of the new beam becomes a threshold value better than the current beam Above feature is subject to UE capability. - Basic feature: Once the L1-RSRP of the new beam becomes a threshold value better than the current beam, UE initiated beam report occurs FFS: Whether the above is captured in RAN1 or RAN2 specification.

In our view, for beam management purposes, since the reported L1-RSRP/L1-SINR is one type of CSI which may also impact UL/DL scheduling, it would be OK if no filtering is applied before the reporting. Therefore, RAN4 shall focus on the basic UE feature.

Proposal 4  RAN4 specify RRM requirements for event-triggered L1 reporting without time-window-based filtering or other filtering configured.

In our view, network shall be responsible for the filtering of L1 measurement results reported by UE, which is the same as legacy. However, for event-triggered L1 reporting, the UE reporting behaviour could be different from legacy. For example, in legacy L3 measurement, UE shall also report when event-leaving condition is met. However, it is still not clear in RAN1 whether event-leaving is necessary for L1 based approach. On the other hand, after event is met, it is still not clear whether UE shall send one report or multiple reports.

Therefore, we would like to wait further progress regarding RAN1/2 conclusions on how the L1 reporting is sent in time-domain manner, e.g. whether single-shot or multi-shot reporting after event-entering condition is met, and whether UE reports when event-leaving condition is met, etc. This would impact the known condition of the TCI state switching. For example, UE may not have a chance to report that the event leaving condition is met, hence UE may not know the best beam for a new to-be-activated TCI if gNB schedule this TCI based on previous reporting.

Proposal 5  Regarding the impact to know conditions of TCI state activation from event-triggered L1 reporting, RAN4 waits further RAN1/2 conclusions.

Conclusions

Based on above analysis, we have following observations and proposals.

Proposal 1  Remove square bracket based on RAN1 latest agreements:

- For event triggered measurement reporting delay, ending point is the point when the UE transmits a first PUCCH triggered by the event.

Proposal 2  To discuss the requirements for event-triggered L1 reporting, it is proposed to firstly discuss the scenario where UE needs not to consider updating the L1 reporting content based on latest results after first PUCCH is sent.

- That means to firstly consider the case that, the condition which will trigger the event is remained fixed during the time UE performs L1 measurements.

- Whether UE shall update the L1 report content based on latest L1 measurement results, or shall even cancel the report in some cases, is up to RAN1/2 discussion.

Proposal 3  To discuss the requirements for event-triggered L1 reporting, it is proposed to firstly discuss the scenario where, during the time UE performs L1 measurements for evaluating the event,

- No change on the indicated TCI state

- No change on the to-be-measured RS of the new beam.

Proposal 4  RAN4 specify RRM requirements for event-triggered L1 reporting without time-window-based filtering or other filtering configured.

Proposal 5  Regarding the impact to know conditions of TCI state activation from event-triggered L1 reporting, RAN4 waits further RAN1/2 conclusions.

- References

[1]  R4-2416856  WF on RRM requirements for NR_MIMO_Ph5_Part1, Samsung.  RAN4 #112bis