3GPP TSG-RAN WG4 Meeting #113															R4-2419108
Orlando, US, 18th – 22nd November, 2024

Source:	Ericsson

Title:	Initial discussions on IoT NTN TDD mode: Study of impact due to periodic pattern

Agenda item:	7.28.2

Document for:	Approval

# 1	Introduction

RAN plenary has approved a new WID [1]. The assumptions are listed as stated below:

… The work item aims to specify enhancements for NB-IoT NTN to enable NTN operation with a NB-IoT TDD mode leveraging commonalities with half-duplex NB-IoT FDD NTN, by defining a new NB-IoT TDD mode for NTN based on minimum necessary changes to the NB-IoT NTN FDD frame structure and procedures for the NB-IoT operation in the targeted unpaired MSS allocated band (TN deployment is not expected in this band). The feature is not intended to be applicable to existing 3GPP bands. The study and work objectives assume the following: - LEO @600 km and @1200 km orbit respectively, with set-1 satellite parameters as reference scenarios (See 3GPP TR 36.763) - Target the 1616-1626.5 MHz MSS allocated band - Standalone deployment with anchor and non-anchor carriers (i.e. operating in carrier(s) used only for NB-IoT) - Operate with Earth fixed Tracking area, with either Earth fixed cells or Earth moving cells for NGSO - The new NB-IoT NTN TDD mode allows configuring the usage of radio resources in the targeted MSS allocated band with a periodic subset of the UL and DL subframes in N radio frames. The periodic pattern should consist of non-overlapping set of usable contiguous UL subframes and set of usable contiguous DL subframes, and guard periods, which is periodic every N radio frames, with N=9 as baseline. No blind detection is assumed at the UE side. The value of N and the configuration of the periodic pattern are fixed per band. …

The RAN4 impacts are labeled below:

… This work item includes the following objectives: - Study the impact due to the periodic pattern, at least on UE downlink synchronization and other aspects (if identified) [RAN1, RAN4] - Checkpoint in RAN#106 for the completion of the study phase. RAN1 start from Oct’24. RAN4 start from Nov’24 - Specify a new NB-IoT TDD NTN mode based on minimum necessary changes to the NB-IoT NTN FDD frame structure and procedures, based on the outcome of the study, including: - Definition, configuration (if needed) and signaling (if needed) of the periodic pattern including confirming the value of N, and associated UE procedures [RAN1, RAN2] - Other necessary impacts on higher layers [RAN2] - RRM and RF core requirements [RAN4] - Specify a new NB-IoT TDD operating NTN band for the MSS allocation spanning 1616-1626.5 MHz for DL and UL, based on the outcome of the study, to be used as example band for this WI [RAN4]. - Specify band numbering - Specify SAN and UE RF characteristics - Specify DL and UL channelization. - Specify channel bandwidth as 200 kHz - Note1: No NTN-NTN coexistence study needed. - Note2: Leverage existing work as much as possible for TN-NTN coexistence of adjacent bands …

In this contribution we provide Initial discussions on IoT NTN TDD mode with focus on study of impact due to periodic pattern.

# 2	Discussion

It is up to RAN1 to settle frame structure and therefore the DL timing reference solution. In RAN1 Final feature lead summary [2,3,4] we have:

… Agreement For the study phase, RAN1 assumes as a baseline for evaluations (which includes DL synchronization as per the WID) that downlink PHY channels and signals are transmitted on the anchor carrier following subframe index/location and SFN mapping of NB-IoT NTN FDD. - For the purpose of study, the DL subframes of the Rel-19 TDD pattern may not fully cover all the instances of the PHY channels and signals and eNB will not transmit the corresponding instances. - FFS: What is the TDD pattern Agreement RAN1 evaluates via link level simulations the following: - NPSS/NSSS detection - NPBCH decoding Agreement The impact of the TDD pattern at least on the following channels / signals are evaluated at least qualitatively (e.g. specification impact, number of available instances, etc.): - SIB1-NB - Other SIBs needed for basic NB-IoT NTN operation (e.g. SIB2, SIB31) - Paging - NPRACH / RAR - NPSS/NSSS/NPBCH - NPDCCH, NPUSCH, NPDSCH - Note: Study for the above channels / signals may continue during the normative phase. - It is up to companies to perform link level evaluations for some of the aspects above Agreement For the study phase, RAN1 evaluates the following combination of N (in radio frames) and D (D = number of consecutive downlink subframes): - N = 9: - D = 8 - D = 20 - D = 30 - N = 8: - D = 20 - Companies to report which subframe indices are included in the set of consecutive downlink subframes. - Other combinations are not precluded to be reported by companies NOTE: This does not imply that the above combinations are the only ones to be considered during the normative phase. Agreement For the study phase, RAN1 evaluates the following combination of N (in radio frames) and U (U = number of consecutive uplink subframes): - N = 9: - U = 8 - U = 20 - U = 30 - N = 8: - U = 20 - Companies to report which subframe indices are included in the set of consecutive uplink subframes. - Other combinations are not precluded to be reported by companies - For evaluations including both uplink and downlink (if any), companies are encouraged to prioritize the case of D=U NOTE: This does not imply that the above combinations are the only ones to be considered during the normative phase. Agreement For the study phase, RAN1 assumes as a baseline for evaluations that uplink PHY channels are transmitted on the anchor carrier following subframe index/location and SFN mapping of NB-IoT NTN FDD. - For the purpose of study, the UL subframes of the Rel-19 TDD pattern may not fully cover all the instances of the PHY channels and eNB will not receive the corresponding instances. - FFS: What is the TDD pattern Proposal The study and specification work for NB-IoT NTN TDD shall ensure consider at least operation compatible with the TDD pattern of the legacy system deployed in the 1616-1626.5 MHz band (shown below), with the following constraints: - N = 9. - D = 8, with the set of consecutive downlink subframes being confined within one downlink slot in the TDD pattern below. - U = 8, with the set of consecutive uplink subframes being confined within one uplink slot in the TDD pattern below. - FFS: Details - NOTE: The figure below represents the TDD pattern of the legacy system at the satellite node. …

In a first analysis of DL sync channels (and SIB) Ericsson concludes in RAN1 that [3]:

Based on the analysis performed in the subsections above, the anchor carrier must reserve DL subframes to accommodate at least NPBCH, NPSS, NSSS, SIB1-NB, SIB2-NB, and SIB31-NB, leaving the rest of the subframes usable for control information and data traffic, where the latter can span from one up to 10 NPDSCH subframes. Thus, as a minimum assigning 2 DL radio frames out of N radio frames is foreseen to be needed, and aiming at minimizing the specification impact, the selection of N can be either equal to the transmission duration of one self-decodable CSB (i.e., 8 radio frames) or a multiple of such a duration. Figure 3 illustrates the presence of the PHY-channels and signals that as a minimum are envisioned to be transmitted in TDD operation for NB-IoT NTN, as well as the remaining subframes in principle usable for NPDCCH and NPDSCH.

Figure 3: Presence of PHY-channels and signals that are envisioned that need to be transmitted in TDD operation for NB-IoT NTN, which are distributed using X = 2 DL radio frames within a period of N =8 radio frames.

- Proposal 2: 	For the support of “TDD mode NB-IoT NTN” on the anchor carrier in DL:

- At least 2 consecutive DL radio frames within a period composed by N radio frames are used to carry at least NPBCH, NPSS, NSSS, SIB1-NB, SIB2-NB, and SIB31-NB, leaving the rest of the subframes usable for NPDCCH and NPDSCH.

- Aiming at minimizing the spec impact, MIB-NB is used as design reference where N can be either equal to the transmission duration of one self-decodable CSB (i.e., 8 radio frames) or a multiple of such transmission duration.

A DL frame structure where 2 consecutive radio frames are repeated, i.e., D = 20, X = 2 DL radio frames, is compatible with existing baseline NB-IoT NTN FDD in TS 36.133 [4], since the reference NRS, used for RAN4 RRM procedures, are specified to be repeated in every 0, 4 and 9:th subframes.

Observation 1: 	A D = 20 or D = 30 (D = number of DL consecutive subframes) is compatible with existing NB-IoT NTN FDD, since RAN4 RRM procedures assume NRS to be transmitted in every 0,4 and 9 subframes.

An NB-IoT NTN FDD devise is in DL mode unless scheduled. When transmitting in UL and existing NB-IoT NTN FDD device is permitted to switch to DL to receive DL synchronization signals at least every 160 ms. This means that a new NB-IoT NTN TDD mode terminal based on NB-IoT NTN FDD baseline can handle then gaps in NPSS, NSSS and NPBCH transmission which will appear based on the RAN1 proposals of N and D (N = 9: D = 8, 20, 30   N = 8: D = 20) currently being proposed.

Observation 2: 	A new NB-IoT NTN TDD mode terminal based on NB-IoT NTN FDD baseline can handle then gaps in NPSS, NSSS and NPBCH transmission which will appear based on the RAN1 proposals of N and D (N = 9: D = 8, 20, 30   N = 8: D = 20) currently being proposed, since when transmitting in UL and existing NB-IoT NTN FDD device is permitted to switch to DL to receive DL synchronization signals at least every 160 ms.

With regards to the Measurements for UE category NB-IoT for Satellite Access specified in the NB-IoT NTN FDD for new NB-IoT NTN TDD mode there are two existing modes, normal (LTE standard) and enhanced (sometimes also called extended) coverage (In legacy in 36.133, NB-IoT supports operation in both normal- and enhanced (also called extended coverage sometimes) coverage.  The two coverage modes are defined as follows in 36.133:

-    The requirements for normal coverage shall apply for UE category NB1 provided that the radio condition of its serving cell are: NSCH Ês/Iot ³ -6 dB and NRS Ês/Iot ³ -6 dB.

-    The requirements for enhanced coverage shall apply for UE category NB1 provided that the radio condition of its serving cell are: -15 dB £ NSCH Ês/Iot < -6 dB and -15 dB £ NRS Ês/Iot < -6 dB.

Proposal 1: 	It is assumed that since we base new NB-IoT NTN TDD mode on NB-IoT NTN FDD baseline that both coverage modes shall be analysed for possible inclusion in NB-IoT NTN TDD.

Existing NB-IoT NTN FDD measurements for Satellite Access assume measurement periods of 800 ms or 1600 ms depending on if normal or extended coverage is assumed. For example, for normal coverage this is based on an assumption between measurement time and accuracy of one sample every 80 ms and 10 samples. The samples are based on NRS which are transmitted in every radio frame.

Observation 3: 	Existing NB-IoT NTN FDD measurements for Satellite Access assume measurement periods of 800 ms or 1600 ms depending on if normal or extended coverage is assumed (sometimes normal coverage includes 1600 ms periods as well). The samples are based on NRS which are transmitted in every radio frame.

Since it is stated in WID [1] that we shall specify a new NB-IoT TDD NTN mode based on minimum necessary changes to the NB-IoT NTN FDD frame structure and procedures, based on the outcome of the study, including RRM and RF core requirements [RAN4] we should reuse existing measurement periods, if possible.

Proposal 2: 	Since it is stated in WID [1] that we shall specify a new NB-IoT TDD NTN mode based on minimum necessary changes to the NB-IoT NTN FDD frame structure and procedures, including RRM and RF core requirements, we should reuse existing measurement periods, unless subsequent RAN4 analysis shows that this is not possible.

# 3 	Summary

Observation 1: 	A D = 20 or D = 30 (D = number of DL consecutive subframes) is compatible with existing NB-IoT NTN FDD, since RAN4 RRM procedures assume NRS to be transmitted in every 0,4 and 9 subframes.

Observation 2: 	A new NB-IoT NTN TDD mode terminal based on NB-IoT NTN FDD baseline can handle then gaps in NPSS, NSSS and NPBCH transmission which will appear based on the RAN1 proposals of N and D (N = 9: D = 8, 20, 30   N = 8: D = 20) currently being proposed, since when transmitting in UL and existing NB-IoT NTN FDD device is permitted to switch to DL to receive DL synchronization signals at least every 160 ms.

Proposal 1: 	It is assumed that since we base new NB-IoT NTN TDD mode on NB-IoT NTN FDD baseline that both coverage modes shall be analysed for possible inclusion in NB-IoT NTN TDD.

Observation 3: 	Existing NB-IoT NTN FDD measurements for Satellite Access assume measurement periods of 800 ms or 1600 ms depending on if normal or extended coverage is assumed (sometimes normal coverage includes 1600 ms periods as well). The samples are based on NRS which are transmitted in every radio frame.

Proposal 2: 	Since it is stated in WID [1] that we shall specify a new NB-IoT TDD NTN mode based on minimum necessary changes to the NB-IoT NTN FDD frame structure and procedures, including RRM and RF core requirements, we should reuse existing measurement periods, unless subsequent RAN4 analysis shows that this is not possible.

# References

[1] 		RP-242415, New WID on introduction of IoT-NTN TDD mode

[2]		R1-2409038, R1-2409039, R1-2409274 feature lead summary

[3]		R1-2408736, On TDD mode for IoT-NTN, Ericsson

[4]		TS 36.133, Evolved Universal Terrestrial Radio Access (E-UTRA); Requirements for support of radio resource management