3GPP TSG-RAN WG4 Meeting #113															R4-2419110
Orlando, US, 18th – 22nd November, 2024

Source:	Ericsson

Title:	Other aspects for Type 4 capable FWA UE for EN-DC NR-CA

Agenda item:	7.5.2.3

Document for:	Approval

# 1	Introduction

In previous meeting the following WF was approved [1]:

… ## 1.1 <Issue 3-1: UE Type definition for EN-DC> Way Forward: - The following table is a starting point, and continue further discussion the following table in the next meeting. - The table will be added into TS 38.101-3: 7.10B power imbalance for DC in FR1. - Table 1: EN-DC UE Capability types [UE Capability Type]; Type description; Dedicated UE capability. Type 1; It supports intra-band RF requirements (with a maximum power spectral density imbalance between downlink carriers, within 6 dB) and an (NG)EN-DC/NE-DC MTTD/MRTD according to clause 7.5.3/7.6.3 in TS 38.133.; N/A. Type 2; The UE supports inter-band RF requirements (with DL power imbalance as specified in caluse 7.10A in TS 38.101-3) and an (NG)EN-DC MTTD/MRTD according to clause 7.5.2/7.6.2 in TS 38.133 and NE-DC MTTD/MRTD according to clause 7.5.5/7.6.5 in TS 38.133; interBandMRDC-WithOverlapDL-Bands-r16, requirementTypeIndication-r18. Type 4; The UE supports inter-band RF requirements (with DL power imbalance as specified in caluse 7.10A in TS 38.101-3) and an (NG)EN-DC MTTD/MRTD according to clause 7.5.2/7.6.2 in TS 38.133 and NE-DC MTTD/MRTD according to clause 7.5.5/7.6.5 in TS 38.133; TBD. ## 1.2 <Issue 3-2: UE Type definition for NR-CA> Way Forward: - The following table is a starting point, and continue further discussion the following table in the next meeting. - The table will be added into TS 38.101-1: 7.10A Power imbalance for CA. - Table 2: NR-CA UE Capability types [UE capability types]; Type description; dedicated UE capability. Type 1; It supports intra-band RF requirements (with a maximum power spectral density imbalance between downlink carriers, within 6 dB) and MRTD requirements according to clause 7.6.4-1 in 38.133; N/A. Type 2; The UE supports the inter-band RF requirements (with DL power imbalance as specified in clause 7.10A.2) and MTTD/MRTD requirements according to Table 7.5.4-1/Table 7.6.4-2 in TS 38.133; intraBandNR-CA-non-collocated-r18. Type 4; The UE supports the inter-band RF requirements (with DL power imbalance as specified in clause 7.10A.2) and MTTD/MRTD requirements according to Table 7.5.4-1/Table 7.6.4-2 in TS 38.133; TBD. …

# 2	Discussion

Regarding issues 3-1 UE Type definition for EN-DC and issue 3-2 UE Type definition for CA we support the proposal in WF [1], since, as we see it, the intention is to simplify specification text in a couple of places. The WF proposals are local to certain particular sections, namely TS 38.101-3 section 7.10B and TS 38.101-1 section 7.10A and will not change any portal paragraphs early in TS 38.101-1 and TS 38.101-3.

Observation 1: 	The WF proposals are local to certain particular sections, namely TS 38.101-3 section 7.10B and TS 38.101-1 section 7.10A and will not change any portal paragraphs early in TS 38.101-1 and TS 38.101-3.

Proposal 1:	Add the following to TS 38-101-3:

- Table 1: EN-DC UE Capability types

| [UE Capability Type] | Type description | Dedicated UE capability |
| --- | --- | --- |
| Type 1 | It supports intra-band RF requirements (with a maximum power spectral density imbalance between downlink carriers, within 6 dB) and an (NG)EN-DC/NE-DC MTTD/MRTD according to clause 7.5.3/7.6.3 in TS 38.133. | N/A |
| Type 2 | The UE supports inter-band RF requirements (with DL power imbalance as specified in caluse 7.10A in TS 38.101-3) and an (NG)EN-DC MTTD/MRTD according to clause 7.5.2/7.6.2 in TS 38.133 and NE-DC MTTD/MRTD according to clause 7.5.5/7.6.5 in TS 38.133 | interBandMRDC-WithOverlapDL-Bands-r16, requirementTypeIndication-r18 |
| Type 4 | The UE supports inter-band RF requirements (with DL power imbalance as specified in caluse 7.10A in TS 38.101-3) and an (NG)EN-DC MTTD/MRTD according to clause 7.5.2/7.6.2 in TS 38.133 and NE-DC MTTD/MRTD according to clause 7.5.5/7.6.5 in TS 38.133 | TBD |

Add the following to TS 38-101-1:

- Table 2: NR-CA UE Capability types

| [UE capability types] | Type description | dedicated UE capability |
| --- | --- | --- |
| Type 1 | It supports intra-band RF requirements (with a maximum power spectral density imbalance between downlink carriers, within 6 dB) and MRTD requirements according to clause 7.6.4-1 in 38.133 | N/A |
| Type 2 | The UE supports the inter-band RF requirements (with DL power imbalance as specified in clause 7.10A.2) and MTTD/MRTD requirements according to Table 7.5.4-1/Table 7.6.4-2 in TS 38.133 | intraBandNR-CA-non-collocated-r18 |
| Type 4 | The UE supports the inter-band RF requirements (with DL power imbalance as specified in clause 7.10A.2) and MTTD/MRTD requirements according to Table 7.5.4-1/Table 7.6.4-2 in TS 38.133 | TBD |

# 3 	Summary

Observation 1: 	The WF proposals are local to certain particular sections, namely TS 38.101-3 section 7.10B and TS 38.101-1 section 7.10A and will not change any portal paragraphs early in TS 38.101-1 and TS 38.101-3.

Proposal 1:	Add the following to TS 38-101-3:

- Table 1: EN-DC UE Capability types

| [UE Capability Type] | Type description | Dedicated UE capability |
| --- | --- | --- |
| Type 1 | It supports intra-band RF requirements (with a maximum power spectral density imbalance between downlink carriers, within 6 dB) and an (NG)EN-DC/NE-DC MTTD/MRTD according to clause 7.5.3/7.6.3 in TS 38.133. | N/A |
| Type 2 | The UE supports inter-band RF requirements (with DL power imbalance as specified in caluse 7.10A in TS 38.101-3) and an (NG)EN-DC MTTD/MRTD according to clause 7.5.2/7.6.2 in TS 38.133 and NE-DC MTTD/MRTD according to clause 7.5.5/7.6.5 in TS 38.133 | interBandMRDC-WithOverlapDL-Bands-r16, requirementTypeIndication-r18 |
| Type 4 | The UE supports inter-band RF requirements (with DL power imbalance as specified in caluse 7.10A in TS 38.101-3) and an (NG)EN-DC MTTD/MRTD according to clause 7.5.2/7.6.2 in TS 38.133 and NE-DC MTTD/MRTD according to clause 7.5.5/7.6.5 in TS 38.133 | TBD |

Add the following to TS 38-101-1:

- Table 2: NR-CA UE Capability types

| [UE capability types] | Type description | dedicated UE capability |
| --- | --- | --- |
| Type 1 | It supports intra-band RF requirements (with a maximum power spectral density imbalance between downlink carriers, within 6 dB) and MRTD requirements according to clause 7.6.4-1 in 38.133 | N/A |
| Type 2 | The UE supports the inter-band RF requirements (with DL power imbalance as specified in clause 7.10A.2) and MTTD/MRTD requirements according to Table 7.5.4-1/Table 7.6.4-2 in TS 38.133 | intraBandNR-CA-non-collocated-r18 |
| Type 4 | The UE supports the inter-band RF requirements (with DL power imbalance as specified in clause 7.10A.2) and MTTD/MRTD requirements according to Table 7.5.4-1/Table 7.6.4-2 in TS 38.133 | TBD |

# References

[1] 		R4-2417114, WF on Rel-19 non-collocated scenario