3GPP TSG-RAN WG4 Meeting # 113	R4-2419115

# Orlando, US, 18th – 22nd November, 2024

Agenda item:	7.17.4.1

Source: 	National Taiwan University

Title: 	On Base Station Tx Beamforming Codebook Design and the Prediction Impact

Document for:	Approval

Introduction

We present our view on partially specified decoder and reference decoder derivation procedure in this contribution.

Discussion

Discussion on Tx Beamforming Codebook

Two sub-options are listed in the previous meeting WF:

Option 1a [corresponding to BS antenna configurations option 1]: 32 beams with grid of 4 elevation angles from [-25 to 25] and 8 azimuth angles from [-60 to 60] Option 1b [corresponding to BS antenna configurations option 1]: The codebook construction can be clarified by companies.

However, in most of the commercial mmWave BS beamforming codebook design, hierarchical codebook with wide and narrow beams are used. From beam prediction perspective, with the same number of beams in Set B, wider spatial domain coverage is better for prediction accuracy, especially in fading scenarios, otherwise UE may fail to predict best beams if all least fading angle is out of Set B beam coverage, and the stronger angles from the dataset are in deep fading. Therefore, transmitting wide beams in Set B and leave most of the narrow beams in Set A is a better design for beam prediction than all beams with uniform beam width, i.e., non-hierarchical codebook. Unfortunately, both option 1a and 1b are codebook with uniform beam-width.

Observation 1: Option 1a and 1b in the previous meeting simulation assumption WF are with uniform beam-width, and expected to have worse performance, can’t show prediction model’s best capability, and deviated from reality.

Therefore, we propose the following hierarchical codebook [1]:

Wide beams: (M, N)=(4,2) for BS antenna configuration, 8 beams with grid of 2 elevation angles from [-30 to 30] and 4 azimuth angles from [-60 to 60].

Narrow beams:  (M, N)=(8,4) for BS antenna configuration, 32 beams with grid of 4 elevation angles from [-30 to 30] and 8 azimuth angles from [-60 to 60].

Proposal 1: Use the following hierarchical codebook:

Wide beams: (M, N)=(4,2) for BS antenna configuration, 8 beams with grid of 2 elevation angles from [-30 to 30] and 4 azimuth angles from [-60 to 60].

Narrow beams:  (M, N)=(8,4) for BS antenna configuration, 32 beams with grid of 4 elevation angles from [-30 to 30] and 8 azimuth angles from [-60 to 60].

Conclusion

Observation 1: Option 1a and 1b in the previous meeting simulation assumption WF are with uniform beam-width, and expected to have worse performance, can’t show prediction model’s best capability, and deviated from reality.

Proposal 1: Use the following hierarchical codebook:

Wide beams: (M, N)=(4,2) for BS antenna configuration, 8 beams with grid of 2 elevation angles from [-30 to 30] and 4 azimuth angles from [-60 to 60].

Narrow beams:  (M, N)=(8,4) for BS antenna configuration, 32 beams with grid of 4 elevation angles from [-30 to 30] and 8 azimuth angles from [-60 to 60].

Reference

[1] C.-H. Huang, Y. Kim, and S.-C. Lin, “Universal Beam Prediction for Mismatched Beam Codebooks,” submitted to IEEE ICC 2025.