3GPP TSG-RAN4 Meeting #113	R4-2419138

# Orlando, US, 18 – 22 November, 2024

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.133 | CR | - | rev | - | Current version: | 18.7.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | draftCR on TCs for SL positioning |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_pos_enh2-Perf |  |  |  |  |  | Date: |  |  | 2024-11-05 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | - Some test setup for SL positioning TCs, e.g. resource pool configuration, Noc, Es/Noc, are TBD. - The slot offset for SL-PRS from different TX UEs are not specified in SL positioning TCs. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Change resource pool configuration, Noc, Es/Noc setup from TBD to exact values in some SL positioning TCs. - Define slot offset for SL-PRS from different TX UEs in SL positioning TCs. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | SL positioning TCs are incomplete. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | A.9A.1.1.2, A.9A.1.1.3, A.9A.1.1.4, A.9A.1.2.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | x |  | Test specifications |  |  |  | TS 38.533 |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | The CR is based on revised version of big draftCR R4-2416892 from RAN4 reflector after RAN4#112-bis |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<Start of Change 1>

#### A.9A.1.1.2	SL Rx-Tx measurement delay tests

##### A.9A.1.1.2.1	Test Purpose and Environment

The purpose of the test is to verify that the SL Rx-Tx measurement meets the requirements specified in clause 12A.2 in an environment with AWGN propagation conditions in FR1 in standalone NR scenario, with additionally configured single frequency layer for SL positioning.

This test is applicable for UEs supporting NR Uu and V2X or 5G ProSe operation, which are capable of performing SL Rx-Tx measurements.

The supported NR Uu test configurations in FR1 are shown in table A.9A.1.1.2.1-1.

Table A.9A.1.1.2.1-1: Supported Test Configurations for FR1 NR cell

| Configuration | Description |
| --- | --- |
| 1 | NR Uu: 15 kHz SSB SCS, 20 MHz BW, FDD duplex mode |
| 2 | NR Uu: 15 kHz SSB SCS, 20 MHz BW, TDD duplex mode |
| 3 | NR Uu: 30 kHz SSB SCS, 40 MHz BW, TDD duplex mode |
| Note 1: The UE is only required to pass in one of the supported test configurations in FR1. |  |

The supported NR SL test configurations are specified in table A.9A.1.1.2.1-2.

Table A.9A.1.1.2.1-2: Supported test configurations for NR SL UEs

| NR SL configuration | Description |
| --- | --- |
| SL_conf1 | NR SL: 15 kHz SSB SCS, 10 MHz bandwidth, HD duplex mode |
| SL_conf2 | NR SL: 30 kHz SSB SCS, 10 MHz bandwidth, HD duplex mode |
| SL_conf3 | NR SL: 30 kHz SSB SCS, 20 MHz bandwidth, HD duplex mode |
| NOTE: The UE is only required to be tested in one of the supported test configurations. |  |

There is one NR active cell (Cell 1) and three active UEs (one target UE and two anchor UEs for SL positioning measurement) in this test. The target UE receives SL-PRS and performs the SL Rx-Tx time difference measurement. The two anchor UEs transmit the SL-PRS for the SL Rx-Tx time difference measurement on NR SL RF channel 2. The target UE and all anchor UEs are in RRC_CONNECTED state, with Cell 1 as their PCell in FR1 on NR Uu RF channel 1.

The test consists of two successive time periods, with time duration of T1 and T2 respectively. Before T2 starts, the UEs have been synchronized to the NR serving cell. And during T2, two anchor UEs transmit SL-PRS for positioning measurements.

The SL-RTT-ProvideAssistanceData and SL-RTT-RequestLocationInformation as defined in TS 38.355 [37, clause 6.9], shall be provided to the target UE via Cell 1 during T1. The last TTI containing the two messages shall be provided to the target UE T ms before the start of T2, where T = 50 ms is the maximum processing time of the SL-RTT assistance data and location information request.

The test parameters are given in table A.9A.1.1.2.1-3, A.9A.1.1.2.1-4, A.9A.1.1.2.1-5 and table A.9A.1.1.2.1-6 below.

Table A.9A.1.1.2.1-3: General Test Parameters for SL Rx-Tx measurement reporting delay

| Parameter | Unit | Value | Comment |
| --- | --- | --- | --- |
| Serving cell |  | Cell 1 | NR PCell of the target UE and all anchor UEs (anchor UE 1, anchor UE 2), in FR1 on NR Uu RF channel 1. This cell is also the synchronization source for SL operation for all UEs in the test. |
| CP length |  | Normal |  |
| DRX |  | OFF |  |
| Measurement gap |  | OFF |  |
| Target UE |  | UE 0 | The UE performing SL Rx-Tx measurements based on SL-PRS transmissions from anchor UEs |
| Other anchor UEs |  | UE 1 and UE 2 | Anchor UE 1 and Anchor UE 2 appear at the first and second places in the anchor UE list SL-RTD-Info in the SL-RTT assistance data. |
| Number of anchor UEs provided in SL-TDOA assistance data |  | 3 | Including the target UE |
| Sidelink communication configuration |  | As specified in table A.3.21.2-2 |  |
| Target UE antenna configuration |  | 1 x 2 |  |
| Timing offset between the anchor UEs at the target UE antenna connector | s | UE 2 to UE 1: 0 UE 3 to UE 1: 3 | Synchronous transmissions |
| T1 | s | 3 | The length of the time interval from the beginning of each test |
| T2 | s | 1.28 | The length of the time interval that follows immediately after time interval T1 |

Table A.9A.1.1.2.1-4: NR Uu specific test parameters for Cell 1

| Parameter |  | Unit | Value | Comment |
| --- | --- | --- | --- | --- |
| NR Uu RF channel number |  |  | 1 | RF channel of Cell 1. |
| SSB configuration | Uu_conf1 |  | SSB.1 FR1 | SSB configuration of Cell 1. |
|  | Uu_conf2 |  | SSB.1 FR1 |  |
|  | Uu_conf3 |  | SSB.2 FR1 |  |
| SMTC configuration | Uu_conf1 |  | SMTC.2 | SMTC configuration of Cell 1. |
|  | Uu_conf2 |  | SMTC.1 |  |
|  | Uu_conf3 |  | SMTC.1 |  |
| PDSCH RMC configuration | Uu_conf1 |  | SR.1.1 FDD |  |
|  | Uu_conf2 |  | SR.1.1 TDD |  |
|  | Uu_conf3 |  | SR.2.1 TDD |  |
| RMSI CORESET RMC configuration | Uu_conf1 |  | CR.1.1 FDD | As specified in clause A.3.1.2.1 |
|  | Uu_conf2 |  | CR.1.1 TDD |  |
|  | Uu_conf3 |  | CR.2.1 TDD |  |
| Dedicated CORESET RMC configuration | Uu_conf1 |  | CCR.1.1 FDD |  |
|  | Uu_conf2 |  | CCR.1.1 TDD |  |
|  | Uu_conf3 |  | CCR.2.1 TDD |  |
| Initial BWP configuration | Uu_conf1,2,3 |  | DLBWP.0.1 ULBWP.0.1 |  |
| Active DL BWP configuration | Uu_conf1,2,3 |  | DLBWP.1.1 |  |
| Active UL BWP configuration | Uu_conf1,2,3 |  | ULBWP.1.1 |  |

Table A.9A.1.1.2.1-5: NR Uu UE-specific test parameters for UE 0, UE 1 and UE 2

| Parameter |  | Unit | Value | Comment |
| --- | --- | --- | --- | --- |
| NR Uu RF channel number |  |  | 1 | RF channel of Cell 1. |
| DRX |  |  | OFF |  |
| OCNG Patterns |  |  | OP.1 |  |
| EPRE ratio of PSS to SSS |  | dB | 0 |  |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSS Note 1 |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS Note 1 |  |  |  |  |
| Note2 | Config 1,2,3 | dBm/15 kHz | -110 |  |
|  | Config 1, 2 | dBm /SCS | -110 |  |
|  | Config 3 |  | -107 |  |
|  | dB |  | 4.5 |  |
|  | dB |  | 4.5 |  |
| SS-RSRPNote3 | Config 1,2 | dBm /SCS | -105.5 |  |
|  | Config 3 |  | -102.5 |  |
| IoNote3 | Config 1,2 | dBm /9.36MHz | -76.2 |  |
|  | Config 3 | dBm/ 38.16MHz | -70.1 |  |
| Propagation condition |  |  | AWGN |  |
| NOTE 1: OCNG shall be used such that cell 1 is fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. NOTE 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. NOTE 3: SS-RSRP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. NOTE 4: SS-RSRP minimum requirements are specified assuming independent interference and noise at each receiver antenna port. |  |  |  |  |

Table A.9A.1.1.2.1-6: Anchor V2X UE specific test parameters for SL Rx-Tx measurement

| Parameter |  | Unit | Anchor UE 1 |  | Anchor UE 2 |  |  | Comment |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | T1 | T2 | T1 | T2 |  |  |
| SL RF Channel number |  |  | 2 |  | 2 |  |  |  |
| SL DRX |  |  | OFF |  | OFF |  |  |  |
| networkControlledSyncTx |  |  | ON |  | ON |  |  |  |
| inCoverage (in MIB-SL) |  |  | TRUE |  | TRUE |  |  |  |
| SL pool configuration | SL_conf1 |  | shared resource pool if supported by UE1, otherwise dedicated |  | shared resource pool if supported by UE1, otherwise dedicated |  |  |  |
|  | SL_conf2 |  |  |  |  |  |  |  |
|  | SL_conf3 |  |  |  |  |  |  |  |
| SL-PRS configuration | SL_conf1 |  | N/A | SL PRS.1.3 FR1 | N/A |  | SL PRS.1.3 FR1 | As specified in table A.3.21A.2.1-1 |
|  | SL_conf2 |  |  |  |  |  |  |  |
|  | SL_conf3 |  |  |  |  |  |  |  |
| SL-PRS slot offset |  |  | N/A | 0 | N/A |  | KNote 4 |  |
| PSCCH RMC (defined in A.3.21.3) |  |  | CC.1A HD | CC.1A HD | CC.1A HD |  | CC.1A HD |  |
| PSSCH RMC (defined in A.3.21.3) |  |  | CD.1A HD | CD.1A HD | CD.1A HD |  | CD.1A HD |  |
| in symbols containing SL-PRS Note 2 |  | dBm/SCS | -94 |  |  |  |  |  |
| in symbols containing PSCCH Note 2 |  | dBm/SCS | -98 |  |  |  |  |  |
| SL-PRS |  | dB | -Infinity | -3 | -Infinity |  | -3 |  |
| PSCCH |  | dB | N/A | 1 | N/A |  | 1 |  |
| Io in symbols containing SL-PRS Note 3 | SL_conf1 | dBm/BW | N/A | -64.63 | N/A |  | -64.63 |  |
|  | SL_conf2 |  |  |  |  |  |  |  |
|  | SL_conf3 |  |  |  |  |  |  |  |
| SL PRS-RSRP Note3 |  | dBm/SCS | -Infinity | -97 | -Infinity |  | -97 |  |
| Propagation Condition |  |  | AWGN |  |  |  |  |  |
| NOTE 1: Interference from other UEs and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. NOTE 2: SL PRS-RSRP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Io level is based on the allocated PRBs for SL PRS symbols. NOTE 3: The UE is only required to be tested in one of the supported test configurations. NOTE 4: If UE supports maxNumOfSlotsWIthActiveSL-PRS-Resources-r18 >1, X equals to 1; otherwise, X equals to the number of slots for the time UE indicates in minTimeAfterEndofSlotCarryActiveSL-PRS-Resources-r18. |  |  |  |  |  |  |  |  |

##### A.9A.1.1.2.2	Test Requirements

The SL Rx-Tx time difference measurement time fulfils the requirements specified in clause 12A.4.5.

The UE shall perform and report the SL Rx-Tx time difference measurements for anchor UE 1 and anchor UE 2 within the specified SL Rx-Tx time difference measurement time starting from the beginning of time interval T2.

NOTE:	The actual overall delays measured in the test may be up to 2xTTIDCCH higher than the time duration above because of TTI insertion uncertainty of the measurement report in DCCH.

The rate of the correct events for each anchor UE observed during repeated tests shall be at least 90%, where the reported SL Rx-Tx measurement for each correct event shall be within the SL Rx-Tx reporting range specified in clause 10.4A.4.1.

#### A.9A.1.1.3	NR SL AoA measurements reporting delay test in FR1 SA

##### A.9A.1.1.3.1	Test Purpose and Environment

The purpose of the test is to verify that the SL AoA measurement meets the requirements specified in clause 12A.6 in an environment with AWGN propagation conditions in FR1 in NR Uu standalone scenario, when a single frequency layer is configured for SL positioning.

The test is applicable for UEs supporting NR Uu and V2X or 5G ProSe operation, which are capable of performing SL AoA measurements.

The supported NR Uu test configurations are specified in table A.9A.1.1.3.1-1.

The supported NR SL test configurations are specified in table A.9A.1.1.3.1-2.

Table A.9A.1.1.3.1-1: Supported Test Configurations for FR1 NR cell

| Configuration | Description |
| --- | --- |
| 1 | NR Uu: 15 kHz SSB SCS, 20 MHz BW, FDD duplex mode |
| 2 | NR Uu: 15 kHz SSB SCS, 20 MHz BW, TDD duplex mode |
| 3 | NR Uu: 30 kHz SSB SCS, 40 MHz BW, TDD duplex mode |
| NOTE 1: The UE is only required to pass in one of the supported test configurations in FR1. |  |

Table A.9A.1.1.3.1-2: Supported test configurations for NR SL UEs

| NR SL configuration | Description |
| --- | --- |
| SL_conf1 | NR SL: 15 kHz SSB SCS, 10 MHz bandwidth, HD duplex mode |
| SL_conf2 | NR SL: 30 kHz SSB SCS, 10 MHz bandwidth, HD duplex mode |
| SL_conf3 | NR SL: 30 kHz SSB SCS, 20 MHz bandwidth, HD duplex mode |
| NOTE: The UE is only required to be tested in one of the supported test configurations. |  |

In the test there is one target UE receiving SL-PRS and performing SL AoA measurements and two anchor UEs (anchor UE 1, anchor UE 2) transmitting SL-PRS for the SL AoA measurements. The target UE and all the anchor UEs are in RRC_CONNECTED state, with Cell 1 as their PCell in FR1. Cell 1 is also the synchronization source of the target UE and all anchor UEs in the test.

The test consists of two consecutive time intervals, with the duration of T1 and T2. During the duration T1, the target UE shall not have any timing information of anchor UE 1 and anchor UE 2. All two anchor UEs transmit SL-PRS during T2.

The SL-AOA-ProvideAssistanceData and SL-AOA-RequestLocationInformation as defined in TS 38.355 [37], shall be provided to the target UE via Cell 1 during T1. The last TTI containing the two messages shall be provided to the target UE T ms before the start of T2, where T = 50 ms is the maximum processing time of the SL-AOA assistance data and location information request.

The general test parameters are listed in table A.9A.1.1.3.1-3. NR Uu specific test parameters for Cell 1 and NR Uu UE-specific test parameters for all UEs in the test are listed in table A.9A.1.1.3.1-4 and A.9A.1.1.3.1-5, respectively. Anchor UE specific test parameters for SL AoA measurement reporting delay during T1 and T2 are listed in table A.9A.1.1.3.1-6.

Table A.9A.1.1.3.1-3: General test parameters for SL AoA measurement reporting delay

| Parameter | Unit | Value | Comment |
| --- | --- | --- | --- |
| Serving cell |  | Cell 1 | NR PCell of the target UE and all anchor UEs (anchor UE 1, anchor UE 2), in FR1 on NR Uu RF channel 1. This cell is also the synchronization source for SL operation for all UEs in the test. |
| CP length |  | Normal |  |
| DRX |  | OFF |  |
| Measurement gap |  | OFF |  |
| Target UE |  | UE 0 | The UE performing SL AoA measurements based on SL-PRS transmissions from anchor UEs |
| Other anchor UEs |  | UE 1 and UE 2 | Anchor UE 1 and Anchor UE 2 appear at the first and second places in the anchor UE list in the SL-AOA assistance data. |
| Number of anchor UEs provided in SL-AOA assistance data |  | 3 | Including the target UE |
| Sidelink communication configuration |  | As specified in table A.3.21.2-2 |  |
| Target UE antenna configuration |  | 1 x 2 |  |
| Timing offset between the anchor UEs at the target UE antenna connector | s | UE 1 to UE 0: 0 UE 2 to UE 1: 3 | Synchronous transmissions |
| T1 | s | 3 | The length of the time interval from the beginning of each test |
| T2 | s | 1.28 | The length of the time interval that follows immediately after time interval T1 |

Table A.9A.1.1.3.1-4: NR Uu specific test parameters for Cell 1

| Parameter |  | Unit | Value | Comment |
| --- | --- | --- | --- | --- |
| NR Uu RF channel number |  |  | 1 | RF channel of Cell 1. |
| SSB configuration | Uu_conf1 |  | SSB.1 FR1 | SSB configuration of Cell 1. |
|  | Uu_conf2 |  | SSB.1 FR1 |  |
|  | Uu_conf3 |  | SSB.2 FR1 |  |
| SMTC configuration | Uu_conf1 |  | SMTC.2 | SMTC configuration of Cell 1. |
|  | Uu_conf2 |  | SMTC.1 |  |
|  | Uu_conf3 |  | SMTC.1 |  |
| PDSCH RMC configuration | Uu_conf1 |  | SR.1.1 FDD |  |
|  | Uu_conf2 |  | SR.1.1 TDD |  |
|  | Uu_conf3 |  | SR.2.1 TDD |  |
| RMSI CORESET RMC configuration | Uu_conf1 |  | CR.1.1 FDD | As specified in clause A.3.1.2.1 |
|  | Uu_conf2 |  | CR.1.1 TDD |  |
|  | Uu_conf3 |  | CR.2.1 TDD |  |
| Dedicated CORESET RMC configuration | Uu_conf1 |  | CCR.1.1 FDD |  |
|  | Uu_conf2 |  | CCR.1.1 TDD |  |
|  | Uu_conf3 |  | CCR.2.1 TDD |  |
| Initial BWP configuration | Uu_conf1,2,3 |  | DLBWP.0.1 ULBWP.0.1 |  |
| Active DL BWP configuration | Uu_conf1,2,3 |  | DLBWP.1.1 |  |
| Active UL BWP configuration | Uu_conf1,2,3 |  | ULBWP.1.1 |  |

Table A.9A.1.1.3.1-5: NR Uu UE-specific test parameters for UE 0, UE 1, and UE 2

| Parameter |  | Unit | Value | Comment |
| --- | --- | --- | --- | --- |
| NR Uu RF channel number |  |  | 1 | RF channel of Cell 1. |
| DRX |  |  | OFF |  |
| OCNG Patterns |  |  | OP.1 |  |
| EPRE ratio of PSS to SSS |  | dB | 0 |  |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSS Note 1 |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS Note 1 |  |  |  |  |
| Note2 | Config 1,2,3 | dBm/15 kHz | -110 |  |
|  | Config 1, 2 | dBm /SCS | -110 |  |
|  | Config 3 |  | -107 |  |
|  | dB |  | 4.5 |  |
|  | dB |  | 4.5 |  |
| SS-RSRPNote3 | Config 1,2 | dBm /SCS | -105.5 |  |
|  | Config 3 |  | -102.5 |  |
| IoNote3 | Config 1,2 | dBm /9.36MHz | -76.2 |  |
|  | Config 3 | dBm/ 38.16MHz | -70.1 |  |
| Propagation condition |  |  | AWGN |  |
| NOTE 1: OCNG shall be used such that cell 1 is fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. NOTE 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. NOTE 3: SS-RSRP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. NOTE 4: SS-RSRP minimum requirements are specified assuming independent interference and noise at each receiver antenna port. |  |  |  |  |

Table A.9A.1.1.3.1-6: Anchor UE specific test parameters on the SL carrier

| Parameter |  | Unit | Anchor UE 1 |  | Anchor UE 2 |  |  | Comment |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | T1 | T2 | T1 | T2 |  |  |
| SL RF Channel number |  |  | 2 |  | 2 |  |  |  |
| SL DRX |  |  | OFF |  | OFF |  |  |  |
| networkControlledSyncTx |  |  | ON |  | ON |  |  |  |
| inCoverage (in MIB-SL) |  |  | TRUE |  | TRUE |  |  |  |
| SL pool configuration | SL_conf1 |  | shared resource pool if supported by UE1, otherwise dedicated |  | shared resource pool if supported by UE1, otherwise dedicated |  |  |  |
|  | SL_conf2 |  |  |  |  |  |  |  |
|  | SL_conf3 |  |  |  |  |  |  |  |
| SL-PRS configuration | SL_conf1 |  | N/A | SL PRS.1.3 FR1 | N/A |  | SL PRS.1.3 FR1 | As specified in table A.3.21A.2.1-1 |
|  | SL_conf2 |  |  |  |  |  |  |  |
|  | SL_conf3 |  |  |  |  |  |  |  |
| SL-PRS slot offset |  |  | N/A | 0 | N/A |  | KNote 4 |  |
| PSCCH RMC (defined in A.3.21.3) |  |  | CC.1A HD | CC.1A HD | CC.1A HD |  | CC.1A HD |  |
| PSSCH RMC (defined in A.3.21.3) |  |  | CD.1A HD | CD.1A HD | CD.1A HD |  | CD.1A HD |  |
| in symbols containing SL-PRS Note 2 |  | dBm/SCS | -94 |  |  |  |  |  |
| in symbols containing PSCCH Note 2 |  | dBm/SCS | -98 |  |  |  |  |  |
| SL-PRS |  | dB | -Infinity | -3 | -Infinity |  | -3 |  |
| PSCCH |  | dB | N/A | 1 | N/A |  | 1 |  |
| Io in symbols containing SL-PRS Note 3 | SL_conf1 | dBm/BW | N/A | -64.63 | N/A |  | -64.63 |  |
|  | SL_conf2 |  |  |  |  |  |  |  |
|  | SL_conf3 |  |  |  |  |  |  |  |
| SL PRS-RSRP Note3 |  | dBm/SCS | -Infinity | -97 | -Infinity |  | -97 |  |
| Propagation Condition |  |  | AWGN |  |  |  |  |  |
| NOTE 1: Interference from other UEs and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. NOTE 2: SL PRS-RSRP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Io level is based on the allocated PRBs for SL PRS symbols. NOTE 3: The UE is only required to be tested in one of the supported test configurations. NOTE 4: If UE supports maxNumOfSlotsWIthActiveSL-PRS-Resources-r18 >1, X equals to 1; otherwise, X equals to the number of slots for the time UE indicates in minTimeAfterEndofSlotCarryActiveSL-PRS-Resources-r18. |  |  |  |  |  |  |  |  |

##### A.9A.1.1.3.2	Test Requirements

The SL AoA measurement time fulfils the requirements specified in clause 12A.2.6.

The UE shall perform and report to LMF the SL AoA measurements for the anchor UE 1 and anchor UE 2, within the time duration specified in clause 12A.6 starting from the beginning of time interval T2.

NOTE:	The actual overall delays measured in the test may be up to 2xTTIDCCH higher than the time duration above because of TTI insertion uncertainty of the measurement report in DCCH.

The rate of the correct events for each anchor UE observed during repeated tests shall be at least 90%, where the reported SL AoA measurement for each correct event shall be within the SL AoA reporting range specified in clause 10.4A.6.1.1, i.e., between A_AoA_0 and SL_AoA_3599, and between Z_AoA_0 and Z_AoA_1799.

#### A.9A.1.1.4	NR SL RTOA measurements reporting delay test in FR1 SA

##### A.9A.1.1.4.1	Test Purpose and Environment

The purpose of the test is to verify that the SL RTOA measurement meets the requirements specified in clause 12A.7 in an environment with AWGN propagation conditions in FR1 in NR Uu standalone scenario, when a single frequency layer is configured for SL positioning.

The test is applicable for UEs supporting NR Uu and V2X or 5G ProSe operation, which are capable of performing SL AoA measurements.

The supported NR Uu test configurations are specified in table A.9A.1.1.4.1-1.

The supported NR SL test configurations are specified in table A.9A.1.1.4.1-2.

Table A.9A.1.1.4.1-1: Supported test configurations for FR1 NR Cell 1

| NR Uu configuration | Description |
| --- | --- |
| Uu_conf1 | NR Uu: 15 kHz SSB SCS, 20 MHz bandwidth, FDD duplex mode |
| Uu_conf2 | NR Uu: 15 kHz SSB SCS, 20 MHz bandwidth, TDD duplex mode |
| Uu_conf3 | NR Uu: 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode |
| NOTE: The UE is only required to be tested in one of the supported test configurations. |  |

Table A.9A.1.1.4.1-2: Supported test configurations for NR SL UEs

| NR SL configuration | Description |
| --- | --- |
| SL_conf1 | NR SL: 15 kHz SSB SCS, 10 MHz bandwidth, HD duplex mode |
| SL_conf2 | NR SL: 30 kHz SSB SCS, 10 MHz bandwidth, HD duplex mode |
| SL_conf3 | NR SL: 30 kHz SSB SCS, 20 MHz bandwidth, HD duplex mode |
| NOTE: The UE is only required to be tested in one of the supported test configurations. |  |

In the test there is one target UE transmitting SL-PRS and performing SL RTOA measurements and one anchor UE (anchor UE 1) receiving SL-PRS for the SL RTOA measurements. The target UE and all the anchor UEs are in RRC_CONNECTED state, with Cell 1 as their PCell in FR1. Cell 1 is also the synchronization source of the target UE and all anchor UEs in the test.

The test consists of two consecutive time intervals, with the duration of T1 and T2. During the duration T1, the target UE shall not have any timing information of anchor UE 1 and anchor UE 2. All two anchor UEs transmit SL-PRS during T2.

The SL-TOA-ProvideAssistanceData and SL-TOA-RequestLocationInformation as defined in TS 38.355 [37], shall be provided to the target UE via Cell 1 during T1. The last TTI containing the two messages shall be provided to the target UE T ms before the start of T2, where T = 50 ms is the maximum processing time of the SL-TOA assistance data and location information request.

The general test parameters are listed in table A.9A.1.1.4.1-3. NR Uu specific test parameters for Cell 1 and NR Uu UE-specific test parameters for all UEs in the test are listed in table A.9A.1.1.4.1-4 and A.9A.1.1.4.1-5, respectively. Anchor UE specific test parameters for SL RTOA measurement reporting delay during T1 and T2 are listed in table A.9A.1.1.4.1-6.

Table A.9A.1.1.4.1-3: General test parameters for SL RTOA measurement reporting delay

| Parameter | Unit | Value | Comment |
| --- | --- | --- | --- |
| Serving cell |  | Cell 1 | NR PCell of the target UE and all anchor UEs (anchor UE 1), in FR1 on NR Uu RF channel 1. This cell is also the synchronization source for SL operation for all UEs in the test. |
| CP length |  | Normal |  |
| DRX |  | OFF |  |
| Measurement gap |  | OFF |  |
| Target UE |  | UE 0 | The UE performing SL RTOA measurements based on SL-PRS receiving from anchor UEs |
| Other anchor UEs |  | UE 1 | Anchor UE 1 appear at the first place in the anchor UE list SL-TOA assistance data. |
| Number of anchor UEs provided in SL-TOA assistance data |  | 2 | Including the target UE |
| Sidelink communication configuration |  | As specified in table A.3.21.2-2 |  |
| Target UE antenna configuration |  | 1 x 2 |  |
| Timing offset between the anchor UEs at the target UE antenna connector | s | Anchor UE 1 to target UE 1: 0 | Synchronous transmissions |
| T1 | s | 3 | The length of the time interval from the beginning of each test |
| T2 | s | 1.28 | The length of the time interval that follows immediately after time interval T1 |

Table A.9A.1.1.4.1-4: NR Uu specific test parameters for Cell 1

| Parameter |  | Unit | Value | Comment |
| --- | --- | --- | --- | --- |
| NR Uu RF channel number |  |  | 1 | RF channel of Cell 1. |
| SSB configuration | Uu_conf1 |  | SSB.1 FR1 | SSB configuration of Cell 1. |
|  | Uu_conf2 |  | SSB.1 FR1 |  |
|  | Uu_conf3 |  | SSB.2 FR1 |  |
| SMTC configuration | Uu_conf1 |  | SMTC.2 | SMTC configuration of Cell 1. |
|  | Uu_conf2 |  | SMTC.1 |  |
|  | Uu_conf3 |  | SMTC.1 |  |
| PDSCH RMC configuration | Uu_conf1 |  | SR.1.1 FDD |  |
|  | Uu_conf2 |  | SR.1.1 TDD |  |
|  | Uu_conf3 |  | SR.2.1 TDD |  |
| RMSI CORESET RMC configuration | Uu_conf1 |  | CR.1.1 FDD | As specified in clause A.3.1.2.1 |
|  | Uu_conf2 |  | CR.1.1 TDD |  |
|  | Uu_conf3 |  | CR.2.1 TDD |  |
| Dedicated CORESET RMC configuration | Uu_conf1 |  | CCR.1.1 FDD |  |
|  | Uu_conf2 |  | CCR.1.1 TDD |  |
|  | Uu_conf3 |  | CCR.2.1 TDD |  |
| Initial BWP configuration | Uu_conf1,2,3 |  | DLBWP.0.1 ULBWP.0.1 |  |
| Active DL BWP configuration | Uu_conf1,2,3 |  | DLBWP.1.1 |  |
| Active UL BWP configuration | Uu_conf1,2,3 |  | ULBWP.1.1 |  |

Table A.9A.1.1.4.1-5: NR Uu UE-specific test parameters for UE 0 and UE 1

| Parameter |  | Unit | Value | Comment |
| --- | --- | --- | --- | --- |
| NR Uu RF channel number |  |  | 1 | RF channel of Cell 1. |
| DRX |  |  | OFF |  |
| OCNG Patterns |  |  | OP.1 |  |
| EPRE ratio of PSS to SSS |  | dB | 0 |  |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSS Note 1 |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS Note 1 |  |  |  |  |
| Note2 | Config 1,2,3 | dBm/15 kHz | -110 |  |
|  | Config 1, 2 | dBm /SCS | -110 |  |
|  | Config 3 |  | -107 |  |
|  | dB |  | 4.5 |  |
|  | dB |  | 4.5 |  |
| SS-RSRPNote3 | Config 1,2 | dBm /SCS | -105.5 |  |
|  | Config 3 |  | -102.5 |  |
| IoNote3 | Config 1,2 | dBm /9.36MHz | -76.2 |  |
|  | Config 3 | dBm/ 38.16MHz | -70.1 |  |
| Propagation condition |  |  | AWGN |  |
| NOTE 1: OCNG shall be used such that cell 1 is fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. NOTE 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. NOTE 3: SS-RSRP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. NOTE 4: SS-RSRP minimum requirements are specified assuming independent interference and noise at each receiver antenna port. |  |  |  |  |

Table A.9A.1.1.4.1-6: Anchor UE specific test parameters on the SL carrier

| Parameter |  | Unit | Anchor UE 1 |  |
| --- | --- | --- | --- | --- |
|  |  |  | T1 | T2 |
| SL RF Channel number |  |  | 2 |  |
| SL DRX |  |  | OFF |  |
| networkControlledSyncTx |  |  | ON |  |
| inCoverage (in MIB-SL) |  |  | TRUE |  |
| SL pool configuration | SL_conf1 |  | shared resource pool if supported by UE1, otherwise dedicated |  |
|  | SL_conf2 |  |  |  |
|  | SL_conf3 |  |  |  |
| SL-PRS configuration | SL_conf1 |  | N/A | SL PRS.1.3 FR1 |
|  | SL_conf2 |  |  |  |
|  | SL_conf3 |  |  |  |
| SL-PRS slot offset |  |  | N/A | 0 |
| PSCCH RMC (defined in A.3.21.3) |  |  | CC.1A HD | CC.1A HD |
| PSSCH RMC (defined in A.3.21.3) |  |  | CD.1A HD | CD.1A HD |
| in symbols containing SL-PRS Note 2 |  | dBm/SCS | -94 |  |
| in symbols containing PSCCH Note 2 |  | dBm/SCS | -98 |  |
| SL-PRS |  | dB | -Infinity | -3 |
| PSCCH |  | dB | N/A | 1 |
| Io in symbols containing SL-PRS Note 3 | SL_conf1 | dBm/BW | N/A | -64.63 |
|  | SL_conf2 |  |  |  |
|  | SL_conf3 |  |  |  |
| SL PRS-RSRP Note3 |  | dBm/SCS | -Infinity | -97 |
| Propagation Condition |  |  | AWGN |  |
| NOTE 1: The resources for NR Uu uplink transmission are assigned to the UE prior to the start of time period T2. NOTE 2: Interference from other UEs and noise sources not specified in the test are assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. NOTE 3: SL PRS-RSRP and Io levels have been derived from other parameters and are given for information purpose. These are not settable test parameters. |  |  |  |  |

##### A.9A.1.1.4.2	Test Requirements

The SL RTOA measurement time fulfils the requirements specified in clause 12A.2.7.

The UE shall perform and report to LMF the SL RTOA measurements for the anchor UE 1, within the time duration specified in clause 12A.7 starting from the beginning of time interval T2.

NOTE:	The actual overall delays measured in the test may be up to 2xTTIDCCH higher than the time duration above because of TTI insertion uncertainty of the measurement report in DCCH.

The rate of the correct events for each anchor UE observed during repeated tests shall be at least 90%, where the reported SL RTOA measurement for each correct event shall be within the SL RTOA reporting range specified in clause 10.4A.7.1.1, i.e., between SL_RTOA_0 and SL_RTOA_985024.

<End of Change 1>

<Start of Change 2>

#### A.9A.1.2.2	SL Rx-Tx measurement accuracy test case in FR1

##### A.9A.1.2.2.1	Test Purpose and Environment

The purpose of the test is to verify that the SL Rx-Tx measurement meets the accuracy requirements specified in clause 10.4A.4 in an environment with AWGN propagation conditions in FR1 in standalone NR scenario, with additionally configured single frequency layer for SL positioning.

This test is applicable for UEs supporting NR Uu and V2X or 5G ProSe operation, which are capable of performing SL Rx-Tx measurements.

The supported NR Uu test configurations in FR1 are shown in table A.9A.1.2.2.1-1.

Table A.9A.1.2.2.1-1: Supported Test Configurations for FR1 NR cell

| Configuration | Description |
| --- | --- |
| 1 | NR Uu: 15 kHz SSB SCS, 20 MHz BW, FDD duplex mode |
| 2 | NR Uu: 15 kHz SSB SCS, 20 MHz BW, TDD duplex mode |
| 3 | NR Uu: 30 kHz SSB SCS, 40 MHz BW, TDD duplex mode |
| NOTE 1: The UE is only required to pass in one of the supported test configurations in FR1. |  |

The supported NR SL test configurations are specified in table A.9A.1.2.2.1-2.

Table A.9A.1.2.2.1-2: Supported test configurations for NR SL UEs

| NR SL configuration | Description |
| --- | --- |
| SL_conf1 | NR SL: 15 kHz SSB SCS, 10 MHz bandwidth, HD duplex mode |
| SL_conf2 | NR SL: 30 kHz SSB SCS, 10 MHz bandwidth, HD duplex mode |
| SL_conf3 | NR SL: 30 kHz SSB SCS, 20 MHz bandwidth, HD duplex mode |
| NOTE: The UE is only required to be tested in one of the supported test configurations. |  |

There is one NR active cell (Cell 1) and three active UEs (one target UE and two anchor UEs for SL positioning measurement) in this test. The target UE receives SL-PRS and performs the SL Rx-Tx time difference measurement. The two anchor UEs transmit the SL-PRS for the SL Rx-Tx time difference measurement on NR SL RF channel 2. The target UE and all anchor UEs are in RRC_CONNECTED state, with Cell 1 as their PCell in FR1 on NR Uu RF channel 1.

The test consists of two successive time periods, with time duration of T1 and T2 respectively. Before T2 starts, the UEs have been synchronized to the NR serving cell. And during T2, two anchor UEs transmit SL-PRS for positioning measurements.

The SL-RTT-ProvideAssistanceData and SL-RTT-RequestLocationInformation as defined in TS 38.355 [37, clause 6.9], shall be provided to the target UE via Cell 1 during T1. The last TTI containing the two messages shall be provided to the target UE T ms before the start of T2, where T = 50 ms is the maximum processing time of the SL-RTT assistance data and location information request.

The test parameters are given in table A.9A.1.2.2.1-3, A.9A.1.2.2.1-4, A.9A.1.2.2.1-5 and table A.9A.1.2.2.1-6 below.

Table A.9A.1.2.2.1-3: General Test Parameters for SL Rx-Tx measurement reporting delay

| Parameter | Unit | Value | Comment |
| --- | --- | --- | --- |
| Serving cell |  | Cell 1 | NR PCell of the target UE and all anchor UEs (anchor UE 1, anchor UE 2), in FR1 on NR Uu RF channel 1. This cell is also the synchronization source for SL operation for all UEs in the test. |
| CP length |  | Normal |  |
| DRX |  | OFF |  |
| Measurement gap |  | OFF |  |
| Target UE |  | UE 0 | The UE performing SL Rx-Tx measurements based on SL-PRS transmissions from anchor UEs |
| Other anchor UEs |  | UE 1 and UE 2 | Anchor UE 1 and Anchor UE 2 appear at the first and second places in the anchor UE list SL-RTD-Info in the SL-RTT assistance data. |
| Number of anchor UEs provided in SL-RTT assistance data |  | 3 | Including the target UE |
| Sidelink communication configuration |  | As specified in table A.3.21.2-2 |  |
| Target UE antenna configuration |  | 1 x 2 |  |
| Timing offset between the anchor UEs at the target UE antenna connector | s | UE 2 to UE 1: 3 | Synchronous transmissions |
| T1 | s | 3 | The length of the time interval from the beginning of each test |
| T2 | s | 1.28 | The length of the time interval that follows immediately after time interval T1 |

Table A.9A.1.2.2.1-4: NR Uu specific test parameters for Cell 1

| Parameter |  | Unit | Value | Comment |
| --- | --- | --- | --- | --- |
| NR Uu RF channel number |  |  | 1 | RF channel of Cell 1. |
| SSB configuration | Uu_conf1 |  | SSB.1 FR1 | SSB configuration of Cell 1. |
|  | Uu_conf2 |  | SSB.1 FR1 |  |
|  | Uu_conf3 |  | SSB.2 FR1 |  |
| SMTC configuration | Uu_conf1 |  | SMTC.2 | SMTC configuration of Cell 1. |
|  | Uu_conf2 |  | SMTC.1 |  |
|  | Uu_conf3 |  | SMTC.1 |  |
| PDSCH RMC configuration | Uu_conf1 |  | SR.1.1 FDD |  |
|  | Uu_conf2 |  | SR.1.1 TDD |  |
|  | Uu_conf3 |  | SR.2.1 TDD |  |
| RMSI CORESET RMC configuration | Uu_conf1 |  | CR.1.1 FDD | As specified in clause A.3.1.2.1 |
|  | Uu_conf2 |  | CR.1.1 TDD |  |
|  | Uu_conf3 |  | CR.2.1 TDD |  |
| Dedicated CORESET RMC configuration | Uu_conf1 |  | CCR.1.1 FDD |  |
|  | Uu_conf2 |  | CCR.1.1 TDD |  |
|  | Uu_conf3 |  | CCR.2.1 TDD |  |
| Initial BWP configuration | Uu_conf1,2,3 |  | DLBWP.0.1 ULBWP.0.1 |  |
| Active DL BWP configuration | Uu_conf1,2,3 |  | DLBWP.1.1 |  |
| Active UL BWP configuration | Uu_conf1,2,3 |  | ULBWP.1.1 |  |

Table A.9A.1.2.2.1-5: NR Uu UE-specific test parameters for UE 0, UE 1 and UE 2

| Parameter |  | Unit | Value | Comment |
| --- | --- | --- | --- | --- |
| NR Uu RF channel number |  |  | 1 | RF channel of Cell 1. |
| DRX |  |  | OFF |  |
| OCNG Patterns |  |  | OP.1 |  |
| EPRE ratio of PSS to SSS |  | dB | 0 |  |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSS Note 1 |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS Note 1 |  |  |  |  |
| Note2 | Config 1,2,3 | dBm/15 kHz | -110 |  |
|  | Config 1, 2 | dBm /SCS | -110 |  |
|  | Config 3 |  | -107 |  |
|  | dB |  | 4.5 |  |
|  | dB |  | 4.5 |  |
| SS-RSRPNote3 | Config 1,2 | dBm /SCS | -105.5 |  |
|  | Config 3 |  | -102.5 |  |
| IoNote3 | Config 1,2 | dBm /9.36MHz | -76.2 |  |
|  | Config 3 | dBm/ 38.16MHz | -70.1 |  |
| Propagation condition |  |  | AWGN |  |
| NOTE 1: OCNG shall be used such that cell 1 is fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. NOTE 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. NOTE 3: SS-RSRP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. NOTE 4: SS-RSRP minimum requirements are specified assuming independent interference and noise at each receiver antenna port. |  |  |  |  |

Table A.9A.1.2.2.1-6: Anchor V2X UE specific test parameters for SL Rx-Tx measurement

| Parameter |  | Unit | Anchor UE 1 |  | Anchor UE 2 |  |  | Comment |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | T1 | T2 | T1 | T2 |  |  |
| SL RF Channel number |  |  | 2 |  | 2 |  |  |  |
| SL DRX |  |  | OFF |  | OFF |  |  |  |
| networkControlledSyncTx |  |  | ON |  | ON |  |  |  |
| inCoverage (in MIB-SL) |  |  | TRUE |  | TRUE |  |  |  |
| SL pool configuration | SL_conf1 |  | dedicated resource pool if supported by UE1, otherwise dedicated |  | dedicated resource pool if supported by UE1, otherwise dedicated |  |  |  |
|  | SL_conf2 |  |  |  |  |  |  |  |
|  | SL_conf3 |  |  |  |  |  |  |  |
| SL-PRS configuration | SL_conf1 |  | N/A | SL PRS.1.3 FR1 | N/A |  | SL PRS.1.4 FR1 | As specified in table A.3.21A.2.1-1 |
|  | SL_conf2 |  |  | SL PRS.1.3 FR1 |  |  | SL PRS.1.4 FR1 |  |
|  | SL_conf3 |  |  | SL PRS.1.3 FR1 |  |  | SL PRS.1.4 FR1 |  |
| SL-PRS slot offset |  |  | N/A | 0 | N/A |  | KNote 4 |  |
| PSCCH RMC (defined in A3.21.3) |  |  | CC.1A HD | CC.1A HD | CC.1A HD |  | CC.1A HD |  |
| PSSCH RMC (defined in A.3.21.3) |  |  | CD.1A HD | CD.1A HD | CD.1A HD |  | CD.1A HD |  |
| in symbols containing SL-PRS Note 2 |  | dBm/SCS | -94 |  |  |  |  |  |
| in symbols containing PSCCH Note 2 |  | dBm/SCS | -98 |  |  |  |  |  |
| SL-PRS |  | dB | -Infinity | -3 | -Infinity |  | -3 |  |
| PSCCH |  | dB | N/A | 1 | N/A |  | 1 |  |
| Io in symbols containing SL-PRS Note 3 | SL_conf1 | dBm/BW | N/A | -64.63 | N/A |  | -64.63 |  |
|  | SL_conf2 |  |  |  |  |  |  |  |
|  | SL_conf3 |  |  |  |  |  |  |  |
| SL PRS-RSRP Note3 |  | dBm/SCS | -Infinity | -97 | -Infinity |  | -97 |  |
| SL-PRS EsIot |  | dB | N/A | -3 | N/A |  | -3 |  |
| Propagation Condition |  |  | AWGN |  |  |  |  |  |
| NOTE 1: Interference from other UEs and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. NOTE 2: SL PRS-RSRP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Io level is based on the allocated PRBs for SL PRS symbols. NOTE 3: The UE is only required to be tested in one of the supported test configurations. NOTE 4: If UE supports maxNumOfSlotsWIthActiveSL-PRS-Resources-r18 >1, X equals to 1; otherwise, X equals to the number of slots for the time UE indicates in minTimeAfterEndofSlotCarryActiveSL-PRS-Resources-r18. |  |  |  |  |  |  |  |  |

##### A.9A.1.2.2.2	Test Requirements

The SL Rx-Tx time difference measurement fulfils the UE Rx-Tx measurement accuracy requirements specified in clause 10.4A.4.2 for both anchor UE 1 and anchor UE 2.

<End of Change 2>