3GPP TSG-RAN4 Meeting #113	R4-2419152

# Orlando, US, 18th – 22nd November 2024

Source:	LG Electronics

Title:	Tx requirements for SL intra-band non-contiguous CA

Agenda item:	7.3.2

Document for:	Discussion

# Introduction

In this paper, we provide our view on MPR requirements based on the Tdoc submitted in last meeting [2] and on LO and image rejection levels, which fall inside the gap region between the two carriers and impact the related ACLR requirement.

# Discussion

## MPR

Based on the Tdoc submitted in the last meeting [2], we propose MPR requirement as follows. The table number is referred in [2].

### < PC2 & PC3 PSSCH MPR >

#### Proposal 1: Consider MPR evaluation scenario in Table 2-3 [2].

#### Proposal 2: Consider PC2 MPR in Table 2-10 for SL non-contiguous CA with 1x26dBm PA + 1LO.

Table 2-10 : PSSCH/PSCCH MPR for SL non-contiguous CA with 1x26dBm PA + 1LO

| B | MPR (dB) for IM3 frequency |  |
| --- | --- | --- |
|  | SEMfreq_-13 | SEfreq_-30 |
| 0 ≤ B < 5.04 | ≤ 11.0 | ≤ 18.5 |
| 5.04 ≤ B < 10.08 | ≤ 10.0 | ≤ 17.0 |
| 10.08 ≤ B |  | ≤ 16.0 |

#### Proposal 3: Consider PC2 MPR in Table 2-11 for SL non-contiguous CA with 2x23dBm PA + 1LO.

Table 2-11 : PSSCH/PSCCH MPR for SL non-contiguous CA with 2x23dBm PA + 1LO

| B | MPR (dB) for IM3 frequency |  |
| --- | --- | --- |
|  | SEMfreq_-13 | SEfreq_-30 |
| 0 ≤ B < 5.04 | ≤ 11.5 | ≤ 19.0 |
| 5.04 ≤ B < 10.08 | ≤ 11.0 | ≤ 17.5 |
| 10.08 ≤ B |  | ≤ 16.5 |

#### Proposal 4: Consider PC2 MPR in Table 2-12 for SL non-contiguous CA with 2x23dBm PA + 2LO.

Table 2-12 : PSSCH/PSCCH MPR for SL non-contiguous CA with 2x23dBm PA + 2LO

| B | MPR (dB) for IM3 frequency |  |
| --- | --- | --- |
|  | SEMfreq_-13 | SEfreq_-30 |
| 0 ≤ B < 5.04 | ≤ 7.0 | ≤ 15.0 |
| 5.04 ≤ B < 10.08 |  | ≤ 13.0 |
| 10.08 ≤ B < 20.16 |  | ≤ 12.0 |
| 20.16 ≤ B |  | ≤ 10.0 |

#### Proposal 5: Consider PC3 MPR in Table 2-21 for SL non-contiguous CA with 1x23dBm PA + 1LO.

Table 2-21 : PSSCH/PSCCH MPR for SL non-contiguous CA with 1x23dBm PA + 1LO

| B | MPR (dB) for IM3 frequency |  |
| --- | --- | --- |
|  | SEMfreq_-13 | SEfreq_-30 |
| 0 ≤ B < 5.04 | ≤ 10.0 | ≤ 16.5 |
| 5.04 ≤ B < 10.08 | ≤ 9.0 | ≤ 15,5 |
| 10.08 ≤ B |  | ≤ 15.0 |

#### Proposal 6: Consider PC3 MPR in Table 2-22 for SL non-contiguous CA with 2x20dBm PA + 1LO.

Table 2-22 : PSSCH/PSCCH MPR for SL non-contiguous CA with 2x20dBm PA + 1LO

| B | MPR (dB) for IM3 frequency |  |
| --- | --- | --- |
|  | SEMfreq_-13 | SEfreq_-30 |
| 0 ≤ B < 5.04 | ≤ 10.5 | ≤ 17.0 |
| 5.04 ≤ B < 10.08 | ≤ 9.5 | ≤ 16.0 |
| 10.08 ≤ B |  | ≤ 15.5 |

#### Proposal 7: Consider PC3 MPR in Table 2-23 for SL non-contiguous CA with 2x20dBm PA + 2LO.

Table 2-23 : PSSCH/PSCCH MPR for SL non-contiguous CA with 2x20dBm PA + 2LO

| B | MPR (dB) for IM3 frequency |  |
| --- | --- | --- |
|  | SEMfreq_-13 | SEfreq_-30 |
| 0 ≤ B < 5.04 | ≤ 6.5 | ≤ 13.5 |
| 5.04 ≤ B < 10.08 | ≤ 6.5 | ≤ 12.5 |
| 10.08 ≤ B < 20.16 |  | ≤ 11.5 |
| 20.16 ≤ B |  | ≤ 10.5 |

### < PC2 & PC3 PSFCH MPR >

#### Proposal 8: Consider PSFCH MPR evaluation scenario in Table 2-24.

#### Proposal 9: Consider PC2 PSFCH MPR in Table 2-29 for SL non-contiguous CA.

Table 2-29 : PC2 PSFCH MPR for SL non-contiguous CA

|  | MPR (dB) for IM3 frequency |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | SEMfreq_-13 |  |  | SEfreq_-30 |  |  |
|  | SCS=15kHz (B=0.36) | SCS=30kHz (B=0.72) | SCS=60kHz (B=1.44) | SCS=15kHz (B=0.36) | SCS=30kHz (B=0.72) | SCS=60kHz (B=1.44) |
| 1x26dBm + 1LO | ≤ 15.0 | ≤ 15.0 | ≤ 14.0 | ≤ 20.0 | ≤ 20.0 | ≤ 19.0 |
| 2x23dBm + 1LO | ≤ 15.0 | ≤ 15.0 | ≤ 14.0 | ≤ 20.0 | ≤ 20.0 | ≤ 19.0 |
| 2x23dBm + 2LO | ≤ 9.5 | ≤ 8.5 | ≤ 7.5 | ≤ 16.5 | ≤ 16.0 | ≤ 15.0 |

#### Proposal 10: Consider PC3 PSFCH MPR in Table 2-34 for SL non-contiguous CA.

Table 2-34 : PC3 PSFCH MPR for SL non-contiguous CA

|  | MPR (dB) for IM3 frequency |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | SEMfreq_-13 |  |  | SEfreq_-30 |  |  |
|  | SCS=15kHz (B=0.36) | SCS=30kHz (B=0.72) | SCS=60kHz (B=1.44) | SCS=15kHz (B=0.36) | SCS=30kHz (B=0.72) | SCS=60kHz (B=1.44) |
| 1x26dBm + 1LO | ≤ 12.0 | ≤ 12.0 | ≤ 11.0 | ≤ 18.0 | ≤ 18.0 | ≤ 17.0 |
| 2x23dBm + 1LO | ≤ 12.0 | ≤ 12.0 | ≤ 11.0 | ≤ 18.0 | ≤ 18.0 | ≤ 17.0 |
| 2x23dBm + 2LO | ≤ 9.0 | ≤ 8.0 | ≤ 7.0 | ≤ 16.0 | ≤ 15.5 | ≤ 15.0 |

### < PC2 & PC3 S-SSB MPR >

#### Proposal 11: Consider SSSB MPR evaluation scenario in Table 2-35.

#### Proposal 12: Consider PC2 SSSB MPR in Table 2-40 for SL non-contiguous CA.

Table 2-40 : PC2 SSSB MPR for SL non-contiguous CA

|  | MPR (dB) for IM3 frequency |  |  |  |
| --- | --- | --- | --- | --- |
|  | SEMfreq_-13 |  | SEfreq_-30 |  |
|  | SCS=15kHz (B=3.96) | SCS=30kHz (B=7.92) | SCS=15kHz (B=3.96) | SCS=30kHz (B=7.92) |
| 1x26dBm + 1LO | ≤ 13.0 | ≤ 12.0 | ≤ 22.0 | ≤ 20.0 |
| 2x23dBm + 1LO | ≤ 14.0 | ≤ 13.0 | ≤ 23.0 | ≤ 21.0 |
| 2x23dBm + 2LO | ≤ 9.0 | ≤ 8.0 | ≤ 17.0 | ≤ 15.0 |

#### Proposal 13: Consider PC3 SSSB MPR in Table 2-45 for SL non-contiguous CA.

Table 2-45 : PC3 SSSB MPR for SL non-contiguous CA

|  | MPR (dB) for IM3 frequency |  |  |  |
| --- | --- | --- | --- | --- |
|  | SEMfreq_-13 |  | SEfreq_-30 |  |
|  | SCS=15kHz (B=3.96) | SCS=30kHz (B=7.92) | SCS=15kHz (B=3.96) | SCS=30kHz (B=7.92) |
| 1x23dBm + 1LO | ≤ 12.0 | ≤ 10.5 | ≤ 20.0 | ≤ 18.0 |
| 2x20dBm + 1LO | ≤ 13.0 | ≤ 11.5 | ≤ 21.0 | ≤ 20.0 |
| 2x20dBm + 2LO | ≤ 8.5 | ≤ 7.5 | ≤ 15.0 | ≤ 14.0 |

## In Gap

Following configurations for SL intra-band NC CA are simulated to check in-Gap ACLR impact with the larger I/Q image and LO leakage suppression. The cases are from [3].

- 34dBc IQ image

- 34dBc LO suppression

Table 3-1 Problematic Configurations of NCCA

| case | LCRB1 | RBSTART1 | LCRB2 | RBSTART2 | SCS | CBW1 | CBW2 | GAP |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 7 | 10 | 41 | 10 | 0 | 15 | 10 | 10 | 10 |
| 23 | 10 | 41 | 20 | 2 | 15 | 10 | 20 | 20 |
| 39 | 10 | 13 | 10 | 0 | 30 | 10 | 20 | 20 |
| 43 | 12 | 11 | 25 | 0 | 30 | 10 | 20 | 20 |

Table 3-2 shows the PSSCH MPR simulation results for cases. Here, QPSK is considered.

Table 3-2 PSSCH MPR simulation result for in-Gap with 34dBc IQ and 34dBc LO leakage

| case | B | Main factor(MPR) | 1x26dBm | 2x23dBm+1LO |
| --- | --- | --- | --- | --- |
| 7 | 3.6 | ACLR(in-Gap) & SEM | 7.2 | 7.5 |
| 23 | 5.4 | ACLR(in-Gap) & SE | 14.2 | 15.6 |
| 39 | 7.2 | ACLR(in-Gap) & SE | 13.7 | 14.3 |
| 43 | 13.32 | ACLR(in-Gap) & SE | 12.3 | 13.1 |

Based on the simulation results, larger LO suppression and I/Q image needs to be considered for MPR evaluation of SL intra-band NC CA. However, it is not necessary to introduce exception for I/Q image and LO leakage requirements.

#### Proposal 14: Consider to use larger LO suppression (-34dBc) and I/Q image suppression  (-34dBc) levels for MPR evaluation of SL intra-band NC CA that what has been used for single carrier. With these levels the IQ-impairmants have negligible impacty on ACLR and it is not necessary to introduce exception(s) due to inadequate I/Q image and LO imperfections.

## A-MPR

In [4], the following observations were made for European regulation related to band n47.

Observation 1: It is required that UE operates on channels that have maximum channel bandwidth of 10MHz

Observation 2: It is required that UE operates on channels with centre frequency listed above with +/-20ppm frequency error

Observation 3: The out-of-band domain is defined as ±250 % of the channel bandwidth, and then the out-of-band (OOB) domain for the 5 GHz ITS frequency band is 5 835 MHz to 5 855 MHz at the lower part and 5 925 MHz to 5 945 MHz at the higher part of the frequency band.

Observation 4: Maximum EIRP power density for UE emissions into the out-of-band domain is -30dBm/MHz.

Observation 5: Spectrum mask is defined for 10MHz channel bandwidth only

Observation 6: Spectrum mask is only applicable within 5855 to 5925MHz.

Observation 7: Considering the observations 3, 4, 5, 6 there is only one 10+10MHz UL NC-CA combination where IM3-products fall inside the band n47. In all other cases -30dBm/MHz OOB or spurious emission requirement needs to be met.

Based on the observations, we propose the following.

#### Proposal 15: Consider the above observations when specifing Rel-19 SL intra-band NC CA A-MPR requirements.

# Conclusion

In this contribution, we provided our view on Tx requirements of SL intra-band non-contiguous CA. Based on those, we propose as follows.

### < PC2 & PC3 PSSCH MPR >

#### Proposal 1: Consider MPR evaluation scenario in Table 2-3 [2].

#### Proposal 2: Consider PC2 MPR in Table 2-10 for SL non-contiguous CA with 1x26dBm PA + 1LO.

Table 2-10 : PSSCH/PSCCH MPR for SL non-contiguous CA with 1x26dBm PA + 1LO

| B | MPR (dB) for IM3 frequency |  |
| --- | --- | --- |
|  | SEMfreq_-13 | SEfreq_-30 |
| 0 ≤ B < 5.04 | ≤ 11.0 | ≤ 18.5 |
| 5.04 ≤ B < 10.08 | ≤ 10.0 | ≤ 17.0 |
| 10.08 ≤ B |  | ≤ 16.0 |

#### Proposal 3: Consider PC2 MPR in Table 2-11 for SL non-contiguous CA with 2x23dBm PA + 1LO.

Table 2-11 : PSSCH/PSCCH MPR for SL non-contiguous CA with 2x23dBm PA + 1LO

| B | MPR (dB) for IM3 frequency |  |
| --- | --- | --- |
|  | SEMfreq_-13 | SEfreq_-30 |
| 0 ≤ B < 5.04 | ≤ 11.5 | ≤ 19.0 |
| 5.04 ≤ B < 10.08 | ≤ 11.0 | ≤ 17.5 |
| 10.08 ≤ B |  | ≤ 16.5 |

#### Proposal 4: Consider PC2 MPR in Table 2-12 for SL non-contiguous CA with 2x23dBm PA + 2LO.

Table 2-12 : PSSCH/PSCCH MPR for SL non-contiguous CA with 2x23dBm PA + 2LO

| B | MPR (dB) for IM3 frequency |  |
| --- | --- | --- |
|  | SEMfreq_-13 | SEfreq_-30 |
| 0 ≤ B < 5.04 | ≤ 7.0 | ≤ 15.0 |
| 5.04 ≤ B < 10.08 |  | ≤ 13.0 |
| 10.08 ≤ B < 20.16 |  | ≤ 12.0 |
| 20.16 ≤ B |  | ≤ 10.0 |

#### Proposal 5: Consider PC3 MPR in Table 2-21 for SL non-contiguous CA with 1x23dBm PA + 1LO.

Table 2-21 : PSSCH/PSCCH MPR for SL non-contiguous CA with 1x23dBm PA + 1LO

| B | MPR (dB) for IM3 frequency |  |
| --- | --- | --- |
|  | SEMfreq_-13 | SEfreq_-30 |
| 0 ≤ B < 5.04 | ≤ 10.0 | ≤ 16.5 |
| 5.04 ≤ B < 10.08 | ≤ 9.0 | ≤ 15,5 |
| 10.08 ≤ B |  | ≤ 15.0 |

#### Proposal 6: Consider PC3 MPR in Table 2-22 for SL non-contiguous CA with 2x20dBm PA + 1LO.

Table 2-22 : PSSCH/PSCCH MPR for SL non-contiguous CA with 2x20dBm PA + 1LO

| B | MPR (dB) for IM3 frequency |  |
| --- | --- | --- |
|  | SEMfreq_-13 | SEfreq_-30 |
| 0 ≤ B < 5.04 | ≤ 10.5 | ≤ 17.0 |
| 5.04 ≤ B < 10.08 | ≤ 9.5 | ≤ 16.0 |
| 10.08 ≤ B |  | ≤ 15.5 |

#### Proposal 7: Consider PC3 MPR in Table 2-23 for SL non-contiguous CA with 2x20dBm PA + 2LO.

Table 2-23 : PSSCH/PSCCH MPR for SL non-contiguous CA with 2x20dBm PA + 2LO

| B | MPR (dB) for IM3 frequency |  |
| --- | --- | --- |
|  | SEMfreq_-13 | SEfreq_-30 |
| 0 ≤ B < 5.04 | ≤ 6.5 | ≤ 13.5 |
| 5.04 ≤ B < 10.08 | ≤ 6.5 | ≤ 12.5 |
| 10.08 ≤ B < 20.16 |  | ≤ 11.5 |
| 20.16 ≤ B |  | ≤ 10.5 |

### < PC2 & PC3 PSFCH MPR >

#### Proposal 8: Consider PSFCH MPR evaluation scenario in Table 2-24.

#### Proposal 9: Consider PC2 PSFCH MPR in Table 2-29 for SL non-contiguous CA.

Table 2-29 : PC2 PSFCH MPR for SL non-contiguous CA

|  | MPR (dB) for IM3 frequency |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | SEMfreq_-13 |  |  | SEfreq_-30 |  |  |
|  | SCS=15kHz (B=0.36) | SCS=30kHz (B=0.72) | SCS=60kHz (B=1.44) | SCS=15kHz (B=0.36) | SCS=30kHz (B=0.72) | SCS=60kHz (B=1.44) |
| 1x26dBm + 1LO | ≤ 15.0 | ≤ 15.0 | ≤ 14.0 | ≤ 20.0 | ≤ 20.0 | ≤ 19.0 |
| 2x23dBm + 1LO | ≤ 15.0 | ≤ 15.0 | ≤ 14.0 | ≤ 20.0 | ≤ 20.0 | ≤ 19.0 |
| 2x23dBm + 2LO | ≤ 9.5 | ≤ 8.5 | ≤ 7.5 | ≤ 16.5 | ≤ 16.0 | ≤ 15.0 |

#### Proposal 10: Consider PC3 PSFCH MPR in Table 2-34 for SL non-contiguous CA.

Table 2-34 : PC3 PSFCH MPR for SL non-contiguous CA

|  | MPR (dB) for IM3 frequency |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | SEMfreq_-13 |  |  | SEfreq_-30 |  |  |
|  | SCS=15kHz (B=0.36) | SCS=30kHz (B=0.72) | SCS=60kHz (B=1.44) | SCS=15kHz (B=0.36) | SCS=30kHz (B=0.72) | SCS=60kHz (B=1.44) |
| 1x26dBm + 1LO | ≤ 12.0 | ≤ 12.0 | ≤ 11.0 | ≤ 18.0 | ≤ 18.0 | ≤ 17.0 |
| 2x23dBm + 1LO | ≤ 12.0 | ≤ 12.0 | ≤ 11.0 | ≤ 18.0 | ≤ 18.0 | ≤ 17.0 |
| 2x23dBm + 2LO | ≤ 9.0 | ≤ 8.0 | ≤ 7.0 | ≤ 16.0 | ≤ 15.5 | ≤ 15.0 |

### < PC2 & PC3 S-SSB MPR >

#### Proposal 11: Consider SSSB MPR evaluation scenario in Table 2-35.

#### Proposal 12: Consider PC2 SSSB MPR in Table 2-40 for SL non-contiguous CA.

Table 2-40 : PC2 SSSB MPR for SL non-contiguous CA

|  | MPR (dB) for IM3 frequency |  |  |  |
| --- | --- | --- | --- | --- |
|  | SEMfreq_-13 |  | SEfreq_-30 |  |
|  | SCS=15kHz (B=3.96) | SCS=30kHz (B=7.92) | SCS=15kHz (B=3.96) | SCS=30kHz (B=7.92) |
| 1x26dBm + 1LO | ≤ 13.0 | ≤ 12.0 | ≤ 22.0 | ≤ 20.0 |
| 2x23dBm + 1LO | ≤ 14.0 | ≤ 13.0 | ≤ 23.0 | ≤ 21.0 |
| 2x23dBm + 2LO | ≤ 9.0 | ≤ 8.0 | ≤ 17.0 | ≤ 15.0 |

#### Proposal 13: Consider PC3 SSSB MPR in Table 2-45 for SL non-contiguous CA.

Table 2-45 : PC3 SSSB MPR for SL non-contiguous CA

|  | MPR (dB) for IM3 frequency |  |  |  |
| --- | --- | --- | --- | --- |
|  | SEMfreq_-13 |  | SEfreq_-30 |  |
|  | SCS=15kHz (B=3.96) | SCS=30kHz (B=7.92) | SCS=15kHz (B=3.96) | SCS=30kHz (B=7.92) |
| 1x23dBm + 1LO | ≤ 12.0 | ≤ 10.5 | ≤ 20.0 | ≤ 18.0 |
| 2x20dBm + 1LO | ≤ 13.0 | ≤ 11.5 | ≤ 21.0 | ≤ 20.0 |
| 2x20dBm + 2LO | ≤ 8.5 | ≤ 7.5 | ≤ 15.0 | ≤ 14.0 |

### < In-Gap >

#### Proposal 14: Consider to use larger LO suppression (-34dBc) and I/Q image suppression  (-34dBc) levels for MPR evaluation of SL intra-band NC CA that what has been used for single carrier. With these levels the IQ-impairmants have negligible impacty on ACLR and it is not necessary to introduce exception(s) due to inadequate I/Q image and LO imperfections.

### < A-MPR >

Observation 1: It is required that UE operates on channels that have maximum channel bandwidth of 10MHz

Observation 2: It is required that UE operates on channels with centre frequency listed above with +/-20ppm frequency error

Observation 3: The out-of-band domain is defined as ±250 % of the channel bandwidth, and then the out-of-band (OOB) domain for the 5 GHz ITS frequency band is 5 835 MHz to 5 855 MHz at the lower part and 5 925 MHz to 5 945 MHz at the higher part of the frequency band.

Observation 4: Maximum EIRP power density for UE emissions into the out-of-band domain is -30dBm/MHz.

Observation 5: Spectrum mask is defined for 10MHz channel bandwidth only

Observation 6: Spectrum mask is only applicable within 5855 to 5925MHz.

Observation 7: Considering the observations 3, 4, 5, 6 there is only one 10+10MHz UL NC-CA combination where IM3-products fall inside the band n47. In all other cases -30dBm/MHz OOB or spurious emission requirement needs to be met.

#### Proposal 15: Consider the above observations when specifing Rel-19 SL intra-band NC CA A-MPR requirements.

# References

[1] R4-2417089, “WF on sidelink intra-band CA part 1”, OPPO

[2] R4-2416066,  “Tx requirements for intra-band non-contiguous CA”, LG Electronics

[3] R4-2415601, “on intra-band NCCA MPR simulation results”, OPPO

[4] R4-2412014, “NR side link requirements and European regulation for band n47”, LG Electronics