# 3GPP TSG-RAN WG4 Meeting # 113                                    R4-2419167

# Orlando, US, 18th – 22nd November, 2024

# Title: 	Discussion on UE demodulation performance requirements for 8Rx with MMSE-IRC

# Source: 	ZTE Corporation, Sanechips

# Agenda item:	7.16.2

# Document for:	Approval

# 1 Introduction

In RAN #104 meeting, the new WID of UE performance requirements for 8Rx CPE/FWA/vehicle/industrial devices, which was approved in [1], wherein the objectives related to 8Rx for CPE/FWA /vehicles are listed as below:

- Define the following UE performance requirements for 8Rx CPE/FWA/vehicle/industrial devices: - PDSCH demodulation and CQI reporting requirements with inter-cell interference with MMSE-IRC receiver - Reuse the Rel-17 interference model for 2/4Rx UE as a baseline - PDSCH demodulation requirements with intra-cell inter-user interference with MMSE-IRC receiver - Reuse the existing interference model for 2/4Rx UE as a baseline

In this contribution, we give some initial discussions on the UE performance requirements for 8Rx CPE/FWA/vehicle/industrial device, which include inter-cell interference cancellation and intra-cell inter-user interference cancellation based on the lasting meeting conclusion [2].

# 2 Discussion

As we all know, co-channel inter-cell and intra-cell inter-user interference cancellation performance requirements were introduced in Rel-17. And in Rel-18, RAN4 introduced 8Rx demodulation requirements for PDSCH and CSI performance. With 8Rx, larger performance gain can be verified, due to larger spatial domain freedom compared to 2/4Rx. As a consequence, from demodulation perspective, there are more benefits to define UE performance requirements for 8Rx CPE/FWA/vehicle/industrial devices under interference scenarios.

  - Sub-topic 2-1 General

8Rx UE MMSE-IRC receiver coverage

Agreement: - Cover both Baseline 8Rx UE MMSE-IRC receiver and simplified 8Rx UE MMSE-IRC Receiver for the initial simulation analysis. - TBD: Criteria used to decide if one set of requirements can be introduced covering both receiver types. - Note: Test coverage shall cover both receivers

Regarding baseline and simplified receivers, we still think that defining two set requirements is necessary, as they represent different implementation methods at the baseband side. At the same time, we provided simulation results for inter interference and intra interference cancellation in reference [3]. From our simulation results, we observed significant performance degradation between baseline and simplified receiver, particularity in Rank 4 about more than 3dB . And regarding the CQI interference scenario, RAN4 has not yet evaluated the results, making it difficult to decide which requirements can be applicable to these two types of receivers.

Observation 1. RAN4 has not yet evaluated all test cases, making it difficult to decide which requirements can be applicable to these two types of receivers.

Proposal 1. Consider the delay in defining criteria until all test cases are finalized.

  - Sub-topic 1-2 PDSCH requirements with inter cell interference

Interference modelling and power levels

Agreement: - Reuse the same interference profiles (i.e., cover HomNet and Hetnet scenarios) and same power levels (i.e., INR values) defined for Rel-17 2/4Rx. - TBD: Further discuss applicability rules to potentially reduce UE effort to only comply to either 1 or 2 interference cells.

Regarding HomNet and HetNet scenarios, we believe that are common practical deployment scenarios for 5G system, e.g. macro cell and co-channel low power node within the macro cell coverage. Hence, HomNet and HetNet shall be considered for 8Rx inter cell interference scenario.

From UE testing perspective, we understand the motivation to reduce UE effort to only capture 1or 2 interference cells. However, as mentioned above that the HomNet and HetNet are different deployment scenarios for 5G system, even if the UE has the capability to handle interference. From the perspective of Network vendor, if certain tests are to be skipped, we must ensure that these do not in impose limitations on the implemented scheduling. In other words, for the test scenarios that are skipped, UE vendors need to confirm that there are no performance issues with these test cases in practical network.

Observation 2. If some test cases need to be skipped, UE vendors need to confirm that there are no performance issues with these test cases in practical network.

Proposal 2. To consider do not reduce UE effort to only comply to either 1 or 2 interference cells.

Rank of serving cell

Agreement - Evaluate rank 1,2 & 4 in initial simulation analysis. - TBD: Down-selection for requirement definition.

From our simulation results[3], we believe that Rank1, 2 and 4 SNR values are all reasonable, which also implies that they are viable in tests. If a down-selection be mandatory , we prefer to define Rank 2 and 4 test cases, as this would enhance system performance more effectively in practical deployment.

Proposal 3. If a down-selection be mandatory, we prefer to define Rank 2 and 4 requirements.

  - Sub-topic 1-3 PDSCH requirements with intra cell inter user interference

Precoding method for the co-scheduled UE

Agreement: For initial simulation alignment use: - Rank 1+1: random and orthogonal. - Rank 2+2: orthogonal. - Rank 4+4: orthogonal. For orthogonal precoding, select the precoder to ensure any column of precoder is orthogonal to any column of precoder for the target PDSCH

  - Regarding precoding method for Rank 1+1, random and orthogonal precoding methods are testable. From our simulation results [3], the SNR point is very low for random and orthogonal precoding methods with MMSE-IRC receiver. However, orthogonal precoding method also can achieve a better performance compared to random precoding with MMSE receiver. We understand that orthogonal precoding actually performs interference cancellation at the transmitter side. Even random and orthogonal are different configurations on the BS side, they to some extent reflect the capabilities of the BS. However, from testing perspective, we believe that random or orthogonal precoding are also testable for 8Rx. Hence, we prefer only to consider random precoding for Rank 1+1.

Proposal 4. Only considering random precoding for Rank 1+1.

  - Sub-topic 1-4 Wideband CQI reporting requirements with inter-cell interference

Propagation condition

Proposals - Option 1: Reuse the same configuration for Rel-17 2/4Rx: - TDLA30-5 for the serving cell - AWGN for the interference cell. - Option 2: Further study whether to also include fading channel conditions. - Option 3: CQI requirements under fading channel (If time allows) - Option 4: Consider the assumption assumed for Rel-17 CQI reporting fading tests with inter-cell interference as baseline - Other options not precluded

In Release 17, RAN4 introduced wideband CQI reporting requirements with inter-cell interference for 2Rx and 4Rx. Since the baseband processing capability of 8Rx is superior to that of 2Rx and 4Rx, from our understanding, we can at least consider reusing the configuration of R17.

Proposal 5. To consider R17 configuration as a starting point.

CQI calculation algorithm

Proposals - Option 1: Define CQI reporting requirements as below: - Baseline SU-MIMO 8Rx receiver with Baseline CQI calculation - Simplified SU-MIMO 8Rx receiver with Simplified CQI calculation. - Option 2: RAN4 to define CQI requirements for two receiver types: - Baseline receiver: UE performs demodulation and CSI calculation with 8Rx joint processing. - Simplified receiver: UE performs demodulation and CSI calculation with two separate 4Rx processing. - Option 3: Only define one CQI calculation assumption to match both receiver types. - Option 4: Introduce one CQI calculation assumption independent of receiver type. Reconsider if later proven that two CQI calculation assumptions matching the two receiver types are needed. - Option 5: RAN4 to evaluate the throughput ratio for both receiver assumptions based on the agreed parameters/settings. If the throughput ratio is close, then we can go for one requirement. Otherwise, RAN4 can define two sets of requirements for different type of receiver. - Other options not precluded.

From inter cell interference and intra cell interference cancellation, we observed varying performance between the baseline and simplified receivers. Since UE needs to perform demodulation and CSI calculation with two separate 4Rx processing, which is different from legacy baseband joint processing. As a consequence, we should not rule out the adoption of different baseband implementation methods during the evaluation phase. This also indirectly reflects the capability to handle interference.

Proposal 6. Different baseband implementation methods need to be considered at least during the evaluation phase.

Rank for the serving cell

Proposals - Option 1: Rank 1 - Option 2: Rank 2 - Option 3: Rank 4 - Option 4: Cover the following for initial study and possible down select based on simulation results: - Rank 1, 2 & 4 - Other options not precluded.

In legacy R-18 8Rx requirements, RAN4 introduced 4T8R with Rank 4 under static channel for CQI requirements. Hence we prefer to consider 4T8R with Rank 4 for 8Rx inter cell interference requirements. However, considering different implementation methods, Rank1 and Rank 2 are also need to be evaluated.

Proposal 7. To consider Rank 1, 2, and 4 for 8Rx inter cell interference requirements.

Antenna configuration and MIMO correlation for the serving cell

Proposals - Option 1: 2T8R - Option 1a: - 2T8R with ULA Low - Option 1b: - 2T8R with rank 1 - Option 2: 4T8R - Option 2a: 4T8R with ULA Low, Rank 4 - Option 2b: 4T8R with Rank 4 Other options not precluded.

Regarding antenna configuration, we prefer to consider 4T8R for Rank4 and 2T8R for Rank1 and Rank2.

Proposal 8. To consider 4T8R for Rank 4 and 2T8R for Rank 1 and Rank 2.

CSI-IM and NZP CSI-RS configurations

Proposals - Option 1: Reuse agreement from R17 2/4Rx inter-cell interference suppressing requirement for simulation: - CSI-IM on serving cell overlapping with PDSCH from interference cell - NZP CSI-RS on serving cell overlapping with NZP CSI-RS from interference cell - Other options not precluded

Regarding CSI-IM and NZP CSI-RS configurations, R17 2/4Rx inter-cell interference suppressing requirement for simulation as follows from TS 38.101-4.

Table 6.2.2.1.2.3-1 Wideband CQI reporting test with inter-cell interference

| ZP CSI-RS configuration | CSI-RS resource Type |  | Periodic | Periodic |
| --- | --- | --- | --- | --- |
|  | Number of CSI-RS ports (X) |  | 4 | 4 |
|  | CDM Type |  | FD-CDM2 | FD-CDM2 |
|  | Density (ρ) |  | 1 | 1 |
|  | First subcarrier index in the PRB used for CSI-RS (k0) |  | Row 5,4 | Row 5,4 |
|  | First OFDM symbol in the PRB used for CSI-RS (l0) |  | 9 | 9 |
|  | CSI-RS periodicity and offset | slot | 5/1 | Same as serving cell |
| NZP CSI-RS for CSI acquisition | CSI-RS resource Type |  | Periodic | Periodic |
|  | Number of CSI-RS ports (X) |  | 2 | 1 |
|  | CDM Type |  | FD-CDM2 | noCDM |
|  | Density (ρ) |  | 1 | 1 |
|  | First subcarrier index in the PRB used for CSI-RS (k0, k1 ) |  | Row 3(6, -) | Row 2(6, -) |
|  | First OFDM symbol in the PRB used for CSI-RS (l0) |  | 13 | 13 |
|  | NZP CSI-RS-timeConfig periodicity and offset | slot | 5/1 | Same as serving cell |
| CSI-IM configuration | CSI-IM resource Type |  | Periodic | Periodic |
|  | CSI-IM RE pattern |  | 0 | 0 |
|  | CSI-IM Resource Mapping (kCSI-IM,lCSI-IM) |  | (4, 9) | (6, 9) |
|  | CSI-IM timeConfig periodicity and offset | slot | 5/1 | Same as serving cell |

Table 6.2.2.2.2.3-1: Wideband CQI reporting test with inter-cell interference (TDD)

| ZP CSI-RS configuration | CSI-RS resource Type |  | Periodic | Periodic |
| --- | --- | --- | --- | --- |
|  | Number of CSI-RS ports (X) |  | 2 | 1 |
|  | CDM Type |  | FD-CDM2 | noCDM |
|  | Density (ρ) |  | 1 | 1 |
|  | First subcarrier index in the PRB used for CSI-RS (k0) |  | Row 3(8) | Row 2(8) |
|  | First OFDM symbol in the PRB used for CSI-RS (l0) |  | 9 | 9 |
|  | CSI-RS periodicity and offset | slot | 10/1 | Same as serving cell |
| NZP CSI-RS for CSI acquisition | CSI-RS resource Type |  | Periodic | Periodic |
|  | Number of CSI-RS ports (X) |  | 2 | 1 |
|  | CDM Type |  | FD-CDM2 | noCDM |
|  | Density (ρ) |  | 1 | 1 |
|  | First subcarrier index in the PRB used for CSI-RS (k0, k1 ) |  | Row 3(6, -) | Row 2(6, -) |
|  | First OFDM symbol in the PRB used for CSI-RS (l0) |  | 13 | 13 |
|  | NZP CSI-RS-timeConfig periodicity and offset | slot | 10/1 | Same as serving cell |
| CSI-IM configuration | CSI-IM resource Type |  | Periodic | Periodic |
|  | CSI-IM RE pattern |  | 0 | 0 |
|  | CSI-IM Resource Mapping (kCSI-IM,lCSI-IM) |  | (4, 9) | (6,9) |
|  | CSI-IM timeConfig periodicity and offset | slot | 10/1 | Same as serving cell |

Fig.1.  FDD CSI-RS resource elements

Fig. 2.  TDD CSI-RS resource elements

From above CSI-RS resource configurations, we find some issues about FDD CSI-RS resource configuration for 2Rx and 4Rx. From TS 38.101-4 CSI-RS configuration, CSI-IM resource on serving cell overlapping with ZP CSI-RS resource from interference cell in Fig .1 and Fig .2. However, the conclusion of the RAN4#101 meeting for R17 is as follows:

Issue 2-2-1: CSI-IM on target cell - Agreement: - Only consider overlapping with PDSCH from interference Issue 2-2-2: NZP CSI-RS on target cell - Agreement: - Overlaps with NZP CSI-RS from interference

We understand that only CRS-IM resource can measure interference, but as shown in Fig .1, CRS-IM resources overlap with ZP CRS-IM, which does not allow for the measurement of interference.

Observation 3. CRS-IM resources are not overlapped with PDSCH from interference in TS 38.101-4 for FDD scenario.

Proposal 9. Reuse agreement from R17 2/4Rx inter-cell interference suppressing requirement for simulation:

- CSI-IM on serving cell overlapping with PDSCH from interference cell

- NZP CSI-RS on serving cell overlapping with NZP CSI-RS from interference cell

    - Sub-topic 1-5 Applicability rule and release independent

Test applicability

- Candidate options: - Option 1: The new requirements for 8Rx MMSE-IRC receiver should be mandatory requirements for all 8Rx UEs without additional test applicability rule - Option 2: Applicability rule to skip 2/4Rx requirements if 8Rx tests are passed, if same test scenarios/configurations as 2RX/4RX are defined

From applicable rule perspective and test burden, if UE supports 8Rx, 4Rx and 2Rx, when UE passes 8Rx test , 2Rx and 4Rx test shall be passed under same scenarios or configurations in legacy 8Rx requirements. However, if we pay attention to applicability rule for inter cell interference as shown in following table. There are no instructions that any requirements need to be skipped. Hence we think that no need to add test applicable rule for 8Rx inter cell interference requirements.

| Tests | Applicability notes |
| --- | --- |
| All tests in Clause 5.2.2.1.15, 5.2.3.1.15, 5.2.2.2.16 and 5.2.3.2.16 | If UE supporting both duplex mode TDD and FDD with 2RX, only test 1-1 in clause 5.2.2.1.15 and test 1-2 in clause 5.2.2.2.16 will be applied. If UE supporting both duplex mode TDD and FDD with 4RX, only test 1-1 in clause 5.2.3.1.15 and test 1-2 in clause 5.2.3.2.16 will be applied. |

Proposal 10. The new requirements for 8Rx MMSE-IRC receiver should be mandatory requirements for all 8Rx UEs without additional test applicability rule.

Release independent

- Candidate options: - Option 1: The new requirements for 8Rx should be release independent from Rel-18 - Other options are not precluded

Considering 8Rx requirements was introduced in Rel-18, and inter interference cancellation is a mandatory feature. We prefer to consider the new requirements shall be release independent form Rel-18.

Proposal 10. The new requirements for 8Rx should be release independent from Rel-18.

# 3 Conclusion

In this contribution, we give some discussions on demodulation performance requirements for UE demodulation performance requirements for 8Rx with MMSE-IRC , The conclusions are:

Observation 1. RAN4 has not yet evaluated all test cases, making it difficult to decide which requirements can be applicable to these two types of receivers.

Observation 2. If some test cases need to be skipped, UE vendors need to confirm that there are no performance issues with these test cases in practical network.

Observation 3. CRS-IM resources are not overlapped with PDSCH from interference in TS 38.101-4 for FDD scenario.

Proposal 1. Consider the delay in defining criteria until all test cases are finalized.

Proposal 2. To consider do not reduce UE effort to only comply to either 1 or 2 interference cells.

Proposal 3. If a down-selection be mandatory, we prefer to define Rank 2 and 4 requirements.

Proposal 4. Only considering random precoding for Rank 1+1.

Proposal 5. To consider R17 configuration as a starting point.

Proposal 6. Different baseband implementation methods need to be considered at least during the evaluation phase.

Proposal 7. To consider Rank 1, 2, and 4 for 8Rx inter cell interference requirements.

Proposal 8. To consider 4T8R for Rank 4 and 2T8R for Rank 1 and Rank 2.

Proposal 9. Reuse agreement from R17 2/4Rx inter-cell interference suppressing requirement for simulation:

- CSI-IM on serving cell overlapping with PDSCH from interference cell

- NZP CSI-RS on serving cell overlapping with NZP CSI-RS from interference cell

Proposal 10. The new requirements for 8Rx should be release independent from Rel-18.

# 4 References

- RP-240831, New WID: WI resulting from discussion on Demodulation performance requirements enhancements.

- R4-2416555, WF for [112bis][320] NR_demod_Ph5_Part2_UE. China Telcom.

- R4-2419168, Simulation results on UE demodulation performance requirements for 8Rx with MMSE-IRC. ZTE Corporation, Sanechips.