# 3GPP TSG-RAN WG4 Meeting # 113                                   R4-2419173

# Orlando, US, 18th – 22nd November, 2024

# Title: 	Discussion on R18 ATG k1 value

# Source: 	ZTE Corporation, Sanechips

# Agenda item:	5.6.4

# Document for:	Approval

# 1 Introduction

In last meeting , RAN4 discussed how to address the k1 value in the ATG scenario. Ultimately, RAN4 agreed to introduce the outOfOrderOperationDL-r16 as a solution to the ATG HARQ timing [1]. Additionally, during the last meeting, some companies proposed other alternative solutions.

In this contribution, we would like to analyze the feasibility of these alternative proposals.

# 2 Discussion

In previous meeting, RAN2 introduced a new signalling for ATG scenarios, i.e. DL-DataToUL-ACK-r18, to address the range of k1 value.

DL-DataToUL-ACK-r18 ::=                    SEQUENCE (SIZE (1..8)) OF INTEGER (0..31)

The remaining issues is how to resolve the k1 value in the HARQ feedback. In last meeting, some companies proposed resolving the timing for ATG HARQ by introducing k_offset and out of order capability, as illustrated below.

| The number of slots between PDSCH and corresponding HARQ-ACK information (Note 3) |  | 34 if mod(i,40) = 0, 1, 2, 3, 4, 5; k1=30, koffset =2 28 if mod(i,40) = 6, 7, 8, 9, 10, 11; k1=24, koffset=2 22 if mod(i,40) = 12, 13, 14, 15, 16, 17; k1=18, koffset =2 16 if mod(i,40) = 18, 19, 20, 21, 22, 23; k1=12, koffset =2 10 if mod(i,40) = 24, 25, 26, 27, 28, 29; k1=6, koffset =2 |
| --- | --- | --- |

With this approach, the k1 value would be constrained to five specific values, aligning with the current RAN1 specifications definition.

Meanwhile, some companies are concerned that the out of order might impose a heavy burden and latency for UE, and thus suggested disabling all out of order HARQ processes.

- 34 if mod(i,40) = 0, 1, 2, 3, 4, 5; k1=30, koffset = 2 - 28 if mod(i,40) = 6, 7, 8, 9, 10, 11; k1=24, koffset = 2 - 22 if mod(i,40) = 12, 13, 14, 15, 16, 17; k1=18, koffset = 2 - 16 if mod(i,40) = 18, 19, 20, 21, 22, 23; k1=12, koffset = 2 - 10 if mod(i,40) = 24, 25, 26, 27, 28, 29; k1=6, koffset = 2

In general, both above methods have some drawbacks compared to legacy HARQ feedback method. Given that RAN1 specifications have been frozen, initiating revisions to the RAN1 spec is exceedingly challenging and beyond the scope of this WI. Therefore, RAN4 should consider a compromise approach to address this issue.

Regarding the disable approach, we understand that HARQ disable is introduced in NTN scenario caused by larger delay. However, the motivation for introducing new pattern in ATG is to increase the network traffic. If too many HARQ processes are disabled in transmission, it implies that the transmission efficiency of many downlink slots cannot be guaranteed, resulting in a significant loss of network traffic. Compared to legacy TDD pattern, even with the introduction of so many downlink slots, we can not guarantee that all potential gains will be realized. In that case, what is significance of introducing a new TDD pattern. Meanwhile, we evaluated the performance of partial HARQ and all HARQ process in Fig. 1 and Fig. 2. From our simulation results, it can be seen that especially at low SNR condition, there is a noticeable decrease in throughput with partial HARQ, which can have a certain impact on system capacity. While under high SNR conditions both approaches achieve the same throughput. However, in real world environments, high SNR conditions are not consistently present.

| Fig . 1 Throughput curve for 30D4S6U with MCS 13 2T2R | Fig. 2 Throughput curve for 30D4S6U with MCS 24 2T2R |
| --- | --- |

Observation 1. From our simulation results, it can be seen that especially at low SNR condition, there is a noticeable decrease in throughput with partial HARQ, which can have a certain impact on system capacity.

On the contrary, outOfOrderOperationDL-r16 may impose some additional burdens on the UE, but it would have little impact on the overall system’s traffic scheduling that all downlink slots transmission could be guaranteed. As a consequence, we prefer to consider the outOfOrderOperationDL-r16 as a solution to the ATG HARQ timing from implementation perspective.

Proposal 1. Confirm that the outOfOrderOperationDL-r16 is a solution to the ATG HARQ timing issue.

# 3 Conclusion

In this contribution, we give some discussions on the HARQ k1 value for ATG, The conclusion are:

Observation 1. From our simulation results, it can be seen that especially at low SNR condition, there is a noticeable decrease in throughput with partial HARQ, which can have a certain impact on system capacity.

Proposal 1. Confirm that the outOfOrderOperationDL-r16 is a solution to the ATG HARQ timing issue.

# 4 References

- [1] R4-2413604, CR to TS 38.101-4 corrections of PDSCH and corresponding HARQ-ACK relationship for 30D4S6U TDD pattern for ATG. CMCC.