3GPP TSG-RAN WG4 Meeting #113 	R4-2419183

Orlando, US, 18th – 22nd November 2024

Agenda item:			7.9.3 System parameters

Source:	LG Electronics

Title:	Discussion on NR NTN Ku-Band system parameters

Document for:	Discussion

# Introduction

The General requirements of the Rel-19 NR NTN Ku band work item were discussed in RAN4#112 and RAN4#112bis meeting. The WF in [1] captures the result of the discussion in the RAN4#112bis meeting and [2] from the RAN4#112. This document further discusses these topics.

# Discussion

## Specification work

Considering the simulation model related agreements in the previous meeting regarding the device type (VSAT) and antenna characteristics (directive high gain) in [1] there’s a lot of similarities between them and the work that was done for Ka-band in Rel-18. The development of the Ku-band requirements could therefore be straightforward task when using the existing Ka-band requirements as a starting point.

Observation 1: Considering the simulation model related agreements in the previous meeting regarding the device type (VSAT) and antenna characteristics (directive high gain) in [2] there’s a lot of similarities between them and the work that was done for Ka-band in Rel-18. The development of the Ku-band requirements could therefore be straightforward task when using the existing Ka-band requirements as a starting point.

## Numerology – timing aspects

The impact of SCS on timing is critical to reliable performance therefore decisions on FR1 and FR2 numerology feasibility should not be made without a thorough analysis of the issues.

The SCS, which directly also sets the duration of the symbol and cyclic prefix (CP) is important for the timing accuracy of the uplink. For OFDMA receiver it is important that received symbols are time-aligned, with channel delay spread included, within the CP to avoid leakage between the symbols in the FFT process. This then requires the timing advance of different uplink users to be adjusted in a way that above mentioned time alignment is achieved at the receiver side.

The duration of the CP with different Sub-carrier spacing in shown in table below

| NTN FR1/FR2 | SCS (kHz) | Short CP duration (us) | Long CP duration (us) |
| --- | --- | --- | --- |
| FR1 | 15 | 4.68 | 5.21 |
| FR1 | 30 | 2.34 | 2.34 |
| FR1 and FR2 | 60 | 1.17 | 1.69 |
| FR2 | 120 | 0.586 | 1.107 |

The 60 and 120kHz SCS are already defined for NR FR2 and therefore operation also with these wider SCS used at Ka-band has been considered feasible and should therefore this should also apply for Ku-band. However, it has also been pointed out that the timing margin for 120kHz SCS is small and may require further analysis by RRM experts. Due to similarities of Ka- and Ku-band in this aspect the outcome of such analysis and/or solutions is naturally also applicable for Ku-band.

Observation 2: The 60 and 120kHz SCS are already defined for NR FR2 and therefore operation also with these wider SCS used at Ka band has been considered feasible, although margins may be small, Further analysis by RRM experts may be necessary. Due to similarities of Ka- and Ku-band in this aspect the outcome of such analysis and/or solutions is naturally also applicable for Ku-band.

## Numerology – efficient channel utilization

The ability to efficiently support in future the required aggregate bandwidths [list] of NTN services for different SCS choices based on existing FR1 and FR2 channel bandwidths needs to be fully studied.

The supported Maximum transmission bandwidth configurations for FR1-NTN and FR-2 NRN are shown in section 2.3.1 with FR1-NTN supporting channel bandwidths from 5 to 30MHz and FR2-NTN from 50 up to 400MHz.

Based on the discussion in the RAN4#112 and RAN4#112bis meeting the FR2-NTN bandwidths are considered too wide and narrower bandwidth options would be necessary for better match with the existing satellite RF configurations to achieve optimum implementation efficiency. As a result of this it was agreed that for FR1-based numerology native bandwidths of 10 and 25MHz can be used in the analysis, although only 10MHz is currently defined in 38.101-5.

When it comes to FR2-based numerology, 10MHz bandwidth is too narrow to be effectively utilized with wider SCS, but there are already mechanisms in the existing specifications that would allow the use of 25MHz channel bandwidth, which were described in more detail in [3]. It’s also to be noted that use of FR2-based numerology would enable the support of wider channel bandwidths up to 400MHz with single carrier the same way as on Ka-band, while maximum single carrier channel bandwidth is limited to 100MHz with FR1.

Observation 3: Maximum channel bandwidth with FR1 based numerology is 100MHz, while FR2 based numerology allows channel bandwidths up to 400MHz as in Ka-band.

Observation 4: It is possible to support narrower than 50MHz channel bandwidths also with FR2 based numerology.

Proposal 1: RAN4 to study how narrower bandwidths below 50MHz could also be introduced to FR2-NTN, like 20, 25and 30MHz as shown in Table 1 as example when progressing the work with FR2-based numerology.

Table 1: Narrower bandwidths for FR2-NTN as example

| SCS | 20 | 25 | 30 | 50 | 100 | 200 | 400 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 60 | 24 | 30 | 38 | 66 | 132 | 264 | N/A |
| 120 | 12 | 15 | 18 | 32 | 66 | 132 | 264 |

### Extract from TS38.101-5

### 5.3.2	Maximum transmission bandwidth configuration

The maximum transmission bandwidth configuration NRB for each UE channel bandwidth and subcarrier spacing is specified in Table 5.3.2-1 for FR1-NTN and table 5.3.2-2 for FR2-NTN.

Table 5.3.2-1: Maximum transmission bandwidth configuration NRB for FR1-NTN

| SCS (kHz) | 5MHz | 10MHz | 15MHz | 20MHz | 30MHz |
| --- | --- | --- | --- | --- | --- |
|  | NRB | NRB | NRB | NRB | NRB |
| 15 | 25 | 52 | 79 | 106 | 160 |
| 30 | 11 | 24 | 38 | 51 | 78 |
| 60 | N/A | 11 | 18 | 24 | 38 |

Table 5.3.2-2: Maximum transmission bandwidth configuration NRB for FR2-NTN

| SCS (kHz) | 50MHz | 100MHz | 200MHz | 400MHz |
| --- | --- | --- | --- | --- |
|  | NRB | NRB | NRB | NRB |
| 60 | 66 | 132 | 264 | N/A |
| 120 | 32 | 66 | 132 | 264 |

# Conclusions

The General requirements of the Rel-19 NR NTN Ku band work item were discussed in RAN4#112 and RAN4#112bis meeting. The WF in [1] captures the result of the discussion in the RAN4#112 bis meeting and [2] from the RAN4#112. This document further discusses these topics.

Observation 1: Considering the simulation model related agreements in the previous meeting regarding the device type (VSAT) and antenna characteristics (directive high gain) in [1] there’s a lot of similarities between them and the work that was done for Ka-band in Rel-18. The development of the Ku-band requirements could therefore be straightforward task when using the existing Ka-band requirements as a starting point.

Observation 2: The 60 and 120kHz SCS are already defined for NR FR2 and therefore operation also with these wider SCS used at Ka band has been considered feasible, although margins may be small, Further analysis by RRM experts may be necessary and the outcome of such analysis and solutions is naturally also applicable for Ku-band.

Observation 3: Maximum channel bandwidth with FR1 based numerology is 100MHz, while FR2 based numerology allows channel bandwidths up to 400MHz as in Ka-band.

Observation 4: It is possible to support narrower than 50MHz channel bandwidths also with FR2 based numerology.

Proposal 1: RAN4 to study how narrower bandwidths below 50MHz could also be introduced to FR2-NTN, like 20, 25and 30MHz as shown in Table 1 as example when progressing the work with FR2-based numerology.

# References

[1] R4-2416593, Way Forward for [112bis][313] NR_NTN_Ku_Band_General, Moderator (Eutelsat Group)

[2] R4-2413519, Way Forward for [112][314] NR_NTN_Ku_Band_General, Moderator (Eutelsat Group)

[3] R4-2416067, Discussion on NR NTN Ku-Band SCS and CBW, LG Electronics