3GPP TSG-RAN WG4 Meeting #113	R4-2419185

Orlando, US, 18th – 22nd November 2024

Source:	Nokia

Title:	Testability and Interoperability aspects for AI/ML Based Beam Management

Agenda item:	7.17.4.2

Document for:	Discussion

Introduction

The discussions on AI/ML enabled BM use case were continued in RAN4#112bis in the scope of Rel-19 WI. Agreements achieved during this meeting are summarized in the WF [1].

Some of the issues related to AI/ML enabled BM-Case1 and BM-Case2 require further discussion. In this paper, we provide some additional views on issues related to testability and interoperability aspects, as listed below:

- Generalization and test coverage related aspects

- Testing setup and Channel models

RRM core requirements related topics are discussed in our accompanying paper [2].

Discussion

Generalization and test coverage related aspects

In the RAN4#110 meeting, the following agreement was reached on the generalization in the general aspects’ agenda item, it was agreed to discuss generalization aspects for each AI feature in case-by-case manner. Therefore, we address Generalization aspects of AI/ML BM in this tdoc:

Agreement: - For AI/ML generalization [tests/requirements] - RAN4 should discuss it and decide the requirements/tests for each AI feature in the case-by-case manner

Agreement:

- Vendors may take into account the test environment/conditions defined by RAN4 when training the UE

  - sufficient test environment/conditions should be defined to enable vendors to create the data needed for training.

- FFS on proposals to augment training data to avoid overfitting of UE models to the test environment

- RAN4 to strive to make the test conditions similar to field deployment conditions

Furthermore, following agreement was achieved in RAN4#111 meeting [4].

In RAN4#111, the concern, regarding overfitting of UE functionalities to the test environment, was raised by some companies and it was captured in the agreement and companies were encouraged to bring proposals to avoid overfitting issue. In our view, testing generalization aspects of UE models/functionalities provide a rather realistic and feasible solution to the overfitting issue.

A functionality, if tested in different generalization scenarios, would be considered not overfitted to a certain test environment.

Testing of generalization aspects should also be considered to avoid overfitting of functionalities to the test environment.

Furthermore, testing of generalization aspect is one of the main challenges for RAN4 testing. RAN4 may define multiple tests with different conditions. In each of the test, TE configures the same specified UE configuration for AI/ML BM, therefore the same UE configuration is tested under different scenarios to verify the UE’s model capability to generalize. (environment for AI/ML based BM-Case1 and BM-Case2 differs in each test but not changing dynamically during the test)

For identified scenarios in AI/ML enabled BM-Case1 and BM-Case2, the minimum level of performance may also need to be defined. Similarly acceptable tolerance margin for other scenarios may also need to be agreed, potentially based on simulation results.

In the tables below, we attempt to capture different parameters that may be needed to define different generalization scenarios for AI/ML enabled BM use case.

- Generalization Scenarios – Known set of parameters/configurations to both DUT / TE.

Parameters that should be considered for the definition of different generalization scenarios:

Table 1: Parameters for Generalization Scenarios

| Parameters | Description |
| --- | --- |
| UE Speed | Slow / Medium / Fast |
| SINR (Deployment Scenario) | Good / Bad Radio conditions |
| Propagation Model | TDL / CDL |

For the verification/testing of generalization related aspects in RAN4 for AI/ML BM, RAN4 should define different scenarios based on parameters listed in Table 1.

In order to address the testability issue of generalization aspects of AI/ML BM use case, we propose a new metric to indicate the generalization capabilities of an AI/ML functionality, which may be referred as “generalization score”. In case of AI/ML BM use case, this may be used to verify/check whether the BM functionality (which may be developed for a specific scenario) generalize well to other scenario(s).

As an example, if it is identified that there are 3 scenarios of interest. Any one scenario of these 3 scenarios can be the baseline setup (depending on functionality implementation) and the remaining 2 scenarios will then be used as other set ups to test the generalization aspects. An overall generalization score can be calculated as a function of individual scenario verdicts, e.g., as simple average or weighted average of the individual generalization verdict of each Scenario ‘j’ (i.e., Scenario ‘1’ to Scenario ‘N), where the verdict of Scenario ‘j’ = 1 if the functionality generalizes to Scenario ‘j’ else it is equal to 0.

Overall Generalization Score= (VerdictScenario1+…+VerdictScenarioN)N×100%

RAN4 needs to design a new metric, indicative of generalization capabilities of AI/ML BM functionality, to verify the generalization performance of the functionality in different scenarios.

Testing Setup and Channel Models

The feasibility of testability aspects of AI/ML enabled BM use case and the corresponding testing setup continued to be discussed since several RAN4 meetings, and the following agreements were made.

RAN4#112: Issue 2-3: Channel models Agreement: Further discuss and down-select the following options considering TE feasibility - Option 2: TDL - Option 5: simplified CDL - CDL channel model with a small number of clusters. CDL simplification needs to be further discussed Issue 2-4: Test setup needs Issues listed below to be further investigated in order to determine the capabilities of the test equipment: - set B Tx beams; set A Tx beams - number of beams transmitted simultaneously - overall number of beams transmitted during the test - AoA - AoD - UE rotation during the test Other aspects can also be considered RAN4#112bis: - Issue 2-9: Test setup requirements Agreement: - For CDL or simplified CDL channel model, RAN4 to study the relationship between AoD range [and granularity], number of beams and number of probes. - Set A and B should be generated in the chamber for ground-truth extraction. - For TDL channel model, RAN4 to study the relationship between [dynamic power range], number of beams and number of probes. - Set A and B should be generated in the chamber for ground-truth extraction. For single AoA, FSS how to derive Set A

Furthermore, the revision of the WID for “NR_AIML_Air” in RAN#105 includes the following objective for the testing of AI/ML-enabled BM use case.

Provide specification support for the following aspects: … - Core requirements for the above two use cases for AI/ML LCM procedures and UE features [RAN4]: - Specify necessary RAN4 core requirements for the above three use cases. This includes the test setup and methodology for the beam management use case. - Specify necessary RAN4 core requirements for LCM procedures including performance monitoring. …

This revision of NR_AIML_Air WID justifies the need for defining the testing setup and procedure for the AI/ML-enabled BM use case. Furthermore, the topic of channel model for conformance testing is still open as no explicit agreement was made in RAN4#112-bis meeting implying that the channel model for conformance testing is still to be decided from the TDL and simplified CDL channel models. Moreover, some of the other important open issues which need further investigation are: (1) number of Tx beams in Set A and Set B during the conformance test, (2) details of the AoAs and AoDs to be supported during the conformance test (i.e., the number of different AoAs and their values), (3) whether to use the UE rotation or not during the conformance test of the AI/ML enabled BM use-case.

In this section, we provide our further views on these open aspects.

Channel Model

After RAN4#112bis meeting, there was still no consensus on which channel model to be used for the conformance testing of the AI/ML-enabled beam management use case and hence the need for further study on the relationship between AoD range [and granularity], number of beams and number of probes for simplified CDL model and the relationship between [dynamic power range], number of beams and number of probes for TDL channel model was captured in the way forward [1]. As some companies are advocating for simplified CDL channel model whereas the others are supporting using simple channel models like TDL. There are pros and cons of each of the CDL and TDL channel models which are well known and need to be considered before deciding the channel model for the conformance testing of AI/ML-enabled BM use case.

The use of CDL channel model during the conformance testing results in:

- More realistic testing approach as compared to TDL channel model as the signal reception from multiple AoAs is possible.

- Increased cost of the test systems.

- Increased complexity of the test system due to the need of large number of test probes to emulate clusters of the CDL channel model.

On the other hand, the use of TDL channel model during the conformance testing results in:

- Simplified testing setup.

- Cost-effective solution.

- Inability to mimic the realistic UE behaviour observed in the field.

- Absence of multi-path propagation environment for conformance testing.

- Difficulty in training an AI/ML functionality with the dataset driven based on TDL channel model.

CDL channel models present a more realistic way for testing AI/ML BM use cases due to the more realistic spatial behaviour.

If the testability objective of AI/ML-enabled BM use case is to verify the AI/ML BM functionality taking only L1-RSRP of Set B beams as input and not to test the multipath reception of the UE, the use of TDL channel model during the conformance testing is sufficient.

Considering the challenges associated with both the CDL and TDL channel models, a third option was discussed in the RAN4#112 meeting as a trade-off which is “Simplified CDL” model. This “Simplified CDL” model can be seen as a simplification of the existing CDL models, which typically have a large number (~ 23 or 24) of clusters, to the CDL model(s) with only a small number of “meaningful” clusters. This proposed simplification aims to simplify the testing setup by allowing a limited number of test probes to model the simplified CDL channel during the conformance testing. But the question of how to simplify the existing CDL model(s) is still open. To answer this question, some solutions were discussed in the RAN4#112 meeting which proposed to group the different clusters having AoA offset below a given threshold (e.g., ±15∘).

As the signal(s) from the test probe(s) inside the OTA test chamber are received by the UE antenna (panel) directly without any change in AoA, the number of available test probes correspond to the number of different AoAs from which the UE can receive the signals during the conformance testing. To visualize the location of each of the clusters of each of the CDL channel models (i.e., CDL A, CDL B, CDL C, CDL D, CDL E) in the angular domain from the UE point of view, we plot the clusters of each of the CDL channel models as the function of azimuth and zenith angles (AoA-ZoA) in Figure 1 and Figure 2 below, where Figure 1 shows the individual plots of each of the CDL channel model whereas  Figure 2 is a superimposed plot generated by plotting all the clusters of all the CDL channel models into a single plot.

Figure 1:Location of CDL channel model clusters in azimuth-zenith (AoA-ZoA) plane.

Figure 2: Positions of all CDL channel model clusters in a single azimuth-zenith (AoA-ZoA) plane (superimposed plot).

It can easily be seen from Figure 1 and Figure 2 that the clusters of ‘CDL A’ channel model spans the widest range in the zenith domain whereas the azimuth range of all CDL channel models is quite similar.

The clusters of ‘CDL A’ channel model spans the widest range in the zenith domain whereas the range of all CDL channel models in azimuth domain is almost similar.

The clusters of ‘CDL D’ channel model spans the widest range in the azimuth domain while the spans of the clusters of ‘CDL D’ and ‘CDL E’ channel models in the zenith domain are almost same and is narrowest relative to the other CDL models.

The AI/ML-enabled BM use case targets FR2 which indeed is a sparse channel resulting in a limited number of dominant clusters in FR2. Hence, simplified CDL model with reduced number of clusters will be a good approach for the conformance testing of this use case.

As the FR2 channel is spatially sparse in nature, simplified CDL model with reduced number of clusters will be a good approach for the conformance testing of AI/ML-enabled BM use-case.

RAN4 to decide the number of clusters and their angular location in the simplified CDL model considering the following aspects:

- Practical number of test probes to be made available in the OTA test chamber for AI/ML-enabled BM conformance testing,

- Angular location of the selected clusters should span the maximum part of the azimuth and zenith domains.

Testing Setup

The topic of testing setup for AI/ML-enabled BM use case was also discussed in the RAN4#112-bis meeting as there was some ambiguity regarding the criteria to select the Set A and Set B beams as well as the need to emulate both the Set A and Set B beams inside the test chamber. At least Set B beams need to be emulated inside the OTA test chamber during the conformance testing to provide the L1-RSRP to the DUT to perform the test inference during the inference phase, Set A beams need to be emulated inside the OTA test chamber during/before/after the conformance testing only to extract the ground truth. RAN4 in 112-bis meeting has agreed that both the Set A and Set B beams should be generated in the chamber for ground-truth extraction.

Ideally, all the beams in Set A and/or Set B should be generated without UE rotation as the similar scenario will be observed in the real deployments. However, based on the TE vendors’ input during the RAN4#112-bis meeting, all Set A and/or Set B beams can’t be generated during the conformance testing without using UE rotation as only a limited number of Tx-beam RX-PAS profiles can be emulated with a given number of test probes.

Considering the above-mentioned aspects, the following important questions need to be addressed before finalizing the design of testing setup.

- How many Set A and/or Set B beams need to be generated inside the test chamber for each test instance/interval/iteration?

- How many test instance/iteration/intervals should be defined to determine the UE pass/fail criteria of the AI/ML BM tests.

During the conformance test, the TE should generate a certain amount of channel instances/snapshots to check if the performance of the DUT can achieve a certain minimum level. Each of the channel instance/snapshot created by the TE should contain a certain number of inputs. As this feature targets FR2 channels which are sparse in nature and does not have as many as delay bin/taps as those in FR1 channels. Furthermore, the number of delay bin/taps of FR2 channels is also determined by the exact FR2 frequency of communication, Tx/Rx beam width, channel scenarios, etc. Therefore, how many beams should be generated in each test instance/interval/iteration in the test chamber is a more relevant topic to the design of the test procedure and the ability of the TE. The number of Set A and/or Set B beams to be emulated during the test is typically larger than the number of beams that can be generated by the TE. Hence, it is worth discussing that whether all Set A and/or Set B beams need to be emulated in the test chamber or some of those can be emulated.

It is not very clear whether all Set A and/or Set B beams can be emulated inside the test chamber or only some of those beams which are “most relevant” Tx-beams need to be emulated.

As the TE vendors clarified that only a limited number of beams with different AoAs can be emulated inside the test chamber, it means that multiple Set A and/or Set B beams will arrive at the DUT from the same/similar direction(s). Hence, a natural question to be asked is that whether it will be possible for a UE to distinguish between different beams which may have different Beam IDs but will arrive at the DUT from the same/similar AoA.

UE vendors to clarify if the Set A and/or Set B beams emulated inside the test chamber can’t arrive at the DUT from different AoAs, will it be possible for the DUT to distinguish between the different beams which may have different Beam IDs but will arrive at the DUT from the same AoA.

Furthermore, since in FR2 OTA testing, it is not feasible to get ideal RSRP measurements to validate predicted RSRP values, in RAN4#111, the following agreement was achieved for approximated ground truth.

Agreement: - The ground truth for the predicted RSRP is the ideal measurement of RSRP on the predicted Tx beam - In RAN4, the ground truth is the approximate as the reported RSRP measurement under the certain SNR on the predicted Tx beam - FFS on SNR level to ensure that SNR is high enough for sufficient accuracy of reported RSRP - FFS on impact of multiple-AoA test setup - FFS on the channel condition - FFS on whether the ground truth will be changing or not - Other solutions are not precluded

However, RSRP value(s) measured and reported by the UE also have several uncertainties. Apart from the non-idealities of the RF components, these uncertainties arise due to spherical coverage, UE gain variations and the allowed tolerance margin in the UE accuracy. The uncertainty factors depend on UE implementations, such as UE Rx beam codebook design, the RX beam used and the RX beam gain. In this context, NW/TE do not have any direct means to evaluate how accurate are the predictions especially if they are compared only with the UE reported measurements.

To obtain more accurate ground truth for the predicted RSRP, one potential alternative may be that Network/TE receives the approximated ground truth via direct measurements in UL. These UL measurements can be used as ground truth for downlink measurements if the beam correspondence is achieved. More specifically, the uncertainty introduced by the UE implementation (e.g., for UE measurements) are taken out when the same beams, in DL, are used for DL TX (at TE/BS) and RX (at UE) are also used in UL for TX (at UE) and RX (at TE/BS). Moreover, the accuracy of ground truth evaluation can be increased even further if the same channel used during the DL measurements are reapplied during UL transmission.

Reported RSRP values as approximated ground truth are error-prone and have several uncertainties.

To obtain a more accurate ground truth, RAN4 should consider the case where UL measurements received at TE/NW can be used as ground truth for DL measurements in case when Beam correspondence is ensured between UL and DL beams.

Conclusion

In this paper we share our views on potential RAN4 impacts from issues related to testability and interoperability of AI/ML enabled BM-Case1 and BM-Case2. Specifically, we cover following aspects:

- Generalization and test coverage related aspects

- Testing setup and Channel models

In the paper, the following Observations and Proposals were made:

Observation 1: A functionality, if tested in different generalization scenarios, would be considered not overfitted to a certain test environment.

Proposal 1: Testing of generalization aspects should also be considered to avoid overfitting of functionalities to the test environment.

Proposal 2: For the verification/testing of generalization related aspects in RAN4 for AI/ML BM, RAN4 should define different scenarios based on parameters listed in the Table below.

Table: Parameters for Generalization Scenarios

| Parameters | Description |
| --- | --- |
| UE Speed | Slow / Medium / Fast |
| SINR (Deployment Scenario) | Good / Bad Radio conditions |
| Propagation Model | TDL / CDL |

Proposal 3: RAN4 needs to design a new metric, indicative of generalization capabilities of AI/ML BM functionality, to verify the generalization performance of the functionality in different scenarios.

Observation 2: CDL channel models present a more realistic way for testing AI/ML BM use cases due to the more realistic spatial behaviour.

Observation 3: If the testability objective of AI/ML-enabled BM use case is to verify the AI/ML BM functionality taking only L1-RSRP of Set B beams as input and not to test the multipath reception of the UE, the use of TDL channel model during the conformance testing is sufficient.

Observation 4: The clusters of ‘CDL A’ channel model spans the widest range in the zenith domain whereas the range of all CDL channel models in azimuth domain is almost similar.

Observation 5: The clusters of ‘CDL D’ channel model spans the widest range in the azimuth domain while the spans of the clusters of ‘CDL D’ and ‘CDL E’ channel models in the zenith domain are almost same and is narrowest relative to the other CDL models.

Observation 6: As the FR2 channel is spatially sparse in nature, simplified CDL model with reduced number of clusters will be a good approach for the conformance testing of AI/ML-enabled BM use-case.

Proposal 4: RAN4 to decide the number of clusters and their angular location in the simplified CDL model considering the following aspects:

- Practical number of test probes to be made available in the OTA test chamber for AI/ML-enabled BM conformance testing,

- Angular location of the selected clusters should span the maximum part of the azimuth and zenith domains.

Observation 7: It is not very clear whether all Set A and/or Set B beams can be emulated inside the test chamber or only some of those beams which are “most relevant” Tx-beams need to be emulated.

Proposal 5: UE vendors to clarify if the Set A and/or Set B beams emulated inside the test chamber can’t arrive at the DUT from different AoAs, will it be possible for the DUT to distinguish between the different beams which may have different Beam IDs but will arrive at the DUT from the same AoA.

Observation 8: Reported RSRP values as approximated ground truth are error-prone and have several uncertainties.

Proposal 6: To obtain a more accurate ground truth, RAN4 should consider the case where UL measurements received at TE/NW can be used as ground truth for DL measurements in case when Beam correspondence is ensured between UL and DL beams.

- References

- R4-2417212, “WF on requirements for AI/ML air interface”, Qualcomm Incorporated, RAN4#112bis meeting, 14th – 18th October 2024, China.

- R4-2419184, RRM core requirements for AI/ML Based Beam Management, Nokia, RAN4#113, Orlando, US, 18 – 22 November 2024.