3GPP TSG-RAN WG4 Meeting #113	R4-2419186

Orlando, US, 18th – 22nd November, 2024

Source:	Nokia

Title:	On RRM Requirement Impacts of AIML Mobility

Agenda item:	7.18.2

Document for:	Discussion

Introduction

The RAN4 initiated discussions on the application of Artificial Intelligence (AI) and Machine Learning (ML) for mobility enhancements in 5G New Radio (NR) during RAN4#112bis. It was agreed to first focus the discussion to the use case of Radio Resource Management (RRM) measurement prediction. Subsequent agreements have been reached regarding other potential use cases, including measurement event prediction and Radio Link Failure (RLF) prediction.[1]:

Agreement: - RAN4 to only start the discussion of Measurement event prediction use-case and RLF use-case if sufficient progress in RAN2. - The earliest time to start the discussion for Measurement event prediction use-case is RAN4#114 meeting (Feb 2025). - The earliest time to start the discussion for RLF use-case is from RAN4#114bis meeting (Apr 2025).

Building on the above agreement, this contribution presents our perspectives and discusses open issues related to RRM requirements and performance evaluation for AI/ML-based mobility measurement prediction.

Discussion

Performance metrics

Regarding the priority of scenarios and study use-cases, following agreements have been achieved during RAN4#112bis.

Issue 1-1-2: Scenario/sub-use-case priorities Agreement: RAN4 to start the discussion for scenario 2, 3, 4. Further clarification can be discussed for the scenarios. 2; High; FR1 to FR1 intra-frequency temporal domain case B; 1st goal; Intra-cell. 3; High; FR1 to FR1 inter-frequency (frequency domain); 1st goal; Inter-cell. 4; High; FR2 to FR2 intra-frequency temporal domain case A; 2nd goal; Intra-cell. Agreement: - Scenario 2, 3 and 4, the 3 cases can be discussed: - Sub-use case 1: L1 beam-level measurement result(s) is predicted based on actual L1 beam-level measurement result(s) and then L3 cell-level measurement result is generated - Sub-use case 2: L3 Cell-level measurement result(s) is predicted based on actual L3 cell-level measurement result(s) - Sub-use case 3: L3 Cell-level measurement result(s) is predicted based on actual L1 beam-level measurement result(s) - Follow the latest definition of the sub use-cases in RAN2. Further clarification on the sub use-cases from RAN4 perspective can be discussed.

In addition, as outlined below, further investigation is needed on the performance metrics used to evaluate the effectiveness of AI/ML measurement predictions, as discussed in the previous meeting.

Issue 2-1-1: Performance Metrics/KPIs for Measurement Prediction use case Agreement: - RAN4 to first study the impacts on requirements for the following case: - L3 cell level RSRP (Point C) prediction - FFS L1 beam-level (Point A1) - FFS L3 beam-level (Point E) - Other cases can be considered based on RAN4 further discussion and RAN2 further progress.

Accordingly, RAN4 has agreed to focus on studying L3 cell-level RSRP (Point C) for evaluating measurement prediction. Additionally, two other options—L1 beam-level (Point A1) and L3 beam-level (Point E)—remain open for further discussion. For L1 measurements, as outlined in sub-use cases 1 and 3, the UE can either derive L3 cell-level measurements from predicted L1 measurements or directly predict cell-level measurements based on actual L1 measurements. Since L3 cell-level measurements and predictions are expected to be reported by the UE or used by the network for RRM and handover decisions, it would be judicious to consider these as the primary evaluation point for performance metrics. As L3 beam-level measurements could further optimize mobility, they can be examined after the study of cell-level measurements or undergo additional evaluation by RAN2.

Figure 1: 3GPP TS 38.300 Measurement Model

Additionally, considering both L3 beam-level and L3 cell-level measurements, RAN2#127 and RAN2#127bis have reached agreements prioritizing L3 cell-level measurements for simulation and performance evaluation.

RAN2#127 FS_NR_AIML_Mob - Release 19 Agreements - Companies can consider to do L3 filtered beam level results for any of this cases. L3 filtered beam level prediction cases are lower priority. - Case 1: To predict L1 filtered beam level results, then generate L3 filtered results based on the predicted L1 beam results. - Case 2: To directly predict L3 filtered beam level results based on the L3 beam level measurement results. - Case 3: To directly predict L3 filtered beam level results based on the L1 beam level measurement results. - If companies do L3 filtered beam level prediction simulations, they should focus on FR2-to-FR2 intra-frequency temporal domain prediction case A

RAN2#127bis FS_NR_AIML_Mob - Release 19 Agreements - To keep two filtering options on the table and up to company to report. - One fixed sampling period of FR2 is introduced for L1/L3 filtering option 1 to replace existing one i.e., 20ms. The detail value is FFS. - In the definition of 3 RRM sub-cases, all cell level measurement result(s) refers to L3 filtered cell level measurement

In all 3 agreed sub-use cases, L3 cell-measurement is predicted (generated) and considered for RRM.

Beam-level L3 predictions are considered with low priority in RAN2 FS_NR_AIML_Mob - Release 19.

RAN4 to focus first on L3 cell level RSRP (Point C) prediction and subsequently on L3 beam-level prediction (Point E).

For L3 cell-level RSRP (Point C) prediction, as agreed in RAN4, measurement prediction can be based on three sub-use cases: (i) predicted L1 beam-level measurement results derived from actual L1 measurements (Sub-use case 1), (ii) actual L3 cell-level measurement results (Sub-use case 2), or (iii) actual L1 beam-level measurement results (Sub-use case 3). Since the accuracy of AI/ML model outputs is highly sensitive to the choice of sub-use case as well as the quality, quantity, and characteristics of the inputs, it is essential to study and potentially prioritize one of the agreed sub-use cases. Based on system-level simulation evaluations conducted in RAN2, the majority of companies have indicated that cell-level prediction based on actual L3 cell-level measurements provides the highest accuracy in Mean Absolute Error (MAE) performance.

The performance of AI/ML depends on the selection of sub-use cases used to predict cell-level measurement.

According to RAN2 observations, L3 cell level RSRP (Point C) prediction based on actual L3 cell-level measurement result(s) (i.e., Sub-use case 2) provides the highest MAE accuracy.

RAN4 first study performance metrics/KPIs for target cell L3 cell level RSRP prediction based on actual serving cell L3 cell-level measurement result(s) (i.e., Sub-use case 2).

Temporal Domain Prediction

In the area of temporal domain prediction, RAN2 has conducted extensive analyses and simulations to evaluate the performance of various techniques. It was agreed that Mean Absolute Error (MAE) is an appropriate metric for quantifying prediction accuracy. The evaluation results have yielded the following observations:

- Higher UE speeds results in larger prediction errors

- For short observation window (OW), increasing the OW length can enhance prediction accuracy. Once the OW exceeds a certain threshold, further increases do not yield significant benefits.

- longer prediction window (PW) correlates with decreased prediction accuracy.

- Majority of companies observed that sub-use case 2, i.e., L3 – L3 prediction, demonstrates the highest prediction accuracy

- UE may be able to provide multiple RSRP value(s) and/or average RSRP value over the entire prediction window

Performance of temporal prediction depends on many input/output factors, e.g., UE speed, observation window, prediction window length, number of input beams, L3 filtering, etc.

RAN4 to study the minimum requirement/characteristic of the input and output of AI/ML model for RRM and testability assessment of temporal domain scenarios.

Regarding temporal domain Case-B, two implementation examples are illustrated in Figure 3. The design target for Case B focuses on measurement reduction, where measurements of previously predicted samples within the prior Prediction Window (PW) are skipped in the new predicted window. This is achieved using either a sliding window or a sample-and-hold method, allowing for significant measurement reduction (e.g., up to 50%, as shown in Figure 3).

Figure 3: Illustration of temporal domain prediction – case B (example 1 at the left, example 2 at the right).

It is expected that the performance of different approaches to be different and dependent on several factors such as input samples, reductions patterns, predicted window length, etc. Some of the factors need standardization and some need RAN2 signaling for coordination between UE and the network.

There are two implementation examples of temporal domain prediction – case B.

RAN4 to study and if needed, prioritize one implementation example of temporal domain prediction – case B for RRM performance/requirement evaluation.

Frequency domain scenario

One of the high-priority study items for AI/ML in NR inter-cell mobility is inter-cell inter-frequency measurement prediction. Inter-frequency measurements require configuring measurement gaps, during which the UE must temporarily stop communicating with the serving cell to measure other frequencies. This process can lead to service interruptions and increased power consumption. AI/ML techniques can help reduce the number of inter-frequency measurements a UE needs to perform, thereby minimizing the need for continuous measurement and enhancing UE energy efficiency.

By utilizing measurement data from two frequency layers, an ML model can be trained to understand the correlation between radio measurements across these layers. Specifically, the model can either predict first-layer measurements based on second-layer measurements or determine if a UE connected to a macro layer can connect to a micro layer. This may involve a clustering approach, where measurements from multiple cells serve as inputs for prediction. A time series of measurements, along with additional inputs, may also be considered. Consequently, for inter-frequency measurement prediction, both the input data and model performance metrics (e.g., accuracy) differ from those used in temporal domain predictions

The input data and characteristics of predicted measurement in inter-cell inter-frequency prediction may be different from temporal domain predictions.

RAN4 to study the minimum requirement/characteristic of the input and output of AI/ML model for RRM and testability assessment of inter-cell inter-frequency prediction domain scenarios.

Measurement Prediction Accuracy

For predicted measurement, one of the key issues which needs to be investigated is predicted measurement accuracy. Based on the current RAN2 progress on AI/ML for Mobility, for measurement prediction accuracy Mean Absolute Error (MAE) which is the average difference between the predicted values and the actual values (e.g., predicted L3 filtered cell-level measurement result and actual L3 filtered cell-level measurement) has been agreed to be used as the performance metric.

However, in RAN4 AI/ML for Air Interface, for AI/ML BM use case, single or a combination of options listed in Table 3 are under investigation for accuracy.

Table 3: RAN4 metric options for beam management in AI/ML BM use case

- Option 1: RSRP accuracy - Option 2: Beam prediction accuracy - Top-1 (%) : the percentage of "the Top-1 strongest beam is Top-1 predicted beam" - Top-K/1 (%) : the percentage of "the Top-1 strongest beam is one of the Top-K predicted beams" - Top-1/K (%) : the percentage of "the Top-1 predicted beam is one of the Top-K strongest beams" - Option 3: The successful rate for the correct prediction which is considered as maximum RSRP among top-K predicted beams is larger than the RSRP of the strongest beam – x dB, - Related measurement accuracy can be considered to determine x - Option 4: combinations of above options

For measurement prediction, different accuracy metrics are considered/under discussion in RAN2 SI for AI/ML mobility and in RAN4 for AI/ML BM use case.

RAN4 to study prediction accuracy for the agreed scenarios.

Impacts of measurement errors

Followings capture candidate options from the moderator’s summary of RAN4#112bis [2] meeting regarding the impacts of measurement errors on the prediction accuracy performance.

Issue 2-1-2: Impacts of measurement errors on the prediction accuracy performance Candidate options: - Option 1: RAN4 to study the prediction accuracy performance with consideration of the impact of measurement error. - Option 2: suggest RAN2 to take into account the measurement errors in RAN2 evaluation. - Option 3: Wait for RAN2 evaluation results and if RAN2 doesn’t consider measurement errors then RAN4 should do it after RAN2 evaluations. - Option 4: Wait for more progress in AI/ML BM to study the impact of measurement error on the prediction accuracy requirements for AI mobility

In mobility scenarios, errors in measurements which may be used as the input or ground truth for AI/ML systems is unavoidable, this can significantly impact network performance by leading to inaccurate predictions that may affect mobility management, handover optimization, and radio resource management (RRM). Hence the impacts of the measurement errors on the prediction accuracy should be studied and evaluated. This is now under discussion in RAN2 and hence RAN4 may wait for the RAN2 agreement and initiate the discussion based on RAN2 agreements/evaluations.

Additionally, as noted in the RAN4#112bis meeting, the AI/ML for Beam Management group has agreed to study the impact of measurement errors in inference inputs and training datasets (specifically, Set B measurement errors) on prediction accuracy. RAN4 may also leverage insights from the outcomes of these discussions.

RAN4#112bis; AI/ML for beam management Agreement: RAN4 will study the impact of measurement error for the input to inference and dataset for training(set B measurement error) on prediction accuracy, companies should bring proposals on how to proceed with such a study. - impact on different prediction accuracy metrics should be taken into account - studies already performance by other groups(RAN1) could also be taken into account Note: intention of the study is just to understand the impact of measurement error on prediction accuracy.

The impact of measurement error has been agreed to study in RAN4 AI/ML for beam management, and it is still under discussion in RAN2 AI/ML for mobility.

To study the impact of measurement error, RAN4 should wait for evaluation outcomes of RAN2 AI/ML for mobility (option3) and RAN4 AI/ML for beam management (option 4).

Impacts on Core requirements

Following options from moderator summary [2] need further discussions on the core RRM requirements.

Impacts on Core requirements - Candidate options: - Option 1: For intra-frequency intra-cell prediction, RAN4 to study the RRM impact of observation window and prediction window defined in RAN2 - Option 2: For inter-frequency inter-cell frequency domain prediction, RAN4 to study the side condition of frequency prediction (e.g., EPRE difference). - Option 3: Wait for more progress in AI/ML BM to study the impact on measurement report delay requirements for AI mobility - Option 4: RAN4 not to discuss impacts on core requirements until RAN2 has a stable mechanism for AI/ML assisted mobility. - Option 5: RAN4 to study if HO requirements including delay, interruption, side conditions and accuracy are needed to be updated for AI/ML-assisted mobility

Currently, discussions are ongoing in both RAN2 AI/ML for mobility and RAN4 AI/ML for beam management. RAN2 is focusing on simulation results, the configuration of RRM temporal prediction, the impact of L3 filtering, and input errors. In RAN4 AI/ML for beam management, several issues are being addressed, including the impact of measurement errors, approximated ground truth, requirements related to candidate beam detection, KPIs for beam prediction, and LCM-related requirements. The findings and agreements from both RAN2 and RAN4 can be leveraged by RAN4 to enhance and optimize the study of impacts on core requirements. Given the limited time allocation in RAN4, RAN4 may prioritize the RAN2-focused scenarios to accelerate progress on defining RRM core requirements

For RAN4#113 meeting, RAN4 to wait for further progress from RAN2 mobility (Option#3) and RAN4 AI/ML for beam management for studying the impact on core requirements (Option#4).

Conclusion

In this paper we provide our view on the Rel-19 AI/ML mobility scenario and RRM requirements to be studied in RAN4 working group.

The following Observations and Proposals were made:

Observation 1: In all 3 agreed sub-use cases, L3 cell-measurement is predicted (generated) and considered for RRM.

Observation 2: Beam-level L3 predictions are considered with low priority in RAN2 FS_NR_AIML_Mob - Release 19.

Proposal 1: RAN4 to focus first on L3 cell level RSRP (Point C) prediction and subsequently on L3 beam-level prediction (Point E).

Observation 3: The performance of AI/ML depends on the selection of sub-use cases used to predict cell-level measurement.

Observation 4: According to RAN2 observations, L3 cell level RSRP (Point C) prediction based on actual L3 cell-level measurement result(s) (i.e., Sub-use case 2) provides the highest MAE accuracy.

Proposal 2: RAN4 first study performance metrics/KPIs for target cell L3 cell level RSRP prediction based on actual serving cell L3 cell-level measurement result(s) (i.e., Sub-use case 2).

Observation 5: Performance of temporal prediction depends on many input/output factors, e.g., UE speed, observation window, prediction window length, number of input beams, L3 filtering, etc.

Proposal 3: RAN4 to study the minimum requirement/characteristic of the input and output of AI/ML model for RRM and testability assessment of temporal domain scenarios.

Observation 6: There are two implementation examples of temporal domain prediction – case B.

Proposal 4: RAN4 to study and if needed, prioritize one implementation example of temporal domain prediction – case B for RRM performance/requirement evaluation.

Observation 7: The input data and characteristics of predicted measurement in inter-cell inter-frequency prediction may be different from temporal domain predictions.

Proposal 5: RAN4 to study the minimum requirement/characteristic of the input and output of AI/ML model for RRM and testability assessment of inter-cell inter-frequency prediction domain scenarios.

Observation 8: For measurement prediction, different accuracy metrics are considered/under discussion in RAN2 SI for AI/ML mobility and in RAN4 for AI/ML BM use case.

Proposal 6: RAN4 to study prediction accuracy for the agreed scenarios.

Observation 9:  The impact of measurement error has been agreed to study in RAN4 AI/ML for beam management, and it is still under discussion in RAN2 AI/ML for mobility.

Proposal 7: To study the impact of measurement error, RAN4 should wait for evaluation outcomes of RAN2 AI/ML for mobility (option3) and RAN4 AI/ML for beam management (option 4).

Proposal 8: For RAN4#113 meeting, RAN4 to wait for further progress from RAN2 mobility (Option#3) and RAN4 AI/ML for beam management for studying the impact on core requirements (Option#4).

- References

- R4-2416855, WF on AI/ML Mobility, Nokia, Hefei, Anhui, China, 14th – 18th October 2024.

- R4-2415658, Topic summary for [212bis][211] FS_NR_AIML_Mob, Nokia, Hefei, Anhui, China, 14th – 18th October 2024.