3GPP TSG-RAN WG4 Meeting #113                                        R4-2419188

Orlando, US, 18th – 22nd November, 2024

Source:	Samsung

Title:              Discussion on AI mobility general aspects

Agenda item:			7.18.2

Document for:	Discussion

1. Introduction

In last meeting, RAN4 had some discussions on AI mobility general aspects. In this meeting, we will give some further discussions for some open issues captured in the agreed WF [1].

2 RAN2 agreements update

The latest progress for RRM measurement prediction is duplicated as below

< Agreements > - The System level performance (e.g. HO performance) evaluation is optional (i.e. companies can bring results if they chose). - System level performance for measurement event prediction can be prioritized by companies if they chose to do it. - RAN2 will prioritize discussions on intermediate KPI discussion before discussing system level performance for the corresponding use case. 3 Discussion on what (type of information)/how generalization study can take place in Nov. meeting 4 The SLS simulation assumption discussion is covered in the post#127bis email discussion by assuming: - The simulation assumptions agreed for measurement event prediction and RLF prediction is taken as baseline for SLS in principle - The HO model in 36.839 is taken as baseline - The HO performance will be HOF and number of HO only and definition in 36.839 is taken as baseline - The baseline of HO performance is R15 legacy measurement and HO procedure. < Agreements on simulation table > 1: The spreadsheets are organized into three separate folders, with each folder corresponding to one of the three use cases. 2: The RRM prediction use case is used as a template for the documentation process. This approach will be similarly applied to the measurement event prediction and RLF/HOF prediction use cases, the spreadsheets of which are subject to revision upon availability of the simulation results. 3: Individual spreadsheet for each identified scenario for the use case of RRM prediction is created, e.g., scenarios 1~6 with the understanding that we can add more spreadsheets as required e.g., when other scenarios are identified. 4: Distinct sheets are initially set up for capturing the simulation assumptions, evaluated KPIs and definitions, and simulation results from companies, with the understanding that we will add more sheets as needed and in accordance with discussions that emerge during the evaluation process. 5: The columns in the simulation results sheet are categorized into five main groups: general information, variable settings, selectable simulation assumption, model-related information and performance metrics for various KPIs. 6: The ‘case’ column considers the three sub-use cases and their combination with additional factors. One colume for additional factors will be introduced. Those factors can be determined through discussions as the evaluation progresses. 7: Adopt the example spreadsheets provided in the attachment as a starting point, understanding that their content is flexible and can be modified as the evaluation progresses. 8: Create a folder on the 3GPP FTP server for companies to upload their simulation results. Within this folder, create individual subfolders for different use cases. For each use case different subfolders under the main directory corresponds to the different identified scenairos.The file name of the excel table follows the format: 'MeetingNumber_CompanyName_TdocNumber_version number'. 9: Mediatek will provide example template, structure and rules to follow. Companies are expected to follow these examples.. < Agreements > 1 It is mandatory to follow the following rules for filling out the table for simulation results: - Adhere to the format provided in the example, except for the specified columns. The columns 'Other Factors,' 'AI Model Type,' 'Details of AI Model,' and 'Non-AI/Simple AI Method' do not have strict content restrictions. - Keep the same parameter units as the template provided. - Companies are not required to fill in all the information, e.g. some performance metrics. If companies can’t provide the information, please leave the cell blank. NOTE: The rapporteur will not include the inputs if these rules are not followed 2 Adopt the agreed spreadsheet (after email discussion) examples of different RRM prediction scenarios to capture companies’s simulation results.. 1 For intra-frequency temporal domain, higher UE speeds result in larger prediction errors 2 Initially, increasing the OW length can enhance prediction accuracy in the temporal domain case A, especially when the OW is relatively short. However, once the OW exceeds a certain threshold, further increases do not yield significant benefits. Conversely, for PW, longer durations correlate with decreased prediction accuracy. RAN2 will not define the actual threshold and fast fading assumption. 3 Majority of companies observe that among sub cases 1, 2, and 3, at least with shorter prediction window sub case 2 demonstrates the highest prediction accuracy 4 Companies can provide multiple real time RSRP value(s) and/or average RSRP value over the entire window and should indicate in their simulation results what they have used. The companies should at least provide the results of only one value it should be the last value at the end of the PW. We will add two columns in the spreadsheet to capture the last value and the average value. 5 Companies need to report whether earlier predicted results are also used as inputs for future RRM prediction. < Agreements on inter-frequency > 6 Companies should report with their simulation the correlation coefficient 7 Higher-to-lower and lower-to-higher frequency prediction is comparable 8 For co-located scenario, the UE speed in the inter-frequency case has minor impact on prediction accuracy.

3 Observations

3.1 Multi-dimensional discussions for AI/ML mobility

Based on our agreements and RAN2 discussions, from our understanding, actually at least 5-dimensional discussions are ongoing for AI mobility

- One-dimensional discussion: Scenarios

- Two-dimensional discussion: Sub use-cases

- Three-dimensional discussion: L1/L3 filtering, i.e., Sliding/Non-sliding filtering

- Four-dimensional discussion: Observation and prediction pattern

- Five-dimensional discussion: Speed

3.1.1 One-dimensional discussion: 3 Scenarios

In last meeting, RAN4 agreed to put the following scenarios as the starting point:

| 2 | High | FR1 to FR1 intra-frequency temporal domain case B | 1st goal | Intra-cell |
| --- | --- | --- | --- | --- |
| 3 | High | FR1 to FR1 inter-frequency (frequency domain) | 1st goal | Inter-cell |
| 4 | High | FR2 to FR2 intra-frequency temporal domain case A | 2nd goal | Intra-cell |

To be specific, for #4, the target study of the scenario is to study AI/ML measurement prediction algorithm that can enhance HO performance. The measurement in the future is predicted based on historical ones of the same cell. For #2, the target is to reduce measurement overhead. The input is historical measurement values and the output is the values at the subsequent time instances that measurements are skipped. For #3, the target is to reduce the measurement overhead, especially the measurement gap needed for the measurements on other frequency bands. For the AI model in #3, the model takes the RSRP of ell X at frequency f1 (e.g., 2 GHz) as input to predict the RSRP at frequency f2 (e.g., 4 GHz) (or use 4 GHz to predict 2 GHz) of the inter-frequency cell Y at the same location and the same site. Both cell-specific and/or cluster approaches can be considered.

    - Two-dimensional discussion: 3 Sub use-cases

- Scenario 2, 3 and 4, the 3 cases can be discussed: - Sub-use case 1: L1 beam-level measurement result(s) is predicted based on actual L1 beam-level measurement result(s) and then L3 cell-level measurement result is generated - Sub-use case 2: L3 Cell-level measurement result(s) is predicted based on actual L3 cell-level measurement result(s) - Sub-use case 3: L3 Cell-level measurement result(s) is predicted based on actual L1 beam-level measurement result(s) - Follow the latest definition of the sub use-cases in RAN2. Further clarification on the sub use-cases from RAN4 perspective can be discussed.

Based on the measurement model defined in TS 38.300, the significant differences of Sub use-case 1 and Sub uses-case 2 is that the position of beam consolidation/selection and Layer 3 filtering. Besides, for Sub use-case 2, which approach: cluster or cell-specific is more outstanding is still suspending. For Sub use-case 3, no such consolidation and L3 filtering. The L3 filtered cell-level measurement results are predicted based on actual L1 beam-level measurements, and the AI model may additionally need to extract the beam consolidation/selection features.

3.1.3 Three-dimensional discussion: Sliding/Non-sliding filtering

- RAN2 agreed two L1/L3 filtering options: Sliding and non-sliding L1/L3 filtering [TR 38.744]

- L3 filtering applied to beam consolidated measurements is defined in TS 38.331 5.5.3.2 with agreed FilterCoefficient = 4. But how to perform L1 filtering is UE implementation currently, and what is the corresponding/subsequent impact on prediction is not standardized.

- Definition of L1/L3 filtering

  - The non-sliding filtering stipulates that the filtered L3 RSRP are produced per measurement period, which is specified by RAN4, and that means one L1/L3 filtered RSRP value can be derived by X L1 samples, e.g., 5 in RAN2. The sliding filtering stipulates that the filtered L3 RSRP are produced per sampling period, or sampling period* UE Rx beam number for FR2 scenarios. The illustrations are copied as below

L1/L3 sliding filtering                              L1/L3 non-sliding filtering

- The PW/OW length vs the sliding pattern for Sub use-cases 1/2/3

  - The PW length is the L1 sample period multiplied by the number of L1 samples, e.g., 40ms*N for Sub use-case 1; The PW length is the measurement of L3 filtered RSRP measurement period multiplied by the number of L3 filtered RSRP, e.g., 200ms*N for Sub use-case 2/3. For all the cases, N related to MRRT, e.g.,   MRRT of 1/2, N=1.

  - From sliding OW or non-sliding OW perspective, Case A can be divided into

Thereinto, Case A with considering sliding window is a typical setting for Goal 2 since no measurement reduction in this pattern

    - Four-dimensional discussion: Observation and prediction pattern

- Two temporal domain case examples of case A

- The length of OW and PW can be different or not based on various OW/PW ratio, which is agreed in RAN2 that {5,4,3,2,1,1/2,1/3,1/4,1/5}.

- For the AI/ML prediction model, the continuous history measurement instances (L3 cell-level RSRP) in OW can be used to predict continuous future results in PW without any measurement reduction

- Two temporal domain case examples of case B

Thereinto, the “grey” block in example 2 is previous predicted measurement result(s) but in OW; Example 1: The input instances within the observation window are temporally continuous measurements; Example 2: The observation window includes interleaved instances (in other words predicted occasions) that are skipped for measurement.

- Observation and prediction for different MRRT for case B prediction

- The number of predicted/actual measured RSRP results within the PW/OW are different, which impacts the prediction performance of AI model

- If the MRRT ratio becomes large, the OW becomes large accordingly if there are two actual measurement results are contained within the OW

At the same time, problem for the skipped pattern is that: The bigger the distance between the measured instance and the predicted instances, the harder the prediction will be

    - Five-dimensional discussion: Speed

The prediction accuracy with UE speed (30,60,90,120 Km/h) variation are extensively discussing under differenct scenarios/Sub use-cases. So far, RAN2 concluded that for intra-frequency temporal domain, higher UE speeds result in larger prediction errors, and for co-located scenario, the UE speed in the inter-frequency case has minor impact on prediction accuracy. Overall,

Observation 1: For RRM measurement prediction, at least 5 dimensional discussions can be involved in AI mobility, for each dimension, multiple elements are contained. The 5 dimensions can be:

- Scenarios

- Sub use-cases

  - Including: Cell-specific and/or cluster approaches

- Sliding/Non-sliding filtering

- Observation and prediction pattern

  - Including: OW length/PW length; MRRT ratio

- Speed

Actually, based on the simultation results, the variation in each element in each dimension can indirectly/directly affects the prediction accuracy. However, considering full combinations of above all dimensions to define RRM requirements are scary and unnecessary. In this sense, it is necessary to down select and focus on some/one of them to define requirements but how to down select the scope should depends on the RAN2’s process, agreements and prediction gain analysis.

Proposal 1: For RRM measurement prediction requirement definition, suggest RAN4 to discuss the priorities of, at least, scenarios, sub use-cases, sliding/non-sliding filtering, observation and prediction pattern, and speed.

- How to combine all the dimensions and down select the scope should takes RAN2’s processes, agreements and the prediction performance gain analysis into consideration

- Discussions

4.1 General aspects

In our opinion, the pivotal role of AI in BM and mobility shall be the same, that is be trained to learn the correlation in features and to predict future measurements, so the most general issues/requirements discussed in Rel-18 AI/ML RAN4 also need to be considered in Rel-19 AI mobility RAN4. For example, the discussions on, baseline/benchmark performance, requirements and tests for training/inference definition, feasibility of requirements/tests for LCM evaluation, requirements for data collection, high level testing framework, AI/ML model generalization, general definition of model complexity. However, considering so far, AI BM only convers FR2 (spatial, spatial+temporal), we have

Proposal 2: The general aspects part of RAN4 RRM work on AI/ML based beam prediction in Rel-18 could be reused for AI/ML based RRM measurement prediction as much as possible

- RAN4 to discuss what general aspects could be reused to avoid the duplicate study

Furthermore, in the scope of the SID, both UE side model and NW side model measurement predictions are included. From our understanding,

- For the UE side model case, NW may configure UE to perform measurement prediction and to report the predicted measurement results (i.e., the output of the AI/ML model). In this case, the AI/ML model inference will be performed by the UE.

- For the NW side model case, NW may configure AI specific signalling. In this case, the AI/ML model inference will be performed by the NW, but the necessary input data of the AI model (e.g., historical measurement results and/or assistance information) should be generated by UE and reported to NW.

Obviously, the following Alt.s can be investigated:

- AI/ML model training and inference at NW side

- AI/ML model training and inference at UE side

Since no concrete conclusions are achieved for the time being. From this,

Proposal 3:  Based on RAN2’s progress and agreements to determine what kind of AI model (i.e., UE side model or NW side model) could be considered to define the AI/ML based requirement for RRM measurement prediction in RAN4

4.2 Specific issues related to use cases for AI/ML mobility

4.2.1 Performance Metrics/KPIs for Measurement Prediction use case

Based on the observations, a lot of issues for cell level measurement prediction, e.g., non-AI benchmark, the evaluation methodology of certain scenarios, KPIs for measurement prediction, the dataset generation, and input/output of the direct/indirect AI model had been discussed in RAN2, some of them were solved with clear conclusions. Even the simulation results evaluation for different cases based on simulation assumptions has begun in RAN2. To avoid the overlapping of similar discussions in RAN2/RAN4, from our understanding, at least

Proposal 4: The performance Metrics/KPIs for Measurement Prediction use case considered in RAN4 shall subject to RAN2’s processes and agreements

From our understanding,

- It is necessary to define a L3 cell level RSRP prediction accuracy requirements. And based on the following clarifications in TR,

## 5.2 RRM measurement prediction ### 5.2.1 Evaluation methodology, metrics and assumptions Measurement prediction accuracy for cell-level RRM measurement prediction is defined as average L3 RSRP difference between predicted L3 filtered cell-level measurement result and actual L3 filtered cell-level measurement result of the same cell for all RRM sub-use cases.

it is very reasonable to define L3 measurement prediction accuracy requiremente at point C

- However, regarding whether to define accuracy requirement at point A1. From our understanding,

  - First, if L1 beam level prediction (Sub-use case 1) is considered, it is better to define accuracy requirements at A1, since the L3 filtered RSRP will be derived based on measured L1 beam results and the predicted beam L1 results. However, the predicted beam qualities are different from measured beam qualities, in this sense, the beam measurement prediction accuracy should be defined to ensure the AI model can learn the best and can provide the acceptable and accurate predictions directly. And to define the accuracy at A1 actually aligns with our traditional non-AI L3 RSRP requirement definition logic.

  - Besides, we have some concerns that if we to define accuracy requirement at A1, whether it is necessary to define an additional prediction accuracy requirement [on top of the requirement at C]. Since 1) For cell-level prediction, the RSRP prediction accuracy threshold and can be large (whether the threshold is related to QoE/throughput is under discussion), where the threshold is used to facilitate an analysis of the prediction accuracy (The threshold can be used for classification or to judge the prediction accuracy is necessarily good or bad); 2) If to define accuracy requirement at A only, we are not sure, at least before enough evaluation, whether the direct predicted cell results and the results combining predicted beam results and subsequent processing are fairly comparable due to the uncertain intermediate processes

- While, regarding whether to define accuracy requirement at E, we suggest to deprioritize the case, since the model may need to learn the logic of the beam consolidation mechanism, which is different from the ones discussed for A1/C

Proposal 5:

- RAN4 to first study the impacts on requirements for L3 cell level RSRP (Point C) prediction

  - How to define the accuracy requirements, e.g., RSRP prediction accuracy/performance degradation threshold, the applicability of the accuracy, i.e., percentage in L3 AI mobility prediction evaluation

- If RAN4 to study the impacts on requirements for L1 beam-level (Point A1)

  - Whether to/How to define an additional prediction accuracy requirement on top of the requirement at C

- RAN4 to deprioritize studying the impacts on requirements for L3 beam-level (Point E) prediction

For the side condition consideration, since the OW length is extremely important for the temporal-domain prediction. For example, if the OW length is too short, the simulation accuracy may be decreased since there are no enough samples/input date can be trained, while if the OW length is too large, the improvement in accuracy is not significant and may even decrease, since the RSRP measured long time ago has already meaningless due to the fast fading characteristic. Besides, based on simulation analysis and agreements, RAN2 found that under some cases, once the OW length exceeds a certain threshold, further increases do not yield significant benefits, while RAN2 will not define the actual threshold.

Proposal 6: For the side condition consideration, RAN4 to discuss whether to/how to define a certain threshold for OW length to evaluate the L3 measurement prediction accuracy for the Sub use-cases

4.2.2 Impacts of measurement errors on the prediction accuracy performance

From RAN2 agreements on channel modelling perspective, for RRM Sub use-case 1 and 3, fast fading is considered.  However, based on our observations and understandings, so far, all the RAN2 simulations are based on ideal conditions. Considering the channel environment, the measurements taken of the channel, especially the fast fading, always contain a certain amount of error, which should be considered as one of the inputs to the AI/ML model. But how to model the error distribution and how to combine the error with filtering operation for FR1, we think, is AI mobility specific issues since AI BM only covers FR2 (spatial domain predicition, spatial domain+temporal domain predicition) at the current stage. In this sense, there is no guarantee that whether the error consideration added to the input measurements and ground truth in AI BM can be accounted in AI mobility before any clear conclusions achieved in such WI.

Proposal 7: RAN4 to study the prediction accuracy performance with consideration of the impact of measurement error

- For FR2 temporal domain prediciton, suggest to wait for more progress in AI/ML BM to study the impact of measurement error on the prediction accuracy requirements for AI mobility

- For FR1 temporal and frequency domain predictions, suggest RAN4 to discuss how to evaluate the impacts of measurement errors on the prediction accuracy performance for AI mobility

Since the simulation assumption including channel models, UE trajectory, simulation results alignment and evaluation methodology are extensively discussed in RAN2, we propose

Proposal 8: The simulation parameters for verifying the feasibility of requirement/testing in RAN4 should follow RAN2’s progress

4.2.3 Impacts on Core requirements

RRM impact of the OW and PW

For the sliding case, the L1/L3 measurements are updated every sampling period, e.g., 40 ms in RAN2. For non-sliding filtering, the L3 filtered results are generated, in other words, the OW and PW move forward with every measurement period, which are different between FR1 and FR2. In RAN2 simulations, 200 ms for FR1 and 400 ms for FR2 are assumed. Based on such assumptions, the window length should be multiple times of the measurements period. If we consider MRRT further, the window length will be also related to N (the number of L1/L3 filtered RSRP). However, the 200 ms/400 ms is the minimum timing granularity for RAN4 FR1/FR2 intra-f measurement period, let alone several factors, such as DRX cycle, MGRP, SMTC and CSSF for intra-f are not considered in RAN2 measurements period assumption.

But the thing is no matter sliding or non-sliding case, the window length is so important for the temporal-domain prediction, which can directly affect the number of samples (with the same MRRT) that can be used as AI model input for training, and the number of samples needed to predict, which will significantly influence the prediction accuracy. In this sense, from our understanding, for RRM requirement discussion, the principle of determining OW and PW based on measurement period defined in RAN2 can be applied in RAN4, but the RRM impact of OW and PW for scenarios 2 and 4 (FR1/FR2 intra-frequency temporal domain case A/B) should be discussed.

Proposal 9: RAN4 to discuss the RRM impact of OW and PW for scenarios 2 and 4 (FR1/FR2 intra-frequency temporal domain case A/B)

- The principle of determining OW and PW based on measurement period defined in RAN2 can be applied in RAN4

RRM impact of HO requirements

From our understanding, the PW is used for mobility event and/or handover determination by the UE/network. And the ultimate aim for 1st goal is not to evaluate measurement overhead reduction, but to, take NW-side mode as example, recover the skipped RRM measurements based on the predicted results what UE reports in order to mitigate the occurrence of unintended events (e.g., ping-pong HO, too late/early/wrong HO, short ToS, RLF due to too late HO, etc.) or to make accurate HO decision by foreseeing the RRM status of UE in mobility. From this, the measurement reduction should keep the mobility performance from degrading.

Furthermore, from our understanding, comparing to the legacy HO mechanism that NW decides HO after Event A3 trigger conditions are fulfilled during TTT, using AI/ML event predictions may reduce the HO execution time to some extent.

Proposal 10: RAN4 to study if HO requirements including delay, interruption, side conditions and accuracy are needed to be updated for AI/ML-assisted mobility

RRM impact of measurement occasions reduction

For our understanding, for inter-frequency prediction, RAN4 can to study the impact on MG reduction in measurement period/scheduling restriction

Proposal 11: RAN4 to discuss the RRM impact of measurement gap reduction in measurement period/ scheduling restriction for inter-frequency prediction

4.3 LCM related requirements

An AI/ML model needs to be developed, deployed, and managed during the entire lifecycle. Taking a look at the 3GPP AI/ML functional framework for the NR air interface, below, we can find that 1) Core functions of the framework contain: data collection, model training, management, inference, and model storage; 2) The AI/ML model LCM procedures need the following model operations: Data collection, Model monitoring, Model selection/deactivation/switching/fallback, Model inference, Model training/update, Model transfer/delivery and Model storage. Besides, there are two distinct methods for managing LCM: functionality-based LCM and model identity based-LCM. Both of them are considered in Rel-18 AI/ML RRM.

3GPP AI/ML functional framework for the NR air interface.

The overall procedure of AI/ML model LCM was discussed in Rel-18 AI/ML BM RAN1, and potential AI mobility specific enhancement, e.g., LCM was agreed in RAN2 scope.

In Rel-18 AI/ML for NR air interface, the necessity and feasibility of defining requirements/tests for LCM was extensively discussed and confirmed. And during the discussions, the agreements on LCM related requirements/functional tests were achieved sequentially. Based on the WFs agreed in Rel-18, the following LCM related requirements are agreed to define based on the specific BM and positioning use cases:

Issue 1-5: LCM Requirements Agreement: - RAN4 considers defining RRM requirements for LCM procedures based on the specific use cases (beam management and positioning), and candidates are: - identification - selection - activation - deactivation - switching - fallback to non-AI operation - performance monitoring - Others are not precluded

And in the latest meeting, companies agreed that the LCM would be tested.

Issue 1-3: Testing goals Agreement: - The testing goal is to verify whether the minimum performance of AI/ML functionality/feature can be achieved - FFS on whether and how many different test configurations/parameters are used for tests. - LCM would be tested

From this, although the above LCM related requirements and functional tests are AI/ML PHY use cases specific, the aim, necessity, feasibility and even the principle of defining the LCM related requirements and functional tests should be inherited to AI/ML mobility use cases.

Proposal 12: LCM related requirements and functional tests should to be defined based on the specific AI/ML mobility use cases and sub-use cases

- The LCM procedures agreed in AI/ML for NR air interface RRM need to be considered as starting point

  - Prefer to use the legacy framework for RRC/MAC-CE/DCI based core requirements as the baseline for LCM procedures

- RAN4 study should be aligned with the agreements in RAN1 and RAN2

5 Conclusion

In this contribution, we provided our initial viewpoints to RRM requirement impact analyses on AI mobility. The following observations and proposals are obtained:

Observation 1: For RRM measurement prediction, at least 5 dimensional discussions can be involved in AI mobility, for each dimension, multiple elements are contained. The 5 dimensions can be:

- Scenarios

- Sub use-cases

  - Including: Cell-specific and/or cluster approaches

- Sliding/Non-sliding filtering

- Observation and prediction pattern

  - Including: OW length/PW length; MRRT ratio

- Speed

Proposal 1: For RRM measurement prediction requirement definition, suggest RAN4 to discuss the priorities of, at least, scenarios, sub use-cases, sliding/non-sliding filtering, observation and prediction pattern, and speed.

- How to combine all the dimensions and down select the scope should takes RAN2’s processes, agreements and the prediction performance gain analysis into consideration

Proposal 2: The general aspects part of RAN4 RRM work on AI/ML based beam prediction in Rel-18 could be reused for AI/ML based RRM measurement prediction as much as possible

- RAN4 to discuss what general aspects could be reused to avoid the duplicate study

Proposal 3:  Based on RAN2’s progress and agreements to determine what kind of AI model (i.e., UE side model or NW side model) could be considered to define the AI/ML based requirement for RRM measurement prediction in RAN4

Proposal 4: The performance Metrics/KPIs for Measurement Prediction use case considered in RAN4 shall subject to RAN2’s processes and agreements

Proposal 5:

- RAN4 to first study the impacts on requirements for L3 cell level RSRP (Point C) prediction

  - How to define the accuracy requirements, e.g., RSRP prediction accuracy/performance degradation threshold, the applicability of the accuracy, i.e., percentage in L3 AI mobility prediction evaluation

- If RAN4 to study the impacts on requirements for L1 beam-level (Point A1)

  - Whether to/How to define an additional prediction accuracy requirement on top of the requirement at C

- RAN4 to deprioritize studying the impacts on requirements for L3 beam-level (Point E) prediction

Proposal 6: For the side condition consideration, RAN4 to discuss whether to/how to define a certain threshold for OW length to evaluate the L3 measurement prediction accuracy for the Sub use-cases

Proposal 7: RAN4 to study the prediction accuracy performance with consideration of the impact of measurement error

- For FR2 temporal domain prediciton, suggest to wait for more progress in AI/ML BM to study the impact of measurement error on the prediction accuracy requirements for AI mobility

- For FR1 temporal and frequency domain predictions, suggest RAN4 to discuss how to evaluate the impacts of measurement errors on the prediction accuracy performance for AI mobility

Proposal 8: The simulation parameters for verifying the feasibility of requirement/testing in RAN4 should follow RAN2’s progress

Proposal 9: RAN4 to discuss the RRM impact of OW and PW for scenarios 2 and 4 (FR1/FR2 intra-frequency temporal domain case A/B)

- The principle of determining OW and PW based on measurement period defined in RAN2 can be applied in RAN4

Proposal 10: RAN4 to study if HO requirements including delay, interruption, side conditions and accuracy are needed to be updated for AI/ML-assisted mobility

Proposal 11: RAN4 to discuss the RRM impact of measurement gap reduction in measurement period/ scheduling restriction for inter-frequency prediction

Proposal 12: LCM related requirements and functional tests should to be defined based on the specific AI/ML mobility use cases and sub-use cases

- The LCM procedures agreed in AI/ML for NR air interface RRM need to be considered as starting point

  - Prefer to use the legacy framework for RRC/MAC-CE/DCI based core requirements as the baseline for LCM procedures

- RAN4 study should be aligned with the agreements in RAN1 and RAN2

6 References

[1] R4-2416855 “WF on AI/ML Mobility”, Nokia