3GPP TSG-RAN WG4 Meeting #113                                        R4-2419189

Orlando, US, 18th – 22nd November, 2024

Source:	Samsung

Title:              Discussion on fast SCell activation for UE supporting eEMR

Agenda item:			7.15.3

Document for:	Discussion

1. Introduction

In last meeting, RAN4 had fruitful discussions on eEMR based fast activation. In this meeting, we will give some further discussions for some open issues captured in the agreed WF [1].

2 Discussions

2.1 Target scenarios of fast SCell activation

Have checked the scenarios discussed in Rel-18 EMR WI and the corresponding test cases, we found that actually FR1 and FR2 cases are both contained.

#106 - Agreement RAN4 shall focus on inter-band target cell in FR2. If final solution to be agreed can cover intra-band and FR1 without extra standardization effort, it is unnecessary to exclude these two scenarios.

From this, we’d better to inherit the agreed scenarios including FR1 and FR2-1. Actually, there were extensive discussions on how to define the eEMR based fast SCell activation requirements for FR1 and FR2 in last meeting.

However, based on the Tactivation_time defined in RAN4 for FR2 intra-band scenario, if the SCell being activated belongs to FR2 and if there is at least one active serving cell on that FR2 band, there is no extra room to reduce the SCell activation delay even the UE supports Rel-18 EMR.

For FR1, when the SCell is contiguous to an active serving cell in the same band, if SCell being activated is unknown: No cell detection provided the conditions specified for intra-band contiguous CA case in TS38.133 section 8.3.2 are satisfied; UE perform AGC adjustment and TF tracking based on temporary RS, which are irrelevant to Rel-18 EMR.

Based on this, we propose that

Proposal 1: For the target scenarios of fast SCell activation,

- RAN4 not to consider the following SCells for the eEMR based fast SCell activation requirements:

  - SCell being activated is contiguous to PCell in FR1 (i.e. FR1 intra-band contiguous CA)

  - SCell being activated is in the same band as PCell in FR2 (i.e. FR2 intra-band CA)

Proposal 2: For SCell being activated belongs to FR2, RAN4 to consider

  - The SCell being activated belongs to FR2 and there is no active serving cell on that FR2 band provided that PCell or PSCell is in FR1.

2.2 How to update the known condition with consideration of valid eEMR

2.2.1 Whole picture

Combining the EMR reporting framework, direct SCell activation framework, enhanced measurements for fast SCell/SCG setup framework, the definition of valid measurement results and the agreements we achieved in last meeting, we have the following diagram.

Fig.1 Direct SCell activation combining with eEMR for IDLE/INACTIVE mode, NW provides measurement configurations for Rel-18 FR2 CA/DC setup at RRCRelease

Thereinto,

Valid measurement results

- The availability indication of the MRs is included in RRCResumeComplete or RRCSetupComplete.

  - The indicated measurement results should already pass the validity check and remain valid when UE reports to NW

    - The UE is allowed to reuse existing measurement, including legacy measurement for cell re-selection and EMR.

    - Definition of valid measurement results

<R4-2321398> - Agreements: - If accuracy requirements are met, the measurement results are valid for IDLE/INACTIVE measurements within the last [X] sec before msg1 transmission for RRC resume/setup request.

- For NR IDLE mode, the UE reports the valid L3-RSRP measurement (s) with SSB index via UEInformationResponse; For NR INACTIVE mode, UE can reports the valid measurement result (s) with SSB index via RRCResumeComplete

Periods

- X is the validity check timer, configured by NW, the candidate values for X: 5s, 10s, 20s, 50s, 100s

- Y is the interval between the UE sending a valid measurement report and the beginning of SCell activation.

- Z is the SCell activation delay

  - Based on the existing requirement, the delay components of the generic FR2 SCell activation time for both unknown and known cases can mainly contain : TCellsearch, TAGC, TL1-RSRP,measure+TL1-RSRP,report, TFineTiming, Tuncertainty.

    - If there is no active serving cell on that FR2, AGC adjustment and cell detection/search with Rx beam sweeping are considered

    - For known case,

| Components | Delay |
| --- | --- |
| TL1-RSRP,measure+TL1-RSRP,report | 0 ms |
| TCellsearch | 0 ms |
| TAGC | 0 ms |

Detectability

- The detected cells in connected mode would also be assumed be detected by the UE after transitioning to idle mode

- Cells which are “known” when the UE is in connected mode are also considered to be detected when the UE moves back to RRC IDLE/INACTIVE mode for the purposes of candidate SCell measurement

2.2.2 3 options for FR2

The following options were extensively discussed in last meeting.

- For FR2: - Option1: The fast SCell activation delay requirements with Rel-18 valid eEMR reporting apply provided that measIdleValidityDuration-r18/measReselectionValidityDuration-r18 is configured smaller than [X] - Option 1 delay components: 1 SSB sample as legacy known case (Apple with short window, OPPO with X=5, MTK, HW) - Option 2: (Ericsson, Nokia) No need to have such side condition in option 1. Need more discussion. - Option 3: use existing SSB based known SCell activation delay requirement with 2 SSB samples and don’t need side condition in option 1 - Option 3 delay components: [2] SSB samples * beam sweeping +1 SSB sample (QC, Apple, OPPO, ZTE, Samsung, VIVO, MTK when the validityduration is configured) - Option 4 (combine option 1 and option 2) delay components:[ K] SSB samples * beam sweeping +1 SSB sample (QC, ZTE, Nokia, HW, CATT) - K = 0 or 1 if when the “validityduration” is smaller than X - K = 1 or 2 otherwise - Option 4a: delay components:[ K] SSB samples * beam sweeping +1 SSB sample (Samsung) - K = 0 or 1 if when the “validityduration” is applied with smaller than X - K = 1 or 2 otherwise - FFS how to test

From our understanding,

- Option 1 puts limitation on NW configurations. And if the configured value smaller than [5], based on our Fig.1, no such candidate value for X.

Proposal 3: For FR2 eEMR based fast SCell activation delay requirements, if validityduration is configured smaller than [X], additional NW configuration limitations shall be introduced. Not prefer to put any limitation on NW configuration

From our understanding, option 3 (K= non zero values) and option 4/4a (K= 0) are two directions to treat known condition, while which direction to choose depends on what is the fast under Rel-19 SCell activation scenario and the fast level we expect

Proposal 4: The option 3 (K= non zero values) and option 4/4a (K= 0) are two directions to treat known condition, while which direction to choose depends on what is the fast under SCell activation scenario in Rel-19 and the fast level we expect

- Option 3

- It is not a conventional sense of known case since the additional samples for cell detection/AGC are introduced. From our understanding, fast for option 3 means realize SCell activation based on eEMR but without any other period limitations consideration, that means the measurement results are only valid

- Even the L3 measurement results pass the validity check, maybe they are unreliable especially when the validity check time X is too large (e.g., 100 s), since the cell/SSB has not been measured for a long period. In this sense, the timing/gain setting obtained from the last measurement may become invalid/not credible to UE, then more measurement samples in the time domain are needed to ensure the same accurate power gain estimation for example. Besides, the difference between FR1 and FR2 is that the latter needs consider Tx-Rx beam pair operation, the subsequent L1-RSRP will likely be needed, since it is necessary to ensure the TCI state for PDSCH/PDCCH after activation is reliable

- Combining the “Reporting condition” we achieved in last meeting, the corresponding UE actions and delay components can be

- eEMR-specific known ( [Y]< [3 s] && measIdleValidityDuration-r18/measReselectionValidityDuration-r18 is configured) a. Measurement via Rel-18 EMR is valid i. No cell search ii. Need AGC iii. Need fine timing adjustment [iv. Re-perform L1-RSRP measurement for valid fine beam information] b. Delay components: 2 SSB samples * beam sweeping + [TL1-RSRP,measure +TL1-RSRP,report]+ 1 SSB sample (Note How to use the 2 samples depeds on UE implementation)

Observation 1: Option 3 is not a conventional sense of known case for FR2 since the additional samples for cell detection/AGC are introduced, fast for option 3 means realize SCell activation based on eEMR valid value

In this sense, the known condition follows the same principle as we have in the current Spec. just to add some eEMR specific limitation, e.g., provided measIdleValidityDuration-r18/measReselectionValidityDuration-r18 is configured…but no additional condition (s) to indicate the valid results are reliable. Whilst, the Tactivation_time timeline will be cut with the reduced samples, for example, maybe from

6ms + TFirstSSB_MAX + 15*TSMTC_MAX + 8*Trs  + TL1-RSRP, measure + TL1-RSRP, report  + THARQ + max(Tuncertainty_MAC + TFineTiming + 2ms, Tuncertainty_SP6ms + TFirstSSB_MAX + 15*TSMTC_MAX + TL1-RSRP, measure + TL1-RSRP, report  + THARQ + max(Tuncertainty_MAC + TFineTiming + 2ms, Tuncertainty_SP

Proposal 5: If to realize fast SCell activation based on eEMR valid value,

- For FR2 single direct SCell activation at SCell addition, provided measIdleValidityDuration-r18/measReselectionValidityDuration-r18 is configured, an SCell is considered as known for the activation procedure

  - Option 3：The delay components: [2] SSB samples * beam sweeping +[TL1-RSRP,measure +TL1-RSRP,report]+ 1 SSB sample

If to realize fast SCell activation by omitting all the Rx beam sweeping operation. In this sense, the Tactivation_time timeline aligns with the legacy known cell one (i.e., TAGC=0, TCellsearch, TL1-RSRP,measure+TL1-RSRP,report=0, and 1 sample is needed to perform fine time and frequency synchronization). The known condition for Rel-19 Scell activation aligns to the legacy known condition in essence, and from our understanding, this is a total known Cell case. However, the critical thing is to define an additional condition to ensure the eEMR valid value is reliable enough.

Proposal 6: If to realize fast SCell activation by omitting all the Rx beam sweeping related operations (i.e., K=0), an additional condition is needed to ensure the eEMR valid value is reliable enough

- Option 4a

- Option 4 is the compromised solution of option 3. And option 4a is that the validityduration is taken as an applicable window, can be explained as measIdleValidityDuration-r18/measReselectionValidityDuration-r18 is configured, but only when the latest L3 measurement is performed within the last [TBD] ([TBD] smaller than X) seconds before it is reported. Based on this, since the cell/SSB has been measured for a shorter period, and the valid measurement was not sent too long ago, it can be assumed that the UE can use the history/prior power & timing information stored safely. At the same time, the Tx-Rx beam pair selected based on measurement sample in a time period of [TBD] is reliable for PDSCH/PDCCH after activation. UE will activate the SCell quickly. The NW can carry out the quick SCell scheduling based on the instant when the accurate CQI sent by the UE if the Tactivation_time can be shorten based on eEMR

Proposal 7: For 4a, for FR2, the fast SCell activation delay requirements with Rel-18 valid eEMR apply provided that measIdleValidityDuration-r18/measReselectionValidityDuration-r18 is configured, and only when the latest measurement is performed within the last [TBD] s ([TBD] smaller than X) before UE sends a valid measurement report for the SCell being activated

- The valid measurement report via Rel-18 EMR is reliable within a given time period

- For option 4a, the corresponding UE actions and delay components can be

a. Measurement via Rel-18 EMR is valid and reliable i. No cell search ii. No AGC iii. Need fine timing adjustment iv. No extra L1-RSRP measurement b. Delay components:1 SSB sample (i.e., 1 SSB sample as legacy known case)

Proposal 8: For 4a, for FR2 single direct SCell activation at SCell addition, provided measIdleValidityDuration-r18/measReselectionValidityDuration-r18 is configured, an SCell is considered as known for the activation procedure

- The delay components:

  - 1 SSB sample, if when the latest measurement is performed within the last [TBD] s, [TBD] smaller than X

  - Traditional unknown components, otherwise

  - FFS: How to test

However, all above are based on the assumption that if the measIdleValidityDuration-r18/ measReselectionValidityDuration-r18 is configured, but from our understanding, at least the case when measReselectionValidityDuration-r18 is not configured is a new feature in Rel-18. Since in Rel-18 eEMR, cell-reselection is taken as a possible measurement in idle mode, the latest result for overlapped carrier/cells configured for cell-reselection is included, and the related new fields are also specified in TS 38.331, the corresponding UE behaviour is new and can belongs to Rel-18 EMR scope. From this,

Proposal 9: RAN4 to discuss whether to/how to define the known condition provided at least measReselectionValidityDuration-r18 is not configured.

2.2.3 FR1

For FR1, if the latest measurement sample is too long ago before the direct SCell activation command is received, the UE may need additional time for AGC setting. when the SCell is known, the AGC setting time could be shorten because the UE has the history power information. Then total 2 samples (AGC + fine timing adjustment) are reasonable.

Proposal 10: For FR1 single direct SCell activation at SCell addition, provided measIdleValidityDuration-r18/measReselectionValidityDuration-r18 is configured,

- Use existing SSB based known SCell activation delay requirement with 2 SSB samples

  - Whether the indication to network is needed

From our understanding, this is another way to expedite SCell activation. The expected indication reported at maybe Msg 5/Msg 3 implicitly means that the valid candidate CA/EN-DC carriers UE measured are good enough, and it can be used to tell network whether the candidate cell can be activated fast or not directly. Although the method accommodates the fast traffic load, there is no need to discuss the controversial eEMR based known conditions in Section 2.2 if it is applied, and how to define “good” measurement is the derived issue. From this, we suggest to keep an eye on the ongoing discussions and not to introduce the indication.

Proposal 11: Valid eEMR report in R18 is enough. No need to introduce additional indication to the network for fast or slow SCell activation.

  - Detected cell requirement during state transition and Idle mode for eEMR

First, based on the agreements we achieved in Rel-18 EMR, the existing measurements considered in eEMR contain cell re-selection and EMR. And the existing cell re-selection requirements (4.2, 38.133) and idle mode CA/DC measurement requirements (4.4, 38.133) can be reused.

Second, the detectable requirements were extensively discussed in euCA. Such assumption, i.e., the detected cells in connected mode would also be assumed be detected by the UE after transitioning to idle mode, would help in decreasing the expected UE measurement delay by excluding Idle mode cell delay from these cells. That is to say, the UE would only be required to measure cells in idle mode which are already detected cells in connected mode. And this is a simple and reasonable way to make the SINR side condition consistent.

Based on the agreements we achieved in last meeting,

- the SSB detectability applies from the earliest possible eEMR measurement (e.g., the configured measIdleValidityDuration or measReselectionValidityDuration before Msg1 transmission) to the end of the defined SCell activation latency (i.e. a valid CQI reporting). - the SSB measured remains detectable according to the IDLE/Inactive mode measurement conditions specified in 4.2 or the CA/DC measurement conditions specified in 4.4 when UE is in IDLE/INACTIVE state and - the SSB measured remains detectable according to the cell identification conditions specified in clause 9.2 and 9.3 when UE is in CONNECTED mode.

and combining the whole picture we gave in Section 2.2.1, we think to keep consistency, the detectability shall be considered in RRC connected mode, during state transition and idle/inactive.

Proposal 12: Detected cell requirement during state transition and Idle/Inactive mode should be satisfied not only for CA/DC measurement but also for cell re-selection measurement for fast CA/DC setup in Rel-18 EMR

Besides, from our understanding, Idle mode CA/DC measurement and cell re-selection measurement are different features in Rel-18 EMR at least from configurations and capability definitions perspective. The limitations on "idle mode CA/DC measurement reporting provided T331 has not expired" or "The UE is provided with a valid timer T331 by dedicated RRC signaling" specified for CA/DC measurement in the current spec. Section 4.4.1/4.4.2 are not needed for cell re-selection measurement any more. In this sense, if the detectability condition during state transition and idle/inactive should be defined for eEMR cell re-selection measurement, maybe adding new sections: Detected cell requirement during state transition and Idle/Inactive mode for cell re-selection measurement for fast CA/DC setup rather than modifying the existing sections for CA/DC measurement is more appropriate.

Observation 2: Detected cell requirement during state transition and Idle/Inactive mode for cell re-selection measurement for fast CA/DC setup is missing in the current Spec. Section 4.2 and Section 5.1

Proposal 13: It is necessary to define detected cell requirement during state transition and Idle/Inactive mode should be satisfied for cell re-selection measurement for fast CA/DC setup.

3 Conclusion

In this contribution, we provided our viewpoints to requirement impact analyses on eEMR based fast Scell activation. The following observations and proposals are obtained:

Observation 1: Option 3 is not a conventional sense of known case for FR2 since the additional samples for cell detection/AGC are introduced, fast for option 3 means realize SCell activation based on eEMR valid value

Observation 2: Detected cell requirement during state transition and Idle/Inactive mode for cell re-selection measurement for fast CA/DC setup is missing in the current Spec. Section 4.2 and Section 5.1

Proposal 1: For the target scenarios of fast SCell activation,

- RAN4 not to consider the following SCells for the eEMR based fast SCell activation requirements:

  - SCell being activated is contiguous to PCell in FR1 (i.e. FR1 intra-band contiguous CA)

  - SCell being activated is in the same band as PCell in FR2 (i.e. FR2 intra-band CA)

Proposal 2: For SCell being activated belongs to FR2, RAN4 to consider

- The SCell being activated belongs to FR2 and there is no active serving cell on that FR2 band provided that PCell or PSCell is in FR1.

Proposal 3: For FR2 eEMR based fast SCell activation delay requirements, if validityduration is configured smaller than [X], additional NW configuration limitations shall be introduced. Not prefer to put any limitation on NW configuration

Proposal 4: The option 3 (K= non zero values) and option 4/4a (K= 0) are two directions to treat known condition, while which direction to choose depends on what is the fast under SCell activation scenario in Rel-19 and the fast level we expect

Proposal 5: If to realize fast SCell activation based on eEMR valid value,

- For FR2 single direct SCell activation at SCell addition, provided measIdleValidityDuration-r18/measReselectionValidityDuration-r18 is configured, an SCell is considered as known for the activation procedure

  - Option 3：The delay components: [2] SSB samples * beam sweeping +[TL1-RSRP,measure +TL1-RSRP,report]+ 1 SSB sample

Proposal 6: If to realize fast SCell activation by omitting all the Rx beam sweeping related operation (i.e., K=0), an additional condition is needed to ensure the eEMR valid value is reliable enough

Proposal 7: For 4a, for FR2, the fast SCell activation delay requirements with Rel-18 valid eEMR apply provided that measIdleValidityDuration-r18/measReselectionValidityDuration-r18 is configured, and only when the latest measurement is performed within the last [TBD] s ([TBD] smaller than X) before UE sends a valid measurement report for the SCell being activated

- The valid measurement report via Rel-18 EMR is reliable within a given time period

Proposal 8: For 4a, for FR2 single direct SCell activation at SCell addition, provided measIdleValidityDuration-r18/measReselectionValidityDuration-r18 is configured, an SCell is considered as known for the activation procedure

- The delay components:

  - 1 SSB sample, if when the latest measurement is performed within the last [TBD] s, [TBD] smaller than X

  - Traditional unknown components, otherwise

  - FFS: How to test

Proposal 9: RAN4 to discuss whether to/how to define the known condition provided at least measReselectionValidityDuration-r18 is not configured.

Proposal 10: For FR1 single direct SCell activation at SCell addition, provided measIdleValidityDuration-r18/measReselectionValidityDuration-r18 is configured,

- Use existing SSB based known SCell activation delay requirement with 2 SSB samples

Proposal 11: Valid eEMR report in R18 is enough. No need to introduce additional indication to the network for fast or slow SCell activation.

Proposal 12: Detected cell requirement during state transition and Idle/Inactive mode should be satisfied not only for CA/DC measurement but also for cell re-selection measurement for fast CA/DC setup in Rel-18 EMR

Proposal 13: It is necessary to define detected cell requirement during state transition and Idle/Inactive mode should be satisfied for cell re-selection measurement for fast CA/DC setup

4 References

[1] R4-2416854 “WF on RRM requirements for NR_RRM_Ph5_Part2”, CATT