3GPP TSG-RAN WG4 Meeting #113                                        R4-2419190

Orlando, US, 18th – 22nd November, 2024

Source:	Samsung

Title:              General discussion on AI mobility regarding testability and interoperability

Agenda item:			7.18.3

Document for:	Discussion

1. Introduction

In last meeting, RAN4 had some discussions on AI mobility regarding testability and interoperability. In this meeting, we will give some further discussions for some open issues captured in the agreed WF [1].

2 Discussion

2.1 AI/ML functionalities for RAN4 test

Theoretically, based on “3GPP AI/ML functional framework for the NR air interface”, testability issues for all the basic functionalities, i.e., Data collection, Model training, Model inference, Model monitoring can be studied in RAN4 RRM. While, as a starting point, we suggest to prioritize Model inference test verification.

Proposal 1: In AI/ML mobility, regarding the test scope, suggest to prioritize discussions on AI/ML model inference in each use case and sub-use case.

- Take Rel-18 AI/ML test principles/procedures/frameworks as much as possible for testing the AI/ML model inference functionality.

2.2 Testing goals

The testing can be taken as a popular technique for ensuring the reliability of AI model. For RRM measurement prediction, the goals determined in RAN2 for our agreed evaluation scenarios are duplicated as below

| No. | Prioritization | Evaluation scenario combination | Deployment of the cell for measurement and the cell for prediction | Target study goal |
| --- | --- | --- | --- | --- |
| 1 | High | FR1 to FR1 intra-frequency intra-cell temporal domain case B (i.e., Intra_F_C_T_Case B) |  | 1st goal |
| 2 | High | FR1 to FR1 inter-frequency inter-cell (frequency domain) (i.e., Inter_F_C ) | - Co-located (Start from) - Same sector | 1st goal |
| 3 | High | FR2 to FR2 intra-frequency intra-cell temporal domain case A (i.e., Intra_F_C_T_Case A) |  | 2nd goal |

The test objectives can be divided into two parts corresponding to the two goals, separately. Combine RAN2 conclusions and our understanding:

The goals

- 1st goal：

  - Test Property: Measurement overhead reduction

    - Verify the trade-off between the measurement reduction and accuracy performance degradation, the legacy performance is as baseline

  - Metrics [TBD]:

    - Measurement reduction rate: MRRT for temporal domain; MRRS for spatial domain

    - Intermediate KPI (Accuracy): Average L3 cell RSRP difference; Average L1-RSRP difference (dB) (optional)

    - FFS: Prediction window

- 2nd goal：

  - Test Property: HO performance enhancement

    - Verify whether the HO performance gain of AI/ML model can be obtained

  - Metrics [TBD]:

    - System KPI: Performance KPIs defined in TR 36.839 as baseline for mobility performance KPIs

- The evaluation of the AI/ML aided mobility benefits should consider HO performance KPIs (e.g., Ping-pong HO, HOF/RLF, Time of stay, Handover interruption, prediction accuracy, and measurement reduction) etc.) and complexity tradeoffs [RAN2] - NOTE: Simulation assumption and methodology can leverage TR 38.901, 38.843 and 36.839. And leave the detail discussion to RAN2

More specifically,

- The ultimate motivation of AI/ML aided mobility can be to improve the system handover performance, the intermediate KPI such as average L3 cell level RSRP difference (dB) is just to examine the predicted capability of AI model. While, since HO decision is always made by the NW, it relies on UE RRM measurement results to some extent. One main intention of intermediate AI-assisted RRM measurement prediction is to reduce UE measurement effort while simultaneously maintaining the system performance.

- For the 1st goal, the major task of the AI/ML mode is to recover the skipped/omitted measurements through prediction. The predictions, either generated by the NW itself or generated by UE and reported to NW, will be used for HO/target cell decision-making. If the predicted results have (very) minimal impact on the HO decision/target cell selection, the measurement overhead reduction goal is achieved

  - This clarification essentially means that the measurement reduction should strive to keep the legacy mobility performance from degrading, for example, to keep the HO successful rate above a threshold (e.g. 95%)

Proposal 2: RAN4 would better to clarify the testing property/aim and methodology when test AI/ML functionalities for each AI/ML mobility use case and sub-use case. The alternatives are as follows.

- RRM measurement prediction

  - 1st goal: To verify the trade-off between the measurement reduction and accuracy performance degradation, the legacy performance is as baseline

    - Performance gain: The measurement reduction should keep the mobility performance from degrading

  - 2nd goal: To verify whether the HO performance gain of AI/ML model can be obtained

2.3 Multi-Cell testing

- For mobility, neighbouring cells are of the same-level significance as the serving cell since handover is a multi-cell related behaviour. The cross-cell RRM measurement contains inter-frequency (Inter_F_C) and intra-frequency should be studied with certain spatial consistency.

- For scenarios FR1 to FR1 intra-frequency temporal domain case B and FR1 to FR1 inter-frequency inter-cell (frequency domain), cell specific and cluster approaches are considered. While, cluster approach refers to the prediction method where the number of cells for measurements and/or predictions is more than one.

- With the input data from two frequencies (e.g., serving cell X at frequency f1 and neighbour cell Y at frequency f2), the AI/ML model can be trained to learn the correlation between the measurements in the two frequencies. Based on RAN2 simulations, if the cell to be measured and the cell to be predicted are co-located (located in the same sector/site with the same antenna configuration), the correlation is high, the RSRP difference between the two cells is relatively stable and a good prediction accuracy is obtained. While, non-collocated inter-frequency prediction is under discussion

Proposal 3: RAN4 to consider the following testability aspects for AI mobility

- Consideration of serving and neighbour cells

  - Multi-cell setup and number of cells to measure

2.4 Testing in FR2

There are some ongoing discussions on the testability and interoperability in AI/ML for NR air interface. Considering the commonality and difference of AI/ML based beam management and AI/ML mobility: FR2 VS FR1+FR2; Spatial and temporal beam prediction, serving cell (intra-cell) prediction VS temporal, spatial, and frequency domain predictions, cross-cell prediction; BM, predicted L1-RSRP VS Cell, predicted L3-RSRP; The Top-1/K (%) beam prediction accuracy KPI VS intermediate KPIs (Average L3 cell level RSRP difference) + system KPIs (HOF, RLF rate, throughput, etc) . From this, for FR2, to avoid the overlapping of similar discussions in the two WIs, especially intra-frequency intra-cell beam level prediction, we’d better to wait for more progress from AI/ML-BM

Proposal 4: For FR2 measurement prediction testing, RAN4 made for AI/ML-BM can be reused as a baseline

- RAN4 to discuss what principle on testability and interoperability for which AI/ML-PHY scenarios/cases could be reused to avoid the duplicate study

4 Conclusion

In this contribution, we provided our initial viewpoints to General discussion on AI mobility regarding testability and interoperability. The following observations and proposals are obtained:

Proposal 1: In AI/ML mobility, regarding the test scope, suggest to prioritize discussions on AI/ML model inference in each use case and sub-use case.

- Take Rel-18 AI/ML test principles/procedures/frameworks as much as possible for testing the AI/ML model inference functionality.

Proposal 2: RAN4 would better to clarify the testing property/aim and methodology when test AI/ML functionalities for each AI/ML mobility use case and sub-use case. The alternatives are as follows.

- RRM measurement prediction

  - 1st goal: To verify the trade-off between the measurement reduction and accuracy performance degradation, the legacy performance is as baseline

    - Performance gain: The measurement reduction should keep the mobility performance from degrading

  - 2nd goal: To verify whether the HO performance gain of AI/ML model can be obtained

Proposal 3: RAN4 to consider the following testability aspects for AI mobility

- Consideration of serving and neighbour cells

  - Multi-cell setup and number of cells to measure

Proposal 4: For FR2 measurement prediction testing, RAN4 made for AI/ML-BM can be reused as a baseline

- RAN4 to discuss what principle on testability and interoperability for which AI/ML-PHY scenarios/cases could be reused to avoid the duplicate study

5 References

[1] R4-2416855 “WF on AI/ML Mobility”, Nokia