3GPP TSG-RAN WG4 Meeting #113                                        R4-2419192

Orlando, US, 18th – 22nd November, 2024

Source:	Samsung

Title:              Discussion on L3 measurement delay by optimizing CSSF

Agenda item:			7.15.2.2

Document for:	Discussion

1. Introduction

In last meeting, RAN4 had extensively discussions on L3 measurement enhancement by optimizing CSSF. In this meeting, we will give some further discussions for some open issues captured in the agreed WF [1].

2 Discussion

2.1 Clarification on the bullets in WID for this CSSF optimization

The principle of the updated clarifications including the independence of features/capabilities and the applicability of the CSSF requirement are fine for us.

Just since we are defining the trigger condition of FBS based L3 measurement, from our understanding using “...only when the conditions to trigger UE using FBS for L3 measurement are not met” instead of “...assuming a UE not operating with multiple-Rx based simultaneous L3 measurement” maybe is more proper and spec-compliant.

To be specific, based on the clarifications, the relationship of L3 CSSF and multi-Rx simultaneous reception can be understood as:

Rel-19 CSSF optimization and multi-Rx simultaneous reception enhancement are independent features, while the two features can be supported independently or together. It can say, for feature group definitions, Rel-19 CSSF optimization and multi-Rx simultaneous reception capabilities are independent but can coexist. More specifically,

- It is likely to introduce (new) capabilities/signallings to support Rel-19 multi-RX simultaneous reception and Rel-19 CSSF enhancement features, separately and define new corresponding UE behaviours.

- Enhancements in the Rel-19 CSSF should be independent of the UEs support of the multi-Rx based simultaneous measurement features/capabilities.

  - Dedicate configurations for Rel-19 CSSF and multi-RX reception enhancements are required.

  - No need to take multi-Rx based simultaneous measurement/reception as prerequisite for Rel-19 enhancement to CSSF based L3 measurement delay.

- If the UE is capable of multi-Rx simultaneous reception features/capabilities, the Rel-19 CSSF enhancement requirement is applied only when the conditions to trigger UE using FBS for L3 measurement are not met

In summary, regarding the clarification on the bullets in WID for this CSSF optimization, we have the following views (including the text revision Alt):

Proposal 1: Rel-19 CSSF optimization and multi-Rx simultaneous reception enhancement are independent features, while the two features can be supported independently or together. For feature group definitions, Rel-19 CSSF optimization and multi-Rx simultaneous reception capabilities are independent but can coexist

Proposal 2 (text revision Alt):

- Rel-19 CSSF optimization applies for CA/DC scenario independently of the UE support of multi-Rx simultaneous reception features/capabilities

- If the UE is capable of multi-Rx simultaneous reception features/capabilities, the Rel-19 CSSF enhancement requirement is applied only when the conditions to trigger UE using FBS for L3 measurement are not met

2.2 UE measurement procedure to use L3 measurement delay reduction by optimizing CSSF

From our understanding, intra-f and inter-f have the same level of importance. But to simplify the CSSF optimization, SSB-based intra-frequency and inter-frequency measurements without MG(aspect 1 and 2) can be taken as a starting point

Proposal 3: To start from SSB-based intra-frequency and inter-frequency measurements without MG(aspect 1 and 2)

2.3 Feature capability of L3 measurement delay reduction by optimizing CSSF

Based on the discussions in issue 2.1, from our understanding, to make everything clear and simple, a reasonable way is to define a new individual feature for CSSF optimization in Rel-19, and a new capability is required to support the feature, accordingly. And, we also agree that it is too early to discuss the details.

Proposal 4:

- Define a new capability to support the new Rel-19 CSSF enhancement feature.

- Delay the capability discussion to the end of the core part

3 Conclusion

In this contribution, we provided our viewpoints to requirement impact analyses on L3 CSSF enhancement. The following observations and proposals are obtained:

Proposal 1: Rel-19 CSSF optimization and multi-Rx simultaneous reception enhancement are independent features, while the two features can be supported independently or together. For feature group definitions, Rel-19 CSSF optimization and multi-Rx simultaneous reception capabilities are independent but can coexist

Proposal 2 (text revision Alt):

- Rel-19 CSSF optimization applies for CA/DC scenario independently of the UE support of multi-Rx simultaneous reception features/capabilities

- If the UE is capable of multi-Rx simultaneous reception features/capabilities, the Rel-19 CSSF enhancement requirement is applied only when the conditions to trigger UE using FBS for L3 measurement are not met

Proposal 3: To start from SSB-based intra-frequency and inter-frequency measurements without MG(aspect 1 and 2)

Proposal 4:

- Define a new capability to support the new Rel-19 CSSF enhancement feature.

- Delay the capability discussion to the end of the core part

4 References

[1] R4-2416853 “WF on RRM requirements for NR_RRM_Ph5_Part1”, Apple