3GPP TSG-RAN WG4 Meeting #113	R4-2419196

Orlando, USA, 18th – 22nd November 2024

Source: 	Nokia

Title: 					n68 NS_26 and NS_36 A-MPR simulations

Agenda Item: 			6.16.1

Document for:	Discussion

# 1	Introduction

We simulate A-MPR for band n68 due to additional spurious emission masks signaled through NS_26 and NS_36. According to the WF of the previous meeting [1], we use the extreme conditions duplex filter.

# 2	Simulation assumptions and scenario

## 2.1	Simulation parameters

- Operating band n68 (UL 698 MHz – 728 MHz)

- Channel bandwidths: 5 MHz, 10 MHz and 15 MHz

- Power class PC3

- WOLA processing

- I/Q image -28 dBc

- Carrier leakage -28 dBc

- CIM3 -60 dBc, CIM5 -70 dBc

- Only additional spurious emission masks from NS_26 and NS_36 are simulated. Other gating factors (ACLR, SEM, etc.) are excluded because they are handled by MPR.

- Extreme conditions duplex filter in Table 1 is used for both NS-values.

## 2.2	Duplex filter

According to the WF [1], we use the extreme conditions duplex filter parameters in Table 1. We use nearest-neighbor interpolation between the given frequencies.

Table 1: Duplex filter.

|  | Vendor B SAW 15MHz passband |  |
| --- | --- | --- |
| MHz | Extreme | Normal |
| 693.5 | 0.0 | 10.0 |
| 692.5 | 4.0 | 16.0 |
| 691.5 | 10.0 | 20.0 |
| 690.5 | 16.0 | 26.0 |
| 689.5 | 20.0 | 30.0 |
| 688.5 | 26.0 | 30.0 |
| 687.5 | 30.0 | 30.0 |
| 686.5 | 30.0 | 30.0 |

## 2.3	NS_26 mask

| Frequency band (MHz) | Channel bandwidth / Spectrum emission limit (dBm) | Measurement bandwidth |
| --- | --- | --- |
|  | 5 MHz, 10 MHz, 15 MHz |  |
| 686 ≤ f ≤ 694 | -25 | 8MHz |

## 2.4	NS_36 mask

According to the WF [1] we use the “normal conditions” limit.

| Frequency band (MHz) | Channel bandwidth / Spectrum emission limit (dBm) | Measurement bandwidth |
| --- | --- | --- |
|  | 5 MHz, 10 MHz and 15 MHz |  |
| 470 ≤ f ≤ 694 | -42 | 8MHz |

# 3	Simulation results and discussion

In this section we present some highlights of the simulation results. In appendix we provide the complete sets of A-MPR triangles at chosen center frequencies. We use the filter attenuations in Table 1 for all channel positions reported in this contribution. However, as the filter data is for 15 MHz passband, other filter attenuations might be more appropriate for allocations above the 698 MHz – 713 MHz range.

## 3.1	NS_26

5 MHz CBW does not need A-MPR anywhere in the operating band. 10 MHz CBW needs A-MPR at the lower edge of the operating band, but not with 5 MHz offset (Fc=708 MHz). 15 MHz CBW needs A-MPR with 5 MHz offset, but not with 10 MHz offset.

In Figure 1 we present examples of the simulation results for 10 MHz and 15 MHz CBWs at the lower edge of the operating band.

Figure 1: Examples of the simulated backoff to satisfy the NS_26 additional spurious emission limit for 10 MHz and 15 MHz CBWs at the lower edge of the operating band (Fc=703 MHz and Fc=705.5 MHz).

## 3.2	NS_36

At 5 MHz CBW small amount of A-MPR is needed with 5 MHz offset from the lower edge of the operating band but at 10 MHz offset no A-MPR is needed. At 10 MHz CBW A-MPR is needed with 10 MHz offset while no A-MPR is needed with 15 MHz offset. At 15 MHz CBW A-MPR is needed everywhere in the operating band.

In Figure 2 we present examples of the A-MPR triangles for 5 MHz and 15 MHz CBWs at the lower edge of the operating band.

Figure 1: Examples of the simulated backoff to satisfy the NS_36 additional spurious emission limit for 5 MHz and 15 MHz CBWs at the lower edge of the operating band (Fc=700.5 MHz and Fc=705.5 MHz).

# 4	Conclusions

We have simulated A-MPR at band n68 for NS_26 and NS_36 using the extreme conditions filter and normal conditions emission limit in accordance with the WF [1].

We note that the A-MPR values presented here are a bit larger than specified in LTE. At 10 MHz and 15 MHz CBWs this may at least in part be due to different spectrum utilization between LTE and NR. At 5 MHz CBW it appears that normal conditions filter was used with normal conditions emission limit in the LTE simulations [2].

# 5	References

[1] R4-2417080, WF on A-MPR for NR band n68, Ericsson

[2] R4-1701021, Band 68 A-MPR simulation results, Nokia

# 6	Appendix

In this section we present complete sets of A-MPR triangles at chosen center frequencies.

## 6.1	NS_26

## 6.2	NS_36