3GPP TSG-RAN WG4 Meeting #113	R4-2419197

Orlando, USA 18th – 22nd November 2024

Source: 	Nokia

Title: 					n1 Japan A-MPR simulations and proposal

Agenda Item: 			4.2.1

Document for:	Approval

# 1	Introduction

We provide A-MPR simulations results on n1 on PC2 and PC3 due to the relaxed emission requirements in Japan [1]. We also make an A-MPR proposals based on our simulation results.

# 2	Simulation assumptions and scenario

- Operating band n1 (UL 1920 MHz – 1980 MHz)

- Power classes PC2 and PC3

- WOLA processing

- I/Q image -28 dBc

- Carrier leakage -28 dBc

- CIM3 -60 dBc, CIM5 -70 dBc

- Only additional spurious emission mask for Japan is simulated. Other gating factors (ACLR, SEM, etc.) are excluded because they are handled by MPR.

- New additional spurious emission mask for Japan [1]:

  - CBW = 5MHz for PC2 and PC3

    - 1884.5-1910.0 MHz: -30 dBm@1MHz

    - 1910.0-1915.7 MHz: -25 dBm@1MHz

  - CBW = 10 MHz, 15 MHz, 20MHz for PC2 and PC3

    - 1884.5-1906.6 MHz: -30 dBm@1MHz

    - 1906.6-1915.7 MHz: -25 dBm@1MHz

# 3	Simulation results

In this section we present examples of the simulation results. The complete sets of simulated A-MPR triangles at chosen center frequencies are presented in the appendix.

## 3.1	PC3

According to our simulations, A-MPR is not needed for the 5 MHz CBW. For higher CBWs A-MPR is needed for certain center frequencies that are presented in Table 1. In Figure 1 we present examples of the A-MPR triangles for 10 MHz and 20 MHz CBWs at the lower edge of the operating band.

Table 1: The center frequencies where A-MPR for PC3 in Japan due to new emission requirements is needed according to our simulations. 5 MHz CBW does not need A-MPR anywhere in the operating band.

| Channel Bandwidth [MHz] | Fc, AMPR [MHz] |
| --- | --- |
| 10 | < 1929.43 |
| 15 | < 1936.70 |
| 20 | < 1944.07 |

Figure 1: Examples of the simulated backoff to satisfy the new Japan additional spurious emission limit for 10 MHz and 20 MHz CBWs at the lower edge of the operating band (Fc=1925 MHz and Fc=1930 MHz) for PC3.

## 3.2	PC2

According to our simulations, A-MPR is not needed for the 5 MHz CBW. For higher CBWs A-MPR is needed for certain center frequencies that are presented in Table 2. In Figure 2 we present examples of the A-MPR triangles for 10 MHz and 20 MHz CBWs at the lower edge of the operating band.

Table 2: The center frequencies where A-MPR for PC2 in Japan due to new emission requirements is needed according to our simulations. 5 MHz CBW does not need A-MPR anywhere in the operating band.

| Channel Bandwidth [MHz] | Fc, AMPR [MHz] |
| --- | --- |
| 10 | < 1929.37 |
| 15 | < 1936.69 |
| 20 | < 1943.87 |

Figure 2: Examples of the simulated backoff to satisfy the new Japan additional spurious emission limit for 10 MHz and 20 MHz CBWs at the lower edge of the operating band (Fc=1925 MHz and Fc=1930 MHz) for PC2.

# 4	A-MPR proposals

In this section we present an A-MPR proposal based on our simulation results. In appendix we present complete sets of A-MPR triangles of our proposal at relevant center frequencies. In Figures 3 and 4 we present some examples of these A-MPR triangles at the lower edge of the operating band.

## 4.1	PC3

Proposal 1: RAN4 shall consider defining A-MPR for n1 PC3 in Japan as shown in tables 3 and 4.

Table 3: The proposed A-MPR regions for n1 PC3 in Japan.

| Channel Bandwidth, MHz | Carrier Center Frequency, Fc, MHz | Regions |  | A-MPR |
| --- | --- | --- | --- | --- |
|  |  | RBstart*12*SCS MHz | LCRB*12*SCS MHz |  |
| 10 MHz | Fc < 1929.43 | ≥ 0 | ≥ 1.5*12*SCS*RBstart + 4.7 | A1 |
|  |  | ≤ 1.44 | ≤ 1.62 | A2 |
| 15 MHz | Fc < 1932.5 | ≥ 0 | ≥ 1.5*12*SCS*RBstart + 4.7 | A1 |
|  |  | > 0, ≤ 3.26 | ≤ 2.16 | A2 |
|  |  | 0 | ≤ 0.72 | A3 |
|  | 1932.5 ≤ Fc < 1936.70 | ≤ 1.44 | ≤ 1.62 | A2 |
|  |  | ≤ 0.54 | ≥ 12.96 | A4 |
| 20 MHz | Fc < 1944.07 | ≥ 0 | ≥ 1.5*12*SCS*RBstart + 4.7 | A1 |
|  |  | ≤ 1.62 | ≤ 2.16 | A3 |
|  |  | ≤ 1.26 | > 2.16, ≤ 3.6 | A4 |
|  |  | > 1.62, ≤ 4.68 | ≤ 2.16 | A2 |

Table 4: The proposed A-MPR values for n1 PC3 in Japan.

| Modulation/Waveform |  | A1 | A2 | A3 | A4 |
| --- | --- | --- | --- | --- | --- |
| DFT-s-OFDM | PI/2 BPSK | 2 | 1.5 | 2.5 | 1 |
|  | QPSK | 3 | 2 | 3 | 1.5 |
|  | 16 QAM | 3 | 2 | 3.5 |  |
|  | 64 QAM | 3 |  | 3.5 |  |
|  | 256 QAM |  |  |  |  |
| CP-OFDM | QPSK | 4 | 3 | 5 |  |
|  | 16 QAM | 4 | 3 | 5 |  |
|  | 64 QAM | 4 |  | 5 |  |
|  | 256 QAM |  |  |  |  |

Figure 3: Examples of the proposed A-MPR to satisfy the new Japan additional spurious emission limit for 10 MHz and 20 MHz CBWs at the lower edge of the operating band (Fc=1925 MHz and Fc=1930 MHz) for PC3.

## 4.2	PC2

Proposal 2: RAN4 shall consider defining A-MPR for n1 PC2 in Japan as shown in tables 5 and 6.

Table 5: The proposed A-MPR regions for n1 PC2 in Japan.

| Channel Bandwidth, MHz | Carrier Center Frequency, Fc, MHz | Regions |  | A-MPR |
| --- | --- | --- | --- | --- |
|  |  | RBstart*12*SCS MHz | LCRB*12*SCS MHz |  |
| 10 MHz | Fc < 1929.37 | ≥ 0 | ≥ 1.0*12*SCS*RBstart + 4.7 | A1 |
|  |  | ≤ 1.44 | ≤ 1.8 | A2 |
|  |  | ≤ 0.18 | > 2.7, ≤ 3.6 | A3 |
| 15 MHz | Fc < 1932.5 | ≥ 0 | ≥ 1.0*12*SCS*RBstart + 4.7 | A1 |
|  |  | > 0, ≤ 3.26 | ≤ 3.42 | A2 |
|  |  | 0 | ≤ 1.62 | A4 |
|  | 1932.5 ≤ Fc < 1936.69 | ≤ 1.26 | ≥ 1.0*12*SCS*RBstart + 10.2 | A5 |
|  |  | ≤ 1.44 | ≤ 1.8 | A2 |
| 20 MHz | Fc < 1943.87 | ≥ 0 | ≥ 1.0*12*SCS*RBstart + 4.7 | A1 |
|  |  | ≤ 1.62 | ≤ 3.6 | A4 |
|  |  | > 1.62, ≤ 4.68 | ≤ 3.6 | A2 |
|  |  | ≤ 0.9 | > 4.14, < 1.0*12*SCS*RBstart + 4.7 | A6 |
|  |  | > 1.8, ≤ 3.42 | > 4.14, ≤ 5.76 | A7 |

Table 6: The proposed A-MPR values for n1 PC2 in Japan.

| Modulation/Waveform |  | A1 | A2 | A3 | A4 | A5 | A6 | A7 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DFT-s-OFDM | PI/2 BPSK | 2.5 | 1.5 |  | 3 | 1.5 | 1 |  |
|  | QPSK | 4 | 2 | 1.5 | 3.5 | 2.5 |  | 1 |
|  | 16 QAM | 4 | 2.5 |  | 4 | 2.5 |  |  |
|  | 64 QAM | 4 | 3 |  | 4 |  |  |  |
|  | 256 QAM |  |  |  |  |  |  |  |
| CP-OFDM | QPSK | 5 | 4 |  | 5.5 | 3.5 |  | 2.5 |
|  | 16 QAM | 5 | 4 |  | 5.5 | 3.5 |  |  |
|  | 64 QAM | 5 | 4 |  | 5.5 |  |  |  |
|  | 256 QAM |  |  |  |  |  |  |  |

Figure 4: Examples of the proposed A-MPR to satisfy the new Japan additional spurious emission limit for 10 MHz and 20 MHz CBWs at the lower edge of the operating band (Fc=1925 MHz and Fc=1930 MHz) for PC2.

# 5	Conclusions

We have simulated the A-MPR for n1 due to the new relaxed emission requirements in Japan [1]. We have made the following A-MPR proposals based on our simulation results.

Proposal 1: RAN4 shall consider defining A-MPR for n1 PC3 in Japan as shown in tables 3 and 4.

Proposal 2: RAN4 shall consider defining A-MPR for n1 PC2 in Japan as shown in tables 5 and 6.

Table 3: The proposed A-MPR regions for n1 PC3 in Japan.

| Channel Bandwidth, MHz | Carrier Center Frequency, Fc, MHz | Regions |  | A-MPR |
| --- | --- | --- | --- | --- |
|  |  | RBstart*12*SCS MHz | LCRB*12*SCS MHz |  |
| 10 MHz | Fc < 1929.43 | ≥ 0 | ≥ 1.5*12*SCS*RBstart + 4.7 | A1 |
|  |  | ≤ 1.44 | ≤ 1.62 | A2 |
| 15 MHz | Fc < 1932.5 | ≥ 0 | ≥ 1.5*12*SCS*RBstart + 4.7 | A1 |
|  |  | > 0, ≤ 3.26 | ≤ 2.16 | A2 |
|  |  | 0 | ≤ 0.72 | A3 |
|  | 1932.5 ≤ Fc < 1936.70 | ≤ 1.44 | ≤ 1.62 | A2 |
|  |  | ≤ 0.54 | ≥ 12.96 | A4 |
| 20 MHz | Fc < 1944.07 | ≥ 0 | ≥ 1.5*12*SCS*RBstart + 4.7 | A1 |
|  |  | ≤ 1.62 | ≤ 2.16 | A3 |
|  |  | ≤ 1.26 | > 2.16, ≤ 3.6 | A4 |
|  |  | > 1.62, ≤ 4.68 | ≤ 2.16 | A2 |

Table 4: The proposed A-MPR values for n1 PC3 in Japan.

| Modulation/Waveform |  | A1 | A2 | A3 | A4 |
| --- | --- | --- | --- | --- | --- |
| DFT-s-OFDM | PI/2 BPSK | 2 | 1.5 | 2.5 | 1 |
|  | QPSK | 3 | 2 | 3 | 1.5 |
|  | 16 QAM | 3 | 2 | 3.5 |  |
|  | 64 QAM | 3 |  | 3.5 |  |
|  | 256 QAM |  |  |  |  |
| CP-OFDM | QPSK | 4 | 3 | 5 |  |
|  | 16 QAM | 4 | 3 | 5 |  |
|  | 64 QAM | 4 |  | 5 |  |
|  | 256 QAM |  |  |  |  |

Table 5: The proposed A-MPR regions for n1 PC2 in Japan.

| Channel Bandwidth, MHz | Carrier Center Frequency, Fc, MHz | Regions |  | A-MPR |
| --- | --- | --- | --- | --- |
|  |  | RBstart*12*SCS MHz | LCRB*12*SCS MHz |  |
| 10 MHz | Fc < 1929.37 | ≥ 0 | ≥ 1.0*12*SCS*RBstart + 4.7 | A1 |
|  |  | ≤ 1.44 | ≤ 1.8 | A2 |
|  |  | ≤ 0.18 | > 2.7, ≤ 3.6 | A3 |
| 15 MHz | Fc < 1932.5 | ≥ 0 | ≥ 1.0*12*SCS*RBstart + 4.7 | A1 |
|  |  | > 0, ≤ 3.26 | ≤ 3.42 | A2 |
|  |  | 0 | ≤ 1.62 | A4 |
|  | 1932.5 ≤ Fc < 1936.69 | ≤ 1.26 | ≥ 1.0*12*SCS*RBstart + 10.2 | A5 |
|  |  | ≤ 1.44 | ≤ 1.8 | A2 |
| 20 MHz | Fc < 1943.87 | ≥ 0 | ≥ 1.0*12*SCS*RBstart + 4.7 | A1 |
|  |  | ≤ 1.62 | ≤ 3.6 | A4 |
|  |  | > 1.62, ≤ 4.68 | ≤ 3.6 | A2 |
|  |  | ≤ 0.9 | > 4.14, < 1.0*12*SCS*RBstart + 4.7 | A6 |
|  |  | > 1.8, ≤ 3.42 | > 4.14, ≤ 5.76 | A7 |

Table 6: The proposed A-MPR values for n1 PC2 in Japan.

| Modulation/Waveform |  | A1 | A2 | A3 | A4 | A5 | A6 | A7 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DFT-s-OFDM | PI/2 BPSK | 2.5 | 1.5 |  | 3 | 1.5 | 1 |  |
|  | QPSK | 4 | 2 | 1.5 | 3.5 | 2.5 |  | 1 |
|  | 16 QAM | 4 | 2.5 |  | 4 | 2.5 |  |  |
|  | 64 QAM | 4 | 3 |  | 4 |  |  |  |
|  | 256 QAM |  |  |  |  |  |  |  |
| CP-OFDM | QPSK | 5 | 4 |  | 5.5 | 3.5 |  | 2.5 |
|  | 16 QAM | 5 | 4 |  | 5.5 | 3.5 |  |  |
|  | 64 QAM | 5 | 4 |  | 5.5 |  |  |  |
|  | 256 QAM |  |  |  |  |  |  |  |

# 6	References

[1] R4-2415439, NR A-MPR reevaluation scenarios on n1 for Japan, KDDI, NTT Docomo

# 7	Appendix: A-MPR triangles

### 7.1	PC3 simulation results

### 7.2	PC3 proposal

### 7.3	PC2 simulation results

### 7.4	PC2 proposal