3GPP TSG-RAN WG4 Meeting#113                             R4-2419211

Orlando, USA, 18th – 22th Nov, 2024

Agenda item:	7.11.4.2

Source: 	ZTE Corporation, Sanechips

Title: 	Discussion on the applicability improvement of TRP measurement

Document for:	Approval

Introduction

In the RAN#103 meeting, new WID for Rel-19 BS RF evolution has been approved and it’s expected to start the normative work from this April meeting. For the applicability improvement of TRP in Annex, it was agreed to have some clarifications for it to avoid the confusion outside of 3GPP especially.

- Identify BS excessive OTA test scope cases and reduce it with the aim to shorten the test duration time for selected RF requirements, e.g., TX IM, RX out of band blocking, for NR FR1.

Discussion

- 2.1. Tx requirements

For BS RF requirement, there are two most time consuming requirement e.g. transmitter spurious emission requirement and and receiver OOBB requirement because its applicable frequency range is extremely large compared with other requirements.

For the 3rd party OTA verification chamber, it usually take 300 Euro per hour and maximum 10 working hours per day, therefore to ensure the acceptable test coverage with reasonable testing time is extremely important for vendors from both implementation cost and also development Iteration. As pointed out in [2], it has been found that the test time is prohibitive taking up to 3 times the time to perform the conformance testing compared to conducted conformance testing. Excessive OTA testing time has become a constraint for AAS product design, development and certification .

In the contribution [2], the TX IMD texting is identified to comprise of 96 test cases testing output power, ACLR and out of band emissions (all TRP requirements) with 6 interferer offsets are tested for each (12 for NC operation), we propose to focus on the testing scope reduction for OTA Tx IMD requirement as indicated in the following test cases.

Proposals 1: propose to focus on the testing scope reduction for OTA Tx IMD requirement as listed in the following table.

- 2.2. RX out of band blocking

It is well known that RX out of blocking requirement is another most time consuming requirement from receiver perspective in addition to transmitter spurious emission requirement. We also have similar discussion for FR2 BS OOBB requirement in terms of step size for CW signal. The corresponding technical justification is captured in TR 38.817-02. For FR1 BS OOBB requirement testing, the same principle could be reused to reduce the amount of time used for the time-consuming OTA conformance testing, while continue to ensure that any spurious response within the receive bandwidth will still be verified with sufficient granularity.

Proposals 2: propose to reuse the principle design for Rel-15 FR2 BS OOBB CW step size e.g. 1/3 of BW as captured in TR 38.817-02.

Conclusions

In this contribution, we shared some initial views on OTA test scope cases in RAN4 and proposals are made as following:

Proposals 1: propose to focus on the testing scope reduction for OTA Tx IMD requirement as listed in the following table.

Proposals 2: to reuse the principle design for Rel-15 FR2 BS OOBB CW step size e.g. 1/3 of BW as captured in TR 38.817-02.

References

- RP-240829,New WID: WI resulting from discussion on BS RF requirement evolution, Approved.

- RP-240430,Views on BS RF topics for Rel-19, Huawei, approved.