3GPP TSG-RAN WG4 Meeting#113                              R4-2419220

Orlando, USA, 18th – 22th Nov, 2024

Agenda item:	7.20.2.1

Source: 	ZTE Corporation

Title: 	Discussion on potentially new requirements for SBFD operation

Document for:	Approval

Introduction

In the RAN#102 meeting, new WID: Evolution of NR duplex operation: Sub-band full duplex (SBFD) has been approved. It’s expected to start the RAN4 meeting from April meeting. Since we have already reached good progress on the SBFD BS RF impacts in SI phase and outcome was captured in TR 38.858. In this contribution, we would like to share some further views on these remaining open issues.

Sub-topic 2-2: In-channel adjacent subband leakage ratio requirements - Issue 2-2-1: Necessity of in-channel adjacent subband leakage ratio requirements [Background] In RAN4#112, the following WF is approved: - Way forward: - FFS the necessity of in-channel adjacent subband leakage ratio requirements - Option 1: No necessity to introduce new requirement for in-channel adjacent subband leakage ratio. - Option 2: New requirement for in-channel adjacent subband leakage ratio should be defined. - Define a requirement on TX sub-band ACLR similar to the ACLR requirement and use existing ACLR requirement as baseline.. Way Forward: - FFS based on the way forward approved in RAN4#112. Sub-topic 2-3: In-channel adjacent subband selectivity/blocking requirements - Issue 2-3-1/2-3-2: Necessity of in-channel adjacent subband selectivity/blocking requirements [Background] In RAN4#112, the following WF is approved: - Way forward: - FFS the necessity of in-channel adjacent subband selectivity requirements - Option 1: No necessity to introduce new requirement for in-channel adjacent subband selectivity. - Option 2: New requirement for in-channel adjacent subband selectivity should be defined. - Define a requirement on RX sub-band ACS similar to the ACS requirement and use existing ACS requirement as baseline. - Way forward: - FFS the necessity of in-channel adjacent subband blocking requirements - Option 1: No necessity to introduce new requirement for in-channel adjacent subband blocking. - Option 2: New requirement for in-channel adjacent subband blocking should be defined.. Way Forward: - FFS based on the way forward approved in RAN4#112. - Consider the possibility of specifying RX requirement to cover both in-channel adjacent subband selectivity/blocking requirements

Discussion

## 2.1. Potential adjacent sub-band leakage

In the previous RAN4 meeting, we have the extensive discussions on the necessity of adjacent sub-band leakage, however there are no consensus reached so far. Especially for the inter-site scenario, BS CLI problem within single carrier in certain scenario is still one major problems. From our understanding, considering the multi-vendor deployment without any coordination on the BS CLI problem within the co-channel carrier, it might be better to define the minimum requirement or otherwise coordination solutions specified in other approach. Based on the following evaluation results with or without considering co-channel BS2BS CLI assuming reusing the BS ACLR/ACS requirements for DL sub-band and also UL sub-band, the additional interference compared with legacy UE transmission from other cells for indoor scenario are quite limited, in other words, the existing BS ACLR/ACS requirement for sub-band ACLR/ACS requirement should be sufficient. In addition, since DL sub-band and UL sub-band configuration could be any PRB number and also guard band between DL and UL is also supposed to be left up to the implementation, therefore to define the ACLR and ACS requirement for any sub-band configuration will be very complicated and it will also increase lots of the implementation complexity since SBFD BS need to design the digital filtering for any PRB configuration between DL and UL (e.g. 1st order of DDC/DUC in the digital IF module) and also very time consuming to do the conformance testing at the end which is somehow similar as the necessity of RF requirement definition of BWP at UE side.

Proposal 1: for the inter-site scenario, not to define in-channel sub-band leakage and leave this up to the implementation.

In the following section, we provided some initial simulation results with 80MHz DL sub-band configuration and also 20MHz UL sub-band configuration with typical Rapp model used BS PA modelling. It could be easily found that roll-off slope away from transmitted PRB in DL sub-band will increase accordingly which is also varying between PRBs in the UL sub-band. If companies still have strong preference to define the Tx sub-band leakage requirement, then it should be defined similar as UE in-band emission general requirement.

Proposal 2: if companies have strong preference to define the Tx in-band leakage requirement, then prefer to define it similar as UE in-band emission general requirement which could be applicable for any DL sub-band transmission bandwidth configuration and also UL sub-band transmission bandwidth configuration.

Table 6.4.2.3-1: Requirements for in-band emissions

| Parameter description | Unit | Limit (NOTE 1) |  | Applicable Frequencies |
| --- | --- | --- | --- | --- |
| General | dB |  |  | Any non-allocated (NOTE 2) |
| IQ Image | dB | -28 | Image frequencies when output power > 10 dBm | Image frequencies (NOTES 2, 3) |
|  |  | -25 | Image frequencies when output power ≤ 10 dBm |  |
| Carrier leakage | dBc | -28 | Output power > 10 dBm | Carrier leakage frequency (NOTES 4, 5) |
|  |  | -25 | 0 dBm ≤ Output power ≤ 10 dBm |  |
|  |  | -20 | -30 dBm ≤ Output power < 0 dBm |  |
|  |  | -10 | -40 dBm ≤ Output power < -30 dBm |  |
| NOTE 1: An in-band emissions combined limit is evaluated in each non-allocated RB. For each such RB, the minimum requirement is calculated as the higher of PRB - 30 dB and the power sum of all limit values (General, IQ Image or Carrier leakage) that apply. PRB is defined in NOTE 10. NOTE 2: The measurement bandwidth is 1 RB and the limit is expressed as a ratio of measured power in one non-allocated RB to the measured average power per allocated RB, where the averaging is done across all allocated RBs. For pi/2 BPSK with Spectrum Shaping, the limit is expressed as a ratio of measured power in one non-allocated RB to the measured power in the allocated RB with highest PSD. NOTE 3: The applicable frequencies for this limit are those that are enclosed in the reflection of the allocated bandwidth, based on symmetry with respect to the carrier leakage frequency, but excluding any allocated RBs. NOTE 4: The measurement bandwidth is 1 RB and the limit is expressed as a ratio of measured power in one non-allocated RB to the measured total power in all allocated RBs. NOTE 5: The applicable frequencies for this limit depend on the parameter txDirectCurrentLocation in UplinkTxDirectCurrent IE, and are those that are enclosed either in the RB containing the carrier leakage frequency, or in the two RBs immediately adjacent to the carrier leakage frequency but excluding any allocated RB. NOTE 6: LCRB is the Transmission Bandwidth (see clause 5.3). NOTE 7: NRB is the Transmission Bandwidth Configuration (see clause 5.3). NOTE 8: EVM is the limit specified in Table 6.4.2.1-1 for the modulation format used in the allocated RBs. NOTE 9: is the starting frequency offset between the allocated RB and the measured non-allocated RB (e.g. ∆RB = 1 or ∆RB = -1 for the first adjacent RB outside of the allocated bandwidth. NOTE 10: is an average of the transmitted power over 10 sub-frames normalized by the number of allocated RBs, measured in dBm. NOTE 11: For almost contiguous allocations defined in clause 6.2.2, LCRB = NRB_alloc + NRB_gap with no in-gap emission requirement. |  |  |  |  |

## 2.2. Potential adjacent receiver requirements

During the last RAN4 meeting, there were some discussions on the necessity of in-channel adjacent subband selectivity, however there are no consensus reached so far. From our understanding, the DL sub-band signal from the aggressor BSs within the single carrier is orthogonal to the UL sub-band signal reception. In other words, there are no such kind of selectivity needed at all. However based on some comments received online, this DL sub-band interference might cause some nonlinearity to LNA, other RF component similar as in-band blocking signal. Based on our initial simulation results as shown in the following figures, Iot level increase due to BS2BS CLI problem in the DL sub-band region if SBFD BS is capable of receiving the whole carrier instead of UL sub-band only and without considering the BS CLI coordination within the carrier is larger than legacy 25dBc (e.g. 16dBc Iot+9dB for 16QAM demod) for FR1 ICS requirement and also 14dBc (e.g. 5dBc Iot+9dB for 16QAM demod) for FR2 ICS requirement except for FR2 WA scenario; However if so, the BS2BS CLI power level is still expected to lower than in-band blocking power level according to the following evaluation results. The absolute power for in-band blocking level could be provided later on.

FR1:

FR2-1:

Observation 1: increasing Iot level due to BS2BS CLI problem in the DL sub-band region if SBFD BS is capable of receiving the whole carrier instead of UL sub-band only and without considering the BS CLI coordination within the carrier is larger than legacy 25dBc for FR1 ICS requirement and also 14dBcfor FR2 ICS requirement except for FR2 WA scenario.

Observation 2: BS co-channel coordination .e.g. beam nulling or beam coordination could alleviate the increasing IoT level for SBFD BS reception.

Proposal 3: not to define in-channel adjacent subband selectivity/blocking requirements if received BS2BS CLI within the carrier manged lower than in-band blocking power level.

Conclusions

In this contribution, we want to share some initial views on RF requirements for SBFD BS and proposals/observations are made as following:

Proposal 1: for the inter-site scenario, not to define in-channel sub-band leakage and leave this up to the implementation.

Proposal 2: if companies have strong preference to define the Tx in-band leakage requirement, then prefer to define it similar as UE in-band emission general requirement which could be applicable for any DL sub-band transmission bandwidth configuration and also UL sub-band transmission bandwidth configuration.

Observation 1: increasing Iot level due to BS2BS CLI problem in the DL sub-band region if SBFD BS is capable of receiving the whole carrier instead of UL sub-band only and without considering the BS CLI coordination within the carrier is larger than legacy 25dBc for FR1 ICS requirement and also 14dBcfor FR2 ICS requirement except for FR2 WA scenario.

Observation 2: BS co-channel coordination .e.g. beam nulling or beam coordination could alleviate the increasing IoT level for SBFD BS reception.

Proposal 3: not to define in-channel adjacent subband selectivity/blocking requirements if received BS2BS CLI within the carrier manged lower than in-band blocking power level.

References

- RP-234035,New WID: Evolution of NR duplex operation: Sub-band full duplex (SBFD), Huawei, Samsung, Approved.

- TR 38.858

- R4-2321091, TP to TR 38.858 Impact on BS RF requirements, ZTE, Agreed

- R4-2413514, Way Forward for [112][307] NR_duplex_evo_General, Samsung

- R4-2416584,Way forward for [112bis][306] NR_duplex_evo_General, Samsung