3GPP RAN WG4 Meeting #113		R4-2419243

# Orlando, US, 18th – 22nd November, 2024

Source: 	Huawei

Title: 	Test case reduction for TX IM

Agenda Item:	7.11.4.2

Document for:	Approval

1 Introduction

New WID on NR base station (BS) RF requirement evolution for FR1/FR2 and testing was approved at RAN#103 meeting. The work item concentrates on 2 work areas where the current BS specification may be optimised, i.e. Expected EIRP mask for upper 6GHz and OTA test enhancement. One of major identified RF requirements with the aim to shorten the test time is TX IM.

In this contribution we provide considerations on test case reduction for TX IM test reduction.

2 Discussion

Active Antenna Systems are widely deployed in NR. The OTA test is a must for BS type 1-O (AAS architecture). It has been found that the test time is prohibitive taking up to 3 times the time to perform the conformance testing compared to conducted conformance testing. Excessive OTA testing time has become a constraint for AAS product design, development and certification.

It was identified that in some cases the number of test cases could be reduced without impacting the coverage of the testing. OTA TRP testing is particularly time consuming. Two examples are shown in Table 2-1 and 2-2 below.

In Table 2-1, for single band CNC case, the TX IMD testing comprises of 72 test cases testing output power, ACLR and out of band emissions (all TRP requirements). By contrast the output power, ALCR and out of band emissions tests have only 7 test cases in total. And in Table 2-2 for single band MSR AAS, in BC3 CNC case, the TX IMD testing comprises of 96 test cases testing output power, ACLR and out of band emissions. By contrast the output power, ALCR and out of band emissions tests have only 8 test cases in total.

Table 2-1: TRP test cases for single-band CNC (C and NC capable BS with identical parameters) in TS 38.141-2

Table 2-2: TRP test cases for AAS single-band BC3 CNC (C and NC capable BS with identical parameters)

Proposal 1: Multiple test configurations and large number of interferer offsets are generating excessive test time so it is appropriate to study if they are all necessary or can be reduced.

2.1	Interferer offsets

6 interferer offsets are tested for each and 12 if AAS supports NC operation, these offsets date back to the UTRA requirements and have been carried through for each subsequent RAT however not they are generating such excessive test time so it is appropriate to see if they are all still necessary or can be reduced.

Table 6.8.5.1-1 in 38.141-2: Interfering and wanted signals for the OTA transmitter intermodulation requirement

| Parameter | Value |
| --- | --- |
| Wanted signal | NR single or multi-carrier, or multiple intra-band contiguously or non-contiguously aggregated carriers |
| Interfering signal type | NR signal, the minimum BS channel bandwidth (BWChannel) with 15 kHz SCS of the band defined in clause 5.3.5 of TS 38.104 [2] |
| Interfering signal power level | min(46 dBm, Prated,t,TRP) |
| Interfering signal centre frequency offset from the lower (upper) edge of the wanted signal or edge of sub-block inside a gap | , for n=1, 2 and 3 |
| NOTE 1: Interfering signal positions that are partially or completely outside of any downlink operating band of the BS are excluded from the requirement, unless the interfering signal positions fall within the frequency range of adjacent downlink operating bands in the same geographical area. NOTE 2: In Japan, note 1 is not applied in Band n77, n78, n79. NOTE 3: For BS type 1-O supporting dual polarization, the interfering signal power shall be equally divided between supported polarizations at the CLTA interfaces. |  |

For each side test 3 offsets (1xCBW offset, 2xCBW offset, 3xCBW offset) started in UMTS specification and continued through LTE, MSR and NR. In the following text we look at each case respectively.

ACLR

UMTS has ACLR1 and ACLR2 requirement, and ACLR2 is 5 dB harder. LTE and NR effectivity have an ACLR1 and ACLR2 requirement but they are both the same. hence the 1xCBW offset interferer and the 1st adjacent channel can be assumed to be the worst case.

Observation 1: For LTE and NR ACLR, the 1xCBW offset is the worst case so other offsets are not needed.

OBUE

LTE/NR OBUE is generally considered that in the 1st adjacent channel the ALCR requirement dominates and as you move away from the carrier the OBUE is tougher. They have a tougher requirement beyond 10 MHz. As we are testing at M only we can pretty safely say 3xCBW case is probably the toughest.

Observation 2: For OBUE, the 3xCBW offset is the worst case so other offsets are not needed.

Spurious emissions

Spurious emissions are applied 10MHz outside the operating band. The only test case which can generate 3rd order products in SE domain is the 3xCBW case, and this is only if the wanted signal is places on the band edge. As we only test TX IM at middle there are very few bands where we will ever see 3rd order products in the SE domain. If so only the 3xCBW offset seems worth testing.

Observation 3: For spurious emissions, only the 3xCBW offset is needed.

Proposal 2: During TX IMD test, it is proposed to use 1xCBW offset for ACLR test and 3xCBW offset to check OBUE and spurious emissions limits.

2.2	Test configurations

For NR TX IMD test, NRTC1is applicable to contiguous spectrum capable BS and NRTC3 is applicable to C and NC capable BS with identical parameters, and both NRTC1 and NRTC3 are applicable to C and NC capable BS with different parameters. Meanwhile for AAS TS 37.145-2, even for C and NC capable BS with identical parameters, both ATCR7 and ANTCR7 are applied. If follow the same logic, only ANTCR7 is needed for the case.

Proposal 3: for AAS TS 37.145-2, only ANTCR7 is needed for C and NC capable BS with identical parameters, so ATCR7 can be removed from the tests.

3	Summary

In this contribution we provide considerations on test case reduction for TX IM test reduction.

Proposal 1: Multiple test configurations and large number of interferer offsets are generating excessive test time so it is appropriate to study if they are all necessary or can be reduced.

Observation 1: For LTE and NR ACLR, the 1xCBW offset is the worst case so other offsets are not needed.

Observation 2: For OBUE, the 3xCBW offset is the worst case so other offsets are not needed.

Observation 3: For spurious emissions, only the 3xCBW offset is needed.

Proposal 2: During TX IMD test, it is proposed to use 1xCBW offset for ACLR test and 3xCBW offset to check OBUE and spurious emissions limits.

Proposal 3: for AAS TS 37.145-2, only ANTCR7 is needed for C and NC capable BS with identical parameters, so ATCR7 can be removed from the tests.