3GPP RAN WG4 Meeting #113		R4-2419244

# Orlando, US, 18th – 22nd November, 2024

Source: 	Huawei, HiSilicon

Title: 	On modification of existing BS TX requirements for SBFD

Agenda Item:	7.20.2.2

Document for:	Approval

Introduction

During RAN4#112bis, BS RF requirements for SBFD have been discussed. Following conclusions for existing TX requirements are reproduced from [4].

- 1.1. ACLR and OBUE WF: - FFS on the followings: - Whether to define additional ACLR and/or OBUE requirement to ensure feasible coexistence with SBFD-capable gNB firstly? - Option 1: Yes, since legacy ACLR and OBUE requirement might not be sufficient to ensure feasible coexistence. - Option 2: No, since specific co-existence requirements can be covered by regional requirements or customer specific proprietary requirement. - How to define the additional co-existence requirements: - Option 1: based on CL assumption for BS2BS CLI interference and BS DESENS level. - [1dB DESENS in the victim SBFD receiver] - [typical CL values based on typical SBFD-capable gNB implementation] - [frequency offset for the additional OBUE requirement is declared] - Option 2: by improving ACLR1 and ACLR2 requirements - Option 3: others - 1.2. Joint measurement for normal DL symbols/slots and SBFD symbols/slots Agreement: - Averaged EVMs for SBFD slots and non-SBFD slots shall be measured separately. - Averaged EVM for SBFD slots shall be calculated only based on the data samples where DLs are allocated within the SBFD slots.

Good progress was made in last meeting. In this contribution, we would like to share our views regarding the open issues.

Discussion

ACLR and OBUE

Per WF in [3], the following agreement was reached.

Agreement:

- When coexisting with legacy TDD system in adjacent channel, RAN4 shall apply the legacy ACLR and OBUE requirement for SBFD-capable BS.

Per WF in [4], the following WF is FFS.

  - Whether to define additional ACLR and/or OBUE requirement to ensure feasible coexistence with SBFD-capable gNB firstly?

    - Option 1: Yes, since legacy ACLR and OBUE requirement might not be sufficient to ensure feasible coexistence.

    - Option 2: No, since specific co-existence requirements can be covered by regional requirements or customer specific proprietary requirement.

  - How to define the additional co-existence requirements:

    - Option 1: based on CL assumption for BS2BS CLI interference and BS DESENS level.

      - [1dB DESENS in the victim SBFD receiver]

      - [typical CL values based on typical SBFD-capable gNB implementation]

      - [frequency offset for the additional OBUE requirement is declared]

    - Option 2: by improving ACLR1 and ACLR2 requirements

    - Option 3: others

For the first issue, there is some divergence on the concerned deployment scenario for FR1 Macro BS when defining RF requirements. For the already deployed band, co-existence with legacy TDD system in adjacent channel should be considered. For a new wide TDD band without deployment yet, it is promising to deploy SBFD BS to increase UL coverage and reduce latency. As such additional requirements should be defined for co-existence performance between different SBFD BSs operating in adjacent frequency. From the previous discussion and discussion in [R4-2416285], additional blocking requirements for RX and additional OBUE requirement are needed for SBFD BSs, since legacy ALCR and OBUE requirements are not sufficient to ensure feasible coexistence with SBFD-capable BSs. It should be noted that legacy ACLR and OBUE requirement shall apply as general requirements as agreed in [3]. So the additional TX OBUE requirements are optional/regional and can be declared by the vendors only when it is necessary.

Proposal 1: Additional TX OBUE requirements should be defined for co-existence performance between different SBFD BSs operating in adjacent frequency.

For coexistence between different SBFD BSs operating in adjacent frequency, it is BS to BS cross link interference. On the level of the requirements, one reference could be additional spurious emissions which is applied as optional protection of the systems that are deployed in the same geographical area as the BS. It can be found that -52 dBm/MHz is defined for protection of DL UE RX and -49 dBm/MHz for protection of BS UL RX. For protection of SBFD UL, the limit -49 dBm/MHz for protection of BS UL can be reused.

Table 2.2-1: Table 6.6.5.2.3-1 in TS 38.104: BS spurious emissions basic limits for BS for co-existence with systems operating in other frequency bands

| System type for NR to co-exist with | Frequency range for co-existence requirement | Basic limits | Measurement bandwidth | Note |
| --- | --- | --- | --- | --- |
| UTRA FDD Band I or | 2110 – 2170 MHz | -52 dBm | 1 MHz | This requirement does not apply to BS operating in band n1 or n65 |
| E-UTRA Band 1 or NR Band n1 | 1920 – 1980 MHz | -49 dBm | 1 MHz | This requirement does not apply to BS operating in band n1 or n65, since it is already covered by the requirement in clause 6.6.5.2.2. |
| UTRA FDD Band II or | 1930 – 1990 MHz | -52 dBm | 1 MHz | This requirement does not apply to BS operating in band n2 or n70. |
| E-UTRA Band 2 or NR Band n2 | 1850 – 1910 MHz | -49 dBm | 1 MHz | This requirement does not apply to BS operating in band n2, since it is already covered by the requirement in clause 6.6.5.2.2. |
| UTRA FDD Band III or | 1805 – 1880 MHz | -52 dBm | 1 MHz | This requirement does not apply to BS operating in band n3. |
| E-UTRA Band 3 or NR Band n3 | 1710 – 1785 MHz | -49 dBm | 1 MHz | This requirement does not apply to BS operating in band n3, since it is already covered by the requirement in clause 6.6.5.2.2. |
| … | … |  |  |  |
| NR Band n77 | 3.3 – 4.2 GHz | -52 dBm | 1 MHz | This requirement does not apply to BS operating in Band n48, n77 or n78 |
| NR Band n78 | 3.3 – 3.8 GHz | -52 dBm | 1 MHz | This requirement does not apply to BS operating in Band n48, n77 or n78 |
| NR Band n79 | 4.4 – 5.0 GHz | -52 dBm | 1 MHz | This requirement does not apply to BS operating in Band n79 |
| … | … |  |  |  |
| NR Band n104 | 6425 – 7125 MHz | -52 dBm | 1 MHz | This requirement does not apply to BS operating in Band n96, n102 or n104 |
| … | … |  |  |  |

In this case a new classification has to be introduced because the adjacent channel interference is generated in a different manner. The classification, based on the amount of BS-to-BS interference, the most destructive interference due to the presence of a not synchronised operator, is the following:

-	worst case scenario: 0 m base station shifting (co-siting);

-	intermediate case scenario: 577/2 m base station shifting;

-	best case scenario: 577 m base station shifting.

…

The base station antenna gain used to calculate the power received in this case is 10 dB, instead of 13 dB, to consider the tilt of the antennas.

Thus, since the distance between BSs of different operators is 577/2 m, the path loss is 87 dB, and, including the antenna gains, 67 dB.                                                  (Text in TS 25.942)

The -49 dBm/MHz assumes 67 dB MCL and 7dB lower than noise floor, which is derived for 3G system at an intermediate case scenario (577/2 m base station shifting, 2GHz UMTS band) in TR25.942, as shown below. These requirements are defined as the same for all existing FR1 bands. It can be anticipated that at higher frequency band, e.g. n104 upper 6GHz, similar MCL assumption cover more scenarios.

Observation 1: For protection of SBFD UL, the limit -49 dBm/MHz for protection of BS UL can be reused.

Table 2.2-2 shows an example link budget for adjacent channel co-existence if 67 dB MCL is assumed.

Table 2.2-2: Adjacent channel co-existence

| Aggressor | Victim | Aggressor (Operator A) Victim (Operator B) | Link budget |
| --- | --- | --- | --- |
| SBFD | SBFD | D D U U |  |
| TX |  | Guard band -15 dBm/MHz | -15 dBm |
| TX Filter | RX Filter |  | Required attenuation from TX filter: -15+49=34 dB |
| Pathloss/coupling loss |  | e.g. co-existence spurious emissions assume 67 dB MCL | 67 dB |
| Emission level |  | -49 dBm/1MHz in UL, assuming 67 dB MCL +7dB lower than noise floor. | -49 dBm/MHz |

As shown in Table 2.2-2, for protection of SBFD UL of operator B, the SBFD DL of operator B can be adopted as guard band for filter implementation of adjacent operator A. In existing FR1 networks, usually operators hold less than or equal to 400 MHz. Using n104 as an example, the available guard band for TX filter is analysed in table 2.2-3. For the band n104 which has 700 MHz band width, considering it is allocated to 2 ~ 3 operators, each may hold 200 MHz, 300 MHz or 400 MHz. Table 2.2-3 shows the available guard band examples for different operator’s spectrum scenarios.

Table 2.2-3: available guard band examples for different operator’s spectrum scenarios

| Scenario No. | Operator's spectrum | Achieved guard band (MHz) |  | min Ratio of transition guard/sub frequency band | Comments |
| --- | --- | --- | --- | --- | --- |
|  |  | 2 operator case | 3 operator case (in middle) |  |  |
| 1 | 100 MHz | 80 | 40 | 40% | Configuration 1: 40-20-40 Configuration 2: 80-20 |
| 2 | 200 MHz | 180 | 80 | 30% | Configuration 1: SBFD 20-80+ TDD 100 |
| 3 | 300 MHz | 260 | 100 | 33.30% | Configuration 1: SBFD 40-60+ TDD 100+ TDD 100 |
| 4 | 400 MHz | 360 | N/A | 90% | Configuration 1: SBFD 40-60+ TDD 100+ TDD 100+ TDD 100 |

Observation 2: available guard band for different operators’ spectrum cases are different, which depends on the spectrum allocation: number of operations in one band and bandwidth for each operator.

Proposal 2: It is proposed to declare the frequency offset for the additional OBUE requirement.

As a reference, in existing TX requirement, ΔfOBUE is defined based on filter implementation. For FR1 operating band less than 100 MHz, ΔfOBUE is 10 MHz, and for band wider than 100 MHz except n104, ΔfOBUE is extended to 40 MHz. ΔfOBUE is further extended to 100 MHz for band n104. It can be found min ratio of transition guard/sub frequency band is much higher than that of ΔfOBUE, which means it would not impose additional implementation difficulty for taking the advantage of per operator filter. For example, we check the performance of filter used for existing 3.5 GHz band dielectric filter with 200 MHz pass band, the rejection is larger than 40 dB at the offset of 100 MHz.

Table 2.2-4: Maximum offset ΔfOBUE outside the downlink operating band

| BS type | Operating band characteristics (MHz) | ΔfOBUE (MHz) | minimum Ratio of ΔfOBUE /bandwidth |
| --- | --- | --- | --- |
| BS type 1-O | FDL,high – FDL,low < 100 | 10 | 10% |
|  | 100 MHz ≤ FDL,high – FDL,low ≤ 900 (Note: operators hold less than or equal to 400 MHz ) | 40 | 10% |
|  | Band n104 | 100 | 14.3% |

Observation 3: The ratio of transition guard/sub frequency band is higher than that of ΔfOBUE, which means it would not impose additional implementation difficulty for taking the advantage of per operator filter.

One example filter with 6 orders and 2 poles for is shown in figure, the insertion loss within 6425-6725 MHz is less than 1.2 dB. The rejection at 100 MHz offset (ΔfOBUE) of lower side is larger than 29 dB, and the rejection is larger than 38 dB at upper side from 100 MHz offset onward. For 2 operator case in Table 2.2-3, the guard band for 300 MHz sub band can be as large as 260 MHz. So the rejection can be achieved without additional cost.

Figure 2.2-1: example filter evaluation for 6425-6725 MHz sub-band

Observation 4: with larger frequency offset than ΔfOBUE, the required rejection for protection of SBFD UL is achievable for the analog filter.

Proposal 3: For protection of SBFD UL, it is proposed to define the limit -49 dBm/MHz as the additional OBUE requirements.

Proposal 4: For protection of SBFD UL, text proposal for the additional OBUE requirement is proposed as below

==== Text proposal for SBFD additional OBUE requirement for TS 38.104====

6.6.4.2.5.8	Additional operating band unwanted emissions limits for SBFD-capable BS

In regions where SBFD-capable BSs are operating in adjacent channel. For protection of SBFD UL sub-band in adjacent channel, emissions shall not exceed the maximum levels specified in table 6.6.4.2.5.8-1. f is the separation between the SBFD DL edge frequency and the nominal -3 dB point of the measuring filter closest to the edge.

Table 6.6.4.2.5.8-1: additional OBUE requirement for protection of SBFD UL

| Declared frequency offset of measurement filter -3dB point, f | Minimum requirement | Measurement bandwidth |
| --- | --- | --- |
| f | -49 dBm | 1MHz |

==== End of Text proposal ====

Conclusion

In this contribution we discussed on the BS TX requirement for SBFD. We have the following observations and proposals:

Proposal 1: Additional TX OBUE requirements should be defined for co-existence performance between different SBFD BSs operating in adjacent frequency.

Observation 1: For protection of SBFD UL, the limit -49 dBm/MHz for protection of BS UL can be reused.

Observation 2: available guard band for different operators’ spectrum cases are different, which depends on the spectrum allocation: number of operations in one band and bandwidth for each operator.

Proposal 2: It is proposed to declare the frequency offset for the additional OBUE requirement.

Observation 3: The ratio of transition guard/sub frequency band is higher than that of ΔfOBUE, which means it would not impose additional implementation difficulty for taking the advantage of per operator filter.

Observation 4: with larger frequency offset than ΔfOBUE, the required rejection for protection of SBFD UL is achievable for the analog filter.

Proposal 3: For protection of SBFD UL, it is proposed to define the limit -49 dBm/MHz as the additional OBUE requirements.

Proposal 4: For protection of SBFD UL, text proposal for the additional OBUE requirement is proposed as below

==== Text proposal for SBFD additional OBUE requirement for TS 38.104====

6.6.4.2.5.8	Additional operating band unwanted emissions limits for SBFD-capable BS

In regions where SBFD-capable BSs are operating in adjacent channel. For protection of SBFD UL sub-band in adjacent channel, emissions shall not exceed the maximum levels specified in table 6.6.4.2.5.8-1. f is the separation between the SBFD DL edge frequency and the nominal -3 dB point of the measuring filter closest to the edge.

Table 6.6.4.2.5.8-1: additional OBUE requirement for protection of SBFD UL

| Declared frequency offset of measurement filter -3dB point, f | Minimum requirement | Measurement bandwidth |
| --- | --- | --- |
| f | -49 dBm | 1MHz |

==== End of Text proposal ====

- References

- R4-2406107, “Way Forward for [110bis][313] NR_duplex_evo”, Samsung, 3GPP TSG-RAN4 Meeting #110bis, Changsha, China, April 15-19, 2024.

- R4-2409958	Way Forward for [111][313] NR_duplex_evo	Samsung

- R4-2413515, Way Forward for [112][308] NR_duplex_evo_BSRF, Huawei, HiSilicon

- R4-2416585, Way Forward for [112bis][307] NR_duplex_evo_BSRF, Huawei, HiSilicon