3GPP RAN WG4 Meeting #113		R4-2419246

# Orlando, US, 18th – 22nd November, 2024

Source: 	Huawei, HiSilicon

Title: 	TP for CW interference cancelation

Agenda Item:	7.21.3.4

Document for:	Approval

1 Introduction

The new SID on solutions for Ambient IoT was approved at the RAN#102 meeting [1] and later revised [2] at the RAN#103 meeting. The RAN4-led objectives are copied as below:

- RAN4-led:

  - Coexistence study of Ambient IoT and NR/LTE.

  - RF requirements study for Ambient IoT:

    - Ambient IoT BS transmission and reception

    - Ambient IoT Device, as per the General Scope, transmission and reception

    - Intermediate node (UE), as per the General Scope, transmission and reception

In this contribution, we provide TP for CW interference cancelation, on top of endorsed TP R4-2417195.

2	Discussion

Two aspects are evaluated: (1) Blocking of D2R receiver; (2) Carrier-wave interference cancellation

Blocking of D2R receiver

If the carrier-wave source is separately deployed from the D2R receiver (e.g., D1T1-A1 and D1T1-B scenarios), the spatial isolation relies on the distance between the CW node and A-IOT node. For example, the path loss is 57 dB for an isolation distance of 20 meters, assuming free-space propagation. Normally the CW node and A-IOT node deploy at the same height, the antenna gain is about -5 dB at 90°offset from the boresight for both TX antenna and RX antenna. For D1T1-A1 and D1T1-B scenarios the received power at the antenna connector is 23-57-10= -44 dBm and 23-51-10= -38 dBm for 20 meters and 10 meters distance respectively, which is close to the narrowband blocking requirements for small cell NR base station. Hence there is no blocking issue for these scenarios.

In the case of integrated carrier-wave transmitter and D2R receiver in D1T1-A2, analog interference cancellation in the RF front-end can be applied when the interference power exceeds the blocking threshold of receiver. The performance heavily depends on the similarity between the reconstructed and actual received interference. A suppression of 10-30 dB can usually be expected for RF interference cancellation. Assuming the same reference clock between the carrier-wave transmitter and D2R receiver (e.g., in D1T1-A1/A2 scenario, or D1T1-B scenario with wired connection between the base station and the carrier-wave node), higher interference suppression can be reached, such as 20 dB or higher.

Carrier-wave interference

In the case of separated carrier-wave transmitter and D2R receiver in D1T1-A1 or D1T1-B, through spatial isolation, RF interference cancellation and high-pass filtering, the overall suppression to the carrier-wave interference can reach about (61+10+80) = 151 dB at 10 meters distance. For larger spatial isolation corresponding to longer distance between the carrier-wave transmitter and D2R receiver (e.g., ~60 dB @ 25 m), the carrier-wave interference can be even higher (e.g., 70+10+80 = 160 dB).

In the case of integrated carrier-wave transmitter and D2R receiver in D1T1-A2, with ~40 dB interference suppression provided by a circulator/directional coupler and/or the antenna with high isolation between Tx and Rx, the carrier-wave interference suppression can reach about (40+20+80) = 140 dB.

2	Conclusion

This contribution provides Text Proposals on Ambient CW cancellation.

Proposal 1: it is proposed to endorse the attached TP for TR 38.769 clause 6.7.5.

References

[1] RP-240829, New WID: NR base station (BS) RF requirement evolution for FR1/FR2 and testing, RAN4 Chair (Huawei)

[2] R4-2410597, WF on impacts of A-IoT on RF requirements, Huawei

[3] R4-2417195, TP for CW interference cancelation, Huawei, HiSilicon, Ericsson

TP for TR 38.769

6.7.5	CW interference cancelation study

6.7.5.1	General

The CW cancellation can be divided into 3 stages sequentially: (1) Spatial isolation for separate TX/RX antenna configuration; (2) Analog cancellation from antenna to ADC and (3) Digital cancellation at baseband.

Spatial isolation for [separate TX/RX antenna configuration]

For separate TX/RX antenna configuration, the spatial isolation can be achieved by antenna isolation, or coupling loss between CW transmitter and D2R receiver.

For example, if the CW transmission source is separately deployed from the D2R receiver, the spatial isolation relies on the distance between the CW node and A-IOT node.

Analog cancellation

Interference cancellation in the RF front-end can be applied to reduce the desensitization of the receiver. In principle, it can be implemented by reconstructing a local CW with the same amplitude but opposite phase of a CW interferer received from antenna at the input of the LNA, and combine this cancellation signal with the received signal. The cancellation performance depends on the phase and amplitude error between the reconstructed and actual received CW interference.

Additionally in the case of integrated carrier-wave transmitter and D2R receiver in sharing antenna case, the interference suppression could be provided by a circulator/directional coupler.

Digital cancellation

In the digital baseband, suppression can be achieved by filtering.

6.7.5.2	Interference cancelation analysis

6.7.5.2.1	Ericsson

The min received power could be -74 dBm or lower, the signal with such low level received power should be decoded in the presence of the CW transmission which is either 33 dBm or 23 dBm. The CW transmission can leak to the receiver chain via either reflection from environment or the imperfection of the isolation between transmitter and receiver when CW is transmitted by A-IoT BS or UE as reader. As the receiver has limited dynamic range and linearity, this CW leakage needs to be attenuated/isolated to mitigate the desensitization of the receiver.  The CW signal frequency is used to modulate the backscatter signal and the frequency separation of the CW signal and modulated signal is around 100kHz for example as illustrated in Figure 6.7.5.2.1-1 according to the current D2R waveform design. Using an RF filter before the LNA to attenuate the CW signal is not possible. As such, a high level CW leakage signal will be present together with a low wanted backscattered signal before LNA.

Figure 6.7.5.2.1-1: D2R waveform in frequency domain

To analysis the impact of the receiver desensitization, the backscattered signal is modelled as below[2]:

Sb (t)=Abcos⁡(ωb∙t+φb)∙cosωc∙t+φc

with ωc representing the CW frequency and ωb is the message frequency.

And the leakage CW signal is modelled as below:

SI (t)=AI∙cosωc∙t+φI

Then the receiver nonlinearity is modelled with a power series below assuming the second-order nonlinear product can be filtered:

yot=a1∙xt+a3∙xt3

Where a1 is the small signal gain of the receiver and a3<0 represents the nonlinear behaviour of the system.

The input signal before LNA is then:

xt=Sb t+SI (t)

Assuming the φb=φc=φI=0, at output of the LNA, only the terms associated with fundamental frequency coswbtcos⁡(ωct) are of interest and they are below:

yoIt=a1Ab+916a3Ab3+94a3AbAI2cosωb∙t∙cosωc∙t

As the CW signal is much larger than backscattered signal, the term 916∙a3∙Ab3 can be dropped and further simplified with

yoIt=a1Ab+94a3AbAI2cosωb∙t∙cosωc∙t

The voltage of the fundamental signal at output of LNA can be reformulated as:

vt=a1∙Ab∙1+94∙a3a1∙AI2

Since a3<0, we can write:

vt=a1∙Ab∙1-94∙a3a1∙AI2

It can be observed from the above that the wanted signal level is degraded with scaling factor which directly reduces the SNR, therefore the desensitization factor is:

SNRD=20log101-94a3a1∙AI2

Further the IIP3 of LNA can be expressed with:

AIIP3_LNA2=|a1||a3|∙43

Therefore, the SNR degradation can be formulated below:

SNRD=20log101-3∙AI2AIIP3LNA2=20log101-3∙PcwleakgePIIP3LNA

Assuming CW signal is transmitted by A-IoT BS or UE as reader and monostatic antenna configuration, with the return loss of 15 dB, the CW leakage power will be 18 dBm. If the SNR degradation is allowed with 3 dB, the IIP3 of LNA should be 28 dBm. Such high IIP3 is demanding for design. For other combination value for SNR degradation and CW leakage power, it is illustrated in Figure 6.7.5.2.1-2 and the IIP3 value is listed in Table 6.7.5.2.1-1. Referring to TR 38.858, the reported IIP3 for WA BS is from the range of 10 dBm to -27.6 dBm, this in turn demands some CW cancellation capability before LNA.

Table 6.7.5.2.1-1:  IIP3 of LNA with different SNR degradation and CW leakage power

|  |  | SNR degradation (dB) |  |  |
| --- | --- | --- | --- | --- |
|  |  | -1 | -3 | -10 |
| CW leakage power (dBm) | 18 | 32 | 28 | 24 |
|  | 8 | 22 | 18 | 14 |
|  | 0 | 14 | 10 | 6 |
|  | -10 | 4 | 0 | -4 |

Figure 6.7.5.2.1-2: IIP3 of LNA with different SNR degradation and CW leakage power

For CW cancellation, there are passive or active cancellation schemes existing for RFID reader front-end. In both schemes, a copy of the CW leakage signal is created with same amplitude but opposite phase. The cancellation scheme is illustrated in Figure 6.7.5.2.1-3. For the passive cancellation scheme, the cancellation path needs to be calibrated and for active cancellation scheme, the cancellation path can be tuned to adapt to the leakage CW signal. However, the cancellation is limited by either the hardware imperfection or the detecting signal amplitude/phase accuracy. In [6], it is mentioned that for 40 dB CW cancellation, the amplitude error should be 0.09 dB and phase error to be 0.5 degree. This is quite demanding in design considering the current RRM measurement accuracy for RSRP is +/-2 dB in normal condition.

When the CW and A-IoT signal are transmitted from different hardware, the active CW cancellation scheme should be used in this case. This applies to the case where the CW node is co-located with A-IoT BS for D1T1-A2 or D2T2-A2.

Figure 6.7.5.1-3: CW cancellation scheme

6.7.5.2.2	Huawei

3 aspects are evaluated: (1) Blocking of D2R receiver; (2) Carrier-wave interference cancellation

Blocking of D2R receiver

If the carrier-wave source is separately deployed from the D2R receiver (e.g., D1T1-A1 and D1T1-B scenarios), the spatial isolation relies on the distance between the CW node and A-IOT node. For example, the path loss is 57 dB for an isolation distance of 20 meters, assuming free-space propagation. Normally the CW node and A-IOT node deploy at the same height, the antenna gain is about -5 dB at 90°offset from the boresight for both TX antenna and RX antenna. For D1T1-A1 and D1T1-B scenarios the received power at the antenna connector is 23-57-10= -44 dBm and 23-51-10= -38 dBm for 20 meters and 10 meters distance respectively, which is close to the narrowband blocking requirements for small cell NR base station. Hence there is no blocking issue for these scenarios.

In the case of integrated carrier-wave transmitter and D2R receiver in D1T1-A2, analog interference cancellation in the RF front-end can be applied when the interference power exceeds the blocking threshold of receiver. The performance heavily depends on the similarity between the reconstructed and actual received interference. A suppression of 10-30 dB can usually be expected for RF interference cancellation. Assuming the same reference clock between the carrier-wave transmitter and D2R receiver (e.g., in D1T1-A1/A2 scenario, or D1T1-B scenario with wired connection between the base station and the carrier-wave node), higher interference suppression can be reached, such as 20 dB or higher.

Overall carrier-wave interference

In the case of separated carrier-wave transmitter and D2R receiver in D1T1-A1 or D1T1-B, through spatial isolation, RF interference cancellation and high-pass filtering, the overall suppression to the carrier-wave interference can reach about (61+10+80) = 151 dB at 10 meters distance. For larger spatial isolation corresponding to longer distance between the carrier-wave transmitter and D2R receiver (e.g., ~60 dB @ 25 m), the carrier-wave interference can be even higher (e.g., 70+10+80 = 160 dB).

In the case of integrated carrier-wave transmitter and D2R receiver in D1T1-A2, with ~40 dB interference suppression provided by a circulator/directional coupler and/or the antenna with high isolation between Tx and Rx, the carrier-wave interference suppression can reach about (40+20+80) = 140 dB.

6.7.5.2.3	Company C

6.7.5.3	Summary

To be added.