3GPP RAN WG4 Meeting #113 		R4-2419247

Orlando, US, 18th – 22nd November, 2024

Agenda item:	7.12.2.2

Source:	CAICT

Title: 	Performance metric for FR1 NTN

Document for:	Discussion

1	Introduction

This document is to provide our furthrer views on OTA performance metric for FR1 NTN devices.

2	Test methodology for FR1 NTN devices

2.1	Performance metric for IoT NTN devices

In RAN4#112 bis, the performance metric for IoT NTN device was FFS. We have surveyed the possible device types for IoT NTN equipment and analyzed the performance parameters applicable to each type of device.

| UE Type |  |  | Describe | Body phantom |
| --- | --- | --- | --- | --- |
| Handheld and wearable devices | A | Handheld wireless satellite terminal that supports voice only. | Its handheld terminal includes a wireless module that supports satellite voice functionality only. | Hand phantom |
|  | B | Handheld wireless satellite terminal that supports data only. | Its handheld terminal includes a wireless module that supports satellite data functionality only. | Free space、Hand phantom |
|  | C | Handheld wireless satellite terminal that supports data and voice. | Its handheld terminal includes a wireless module that supports both satellite voice and data functionalities. | Free space、Hand phantom、Arm phantom |
|  | F | Non-handheld terminal that does not have an external antenna and supports either voice or data. | Its non-handheld terminal has an integrated antenna unit with the main body, designed to be worn on the arm or head. | Free space、Hand phantom、 |
| Portable devices | D | A non-handheld integrated terminal that is equipped with a small-aperture external parabolic antenna or other types of external antennas, and supports either voice or data. | The antenna unit is separate from the main body. | Free space |
|  | E | A non-handheld terminal equipped with an external antenna that supports either voice or data. | The antenna unit is integrated with the main body. | Free space |

Observation 1: The antenna situation of the  IoT NTN equipment under test is uncertain. Global measurements can apply to all devices, especially those only have IoT NTN satellite communication capabilities and use data or voice for emergency calls.

Observation 2: Smartphones are particularly different; theoretically, it is challenging to meet both OTA and SAR requirements globally at the same time. It is reasonable to use directional antennas in antenna design to concentrate energy in one direction, and smartphone software can guide users to find and align with satellites, making the direction more precise. Therefore, testing in a specific direction during testing would be more accurate and time-saving. In R&D testing scenarios, such tests are necessary.

Observation 3: For certification testing, the antenna directivity remains a black box unless the manufacturer agrees to provide the radiation pattern information. Therefore, to make measurements more accurate when it is uncertain whether the manufacturer will provide this information, global measurements should be used.

Observation 4: It is also reasonable to give greater weight to the upper hemisphere. However, it is premised on specifying the placement of the UE, such as keeping the coordinate system unchanged and placing it horizontally (e.g., the phone screen parallel to the XOY plane) or vertically (e.g., the phone screen parallel to the XOZ plane), and giving higher weight to the upper hemisphere of the UE under different placement conditions.

Proposal 1:	For handheld IoT NTN devices, full sphere metrics, namely TRP and TRS should be adopted.

Observation 5: Non-handheld devices typically have a specific installation form. For example, the aperture of the external antenna points towards the sky, and the base of the antenna is installed in open areas on rooftops, car roofs, aircraft, boats, etc., while the main unit is placed separately from the antenna. Integrated antenna-main units also usually follow certain installation procedures. Wearable devices, such as watches, are typically used with the screen facing upwards.

Observation 7: Under a certain physical installation form, the terminal antenna has initial directivity (e.g., θ≤30°). Therefore, the main lobe of the antenna radiation pattern is within the upper hemisphere.

Observation 7: It is suggested that non-global metrics consider distinguishing between terminals using high-orbit satellites and those using low-orbit satellites. The main beamwidth of the antenna for terminals using high-orbit satellites may be smaller than that for terminals using low-orbit satellites.

Proposal 2:	For non-handheld IoT NTN devices, partial sphere metrics, such as the radiated power and receive sensitivity within a certain angular range, should be adopted.

3	Conclusions

Proposal 1:	For handheld IoT NTN devices, full sphere metrics, namely TRP and TRS should be adopted.

Proposal 2:	For non-handheld IoT NTN devices, partial sphere metrics, such as the radiated power and receive sensitivity within a certain angular range, should be adopted.