3GPP TSG-RAN WG4 Meeting #113	R4-2419250

# Orlando, US, November 18th – 22th 2024

Source:	Ericsson

Title:	Simulation results for Rel-19 MMSE-IRC BS demodulation requirements

Agenda item:	7.16.3

Document for:	Information

# Introduction

In RAN4#112bis meeting, companies agreed to reuse LTE signal/interference model and receiver model as baseline for Rel-19 NR MMSE-IRC BS requirement discussion [1]. In this contribution, the trial simulation results are delivered.

# Simulations

In this simulation, the DIP values in LTE MMSE-IRC BS interference profile [2] are converted to INR values based on the formula in [3]. The trial simulations for HPUE considered for HetNet deployment assuming INR1 is 7dB higher than INR1 from LTE profile.

2.1	Simulation assumptions

Table 2.1-1 Mapping from LTE DIP values to INR values

| Scenario | Interferer number | (DIP1, DIP2) [dB] | (INR1, INR2) [dB] |
| --- | --- | --- | --- |
| HomNet | 1 interferer | (-1.11, NA) | (5.36, NA) |
|  | 2 interferers | (-1.11, -10.91) | (7.29, -2.5) |
| HetNet | 1 interferer | (-0.43, NA) | (9.83, NA) |
|  | 2 interferers | (-0.43, -13.78) | (12.38, -0.97) |

Table 2.1-2: Test parameters

| Parameter |  |  |  | Value |
| --- | --- | --- | --- | --- |
| Transform precoding |  |  |  | Disabled/Enabled |
| Default TDD UL-DL pattern (Note 1) |  |  |  | 15 kHz SCS: 3D1S1U, S=10D:2G:2U 30 kHz SCS: 7D1S2U, S=6D:4G:4U |
| HARQ |  | Maximum number of HARQ transmissions |  | 4 |
|  |  | RV sequence |  | 0, 2, 3, 1 |
| DM-RS |  | DM-RS configuration type |  | 1 |
|  |  | DM-RS duration |  | single-symbol DM-RS |
|  |  | Additional DM-RS position |  | pos1 |
|  |  | Number of DM-RS CDM group(s) without data |  | 2 |
|  |  | Ratio of PUSCH EPRE to DM-RS EPRE |  | -3 dB |
|  |  | DM-RS port(s) |  | {0} |
|  |  | DM-RS sequence generation |  | NID0=0, nSCID =0 |
| Time domain resource assignment |  | PUSCH mapping type |  | A |
|  |  | Start symbol |  | 0 |
|  |  | Allocation length |  | 14 |
| Frequency domain resource assignment |  | RB assignment |  | Full applicable test bandwidth |
|  |  | Frequency hopping |  | Disabled |
| TPMI index for 2Tx two layer or 4Tx four layer spatial multiplexing transmission |  |  |  | 0 |
| Code block group based PUSCH transmission |  |  |  | Disabled |
| Interference (Note 2) | Set 1 (HomNet) | 1 interferer | INR1 (dB) | 5.36 |
|  |  | 2 interferers | INR1 (dB) | 7.29 |
|  |  |  | INR2 (dB) | -2.5 |
|  | Set 2 (HetNet) | 1 interferer | INR1 (dB) | 9.83, HPUE: 16.83 |
|  |  | 2 interferers | INR1 (dB) | 12.38, HPUE: 19.38 |
|  |  |  | INR2 (dB) | -0.97 |
| NOTE 1: The same requirements are applicable to FDD and TDD with different UL-DL patterns. NOTE 2: The receiver model and the interference model are defined in Annex A. |  |  |  |  |

2.2	Simulation results

Table 1: Minimum requirements for PUSCH with 70% of maximum throughput, Type A, 1 Transmission Layer, 5 MHz channel bandwidth, 15 kHz SCS with Transform Precoding Disabled with 1 Interferer

| TX | RX | Propagation conditions and correlation matrix |  | INR1 [dB] | MCS In Table 1 | FRC | SNR [dB] |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Tested signal | Interferer 1 |  |  |  | IRC | MRC |
| 1 | 2 | TDLB100-100 |  | 9.83 | 2 | G_FR1_A3-8 | -0.47 | 3.33 |
|  |  |  |  | 5.36 | 2 | G_FR1_A3-8 | -1.78 | 0.10 |
|  | 4 | TDLC300-100 |  | 9.83 | 16 | G_FR1_A4-8 | 7.65 | 12.53 |
|  |  |  |  | 5.36 | 16 | G_FR1_A4-8 | 6.69 | 9.39 |
|  | 8 | TDLA30-10 |  | 9.83 | 20 | G_FR1_A5-8 | 4.92 | 9.69 |
|  |  |  |  | 5.36 | 20 | G_FR1_A5-8 | 4.79 | 7.04 |

Table 2: Minimum requirements for PUSCH with 70% of maximum throughput, Type A, 1 Transmission Layer, 5 MHz channel bandwidth, 15 kHz SCS with Transform Precoding Disabled with 2 Interferer

| TX | RX | Propagation conditions and correlation matrix |  | (INR1, INR2) [dB] | MCS In Table 1 | FRC | SNR [dB] |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Tested signal | Interferer 1&2 |  |  |  | IRC | MRC |
| 1 | 2 | TDLB100-100 |  | (12.38, -0.97) | 2 | G_FR1_A3-8 | 2.05 | 5.90 |
|  |  |  |  | (7.29, -2.5) | 2 | G_FR1_A3-8 | 0.035 | 2.08 |
|  | 4 | TDLC300-100 |  | (12.38, -0.97) | 16 | G_FR1_A4-8 | 9.69 | 15.08 |
|  |  |  |  | (7.29, -2.5) | 16 | G_FR1_A4-8 | 8.18 | 11.31 |
|  | 8 | TDLA30-10 |  | (12.38, -0.97) | 20 | G_FR1_A5-8 | 5.55 | 12.08 |
|  |  |  |  | (7.29, -2.5) | 20 | G_FR1_A5-8 | 5.33 | 8.82 |

Table 3: Minimum requirements for PUSCH with 70% of maximum throughput, Type A, 1 Transmission Layer, 10 MHz channel bandwidth, 30 kHz SCS with Transform Precoding Disabled with 1 Interferer

| TX | RX | Propagation conditions and correlation matrix |  | INR1 [dB] | MCS In Table 1 | FRC | SNR [dB] |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Tested signal | Interferer 1 |  |  |  | IRC | MRC |
| 1 | 2 | TDLB100-100 |  | 9.83 | 2 | G_FR1_A3-11 | -0.12 | 3.35 |
|  |  |  |  | 5.36 | 2 | G_FR1_A3-11 | -1.64 | 0.20 |
|  | 4 | TDLC300-100 |  | 9.83 | 16 | G_FR1_A4-11 | 7.85 | 12.70 |
|  |  |  |  | 5.36 | 16 | G_FR1_A4-11 | 6.82 | 9.54 |
|  | 8 | TDLA30-10 |  | 9.83 | 20 | G_FR1_A5-11 | 5.07 | 9.90 |
|  |  |  |  | 5.36 | 20 | G_FR1_A5-11 | 4.88 | 7.23 |

Table 4: Minimum requirements for PUSCH with 70% of maximum throughput, Type A, 1 Transmission Layer, 10 MHz channel bandwidth, 30 kHz SCS with Transform Precoding Disabled with 2 Interferer

| TX | RX | Propagation conditions and correlation matrix |  | (INR1, INR2) [dB] | MCS In Table 1 | FRC | SNR [dB] |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Tested signal | Interferer 1&2 |  |  |  | IRC | MRC |
| 1 | 2 | TDLB100-100 |  | (12.38, -0.97) | 2 | G_FR1_A3-11 | 2.38 | 5.92 |
|  |  |  |  | (7.29, -2.5) | 2 | G_FR1_A3-11 | 0.18 | 2.14 |
|  | 4 | TDLC300-100 |  | (12.38, -0.97) | 16 | G_FR1_A4-11 | 9.94 | 15.27 |
|  |  |  |  | (7.29, -2.5) | 16 | G_FR1_A4-11 | 8.29 | 11.47 |
|  | 8 | TDLA30-10 |  | (12.38, -0.97) | 20 | G_FR1_A5-11 | 5.80 | 12.34 |
|  |  |  |  | (7.29, -2.5) | 20 | G_FR1_A5-11 | 5.45 | 9.03 |

Table 5: Minimum requirements for PUSCH with 70% of maximum throughput, Type A, 1 Transmission Layer, 5 MHz channel bandwidth, 15 kHz SCS with Transform Precoding Enabled with 1 Interferer

| TX | RX | Propagation conditions and correlation matrix |  | INR1 [dB] | MCS In Table 1 | FRC | SNR [dB] |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Tested signal | Interferer 1 |  |  |  | IRC | MRC |
| 1 | 2 | TDLB100-100 |  | 9.83 | 2 | G_FR1_A3-31 | -0.12 | 3.55 |
|  |  |  |  | 5.36 | 2 | G_FR1_A3-31 | -1.57 | 0.28 |
|  | 4 | TDLC300-100 |  | 9.83 | 16 | G_FR1_A3-31_1 | 8.24 | 13.16 |
|  |  |  |  | 5.36 | 16 | G_FR1_A3-31_1 | 7.18 | 9.92 |
|  | 8 | TDLA30-10 |  | 9.83 | 20 | G_FR1_A3-31_2 | 5.73 | 10.67 |
|  |  |  |  | 5.36 | 20 | G_FR1_A3-31_2 | 5.56 | 8.02 |

Table 6: Minimum requirements for PUSCH with 70% of maximum throughput, Type A, 1 Transmission Layer, 5 MHz channel bandwidth, 15 kHz SCS with Transform Precoding Enabled with 2 Interferer

| TX | RX | Propagation conditions and correlation matrix |  | (INR1, INR2) [dB] | MCS In Table 1 | FRC | SNR [dB] |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Tested signal | Interferer 1&2 |  |  |  | IRC | MRC |
| 1 | 2 | TDLB100-100 |  | (12.38, -0.97) | 2 | G_FR1_A3-31 | 2.44 | 6.17 |
|  |  |  |  | (7.29, -2.5) | 2 | G_FR1_A3-31 | 0.35 | 2.28 |
|  | 4 | TDLC300-100 |  | (12.38, -0.97) | 16 | G_FR1_A3-31_1 | 10.48 | 15.75 |
|  |  |  |  | (7.29, -2.5) | 16 | G_FR1_A3-31_1 | 8.87 | 11.87 |
|  | 8 | TDLA30-10 |  | (12.38, -0.97) | 20 | G_FR1_A3-31_2 | 6.50 | 13.12 |
|  |  |  |  | (7.29, -2.5) | 20 | G_FR1_A3-31_2 | 6.24 | 9.82 |

Table 7: Minimum requirements for PUSCH with 70% of maximum throughput, Type A, 1 Transmission Layer, 10 MHz channel bandwidth, 30 kHz SCS with Transform Precoding Enabled with 1 Interferer

| TX | RX | Propagation conditions and correlation matrix |  | INR1 [dB] | MCS In Table 1 | FRC | SNR [dB] |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Tested signal | Interferer 1 |  |  |  | IRC | MRC |
| 1 | 2 | TDLB100-100 |  | 9.83 | 2 | G_FR1_A3-32 | 0.30 | 3.81 |
|  |  |  |  | 5.36 | 2 | G_FR1_A3-32 | -1.35 | 0.53 |
|  | 4 | TDLC300-100 |  | 9.83 | 16 | G_FR1_A3-32_1 | 8.57 | 13.45 |
|  |  |  |  | 5.36 | 16 | G_FR1_A3-32_1 | 7.43 | 10.15 |
|  | 8 | TDLA30-10 |  | 9.83 | 20 | G_FR1_A3-32_2 | 5.95 | 10.90 |
|  |  |  |  | 5.36 | 20 | G_FR1_A3-32_2 | 5.71 | 8.25 |

Table 8: Minimum requirements for PUSCH with 70% of maximum throughput, Type A, 1 Transmission Layer, 10 MHz channel bandwidth, 30 kHz SCS with Transform Precoding Enabled with 2 Interferer (prio1)

| TX | RX | Propagation conditions and correlation matrix |  | (INR1, INR2) [dB] | MCS In Table 1 | FRC | SNR [dB] |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Tested signal | Interferer 1&2 |  |  |  | IRC | MRC |
| 1 | 2 | TDLB100-100 |  | (12.38, -0.97) | 2 | G_FR1_A3-32 | 2.91 | 6.40 |
|  |  |  |  | (7.29, -2.5) | 2 | G_FR1_A3-32 | 0.63 | 2.50 |
|  | 4 | TDLC300-100 |  | (12.38, -0.97) | 16 | G_FR1_A3-32_1 | 10.96 | 16.05 |
|  |  |  |  | (7.29, -2.5) | 16 | G_FR1_A3-32_1 | 9.10 | 12.13 |
|  | 8 | TDLA30-10 |  | (12.38, -0.97) | 20 | G_FR1_A3-32_2 | 6.90 | 13.47 |
|  |  |  |  | (7.29, -2.5) | 20 | G_FR1_A3-32_2 | 6.44 | 10.12 |

# Conclusions

The simulation results for NR MMSE-IRC PUSCH are delivered.

# References

[1] 	R4-2416548, ‘WF on NR demodulation performance: Phase 5’, China Telecomm

[2]