3GPP TSG-RAN WG4 Meeting #113	R4-2419268

Orlando, US, 18th – 22nd November, 2024

Source:	ZTE Corporation, Sanechips

Title:	Discussion on other RRM aspects of R19 NR MIMO Phase 5

Agenda Item:	7.19.3.2

Document for:	Discussion

- Introduction

The the latest RAN plenary 105 meeting, a revised WID on Rel-19 WID of NR MIMO Phase 5 was approved in [1]. We listed all the objectives with revision as below:

- Specify enhancement to facilitate UE-initiated/event-driven beam management for reducing overhead and/or latency, assuming the unified TCI while leveraging (as much as possible) legacy CSI measurement and reporting configuration frameworks, targeting FR2 and sTRP with intra- and inter-cell beam management - UL signaling content(s) (and procedure(s) as required) for UE-initiated/event-driven beam reporting facilitating fast beam switching - UL signaling medium/container considering the UE-initiated/event-driven nature of the UL transmission, designed primarily for the purpose of beam reporting - Specify CSI support for up to 128 CSI-RS ports, targeting FR1 - Type-I codebook refinement supporting up to a total of 128 CSI-RS ports across all resources, assuming legacy CSI-RS resources (with up to 32 CSI-RS ports per resource), based on extension of legacy codebooks - Type-II codebook refinement supporting up to a total of 128 CSI-RS ports across all resources, assuming legacy CSI-RS resources (with up to 32 CSI-RS ports per resource), based on extension of legacy codebooks, without modifying any codebook parameter other than introducing additional values for the number of ports codebook parameter(s) - Extension of CRI(s)-based CSI reporting (CQI/PMI/RI calculated per CRI for ≥1 CRIs) for hybrid beamforming supporting up to a total of 128 CSI-RS ports across all resources, with up to 32 CSI-RS ports per resource, without new codebook design - SRS port grouping and its association to the two codewords for the 6/8Rx low complexity receiver supporting more than 4 layers, with legacy codebook - No enhancement on codeword-to-layer mapping, DL resource allocation, CSI feedback, and DCI format - Note: Whether to support 6Rx with more than 4 layers is to be decided in RAN4 Rel-19 RF enhancements WI - Specify UE reporting enhancement for CJT deployments under non-ideal synchronization and backhaul, targeting FR1, both FDD and TDD - Inter-TRP time misalignment and frequency/phase offset measurement and reporting, assuming legacy CSI-RS design, with stand-alone aperiodic reporting on PUSCH - Specify non-coherent UL codebook to facilitate 3-antenna-port codebook-based transmissions, enhancement(s) to enable 3T6R SRS antenna switching, as well as UE capability signaling for 3T3R antenna switching and 3-antenna-port non-codebook-based transmissions, without enhancement on UL full power transmission and without enhancement on SRS resource Note: UL full power transmission mode 1 and 2 are not supported. Note: Other than UE capability signaling, no other enhancement is specified for 3T3R SRS antenna switching. - Specify enhancement for asymmetric DL sTRP/UL mTRP deployment scenarios, assuming intra-band intra-DU non-co-located mTRP scenarios, without changing existing cell definition or defining a new cell (e.g. UL-only cell), assuming the Rel-17/18 unified TCI framework and fully reusing the legacy QCL/UL spatial relation rules, targeting FR1 and FR2 - Two closed-loop PC adjustment states for SRS, both separate from PUSCH; and pathloss offset configurations for pathloss calculation to UL TRP(s), when the pathloss RS is from DL sTRP. - Two TAs through reusing Rel-18 specification of two TAs for multi-DCI-based multi-TRP and removing the restriction that coresetPoolIndex needs to be configured, assuming legacy PRACH resources

All these five objectives are covered by RAN1. From RAN4 RRM perspective, we believe partial of them are less related to RRM. In last RAN4 meeting, whether RRM impacts exist for each objective, RAN4 achieved some initial conclusion and was captured in [2]. In summary, the RRM impact was identified in the UE-initiated/event-driven beam management. The codebook/CSI refinement for the support of 128 CSI-RS ports were identified no RRM impact. The other objectives are still need further identification whether there is RRM impact. In this paper, regarding these uncertain objectives, we will provide our views.

Furthermore, the following agreements were achieved in last meeting and summarized in [2], we list them as below:

- <Topic 1: CSI enhancement to support up to 128 ports > - No agreement - <Topic 2: UE reporting enhancement for CJT calibration> Issue 2-1: Whether RRM core requirements impacts exist by UE reporting enhancement for CJT calibration? Agreement: - RAN4 to simulate the performance of offsets for phase, time delay and frequency, based on one-shot measurement and multiple-sample measurement, and discuss the impact on RRM core requirement later. - Further discuss the simulation assumptions offline during this meeting. - <Topic 3: 3TX support> Issue 3-1: RRM core/performance impacts by introducing 3TX support? Agreement: - No new RRM core requirements by introducing 3TX support. - <Topic 4: Enhancement for asymmetric DL sTRP/UL mTRP scenarios > Issue 4-1: Whether RRM impacts exist by 2TA enhancement? Agreement: - RAN4 to update the RRM requirements for 2TA enhancement [for asymmetric DL sTRP/ UL mTRP] - As baseline, the Rel-18 framework can be considered as starting point, and the update is needed to accommodate the new scenario in this WID. - Further discuss the details in the next meeting. Issue 4-3: Whether to define RRM core requirements of pathloss offset update? Agreement: - Previous RAN4 agreement captured below still apply and close the issue. Issue 2-5-5: Whether to define RRM core requirements of pathloss offset update? Agreement: Whether to define RRM core requirements of pathloss offset update? - Not define RRM requirement for MAC CE decoding delay. - The agreement can be revisited depending on further RAN1 progress..

In this paper, we provide our detailed analysis mainly around the following objectives:

- CSI enhancement to support up to 128 ports

- Aperiodic standalone CJT calibration reporting

- The enhancement for asymmetric DL sTRP/UL mTRP deployment scenarios

- Discussion

- 2.1 CSI enhancement to support up to 128 ports

Another sub-bullet was approved in the updated WID, this new objective is SRS group relevant. It impacts both the association between the SRS port and codewords for more than 4 layers, furthermore, it also impacts the SRS transmission, i.e. 2 SRS port groups are allowed. In RAN 105 meeting, a new sub-objective was approved to support SRS port grouping. To facilitate group based CSI acquisition with the inter-group interference consideration, the Rx was divided into two groups. For UL, the SRS ports are also divided into two groups. From RRM perspective, we focus on the interruption requirements of SRS antenna switching. In legacy SRS antenna switching, the SRS antenna switching happens in aggressor cell, and which leads to interruption in victim cell. With such additional SRS port grouping, it seems no impact on the existing interruption requirements since the port grouping is per cell.

Proposal 1: No RRM impact by the introduction of SRS port grouping.

- 2.2 Aperiodic standalone CJT calibration reporting

To overcome the non-ideal backhaul and non-ideal sychronization in real NW deployment for CJT, some new reporting quantities were identified, including the frequency offset reporting, delay offset reporting, DL/UL phase offset reporting. The reporting of these information by UE could facilitate the compensation on delay offset, frequency offset and DL/UL Rx-Tx phase so as to enhance the reciprocity. Based on RAN1 progress, all of these reporting is aperiodically requested by the NW, and the calibration measurements are based on periodic TRS.

So based on the methods of measurements and reporting, it seems that just following the existing CSI measurement and reporting framework is fine. But during last RAN4 meeting, companies proposed to identify either one-shot or multi-shot measurement to be applied, this would impact the measurement period.

Proposal 2: For aperiodic standalone CJT calibration reporting, determine the measurement period and the accuracy based on evaluation results.

- 2.3 The enhancement for asymmetric DL sTRP/UL mTRP deployment scenarios

To improve UL throughput and power consumption saving, heterogeneous NW can be deployed with macro nodes and micro nodes in non-co-located deployment. Wherein the micro nodes only apply to provide UL load balance. Not any DL transmission happens in the micro link, so that the micro node can turn off the DL Tx to save power consumption. The following picture depicts the scenario as below:

Figure 1. Asymmetric DL sTRP/UL mTRP scenario

As the update in recent WID revision, 2TA through reusing the 2TA for mDCI based mTRP was approved for this scenario. Furthermore, it should be noted that the restriction that coresetPoolIndex needs to be configured was removed. Which means either one CORESET pool or two CORESET pools are both allowed for this scenario.

- Two TAs through reusing Rel-18 specification of two TAs for multi-DCI-based multi-TRP and removing the restriction that coresetPoolIndex needs to be configured, assuming legacy PRACH resources

Besides the confirmation of 2TA, it is also mentioned that the Rel-17/18 unified TCI framework should be assumed for this scenario. RAN1 has also approved the following conclusion:

RAN1#117: Conclusion For the asymmetric DL sTRP/UL mTRP deployment scenario, reuse the rel-17 unified TCI/ICBM and rel-18 unified TCI framework: - When rel-17 unified TCI/ICBM is configured: - For FR1: one joint TCI state or {one DL TCI state + one UL TCI state} can be applied. - For FR2: {one DL TCI state + one UL TCI state} can be applied. - When rel-18 unified TCI is configured: - For FR1: up to two joint TCI states or {one DL TCI state + up to two UL TCI state} can be applied. - For FR2: {one DL TCI state + up to two UL TCI states} can be applied.

So it can be determined that both R17 unified TCI and R18 unified TCI framework are assumed for the asymmetric scenario, and to support 2TA, both joint TCI state and separate TCI state indication could be considered.

Observation 1: Both R17 unified TCI and R18 unified TCI framework are assumed for the asymmetric scenario. Both joint and separate TCI state indication could be considered.

In R17 unified TCI framework, since target to sTRP case, so only one CORESET pool is configured, for each joint TCI state or UL TCI state, a single UL TCI state is involved in.

In R18 MIMO, sDCI based and mDCI based mTRP scenario have been supported. Especially for mDCI, 2TA based UL transmission is introduced for the capable UE. While for sDCI case, 2TA based UL transmission is not supported due to a lack of time and scope management. To support 2TA mechanism, the association of TAG id with CORESET Pool id would be configured to UE.

In last RAN4 meeting, supporting 2TA for this scenario has been confirmed, so 2TA relevant RRM impact should be discussed.

As a result, combined the 2TA demand with the unified TCI state framework, it seems the following cases are all in the scope:

- Case 1: 2TA +R17 unified TCI state framework

- Case 3: 2TA +R18 unified TCI state framework + sDCI

- Case 2: 2TA +R18 unified TCI state framework + mDCI

However, in RAN1 118bis meeting, to support 2TA and remove the restriction on coresetPoolIndex configuration, some further progress was achieved as below:

It can be observed that a single DL reference timing is applied for both TAGs, and the mDCI case is excluded. So the above Case 3 would not happen.

Observation 2: Based on RAN1 progress, a single DL reference timing for both TAGs is confirmed, and mDCI case is excluded. So the two cases should be considered:

- Case 1: 2TA +R17 unified TCI state framework

- Case 2: 2TA +R18 unified TCI state framework + sDCI

For Case 1, only one CORESET pool is configured under R17 unified TCI framework. To support multiple UL links, some joint/UL TCI states associates to TRP1, and others associates to TRP2. Accordingly, TAG1 associates to the joint/UL TCI states associated to TRP1, and TAG2 associates to the joint/UL TCI states associated to TRP2. For the both two types of joint/UL TCI states, a common DL timing is assumed. Based on R17 unified TCI state framework, the source RS of UL TCI state could be SSB, CSI-RS or SRS. So it seems no RRM impact to reuse existing R17 UL TCI state. But for Joint TCI state, the source RS could only be SSB or CSI-RS, not SRS, while in this asymmetric scenario, to our understanding, not any DL signal including DL RS could be transmitted by the UL only TRP, if this can be confirmed, then RAN4 needs to discuss whether and how to apply R17 joint TCI state in the asymmetric scenario. In last RAN4 meeting, RF session sent 1 LS[3] to RAN1 to request the clarification on whether SSB could be transmitted through the UL-only TRP, we can wait for RAN1 reply.

Actually Case 2 is very similar as Case 1, only differenciate at the number of UL TCI state.

Observation 3: UL TCI state in R17/18 unified TCI state could associate to SSB, CSI-RS and SRS. Joint TCI state in R17/18 unified TCI state could not associate to SRS. No existing requriements defined for SRS associated case.

Proposal 3: When applying R17/18 unified TCI state framework to support 2TA in asymmetric scenario, wait for RAN1 reply on whether SSB is allowed to transmit from UL-only TRP.

- If yes, both existing R17/18 joint and UL TCI state could be used.

- Otherwise, need to discuss how to revise the R17/18 joint TCI state to facilitate the asymmetric scenario.

How to obtain and maintain two TAs referring to a single DL timing, this issue is still discussed in RAN1 and without progress until RAN1#117 meeting. To our understanding, since the propagation delay between multiple links are different, to derive the TA for each, UE needs to send UL signals to each TRP.

In R18 MIMO, how to identify a common DL timing was discussed, the following progress were achieved:

Agreement: For multi-DCI based multi-TRP operation with two TAs, for each TAG, the uplink transmission timing takes place before the reception of the first detected path (in time) of the corresponding downlink frame of the reference signal associated with UL/joint TCI state.

To save the effort, here it is better to leverage the R18 conclusion as much as possible, i.e. the single DL timing is determined as the reception of the first detected path (in time) of the corresponding downlink frame of the reference signal associated with UL/joint TCI state of the DL&UL link.

Proposal 4: Regarding how to identify the common DL timing for multiple links, leverage the DL timing derivation identified in R18 MIMO, i.e. the single DL timing is determined as the reception of the first detected path (in time) of the corresponding downlink frame of the reference signal associated with UL/joint TCI state of the DL&UL link.

After the two TA maintaining solution determined, RAN4 can further discuss the requirements of UL transmission for UL-only link, including the Te, Tp/Tq. RAN1&2 are still discussing how to derive TA for the UL only link, so we can wait for further progress from RAN1/2.

- Conclusion

In this contribution, we have the following observations and proposals:

Proposal 1: No RRM impact by the introduction of SRS port grouping.

Proposal 2: For aperiodic standalone CJT calibration reporting, determine the measurement period and the accuracy based on evaluation results.

Observation 1: Both R17 unified TCI and R18 unified TCI framework are assumed for the asymmetric scenario. Both joint and separate TCI state indication could be considered.

Observation 2: Based on RAN1 progress, a single DL reference timing for both TAGs is confirmed, and mDCI case is excluded. So the two cases should be considered:

- Case 1: 2TA +R17 unified TCI state framework

- Case 2: 2TA +R18 unified TCI state framework + sDCI

Observation 3: UL TCI state in R17/18 unified TCI state could associate to SSB, CSI-RS and SRS. Joint TCI state in R17/18 unified TCI state could not associate to SRS. No existing requriements defined for SRS associated case.

Proposal 3: When applying R17/18 unified TCI state framework to support 2TA in asymmetric scenario, wait for RAN1 reply on whether SSB is allowed to transmit from UL-only TRP.

- If yes, both existing R17/18 joint and UL TCI state could be used.

- Otherwise, need to discuss how to revise the R17/18 joint TCI state to facilitate the asymmetric scenario.

Proposal 4: Regarding how to identify the common DL timing for multiple links, leverage the DL timing derivation identified in R18 MIMO, i.e. the single DL timing is determined as the reception of the first detected path (in time) of the corresponding downlink frame of the reference signal associated with UL/joint TCI state of the DL&UL link.

- Reference

- RP-242394, WID revision: NR MIMO Phase 5, RAN#105

- R4-2416921, WF on RRM requirements for NR_MIMO_Ph5(part 2), RAN4#112bis

- R4-2417117, LS on UE RF issues for Rel-19 MIMO enhancement, RAN4#112bis