3GPP TSG RAN WG4 Meeting #113	R4-2419285

Orlando, USA, 18 Nov. 2024 – 22 Nov. 2024

Agenda Item:	5.27.1

Source: 	Ericsson

Title:	Discussions on the remaining open issues for Rel-18 NES

Document for:	Discussion

# Introduction

In last meeting, RAN4 achieved agreements on most open issues. In this contribution, we continue to discuss the remaining issue on NES.

- SSB less SCell activation for inter-band collocated CA

In last meeting following agreement is reached for the power difference side condition to support SSB less SCell on inter-band CA.

Agreement: - For EPRE difference within X dB, keep the existing requirement. - X equals to or larger than 12 dB - For EPRE difference beyond X dB and no larger than Y dB, add one more P-TRS occurrence. - Further discuss whether the upper bound of Y is needed. If needed, Y is larger than 25dB. - From RAN4 perspective, AGC compensation for carrier frequency difference can be optionally implemented by UE. Remove “[after pre-compensation]” from the spec, i.e., not specify “AGC compensation” for carrier frequency difference in the spec. - Further discuss the X and Y values in this meeting. Try to conclude in the 2nd round on Thursday afternoon. Additional Agreement: X = 12; Y = [30].

In last meeting, the wording precompensation was agreed to be removed from the spec. Following two scenarios can be considered for deriving the EPRE difference.

- Scenario 1: When the CC are close by

- Scenario 2: When the CC are far apart

In last meeting for scenario 1 (i.e., when the reference cell and SSB less SCell are very close in frequency and using same BW), companies mentioned that there is no precompensation UE can perform. For scenario 1, it was agreed that 12 dB is absolute maximum EPRE UE can support. That means the 12 dB is the maximum power difference UE can support without any propagation delay difference arising from the frequency separation. This can be taken as baseline for the scenario 2.

NR CA band combination supports wide range of frequency separation such as n71, n77 (630 MHz, 3.75Ghz), which corresponds to free space propagation delay difference of around 16 dB. Since the 16 dB is free space propagation delay difference, we might need to add additional margin on top of free space propagation delay difference to consider the realistic propagation conditions. Towards that end we can consider additional margin as 6 dB. Combinaing 12 dB (which UE can support when the CC are close by), additional margin and the 16dB power difference due to propagation delay difference due to frequency separation, we think 34 dB can be maximum EPRE for the scenario 2. We further think that 34 dB seems reasonable as we are allowing one additional P-TRS sample for the AGC.

- When EPRE difference is beyond 12 dB and below 34 dB, UE is allowed one additional P-TRS for the AGC settling.

Other issue discussed in last meeting without agreement was how does the measurements on SSB less carrier was categorized. There were two set of views presented during the discussion, one view is, since there is no SSB on the serving cell, as per the existing definition, neighbour cells on SSB less SCell carrier are measured as inter-frequency measurements. Other view is that, though SSB is not present on SSB less SCell, as per RAN2, there is no restriction on configuring or providing SSB center frequency. If the SSB less SCell MO is provided, then the measurements can be treated as per existing defintion. Otherwise, all the measurements are inter-frequency measurements. In last meeting, RAN2 discussed this without agreement. RAN2 agreements are copied below for reference.

Agreements 1 From RAN2 perspective, no benefit is identified for configuring servingCellMO for inter-band SSB-less SCell. 2 No LS is sent to R4. 3 Discuss in the next meeting on whether to capture “For inter-band SSB-less SCell(s), this field is not present” in the conditional presence of servingCellMO, depending on R4 progress. 4 Postpone the discussion on intra-band SSB-less SCell.

If RAN2 do not introdcue the servingCellMO for inter-band SSB-less SCell, all the measurements will be considered as an inter-freqeuncy measurements as per the existing definition. We think the main disadvantages of performing the neighbour cell measurements on SSB less SCell carrier as inter-frequnecny measurements is measurement delay is longer for inter-frequency measurements and a smaller number of cells can be measured on the inter-frequency. Since RAN2 could not achieve consensus, we think it may be helpful if RAN4 send a LS to RAN2 about the disadvantage of not configuring the servingCellMO for inter-band SSB-less SCell. Decision on whether to configure the MO or not can be upto RAN2 based on the RAN4 feedback.

- RAN4 to send LS to RAN2 informing about disadvanges of not configuring the servingCellMO for inter-band SSB-less SCell. In the LS, the disadvantages to be listed as following.

  - measurement delay is longer for inter-frequency measurement; and

  - number of cells UE can measure on inter-frequncy layer are lower than intra-freq layer.

- Summary

In this contribution, we have discussed the MUSIM gaps requirements. Based on the discussions, we have made following proposals and observations:

- When EPRE difference is beyond 12 dB and below 34 dB, UE is allowed one additional P-TRS for the AGC settling.

- RAN4 to send LS to RAN2 informing about disadvanges of not configuring the servingCellMO for inter-band SSB-less SCell. In the LS, the disadvantages to be listed as following.

  - measurement delay is longer for inter-frequency measurement; and

  - number of cells UE can measure on inter-frequncy layer are lower than intra-freq layer.

- References

[1]. R4-2410269, “WF on RRM requirements for NR network energy saving”, Huawei, HiSilicon