3GPP TSG-RAN WG4 Meeting #113 	R4-2419287

Orlando, USA, 18 Nov. 2024– 22 Nov. 2024

Agenda Item:	5.10.1

Source: 	Ericsson

Title:	Discussion on test case scenario for mDCI based TCI state switch

Document for:	Discussion

# Introduction

In this contribution we provide our views on the test cases for FR2 multi-RX WI.

# Discussion

In previous meetings, the test procedure for dual TCI state switching for m-DCI was discussed and the test case was agreed. However, few details in test procedure needs to be further discussed. Following WF was agreed in RAN4#112 meeting.

Issue 2-3: Test setup for dual TCI state switching for m-DCI Use the following proposals as starting point, and further check when drafting the CR: - P1: When three probes are used to transmit four RS, the test probe transmitting more than one RS should emulate different DL transmit beams by transmitting different RS [with different power and delay]. This will ensure that the UE has done a time/frequency synchronization with the target RS before switching to the target TCI state when receiving a DCI command for a TCI state switch - P2: In m-DCI TCI state switching test case, because support of simultaneous PDCCH reception is not mandatory for a Rel-18 UE, the TE shall send the two DCIs to switch to the target TCI states in consecutive slots n and n+1. The UE shall be able to receive the target TCI states simultaneously at slot n + 1 + timeDurationForQCL. - P3: Distinguish PDCCH and PDSCH TCI states clearly in the m-DCI TCI state switching test case. The TCI states and associated probes and SSBs should be - TCI states before TCI state swtiching: - For CORESETPoolIndex 0 - PDCCH TCI state: TCI state 0 (probe 0, SSB0) - [FFS PDSCH TCI states: TCI state 0 (probe 0, SSB0)] - For CORESETPoolIndex 1 - PDCCH TCI state: TCI state 1 (probe 1, SSB1) - [FFS PDSCH TCI states: TCI state 1 (probe 1, SSB1)] - TCI states after TCI state swtiching: - For CORESETPoolIndex 0 - PDCCH TCI state: TCI state 0 (probe 0, SSB0) - PDSCH TCI states: TCI state 3 (probe 0, SSB3) - For CORESETPoolIndex 1 - PDCCH TCI state: TCI state 1 (probe 1, SSB1) - PDSCH TCI states: TCI state 2 (probe 2, SSB2)

When the testability aspects were discussed for 4 probes vs 3 probes, RAN4 agreed to go with 3 probes to test the mDCI based dual TCI state switching. Regarding P1, since SSB index are different, and different SSB index are transmitted at different time intervals in time domain. Hence, the delay aspect would come naturally without introducing new delay in the test. Further, as long as the SSB RP is above -3dB, UE should be able to measure the SSB for T/F synchronization. Even if the test setup changes the power, that is not useful for this purpose of the test as SSB is already detectable. Hence, we do not see why we need to use different power setting. Hence, we think P1 is not needed to be captured explicitly in the test procedure.

Regarding P2, we think P2 is already captured in the spec. We produce the part of the spec, where it is captured.

At the beginning of T2, at slot n, UE receives DCI based PDSCH TCI state switch on CORESETPoolIndex 0 for indicating the TCI state switch of PDSCH to TCI state 3. At slot n+1, UE receives DCI based PDSCH TCI state switch on CORESETPoolIndex 1 for indicating the TCI state switch of PDSCH to TCI state 4. During T2, the time multiplexed (allocation in Frequency is symbolic) downlink transmissions from each Angle of Arrival is shown in Figure A.7.5.8.4.1-2.

Regarding P3: We think PDSCH TCI state can be explicitly stated in the test. We would like to point that UE behaviour should reflect the real-world configuration as test is supposed to be testing the UE behaviour in real NW. It would be unrealistic to not have any PDSCH transmissions before the TCI state switch command was sent. Hence, we suggest removing the FFS from P3.

- Distinguish PDCCH and PDSCH TCI states clearly in the m-DCI TCI state switching test case. The TCI states and associated probes and SSBs should be

- TCI states before TCI state switching:

  - For CORESETPoolIndex 0

    - PDCCH TCI state: TCI state 0 (probe 0, SSB0)

    - PDSCH TCI states: TCI state 0 (probe 0, SSB0)

  - For CORESETPoolIndex 1

    - PDCCH TCI state: TCI state 1 (probe 1, SSB1)

    - PDSCH TCI states: TCI state 1 (probe 1, SSB1)

- TCI states after TCI state switching:

  - For CORESETPoolIndex 0

    - PDCCH TCI state: TCI state 0 (probe 0, SSB0)

    - PDSCH TCI states: TCI state 3 (probe 0, SSB3)

  - For CORESETPoolIndex 1

    - PDCCH TCI state: TCI state 1 (probe 1, SSB1)

    - PDSCH TCI states: TCI state 2 (probe 2, SSB2)

# Summary

The following have been proposed in this contribution.

- Distinguish PDCCH and PDSCH TCI states clearly in the m-DCI TCI state switching test case. The TCI states and associated probes and SSBs should be

- TCI states before TCI state switching:

  - For CORESETPoolIndex 0

    - PDCCH TCI state: TCI state 0 (probe 0, SSB0)

    - PDSCH TCI states: TCI state 0 (probe 0, SSB0)

  - For CORESETPoolIndex 1

    - PDCCH TCI state: TCI state 1 (probe 1, SSB1)

    - PDSCH TCI states: TCI state 1 (probe 1, SSB1)

- TCI states after TCI state switching:

  - For CORESETPoolIndex 0

    - PDCCH TCI state: TCI state 0 (probe 0, SSB0)

    - PDSCH TCI states: TCI state 3 (probe 0, SSB3)

  - For CORESETPoolIndex 1

    - PDCCH TCI state: TCI state 1 (probe 1, SSB1)

    - PDSCH TCI states: TCI state 2 (probe 2, SSB2)

# References

[1] RP-221753, WI description, Requirement for NR frequency range 2 (FR2) multi-Rx chain DL reception, Qualcomm Inc., June 2022.

[2] R4-2414044