3GPP TSG RAN WG4 Meeting #113	R4-2419296

Orlando, USA, 18 Nov. 2024 – 22 Nov. 2024

Agenda Item:	7.19.3.2

Source: 	Ericsson

Title:	Discussion on other RRM requirements for MIMO

Document for:	Discussion

# Introduction

In RP-234007, WID for MIMO phase 5 is agreed. In RAN4#112 meeting, RRM requirements impact was analyzed and the following WF [1] is agreed.

| Topic | Description | RRM impact |
| --- | --- | --- |
| UE-initiated/event-driven beam management | UE-initiated/event-driven beam management | YES |
| CSI enhancement | Type-I codebook refinement supporting up to a total of 128 CSI-RS ports | NO |
|  | Type-II codebook refinement supporting up to a total of 128 CSI-RS ports | NO |
|  | CRI-based CSI refinement for up to 128 CSI-RS ports | NO |
|  | Aperiodic standalone CJT calibration reporting | FFS |
| 3TX | 3-antenna-port codebook-based UL transmission | FFS |
| Asymmetric DL sTRP/UL mTRP |  | FFS |

In this contribution we analyze the potential RAN4 RRM impacts for the FFS parts of the above WF.

# Discussion

CJT enhancements

In last meeting, RAN4 agreed to specify the core requirements based on the outcome of the simulation study and made following agreement.

•	RAN4 to simulate the performance of offsets for phase, time delay and frequency, based on one-shot measurement and multiple-sample measurement, and discuss the impact on RRM core requirement later.

•	Further discuss the simulation assumptions offline during this meeting.

Way forward:

•	Companies are encouraged to bring simulation assumption/results in the next meeting. Simulation assumption in Appendix A can be used as a starting point for information only.

We suggest RAN4 agreeing on the following simulation assumptions.

| Parameter | Value |
| --- | --- |
| Duplex, Waveform | TDD, OFDM |
| Carrier Frequency | 3.5 GHz |
| Channel Model | TDL-C |
| Delay Spread | 300ns |
| antenna configuration | 1X2 |
| TRP number | 2 |
| TRS Bandwidth | 48RB, 106 RB |
| Numerology | 14 OFDM symbol per slot, 30kHz SCS |
| UE speed | 3km/h |
| SNR | -3dB, 0dB, 3dB, 6dB, 12 dB |
| Frequency offset between TRP (only for freq. offset simulations) | 100ppb, 50ppb, 20ppb |
| Delay difference between TRP (only for delay offset simulations) | 0.5CP, CP |
| Number of samples for measurement | 1, 2, 3, 4, 5, 10 |
| CSI-RS periodicity (TRS) | 20ms / 40 slots |
| CSI-RS offset | Slot 20, 21 |

Since it will be more convenient to refer to simulation assumptions agreed as a separate document, we provide the simulation assumption in R4-2419297.

- RAN4 to agree on simulation assumptions presented in R4-2419297

In last meeting, we made following proposals for sending LS to RAN1 on AFO and MPO. However, since there is no RAN4 study backing up the proposals, companies could not achieve consensus.

Proposal: Propose to RAN1 to add AFO = {20 ppb, 50 ppb} to the set of configurable ranges to frequency offset report. Please find an LS draft in appendix. Proposal: The required UE Resolution, i.e., more than the current 16 or 32 levels, MPO, for DL/UL phase offset report, from a RAN4 point of view, has to be feed back to RAN1. Please find an LS draft in appendix.

However, we cannot rule out the proposals without further study as it can be critical for the feature performance. Since RAN4 agreed to perform the simulations to check the necessity of the core requirements, we suggest companies to study the necessity of these proposals using the simulations. If RAN4 finds the proposals are necessary for the feature to work, RAN4 can revisit the discussion on sending LS to RAN1.

- AFO level of 50ppb and 20 ppb to be part of the simulation study. MPO levels to be studied based on the phase accuracy and phase granularity estimation from the simulations.

We also present initial set of simulation results based on the above simulation assumptions and make few observations.

Simulation results for frequency offset:

| Number of TRS resource sets in a measurement | A_FO=100 ppb, 500 UEs; Mean error (absolute) in ppb & estimated over 500 UEs |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | SNR = -3 dB |  |  |  |  | SNR= +3dB |  |  |  |  |
|  | Mean error (ME) | ME in # levels (32) |  | ME in # levels (128) | ME in # levels (256) | Mean error (ME) | ME in # levels (32) | ME in # levels (128) | ME in # levels (256) |
| 1 TRS | 8.110 |  | 3 | 11 | 21 | 3.588 | 2 | 5 | 10 |
| 2 TRS | 5.453 |  | 2 | 7 | 14 | 2.527 | 1 | 4 | 7 |
| 3 TRS | 4.651 |  | 2 | 6 | 12 | 2.151 | 1 | 3 | 6 |
| 5 TRS | 3.744 |  | 2 | 5 | 10 | 1.713 | 1 | 3 | 5 |
| 10 TRS | 2.642 |  | 1 | 4 | 7 | 1.264 | 1 | 2 | 4 |

- For frequency offset, at low SNR of -3dB, frequency offset averaged over multiple samples gives same level of accuracy as for a single sample at SNR of +3dB.

| Number of TRS resource sets in a measurement | A_FO=50 ppb, 500 UEs; Mean error (absolute) in ppb & estimated over 500 UEs |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | SNR = -3 dB |  |  |  | SNR= +3dB |  |  |  |  |
|  | Mean error (ME) | ME in # levels (32) | ME in # levels (128) | ME in # levels (256) | Mean error (ME) | ME in # levels (32) | ME in # levels (128) | ME in # levels (256) |
| 1 TRS | 8.032 | 6 | 21 | 42 | 3.598 | 3 | 10 | 19 |
| 2 TRS | 5.317 | 4 | 14 | 28 | 2.449 | 2 | 7 | 13 |
| 3 TRS | 4.490 | 3 | 12 | 23 | 2.093 | 2 | 6 | 11 |
| 5 TRS | 3.690 | 3 | 10 | 19 | 1.677 | 2 | 5 | 9 |
| 10 TRS | 2.556 | 2 | 7 | 14 | 1.199 | 1 | 4 | 7 |

| Number of TRS resource sets in a measurement | A_FO=20 ppb, 500 UEs; Mean error (absolute) in ppb & estimated over 500 UEs |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | SNR = -3 dB |  |  |  | SNR= +3dB |  |  |  |  |
|  | Mean error (ME) | ME in # levels (32) | ME in # levels (128) | ME in # levels (256) | Mean error (ME) | ME in # levels (32) | ME in # levels (128) | ME in # levels (256) |
| 1 TRS | 8.020 | 13 | 52 | 103 | 3.599 | 6 | 24 | 47 |
| 2 TRS | 5.332 | 9 | 35 | 69 | 2.442 | 4 | 16 | 32 |
| 3 TRS | 4.511 | 8 | 29 | 58 | 2.092 | 4 | 14 | 27 |
| 5 TRS | 3.542 | 6 | 23 | 46 | 1.646 | 3 | 11 | 22 |
| 10 TRS | 2.614 | 5 | 17 | 34 | 1.237 | 2 | 8 | 16 |

- Absolute mean error UE can estimate for 20, 50 ppb and 100 ppb do not change, which means UE can estimate the FO with same level of accuracy for 20 ppb, 50 ppb and 100 ppb. In other words, absolute error UE can estimate is independent to 20 ppb or 50 ppb or 100 ppb.

| Number of TRS resource sets in a measurement | A_DO=0.5CP, 500 UEs; Mean error (absolute) in CP & estimated over 500 UEs |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | SNR = -3 dB |  |  |  | SNR= +3dB |  |  |  |  |
|  | Mean error (ME) | ME in # levels (32) | ME in # levels (128) | ME in # levels (256) | Mean error (ME) | ME in # levels (32) | ME in # levels (128) | ME in # levels (256) |
| 1 TRS | 0.016 | 2 | 5 | 9 | 0.015 | 1 | 4 | 8 |
| 2 TRS | 0.014 | 1 | 4 | 8 | 0.014 | 1 | 4 | 8 |
| 3 TRS | 0.013 | 1 | 4 | 7 | 0.013 | 1 | 4 | 7 |
| 5 TRS | 0.011 | 1 | 3 | 6 | 0.011 | 1 | 3 | 6 |
| 10 TRS | 0.008 | 1 | 3 | 5 | 0.008 | 1 | 3 | 5 |

- For delay offset, 2 TRS samples gives good accuracy level for SNR at -3dB.

3TX UL

Specify non-coherent UL codebook to facilitate 3-antenna-port codebook-based transmissions, without enhancement on UL full power transmission and without enhancement on SRS resource

Note: UL full power transmission mode 1 and 2 are not supported.

As per our understanding objective 4 may have some RF impact and Demod impact but we do not foresee any RRM requirements impact.

- specification of 3-TX UL may not have RRM requirements impact.

Asymmetric DL sTRP/UL mTRP

2TA for the asymmetric DL sTRP/UL mTRP

RAN1 agreements from last meeting are copied below.

RAN1 Agreement

Support 2TA for the asymmetric DL sTRP/UL mTRP deployment scenarios:

- Remove the restriction that coresetPoolIndex needs to be configured for the 2TA feature.

- One downlink reference timing is supported and applied to both TAGs.

  - (FFS) Note: UE autonomous TA adjustment is only applicable to the first TAG

- One single n-TimingAdvanceoffset is configured and applied to both TAGs.

- Any of the TCI states can be associated with any one of the two TAGs.

- The RAR carrying TA adjustment for those 2 TAGs is reused for Rel-19 2TA

- The MAC CE based TA adjustment for 2 TAGs is reused for Rel-19 2TA.

- Introduce the optional UE capability of “Overlapping UL transmission reduction” for Rel-19 2TA

  - If UE does not report this UE capability, UE does not expect two UL transmissions associated with different TAGs are overlapped.

- FFS: UE does not expect that in intra-slot TDM PUSCH type-B repetition transmission, two consecutive repetitions associated with different TAGs are overlapped.

In last meeting, Rel-18 2TA requirements were agreed to be considered as a starting point for defining the requirements for 2TA for asymmetric DL sTRP/UL mTRP. We copy some of the requirements introduced in Rel-18 in the table below. On a high level, RAN4 defined following requirements for 2TA in Rel-18.

- DL reference point for PUCCH/PUSCH/SRS/PRACH

- UL transmit timing requirements

- Gradual timing adjustment requirements

In short, the requirements defined in Rel-18 consider mDCI and SSB being transmitted from multiple TRP. Since it is mDCI, either of the TRP can schedule the RACH and since SSB is not association to a TRP was not defined, we had to define somewhat complex requirements. However, in the asymmetric DL sTRP/UL mTRP scenario, though two TA are supported, based on the RAN1 agreement, only one downlink reference timing is supported and same is applied to both the TAGs. The same reference timing is expected to be used for all the UL channels. Since there is only one TRP transmitting SSB, DL reference timing can be defined as the first detected path (in time) of the downlink frame from the reference TRP.

For multi-DCI based multi-TRP operation with two TAs, for each TAG, the uplink transmission timing takes place (NTA+NTA offset)×Tc before the reception of the downlink reference point. . For multi-DCI based multi-TRP operation with two TAs, the reference point for PUCCH/PUSCH/SRS, is the first detected path (in time) of one of the corresponding downlink reference signal(s) of the reference cell associated with one of the DLorJointTCIState [TS 38.331] having the same TAGas the uplink signal. For intra-cell multi-DCI based multi-TRP operation with two TAs, the reference point for PRACH transmission triggered by PDCCH order, is the first detected path (in time) of one of the downlink reference signal(s) in the DL or joint TCI state of the reference cell associated with the same coresetPoolIndex as PDCCH carrying PDCCH order if PRACH association indicator is 0, and that of the other coresetPoolIndex if PRACH association indicator is 1. For inter-cell multi-DCI based multi-TRP operation with two TAs, the reference point for PRACH transmission triggered by PDCCH order, - if “PRACH association indicator” is 0, the reference timing is the first detected path (in time) of one of the corresponding downlink reference signal(s) of DL TCI state(s) of the reference cell associated with the first TAG. - if “PRACH association indicator” is 1, the reference timing is the first detected path (in time) of one of the corresponding downlink reference signal(s) of DL TCI state(s) of the reference cell associated with the second TAG. Where NTA and NTA offset are commanded by the network independently for each TAG [TS 38.331]. NTA offset for first TAG and second TAG are as defined in n-TimingAdvanceOffset and n-TimingAdvanceOffset2 respectively, if configured, otherwise it is NTA offset defined in Table 7.1.2-2. In the requirements of clause 7.1.2, the SSB applies for both CD-SSB and NCD-SSB if the UE supports ncd-SSB-BWP-Wor-r18.

- For asymmetric DL sTRP/UL mTRP with two TAs, DL reference timing can be defined as the first detected path (in time) of the downlink frame from the reference TRP.

UE UL transmit timing requirements:

Transmit timing requirements can follow the legacy principle. That means, for each TAG, the uplink transmission timing takes place (N_"TA" +N_"TA offset" )×T_c before the reception of the downlink reference point. N_TA offset can be used as per the RAN1 definition.

- For each TAG, the uplink transmission timing takes place (N_"TA" +N_"TA offset" )×T_c before the reception of the downlink reference point.

Gradual timing adjustment requirements:

We think Rel-18 principles can be reused for the requirements definition. We copy the Rel-18 requirement below for reference.

For multi-DCI based multi-TRP operation with two TAs, gradual timing adjustment specified in this clause applies for each TAG and shall be met for each TAG. The reference point for each TAG for the PUCCH/PUSCH/SRS and PRACH transmission triggered by PDCCH order is defined in clause 7.1.1.

- For asymmetric DL sTRP/UL mTRP with two TAs, gradual timing adjustment applies for each TAG and shall be met for each TAG.

- For each TAG/TRP, when the transmission timing error between the UE and the reference timing exceeds ±Te then the UE shall adjust the timing such that timing error is within ±Te. The reference timing shall be  before the downlink timing of the reference TRP.

Active uplink TCI state switching delay for UL only TRP

Following WF is agreed in last meeting.

Way forward:

- Option 1: No RRM impacts on UL TCI state switching delay requirement for unified TCI states.

- Option 2: RAN4 to specify UL TCI state switching requirements.

- Option 3: FFS.

UL TCI state switch involves following steps:

- Pathloss estimation

- Timing acquisition

- QCL acquisition

Pathloss estimation: As per our understanding, RAN1 is discussing how to configure the PL-RS for the UL only TRP. One of the proposals under discussion is anchor TRP DL-RS can be used for pathloss computation along with PL-offset indicated in the TCI state configuration.

Timing acquisition: As per our understanding, RAN1 is discussing the Dl reference timing aspect. One of the proposals was to use anchor TRP DL reference timing with some DL offset.

QCL acquisition: We think SRS can be configured as a QCL source for deriving spatial info relation. With SRS being configured as QCL RS, next question that arises is how the UE acquires the spatial info for the SRS. As per RAN1 specification, we think the spatial relation info for the SRS can be obtained using the SRS resource set configured with beamManagement. When NW configures SRS resource set with usage set to beamManagement, UE transmits the SRS resources in the SRS resource set in different spatial direction. In the UL TCI state, NW can indicate SRS resource ID as the QCL source and UE uses the spatial relation acquired from SRS beam management for the UL TCI state spatial relation information.

Since the three components are needed to define UL TCI state switching requirements, first two components may need further RAN1 progress. We understand that SRS based beam management requirements are not defined in RAN4 at present. Hence, we suggest RAN4 to study and specify SRS based beam management requirements.

- RAN4 to specify SRS based beam management requirements.

- RAN4 to specify UL TCI state switching requirements for UL only TRP.

# Summary and Conclusion

In this contribution we analyzed potential RAN4 RRM requirements impact and made following observations and proposals.

CJTC reporting

- For frequency offset, at low SNR of -3dB, frequency offset averaged over multiple samples gives same level of accuracy as for a single sample at SNR of +3dB.

- Absolute mean error UE can estimate for 20, 50 ppb and 100 ppb do not change, which means UE can estimate the FO with same level of accuracy for 20 ppb, 50 ppb and 100 ppb. In other words, absolute error UE can estimate is independent to 20 ppb or 50 ppb or 100 ppb.

- For delay offset, 2 TRS samples gives good accuracy level for SNR at -3dB.

- RAN4 to agree on simulation assumptions presented in R4-2419297.

- AFO level of 50ppb and 20 ppb to be part of the simulation study. MPO levels to be studied based on the phase accuracy and phase granularity estimation from the simulations.

3Tx UL:

- specification of 3-TX UL may not have RRM requirements impact.

Asymmetric DL sTRP/UL mTRP:

- For asymmetric DL sTRP/UL mTRP with two TAs , DL reference timing can be defined as the first detected path (in time) of the downlink frame from the reference TRP.

- For each TAG, the uplink transmission timing takes place (N_"TA" +N_"TA offset" )×T_c before the reception of the downlink reference point.

- For asymmetric DL sTRP/UL mTRP with two TAs, gradual timing adjustment applies for each TAG and shall be met for each TAG.

- For each TAG/TRP, when the transmission timing error between the UE and the reference timing exceeds ±Te then the UE shall adjust the timing such that timing error is within ±Te. The reference timing shall be  before the downlink timing of the reference TRP.

- RAN4 to specify SRS based beam management requirements.

- RAN4 to specify UL TCI state switching requirements for UL only TRP.

# References

[1] RP-234007, New WID: NR MIMO Phase 5
[2] R4-2414035, WF on RRM requirements for NR_MIMO_Ph5