3GPP TSG RAN WG4 Meeting #113	R4-2419299

Orlando, USA, 18 Nov. 2024 – 22 Nov. 2024

Agenda Item:	7.24.2.1

Source: 	Ericsson

Title:	Discussion on measurements enhancements for purpose of supporting LTM

Document for:	Discussion

# Introduction

In this contribution we discuss CSI-RS based L1 measurement requirements.

# Discussion

## CSI-RS based L1 measurement requirements

In last meeting, RAN4 started initial discussion on the CSI-RS based L1-RSRP measurement requirements. Following requirements are identified as the set of requirements that may need to be defined.

- The following requirements need to be discussed for CSI-RS based L1 RSRP measurement on LTM candidate cell(s)

  - Measurement delay

  - Measurement reporting requirements

  - Measurement restrictions

  - Scheduling availability

- FFS on other potential impact.

Following summarizes the RAN1 agreements in last meeting.

There is no consensus in RAN1 on the support L1-SINR measurement based on CSI-RS for candidate cells. Further, from RAN1 perspective, there is no restriction with regards to the frequency location of CSI-RS used for L1-measurement In addition to periodic CSI-RS, semi-persistent CSI-RS is supported for candidate cell L1-RSRP measurement for gNB scheduled reporting from RAN1 perspective - Send an LS to RAN3 (CC RAN2) to ask for the feasibility of specifying the signalling for coordination between serving cell and candidate cell(s) on the transmission of semi-persistent CSI-RS(s) and any other potential issues (e.g. RAN3 workload). Support of semi-persistent CSI-RS is subject to UE capability. Final LS in R1-2409283.

### Intra-frequency and Inter-frequency defintion

As per the RAN1 agreement, since the frequency location of the CSI-RS can be in any location, RAN4 should discuss how to categorize the measurements (e.g., intra-frequency and inter-frequency definition). Following options were discussed in last meeting.

- Option 1: Following CSI-RS based L3 measurement (CTC, CMCC, Apple, Samsung, OPPO, [Nokia?]) - A measurement is defined as a CSI-RS based intra-frequency L1 measurement provided that: - the SCS of the CSI-RS resource of the neighbour cell configured for L1 measurement is the same as the SCS of the CSI-RS resource on the serving cell indicated for L1 measurement, and - the CP type of the CSI-RS resource of neighbour cell configured for L1 measurement is the same as the CP type of the CSI-RS resource of the serving cell indicated for L1 measurement, and - It is applied for SCS = 60KHz - the center frequency of the CSI-RS resource of the neighbour cell configured for L1 measurement is the same as the center frequency of the CSI-RS resource of the serving cell indicated for L1 measurement - Option 2: For definition of intra-frequency and inter-frequency, suggest waiting for more progress in other WGs. (Xiaomi) - Option 3: For backward compatibility, RAN4 better to have a definition for intra- and inter-frequency CSI-RS based L1-RSRP measurement as legacy. (CATT) - One possible way is to revise the legacy definition and clarify that the reference center frequency can be any of center frequencies of the CSI-RS resources of the serving cell. - Option 4: Considering the introduction of CSI-RS based LTM L1 measurement, whether an LTM cell switch is intra-frequency or inter-frequency may depend on whether the associated SSB reported in the CSI-RS-based measurement result is intra-frequency or inter-frequency. (vivo) - Whether the associated SSB is intra-frequency or inter-frequency follows the R18 rules - No additional definition of intra-F/inter-F for CSI-RS based L1 measurements. - Option 5: Intra-frequency CSI-RS based L1 measurement can be defined that the NZP-CSI-RS resource(s) of a candidate cell is within the DL active BWP of the serving cell and the SCS and CP of NZP-CSI-RS resource(s) of a candidate cell is same as those of the DL active BP of the serving cell. (HW) - It should be further discussed when the DL active BWP of the serving cell is within the bandwidth of NZP-CSI-RS resource(s) of a candidate cell.

Option 1 and option 3 were motivated from the CSI-RS based L3 measurement intra-frequency defintion. One disadvantage we see in option 1 is that NW always need to configure serving cell CSI-RS based L1 measurements for the comparison of intra-frequency definition even when NW may not use CSI-RS for L1-RSRP in the serving cell (that means NW do not use fine beams in serving cell but may plan to use them in candidate cells). As per option 1, if NW do not configure CSI-RS based L1 measurement, though the CSI-RS of the neighbour cell is in the active BWP, it is considered as inter-frequency measurement. If we define intra-frequency defintion using this approach, following requirements should be defined.

- Intra-frequency without gap

- Intra-frequency with gap

  - As per option 1, CSI-RS may be defined as intra-frequency measurement and as the option 1 do not have condition on the BW of the CSI-RS, CSI-RS may not be fully contained within the active BWP and UE may need gap for intra-frequency measurement

- Inter-frequency measurement without gap

  - CSI-RS may be within the active BWP though the center frequency is not same as serving cell CSI-RS or serving cell CSI-RS is not configured hence it is inter-frequency even if it is contained within BWP.

- Inter-frequency measurement with gap

Option 4: RAN4 did not define LTM cell switch as an intra-frequency or inter-frequency explicitly. From that sense option 4 makes sense not to define the intra-frequency defintion. However, the UE capability definition may be bit difficult to define without frequency layer definition.   If we do not define the frequency layer and what is intra or inter-frequency, we need to define the number of CSI-RS resource within active BWP UE can measure and number of CSI-RS resources UE can measure outside active BWP. Without frequency layer definition, the number of CSI-RS resources UE can measure outside active BWP may be limited to 1.

Option 5: Two set of requirements should be defined using this approach. Which are intra-frequency without gap and inter-frequency with gap. We see one issue with this option. Whenever the active BWP changes, definition of the frequency layer changes. If we define the UE capability in terms of number of intra-frequency layers UE can measure, and number of inter-frequency layers UE can measure, the capability kind of becomes dynamic.

We think all the options have their pros and cons and all of them can work. We think each option comes with different amount of workload and while choosing which option to select, RAN4 can consider the workload into consideration. Defining intra-frequency measurement based on the serving cell L1 CSI-RS, RAN4 needs to define 4 set of requirements. Defining intra-frequency measurement based on active BWP, RAN4 need to define two set of requirements. Not defining intra-frequency defintion also results in defining 2 set of requirements. Since RAN4 so far had the defintion of intra-frequency and inter-frequency measurement definition, not defining the measurement as intra-frequency or inter-frequency may result in confusion for the spec readers. Moreover, the workload also is not lower if we consider not to define the intra-frequency defintion. Hence, we make following proposal.

- RAN4 to define intra-frequency defintion as the following.

  - A frequency layer is called an intra-frequency measurement if the CSI-RS resources of the frequency layer are within the active BWP.

### Defintion of the frequency layer

If two CSI-RS resources belonging to same frequency layer, UE is expected to measure them at the same time using the same FFT. UE can measure the two CSI-RS resources at the same time using a same FFT if their SCS, CP type, BW, center frequency, and has overlapping periodicity (are fully/partially overlapping in time domain).

- Two CSI Resource are of same frequency layer if they have same SCS, CP type, BW, center frequency, and the CSI-RS occasions are fully/partially overlapping in time domain.

### Supported measurement types

When a measurement is within the active BWP it can be assumed to be performed without measurement gap and if the measurement is outside active BWP, it can be assumed to be performed with measurement gap. Since CSI-RS is used for L1-RSRP of candidate cells for the mobility purpose, RTD > CP shall be considered. In short, measurements inside BWP, outside BWP, RTD > CP and RTD < CP shall be considered.

- CSI-RS based L1 measurements for candidate cells shall support

- Measurement within active BWP, Measurements outside active BWP

- RTD < CP, and RTD > CP

### Measurement behaviour

In RAN4#112bis meeting, the following alternatives were discussed for the UE measurement behaviour for CSI-RS based L1-RSRP measurement.

Alternative 1: - RAN4 to consider the following measurement procedure for CSI-RS based LTM candidate cell measurements: - Step 1: SSB based L3 measurements [and reports] - Step 2: For FR2-1, SSB-based L1 measurements and reports on the same candidate cell - Further discuss whether Step 2 can be skipped under certain conditions. FFS the details of the conditions. - Note: Step 2 is removed for FR1. - Step 3: CSI-RS based L1 measurements on the same candidate cell - Note: The CSI-RS in Step 3 should be QCL’ed with the SSB in Step 1 or 2. - Note: For FR1, the CSI-RS in Step 3 should be QCL’ed with the SSB in Step 1. Alternative 2: - RAN4 to consider the following measurement procedure for CSI-RS based LTM candidate cell measurements: - Step 1: SSB based L3 measurements [and reports] - Step 3: CSI-RS based L1 measurements on the same candidate cell - Note: The CSI-RS in Step 3 should be QCL’ed with the SSB in Step 1.

Further, in last meeting at least periodic CSI-RS is agreed in RAN1. One potential way of using the CSI-RS on candidate cells to achieve better gain may be periodic or semi-persistent CSI-RS combined with aperiodic or semi-persistent CSI report. After the early sync, NW may request the aperiodic/semi-persistent report with repetition OFF and/or repetition ON (pending based on RAN1 agreement) to make better use of the feature. Hence, it is reasonable to assume that CSI-RS report may be requested by NW after the SSB based measurement configuration or reports. Since we cannot put requirement on the NW configuration, we can say that CSI-RS requirements are applicable if the SSB based measurement is performed at least X seconds before the CSI-RS report request.

- CSI-RS based measurement requirements are applicable if the UE measured SSB based L3 measurements less than X seconds before the CSI-RS measurement.

Since the CSI-RS needs to be QCLed with SSB for the timing and QCL derivation, it is reasonable to assume that UE needs to measure the SSB at least X ms before CSI-RS measurement occasion.

In FR2, UE may need to perform RX beam sweeping. The measurement flow depends on whether CSI-RS is configured with repetition ON/OFF. If the repetition ON is not configured, and SSB based L1-RSRP is configured on the candidate cell, UE may perform CSI-RS based L1 measurements based on alternative 1 or 2 as shown in the figure 1, 2 and 3. We provide our analysis of alternative 1 and 2.

Figure 1: Measurement procedure using Alternative 1

Alternative 1:

Step 1: UE performs SSB based L3 measurements with RX beam sweeping in FR2 and determine the RX beam for L3 measurements.

Step 2: If the same SSB is configured for L1 measurements, UE performs RX beam sweeping to find the best Rx beam for the SSB based L1-RSRP measurements. In this step UE may find a finer RX beam for receiving the SSB. This step is useful for finding the best Tx and Rx beam combination for SSB based L1-RSRP measurement. Due to the Rx beam gain and able to receive the SSB in Tx beam peak direction, UE may get higher L1-RSRP compared to L3-RSRP. In this step UE determines the best RX beam for a given SSB and uses the same for the receiving CSI-RS which was QCLed with SSB. In the figure 1, red solid beam may be the best Rx beam.

Step 3: the Rx beam determined in step 2 may be used to receive the CSI-RS which was configured with same SSB as QCL source RS. In the figure 1, UE uses the red solid beam to receive all the three CSI-RS (as three CSI-RS has same SSB as QCL source).

Why Alternative 1 may not work well: Based on the Rx beam width selected by UE, it may happen that the RX beam may be good for receiving one CSI-RS but may not be good for receiving the other CSI-RS. In the figure 1, the red RX beam (best Rx beam for SSB based L1-RSRP) may be good for receiving the green CSI-RS beam but may be not so good for receiving other two CSI-RS. We think wide beam used for SSB based L3 measurement may work well for receiving three CSI-RS resources than the SSB based fine beam for L1 measurement.

Figure 2: Measurement procedure using alternative 2

Alternative 2:

In this approach, UE performs the RX beam sweeping using the periodic or semi-persistent CSI-RS. In step 1, UE performs SSB based L3 measurements and determine the wide beam to be used for SSB based L3 measurements. In step 2, instead of doing Rx beam sweeping on SSB to determine the fine beam, UE uses the periodic or semi-persistent CSI-RS for the Rx beam sweeping. We show the periodic CSI-RS based Rx beam sweeping in figure 3.

Figure 3: Rx beam sweeping using CSI-RS

From the figure 3, we can observe that UE can get the best Tx and Rx beam pair if the UE has performed the RX beam sweeping using CSI-RS. As shown in figure 3, orange CSI-RS beam can be best received with black Rx beam, green CSI-RS beam can be best received with green Rx beam, and blue CSI-RS beam can be received with red Rx beam.

If a UE supports early CSI acquisition and reporting, best Tx and Rx beam pair can be useful in determining the CQI more accurately. When NW schedules data in the candidate cell using fine beam UE will be able to support better throughput using alternative 2.

- RAN4 to consider the following measurement procedure for CSI-RS based LTM candidate cell measurements:

  - Step 1: SSB based L3 measurements [and reports]

  - Step 3: CSI-RS based L1 measurements on the same candidate cell

### Known cell conditions

For CSI-RS based L1-RSRP measurement, known cell conditions are discussed in last meeting and the following WF is agreed.

.

- Agreement: - The UE has performed L3 measurement on the target cell during the last 5 seconds, and - Further check whether to consider SSB-based L3 measurement only - The SSB from the target cell configured for L3[/L1] measurement remains detectable according to the cell identification requirements in clause 9.2. - L1 measurement in the 2nd bullet is not needed for FR1. - Further discuss whether to keep or remove L1 measurement for FR2-1 in the 2nd bullet. - The CSI-RS from the target cell configured for L1 measurement remains [detectable/measurable]. - Further discuss whether to use detectable or measurable. - Further discuss the side condition

Couple of open issues in the above agreement.

Detectability of SSB for L3/L1: If the same SSB is configured for L3 and L1 measurement, detectability condition for L3 measurement is SSB_RP ≥ -6dB and detectability condition for L1 is SSB_RP ≥ -3dB. Since the SSB is used for timing and QCL relations, we think L3 detectability should be enough as SSB based timing and QCL can be taken as a rough timing assumption for the CSI-RS reception.

Measurable or detectable: We think detectable is used for RS which can be detected without any assistance from the NW. Since CSI-RS needs SSB as a QCL relation, we think it cannot be detected on its own. Having said that there are some occasions in the spec which used measurable for SSB also. Hence, we could use either measurable or detectable and define what the measurable or detectable means. We think CSI-RS is measurable if the side condition for the SNR is satisfied and SNR side condition (-3dB) of serving cell CSI-RS measurements can be reused.

We think CSI-RS based L3 measurements needs SSB as a pre-requisite. Hence, we suggest considering SSB-based L3 measurement only.

- For CSI-RS based L1-RSRP measurements on neighbour cells, an LTM candidate cell is known if the following conditions are met

  - The UE has performed SSB based L3 measurement on the target cell during the last 5 seconds, and

  - The SSB QCL’ed with the CSI-RS remains detectable (Es/Io ≥ -6 dB) according to the cell identification requirements in clause 9.2.

  - The CSI-RS from the target cell configured for L1 measurement remains measurable.

    - CSI-RS is considered measurable if the CSI_RS Es/Io is ≥ -3dB.

### Measurement gaps

When there is only one single GAP configuration is supported, UE should share MG between the SSB and CSI-RS. Measurement gap shared with SSB may have larger impact on the RRM measurements. If the CSI-RS is not overlapping with SMTC, UE may perform both the CSI-RS based and SSB based measurements in different gaps.

Concurrent MG may be useful for CSI-RS. However, it may bring lot of additional workload. We think RAN4 can further discuss which type of measurement gap can be used for CSI-RS based L1-RSRP.

- RAN4 to further discuss type of MG for the CSI-RS based L1-RSRP measurements.

# Summary and Conclusion

In this contribution we analyzed potential RAN4 RRM requirements impact and made following observations and proposals.

- RAN4 to define intra-frequency defintion as the following.

  - A frequency layer is called an intra-frequency measurement if the CSI-RS resources of the frequency layer are within the active BWP.

- Two CSI Resource are of same frequency layer if they have same SCS, CP type, BW, center frequency, and the CSI-RS occasions are fully/partially overlapping in time domain.

- CSI-RS based L1 measurements for candidate cells shall support

- Measurement within active BWP, Measurements outside active BWP

- RTD < CP, and RTD > CP

- CSI-RS based measurement requirements are applicable if the UE measured SSB based L3 measurements less than X seconds before the CSI-RS measurement.

- RAN4 to consider the following measurement procedure for CSI-RS based LTM candidate cell measurements:

  - Step 1: SSB based L3 measurements [and reports]

  - Step 3: CSI-RS based L1 measurements on the same candidate cell

- For CSI-RS based L1-RSRP measurements on neighbour cells, an LTM candidate cell is known if the following conditions are met

  - The UE has performed SSB based L3 measurement on the target cell during the last 5 seconds, and

  - The SSB QCL’ed with the CSI-RS remains detectable (Es/Io ≥ -6 dB) according to the cell identification requirements in clause 9.2.

  - The CSI-RS from the target cell configured for L1 measurement remains measurable.

    - CSI-RS is considered measurable if the CSI_RS Es/Io is ≥ -3dB.

- RAN4 to further discuss type of MG for the CSI-RS based L1-RSRP measurements.

# References

[1] 	RP-241515, New WID: NR mobility enhancements Phase 4

[2] 	R4-2414060, WF on NR mobility enhancements Phase 4