3GPP TSG-RAN WG4 Meeting #113	R4-2419324

Orlando, Florida, USA, November 18 – 22, 2024

Source:	Nokia, Nokia Shanghai Bell

Title:	RRM requirements for SSB adaptation

Agenda item:	7.22.2.2

Document for:	Discussion

Introduction

The Rel-19 WID on Enhancements of network energy savings for NR, revised in [1], specifies for the objective “adaptation of common signal/channel transmissions”, among other enhancements:

- SSB adaptation in time domain, e.g. adapting periodicity”.

At RAN4#112bis meeting, the RAN4 scope of the WI was further discussed, and the agreements were captured in the WF [2]. In this contribution, we continue the discussion on the RRM impact from SSB adaptation.

Discussion

  - Progress status in RAN4 and RAN1

At RAN4#112bis meeting, the following has been agreed on RRM requirements for SSB adaptation [2].

### Sub-topic 1-1 SSB adapation – Scenario Issue 1-1-1: Scenario clarification based on RAN1 progress Agreement: - Per RAN1 agreements, RAN4 to take following as starting point for SSB adaption discussion: - CONNECTED mode, and - SSB adaptation for SCell, and - SSB adaptation for NCD-SSB, and - SSB periodicity adaptation in time-domain Note: Above could be further updated based on further RAN1 progress (e.g. PCell, other adaptation mechanism) Issue 1-2-1: Requirements need updating Agreement: - RAN4 to discuss whether there are any impacts on at least the following RRM requirements due to SSB adaption: - L1-RSRP for serving cell - L1-SINR - BFD - CBD - SCell activation - TCI state switching - NR intra-frequency L3 measurement - NR inter-frequency L3 measurement Note: Above list could be further updated based on RAN4 discussion.

Open issues for SSB adaptation are contained in [3].

RAN1 has discussed this enhancement in their last meetings. At RAN1#118bis, the following conclusion was reached:

Agreements (RAN1#118bis) [Adaptation of SSB in time domain] Conclusion​ There is no consensus on the support of adaptation of SSB for idle mode UEs in Rel-19​. Proposal 2.1.4-rev1​ For Cell DTX extension to SSBs not on sync-raster for connected mode UEs, select Option 3, i.e. Cell DTX does not impact UE assumption on SSB transmissions (i.e. legacy behavior).​ - No spec impact

At RAN1 #118 meeting, the following agreements were reached:

Agreements (RAN1#118) [Adaptation of SSB in time domain] Agreement For adaptation mechanism(s) of SSB in time-domain, - For Rel-19 NES-capable UE’s PCell (Connected mode), adaptation of CD-SSB on sync raster is not supported - FFS: Adaptation for SSB that is not CD-SSB is supported (A2) - FFS: Adaptation for SSB not on sync raster is supported (A3) - For Rel-19 NES-capable UE’s SCell - Adaptation of SSB configured for the SCell is supported for the following cases - FFS: Adaptation for CD-SSB (B1) including UE impact compared to legacy operation where the SSB is configured with periodicity>20msec for SCell - Adaptation for SSB that is not CD-SSB on sync raster (B2’) - Adaptation for SSB that is not CD-SSB not on sync raster (B3’) Agreement For Cell DTX extension to SSBs not on sync-raster for connected mode UEs, select from following options - Option 1: One SSB burst periodicity is configured for the UE and UEs assumes SSB transmissions are not present during Cell DTX non-active period - Option 2: UE assumes SSB transmission with different periodicities during Cell DTX non-active period and during Cell DTX active period - Option 3: Cell DTX does not impact UE assumption on SSB transmissions (i.e. legacy behavior) – no spec impact

At RAN1#117 meeting, the following agreement was reached:

Agreement (RAN1#117) For adaptation of SSB in time-domain, Option 1 is supported - Option 1: Adaptation of SSB burst periodicity using one or more SSB burst periodicity value(s) - Note: Using Option 2 to realize Option 1 is not precluded - Option 2: Adaptation based on two SSB configurations [where up to two configurations can be active] - FFS: details of the differences between the two SSB configurations, e.g., two different periodicities - FFS: Details including applicable scenarios - FFS: Support of Cell DTX for connected mode UEs for SSB

  - Scenarios for SSB adaptation

It is noted RAN1#118bis concluded that there is no support for SSB adaptation in idle mode. Hence RAN4 can stick to its RAN4#112bis agreement on the scenarios to be considered as a starting point [2].

Related to the above RAN1#117 agreement on SSB adaptation in time domain, in our understanding the agreed Option 1 is related to adaptation of SSB burst periodicity for a configured single SSB pattern by defining a set of SSB burst periodicities for the SSB transmission. Option 2 is based on definition of two SSB configurations, where either one or both can be active. It is currently not precluded realizing Option 1 using option 2, i.e. single active SSB pattern. However, there is an open issue for Option 2, that the two SSB configurations may differ in the SSB burst periodicity and suitable other parameters such as SSB type or others, the latter requiring further study in RAN1. For Option 2 it is also unclear, whether it is related to single active SSB pattern or two concurrent active SSB patterns. As RAN1 is discussing the change of SSB burst periodicity before paging to improve the UE synchronization when being paged (although a Rel-18 solution based on TRS exists), it is assumed that both Options 1 and 2 are related to alternating between SSB burst periodicities or between two SSB configurations with different SSB burst periodicities which allows to activate up to two SSB patterns at a time (e.g. resulting in one pattern for legacy UEs and two patterns for Rel-19 NES UEs). Although, RAN1 clarification of these scenarios for Option 2 is needed here.

Another scenario for SSB adaptation is considered by RAN1 related to on-demand SSB, namely Option 4 of the RAN4#116 agreement:

Agreement (RAN1#116) … Option 4: UE expects that on-demand SSB burst(s) is transmitted with a periodicity from time instance A to time instance B and with the other periodicity after time instance B. …

Regarding Option 4 in the RAN1#116 agreement, based on our understanding, it is intended for Case-2 with always-ON periodic SSB transmission in SCell, where the NW triggers on-demand denser SSB transmission with shorter SSB periodicity from time instance A, i.e. when NW sends SCell activation command, and later NW starts sending SSB with longer periodicity after time instance B, i.e. when NW receives UE measurement report. The Option 4 can thus be jointly considered with the RAN1#117 agreement about adaptation of SSB burst periodicity in time-domain.

Observation #1: Options 1 and 2 of the RAN1#117 agreement are discussing the alternating between different SSB periodicities and the operation of up to two SSB configurations, respectively.

Observation #2: For Option 1, only one SSB pattern is active at a time, and this may have different SSB burst periodicities.

Observation #3: Option 4 of the RAN1#116 agreement is intended for Case-2 with always-ON periodic SSB transmission in SCell, where the NW triggers on-demand denser SSB transmission with shorter SSB periodicity from time instance A, i.e. when NW sends SCell activation command, and later NW starts sending SSB with longer periodicity after time instance B, i.e. when NW has received UE measurement report.

In summary, until RAN1 concludes the discussion on Option 2 in view of applicable scenarios, e.g. if this is related to CD-SSB and NCD-SSB configurations, and whether two SSB configurations can be active at a time, RAN4 should focus on Option 1. Moreover, RAN1 in our view needs also to discuss applicable scenarios for Option 1 triggering SSB adaptation in greater detail. Based on the above RAN1 agreements, RAN4 can start to investigate RRM impacts for SSB adaptation in the time domain for a single active SSB pattern, due to usage of different SSB burst periodicities.

Proposal 1: 	RAN4 to start investigation of RRM impacts for SSB adaptation in time domain for a single active SSB pattern, due to usage of different SSB burst periodicities. Further scenarios can be considered based on further RAN1 agreements.

Regarding the range of SSB periodicities, we observe that 160ms is the longest periodicity in current spec. It is noted in the RAN1 study item [4] few companies have investigated NW energy saving potential for SSB periodicities beyond 160ms, such as 640ms and 1280ms, but the majority of companies provided results for NW energy savings for SSB periodicities up to and including 160ms. Furthermore, considering the identified impact of SSB periodicity adaptation to L1 and L3 measurement delays, as depicted further below, RAN4 should maintain the existing specified range of SSB burst periodicity for SSB periodicity adaptation.

Proposal 2: 	RAN4 to maintain the specified range of SSB burst periodicity (5ms to 160ms) for SSB periodicity adaptation.

  - Layer 3 measurement impact

RAN4#112bis achieved the above listed agreement on requirements need updating. Based on this, SSB based L3 measurements in RRC CONNECTED state, agreed to be affected by SSB adaptation, are identified to be the following ones:

- NR intra-frequency NR measurements,

- NR inter-frequency NR measurements.

The impact to these measurements is analyzed in this section.

According to TS 38.133, NCD-SSB based L3 measurements can be used for serving cell, intra-frequency and inter-frequency neighbor cell measurements in RRC_CONNECTED state, if the NW indicates it as reference SSB.

For NR intra-frequency measurements in RRC_CONNECTED state, RRM impacts exist for non-serving cell detection delay and the measurement delay for known non-serving cell, as listed below in further detail, referring to existing RRM requirements in TS 38.133.

Intra-frequency cell detection

Requirements for intra-frequency cell detection, i.e. for intra-frequency PSS/SSS detection in RRC_CONNECTED state without measurement gaps, are based on the configured period for the SMTC window. If, due to SSB periodicity adaptation, the SMTC period (highlighted) is adjusted, which is to be confirmed by RAN2, the cell detection delay increases proportionally up to the defined limit in the max calculation as depicted in Table 9.2.5.1-1 for the case of FR1.

Table 9.2.5.1-1: Time period for PSS/SSS detection, (Frequency range FR1)

| DRX cycle | TPSS/SSS_sync_intra |
| --- | --- |
| No DRX | max( 600ms, ceil( 5 x Kp) x SMTC period )Note 1 x CSSFintra |
| DRX cycle≤ 320ms | max( 600ms, ceil(M2 Note 2x 5 x Kp) x max(SMTC period,DRX cycle)) x CSSFintra |
| DRX cycle>320ms | ceil(5 x Kp) x DRX cycle x CSSFintra |
| NOTE 1: If different SMTC periodicities are configured for different cells, the SMTC period in the requirement is the one used by the cell being identified NOTE 2: When highSpeedMeasFlag-r16 is not configured, M2 = 1.5; When highSpeedMeasFlag-r16 is configured, M2 = 1.5 if SMTC periodicity > 40 ms;,otherwise M2=1. NOTE 3: When highSpeedMeasFlag-r16 is configured, the requirements apply only to UE supporting either measurementEnhancement-r16 or intraNR-MeasurementEnhancement-r16 on measurements of the primary component carrier and do not apply to measurements of a secondary component carrier with active SCell. NOTE 4: When highSpeedMeasCA-Scell-r17 is configured and UE supports measurementEnhancementCA-r17, M2 = 1.5 if SMTC periodicity > 40 ms; otherwise M2=1. |  |

Similar requirements apply for cell detection delay for UEs indicating support for “no-gap-with-interruption”, hence also here there is an impact due to SSB periodicity adaptation.

For the case of configured measurement gap, the cell detection delay increases proportionally to the SMTC period up to the defined limit as depicted in Table 9.2.6.2-1 for the case of FR1. Thus, similar impact is observed as for intra-frequency cell detection without measurement gaps.

Table 9.2.6.2-1: Time period for PSS/SSS detection (FR1)

| DRX cycle | TPSS/SSS_sync_intra |
| --- | --- |
| No DRX | max(600ms, 5 x Kgap x max(MGRP, SMTC period)) x CSSFintra |
| DRX cycle≤ 320ms | max(600ms, ceil(M2Note 1x 5 x Kgap) x max(MGRP, SMTC period,DRX cycle)) x CSSFintra |
| DRX cycle>320ms | Ceil( 5 x Kgap ) x max(MGRP, DRX cycle) x CSSFintra |
| NOTE 1: When highSpeedMeasFlag-r16 is not configured, M2 = 1.5; When highSpeedMeasFlag-r16 is configured, M2 = 1.5 if SMTC periodicity > 40 ms, otherwise M2=1. NOTE 2: When highSpeedMeasFlag-r16 is configured, the requirements apply only to UE supporting either measurementEnhancement-r16 or intraNR-MeasurementEnhancement-r16 on measurements of the primary component carrier and do not apply to measurements of a secondary component carrier with active SCell. NOTE 3: For a UE supporting concurrent GAPs, if multiple concurrent GAPs are configured, the MGRP is the periodicity of the activated Pre-MG or the MG pattern associated to the intra-frequency layer. NOTE 4: When highSpeedMeasCA-Scell-r17 is configured, the requirements apply to UE on measurements of secondary component carrier with active SCell. |  |

For the case of deactivated SCell, the cell detection delay is proportional to the measCycleSCell parameter (highlighted) up to a defined limit as depicted in Table 9.2.5.1-4 for the case of FR1. This parameter is configured as part of the MeasObjNR IE and ranges between 160ms and 1280ms, hence it is greater or equal to the largest existing SSB periodicity (160ms). Thus, there is no impact from SSB periodicity adaptation to cell detection of deactivated SCells based on use of measCycleSCell.

Table 9.2.5.1-4: Time period for PSS/SSS detection, deactivated SCell (FR1)

| DRX cycle | TPSS/SSS_sync_intra |
| --- | --- |
| No DRX | Ceil(5 x Kp) x measCycleSCell x CSSFintra |
| DRX cycle≤ 320ms | Ceil(5 x Kp) x max(measCycleSCell, 1.5xDRX cycle) x CSSFintra |
| DRX cycle> 320ms | Ceil(5 x Kp) x max(measCycleSCell, DRX cycle) x CSSFintra |
| NOTE 1: The requirements also apply to deactivated SCG SCell. |  |

Similar requirements apply for cell detection delay related to deactivated PSCells. Thus, there is no impact from SSB periodicity adaptation.

For cell detection using NCSG, the delay requirements are also defined proportional to SMTC period up to the defined limit in the max calculation as depicted in Table 9.2.7.1-1 for the case of FR1. Thus, there is similar impact from SSB adaptation as for cell detection using measurement gaps.

Table 9.2.7.1-1: Time period for PSS/SSS detection with NCSG (FR1)

| DRX cycle | TPSS/SSS_sync_intra |
| --- | --- |
| No DRX | max(600ms, 5 x KNCSG x max(VIRP, SMTC period)) x CSSFintra |
| DRX cycle≤ 320ms | max(600ms, ceil(M2Note 1x 5 x KNCSG) x max(VIRP, SMTC period,DRX cycle)) x CSSFintra |
| DRX cycle>320ms | 5 x KNCSG x max(VIRP, DRX cycle) x CSSFintra |
| NOTE 1: When highSpeedMeasFlag-r16 is not configured, M2 = 1.5; When highSpeedMeasFlag-r16 is configured, M2 = 1.5 if SMTC periodicity > 40 ms, otherwise M2=1. NOTE 2: When highSpeedMeasFlag-r16 is configured, the requirements apply only to UE supporting either measurementEnhancement-r16 or [intraRAT-MeasurementEnhancement-r16] on measurements of the primary component carrier and do not apply to measurements of a secondary component carrier with active SCell. |  |

Intra-frequency Cell identification

Additionally, for the UE to perform a cell identification, a round of measurements would be needed when considering cell identification without index reading. If index reading is needed, then an additional delay for time index detection is added.

Intra-frequency Time index detection

Requirements for intra-frequency time index detection in RRC_CONNECTED state without measurement gaps, are based on the configured SMTC window. If, due to SSB periodicity adaptation, the SMTC period (highlighted) is adjusted, which is to be confirmed by RAN2, the time index detection delay increases proportionally up to the defined limit in the max calculation as depicted in Table 9.2.5.1-3 for the case of FR1.

Table 9.2.5.1-3: Time period for time index detection (FR1)

| DRX cycle | TSSB_time_index_intra |
| --- | --- |
| No DRX | max(120ms, ceil( 3 x Kp ) x SMTC period)Note 1 x CSSFintra |
| DRX cycle≤ 320ms | max(120ms, ceil (M2 Note 2 x 3 x Kp) x max(SMTC period,DRX cycle)) x CSSFintra |
| DRX cycle>320ms | Ceil(3 x Kp) x DRX cycle x CSSFintra |
| NOTE 1: If different SMTC periodicities are configured for different cells, the SMTC period in the requirement is the one used by the cell being identified NOTE 2: When highSpeedMeasFlag-r16 is not configured, M2 = 1.5; When highSpeedMeasFlag-r16 is configured, M2 = 1.5 if SMTC periodicity > 40 ms;,otherwise M2=1 NOTE 3: When highSpeedMeasFlag-r16 is configured, the requirements apply only to UE supporting either measurementEnhancement-r16 or intraNR-MeasurementEnhancement-r16 on measurements of the primary component carrier and do not apply to measurements of a secondary component carrier with active SCell. NOTE 4: When highSpeedMeasCA-Scell-r17 is configured and UE supports measurementEnhancementCA-r17, M2 = 1.5 if SMTC periodicity > 40 ms; otherwise M2=1 |  |

For the case of configured measurement gap, the time index detection delay increases proportionally to the SMTC period up to the defined limit as depicted in Table 9.2.6.2-3 for the case of FR1. Thus, similar impact due to SSB periodicity adaptation is observed as for intra-frequency time index detection without measurement gaps.

Table 9.2.6.2-3: Time period for time index detection (Frequency range FR1)

| DRX cycle | TSSB_time_index_intra |
| --- | --- |
| No DRX | max(120ms, ceil(3 x Kgap ) x max(MGRP, SMTC period)) x CSSFintra |
| DRX cycle≤ 320ms | max(120ms, ceil(M2Note 1x 3 x Kgap) x max(MGRP, SMTC period,DRX cycle) x CSSFintra) |
| DRX cycle>320ms | Ceil(3 x Kgap )x max(MGRP, DRX cycle) x CSSFintra |
| NOTE 1: When highSpeedMeasFlag-r16 is not configured, M2 = 1.5; When highSpeedMeasFlag-r16 is configured, M2 = 1.5 if SMTC periodicity > 40 ms, otherwise M2=1. NOTE 2: When highSpeedMeasFlag-r16 is configured, the requirements apply only to UE supporting either measurementEnhancement-r16 or intraNR-MeasurementEnhancement-r16 on measurements of the primary component carrier and do not apply to measurements of a secondary component carrier with active SCell. NOTE 3: For a UE supporting concurrent GAPs, if multiple concurrent GAPs are configured, the MGRP is the periodicity of the activated Pre-MG or the MG pattern associated to the intra-frequency layer. NOTE 4: When highSpeedMeasCA-Scell-r17 is configured, the requirements apply to UE on measurements of secondary component carrier with active SCell. |  |

For the case of deactivated SCell, the time index detection delay is proportional to the measCycleSCell value (highlighted) up to a defined limit as depicted in Table 9.2.5.1-6 for the case of FR1. This parameter is configured as part of the MeasObjNR IE and ranges between 160ms and 1280ms, hence it is greater or equal to the largest existing SSB periodicity (160ms). Thus, there is no impact from SSB periodicity adaptation to time index detection of deactivated SCells based on use of measCycleSCell.

Table 9.2.5.1-6: Time period for time index detection, deactivated SCell (FR1)

| DRX cycle | TSSB_time_index_intra |
| --- | --- |
| No DRX | Ceil(3 x Kp) x measCycleSCell x CSSFintra |
| DRX cycle≤ 320ms | Ceil(3 x Kp) x max(measCycleSCell, 1.5xDRX cycle) x CSSFintra |
| DRX cycle> 320ms | Ceil(3 x Kp) x max(measCycleSCell, DRX cycle) x CSSFintra |
| NOTE 1: The requirements also apply to deactivated SCG SCell. |  |

For the case of deactivated SCell, the time index detection delay using NCSG is proportional to the measCycleSCell value up to a defined limit as depicted in Table 9.2.7.1-6 for the case of FR1. Thus, there is no impact from SSB periodicity adaptation to time index detection of deactivated SCells based on use of measCycleSCell.

Table 9.2.7.1-6: Time period for time index detection with NCSG (deactivated SCell) (FR1)

| DRX cycle | TSSB_time_index_intra |
| --- | --- |
| No DRX | 3 x KNCSG x max(measCycleSCell, VIRP) x CSSFintra |
| DRX cycle≤ 320ms | 3 x KNCSG x max(measCycleSCell, VIRP, 1.5xDRX cycle) x CSSFintra |
| DRX cycle> 320ms | 3 x KNCSG x max(measCycleSCell, VIRP,DRX cycle) x CSSFintra |

Intra-frequency measurements

Requirements for intra-frequency measurements in RRC_CONNECTED state without measurement gaps are based on the configured SMTC window. If, due to SSB periodicity adaptation, the SMTC period (highlighted) is adjusted, which is to be confirmed by RAN2, the intra-frequency measurement period increases proportionally up to the defined limit as depicted in Table 9.2.5.2-1 for the case of FR1.

Table 9.2.5.2-1: Measurement period for intra-frequency measurements without gaps (FR1)

| DRX cycle | T SSB_measurement_period_intra |
| --- | --- |
| No DRX | max(200ms, ceil( 5 x Kp) x SMTC period)Note 1 x CSSFintra |
| DRX cycle≤ 320ms | max(200ms, ceil(1.5x 5 x Kp) x max(SMTC period,DRX cycle)) x CSSFintra |
| DRX cycle>320ms | ceil( 5 x Kp ) x DRX cycle x CSSFintra |
| NOTE 1: If different SMTC periodicities are configured for different cells, the SMTC period in the requirement is the one used by the cell being identified |  |

Similar requirements apply for intra-frequency measurement period for UEs indicating support for “no-gap-with-interruption”, hence also here there is an impact due to SSB periodicity adaptation.

For the case of configured measurement gap, the intra-frequency measurement period increases proportionally to the SMTC period up to the defined limit as depicted in Table 9.2.6.3-1 for the case of FR1, hence also here there is an impact due to SSB periodicity adaptation.

Table 9.2.6.3-1: Measurement period for intra-frequency measurements with gaps(FR1)

| DRX cycle | T SSB_measurement_period_intra |
| --- | --- |
| No DRX | max(200ms, ceil(5 x Kgap )x max(MGRP, SMTC period)) x CSSFintra |
| DRX cycle≤ 320ms | max(200ms, ceil(1.5x 5 x Kgap) x max(MGRP, SMTC period,DRX cycle)) x CSSFintra |
| DRX cycle>320ms | Ceil(5 x Kgap ) x max(MGRP, DRX cycle) x CSSFintra |
| NOTE 1: For a UE supporting concurrent GAPs, if multiple concurrent GAPs are configured, the MGRP is the periodicity of the activated Pre-MG or the MG pattern associated to the intra-frequency layer. |  |

For the case of deactivated SCell, the intra-frequency measurement period is proportional to the measCycleSCell value up to a defined limit as depicted in Table 9.2.5.2-3 for the case of FR1, hence here there is no impact due to SSB periodicity adaptation.

Table 9.2.5.2-3: Measurement period for intra-frequency measurements without gaps (deactivated SCell) (FR1)

| DRX cycle | T SSB_measurement_period_intra |
| --- | --- |
| No DRX | Ceil(5 x Kp) x measCycleSCell x CSSFintra |
| DRX cycle≤ 320ms | Ceil(5 x Kp) x max(measCycleSCell, 1.5xDRX cycle) x CSSFintra |
| DRX cycle> 320ms | Ceil(5 x Kp) x max(measCycleSCell, DRX cycle) x CSSFintra |
| NOTE 1: The requirements also apply to deactivated SCG SCel |  |

Similar requirements apply for intra-frequency measurement period related to deactivated PSCells, hence also here there is no impact due to SSB periodicity adaptation.

Requirements for intra-frequency measurements period in RRC_CONNECTED state with NCSG, are also based on the configured SMTC window. If, due to SSB periodicity adaptation, the SMTC period (highlighted) is adjusted, which is to be confirmed by RAN2, the intra-frequency measurement period increases proportionally up to the defined limit as depicted in Table 9.2.7.2-1 for the case of FR1, hence there is an impact due to SSB periodicity adaptation.

Table 9.2.7.2-1: Measurement period for intra-frequency measurements with NCSG (FR1)

| DRX cycle | T SSB_measurement_period_intra |
| --- | --- |
| No DRX | max(200ms, 5 x KNCSG x max(VIRP, SMTC period)) x CSSFintra |
| DRX cycle≤ 320ms | max(200ms, ceil(1.5x 5 x KNCSG) x max(VIRP, SMTC period,DRX cycle)) x CSSFintra |
| DRX cycle>320ms | 5 x KNCSG x max(VIRP, DRX cycle) x CSSFintra |

Inter-frequency cell detection

For NR inter-frequency cell detection in RRC_CONNECTED state, similar RRM impacts exist for non-serving cell detection delay, as listed above for intra-frequency measurements.

Inter-frequency measurements

For NR inter-frequency measurement in RRC_CONNECTED state, similar RRM impacts exist for the measurement period for non-serving cell, as listed above for intra-frequency measurements.

SCell activation

When an SCell is activated, the measurement period which was based on a multiple of the parameter measCycleSCell for the deactivated SCell, can be adjusted to a new SSB periodicity based on the SMTC configuration signalled to UE. Thus, the network can indicate a change of the SSB periodicity when the SCell is activated. Likewise, when the SCell is deactivated, the measurement period is again based on a multiple of the parameter measCycleSCell, which is configured independently of the SSB periodicity.

Thus, RAN4 needs to consider the above depicted L3 measurement impact.

Proposal 3: 	RAN4 to consider the L3 measurement impact from SSB periodicity adaptation to NR intra-/inter-frequency cell detection delay and time index delay as well as to NR intra-/inter-frequency measurement period, i.e. the impact to configured SMTC period in case of active SCell.

  - Layer 1 measurement impact

RAN4#112bis achieved the above listed agreement on requirements need updating. Based on this, SSB based L1 measurements in RRC CONNECTED state, agreed to be affected by SSB adaptation, are identified to be the following ones:

- L1-RSRP,

- L1-SINR.

The impact to these measurements is analyzed in this section.

According to TS 38.133, clause 9.2.1, NR intra-frequency measurements are based on the reference SSB, which may be CD-SSB or NCD-SSB. The same applies for serving cell L1 measurements, i.e. NCD-SSB based L1 measurements can be used to derive serving cell L1-RSRP and L1-SINR measurements in RRC_CONNECTED state, if the NW indicates NCD-SSB as reference SSB.

For NR intra-frequency measurements in RRC_CONNECTED state, RRM impacts exist for non-serving cell detection delay and the measurement delay for detected non-serving cell, as listed below in further detail, referring to existing RRM requirements in TS 38.133.

SSB based radio link monitoring

According to the above RAN4 #112bis agreement, not to consider SSB adaptation in PCell, if not agreed in RAN1, no impact to SSB based radio link monitoring in TS 38.133 clause 8.1.2.2 is observed.

SSB based beam failure detection

According to clause 8.5.2.2 for SSB based beam failure detection requirements, the evaluation period TEvaluate_BFD_SSB for the active serving cell (including active SCell) depends on periodicity of SSB (TSSB) in the set , configured for BFD, except for DRX cycles > 320ms, as shown below for FR1 and FR2 for serving cell.

For the deactivated PSCell, the evaluation periods depend on the configured measCyclePSCell parameter independent of DRX enabled/disabled and on the DRX cycle length. However, the measCyclePSCell period is configured independently of the SSB periodicity, hence there is no impact from SSB periodicity adaptation in this case.

Table 8.5.2.2-1: Evaluation period TEvaluate_BFD_SSB for FR1

| Configuration | TEvaluate_BFD_SSB (ms) |
| --- | --- |
| no DRX | Max(50, Ceil(5 ´ P) ´ TSSB) |
| DRX cycle ≤ 320ms | Max(50, Ceil(7.5 ´ P) ´ Max(TDRX,TSSB)) |
| DRX cycle > 320ms | Ceil(5 ´ P) ´ TDRX |
| Note: TSSB is the periodicity of SSB in the set . TDRX is the DRX cycle length. |  |

Table 8.5.2.2-2: Evaluation period TEvaluate_BFD_SSB for FR2

| Configuration | TEvaluate_BFD_SSB (ms) |
| --- | --- |
| no DRX | Max(50, Ceil(5 ´ P ´ N) ´ TSSB) |
| DRX cycle ≤ 320ms | Max(50, Ceil(7.5 ´ P ´ N) ´ Max(TDRX,TSSB)) |
| DRX cycle > 320ms | Ceil(5 ´ P ´ N) ´ TDRX |
| Note: TSSB is the periodicity of SSB in the set . TDRX is the DRX cycle length. |  |

Table 8.5.2.2-4: Evaluation period TEvaluate_BFD_SSB for deactivated PSCell in FR1

| Configuration | TEvaluate_BFD_SSB (ms) |
| --- | --- |
| no DRX | Ceil(5 ´ P) ´ measCyclePscell |
| DRX cycle ≤ 320ms | Ceil(7.5 ´ P) ´ Max(measCyclePscell, TDRX) |
| DRX cycle > 320ms | Ceil(5 ´ P) ´ Max(measCyclePscell, TDRX) |
| Note: DRX cycle is the configured DRX cycle of the PSCell. measCyclePSCell is the measurement cycle length of the deactivated PSCell. |  |

Table 8.5.2.2-5: Evaluation period TEvaluate_BFD_SSB for deactivated PSCell in FR2

| Configuration | TEvaluate_BFD_SSB (ms) |
| --- | --- |
| no DRX | Ceil(5 ´ P ´ N) ´ measCyclePscell |
| DRX cycle ≤ 320ms | Ceil(7.5 ´ P ´ N) ´ Max(measCyclePscell, TDRX) |
| DRX cycle > 320ms | Ceil(5 ´ P ´ N) ´ Max(measCyclePscell, TDRX) |
| Note: DRX cycle is the configured DRX cycle of the PSCell. measCyclePSCell is the measurement cycle length of the deactivated PSCell. |  |

For relaxed measurement criteria, clause 8.5.2.4 specifies the evaluation period TEvaluate_BFD_SSB for the active serving cell (including active SCell) depending on periodicity of SSB in the set , configured for BFD, as shown below.

Table 8.5.2.4-1: Evaluation period TEvaluate_BFD_SSB_Relax for FR1

| Configuration | TEvaluate_BFD_SSB_Relax (ms) |
| --- | --- |
| Max(TDRX, TSSB) ≤ 80 ms | Max(50 ´ K3, Ceil(7.5 ´ K1 ´ P) ´ Max(TDRX,TSSB)) |
| 80ms＜Max(TDRX, TSSB) ≤ 160 ms | Max(50, Ceil(7.5 ´ P) ´ Max(TDRX,TSSB)) |
| Note 1: TSSB is the periodicity of SSB in the set . TDRX is the DRX cycle length and no longer than 80ms. Note 2: K1 is the relaxation factor. K1 = 2 for 40ms＜Max(TDRX, TSSB) ≤ 80 ms, K1 = 4 for Max(TDRX, TSSB) ≤ 40 ms Note 3: K3 is the relaxation factor for the lower bound. K3 = K1, if 1 < K1 ≤ 2; K3 = 1 otherwise. |  |

Table 8.5.2.4-2: Evaluation period TEvaluate_BFD_SSB_Relax for FR2

| Configuration | TEvaluate_BFD_SSB_Relax (ms) |
| --- | --- |
| Mas(TDRX, TSSB) ≤ 80 ms | Max(50 ´ K4, Ceil(7.5 ´ K2 ´ P ´ N) ´ Max(TDRX,TSSB)) |
| 80ms＜Max(TDRX, TSSB) ≤ 160 ms | Max(50, Ceil(7.5 ´ P´ N) ´ Max(TDRX,TSSB)) |
| Note 1: TSSB is the periodicity of SSB in the set . TDRX is the DRX cycle length and no longer than 80ms. Note 2: K2 is the relaxation factor. K2 = 2. Note 3: K4 is the relaxation factor for the lower bound. K4 = K2, if 1 < K2 ≤ 2; K4 = 1 otherwise. |  |

Similar requirements exist for TRP specific SSB beam failure detection in clause 8.18.2.

SSB based candidate beam detection

According to clause 8.5.5.2 for SSB based candidate beam detection requirements, the evaluation period TEvaluate_CBD_SSB for the active serving cell (including active SCell) depends on periodicity of SSB (TSSB) in the set , configured for CBD, except for DRX cycles > 320ms, as shown below for FR1 and FR2 for serving cell.

Table 8.5.5.2-1: Evaluation period TEvaluate_CBD_SSB for FR1

| Configuration | TEvaluate_CBD_SSB (ms) |
| --- | --- |
| non-DRX, DRX cycle ≤ 320ms | Max(25, Ceil(3 ´ P ´ PCBD) ´ TSSB) |
| DRX cycle > 320ms | Ceil(3 ´ P ´ PCBD) ´ TDRX |
| Note: TSSB is the periodicity of SSB in the set . TDRX is the DRX cycle length. |  |

Table 8.5.5.2-2: Evaluation period TEvaluate_CBD_SSB for FR2

| Configuration | TEvaluate_CBD_SSB (ms) |
| --- | --- |
| non-DRX, DRX cycle ≤ 320ms | Max(25, Ceil(3 ´ P ´ N ´ PCBD) ´ TSSB) |
| DRX cycle > 320ms | Ceil(3 ´ P ´ N ´ PCBD) ´ TDRX |
| Note: TSSB is the periodicity of SSB in the set . TDRX is the DRX cycle length. |  |

Similar requirements exist for TRP specific SSB based candidate beam detection in clause 8.18.5.

L1-RSRP measurements

According to clause 9.5.4.1 for SSB based L1-RSRP reporting requirements, the SSB based L1-RSRP measurement period depends on periodicity of SSB (TSSB), as shown below for FR1 and FR2 for serving cell.

Table 9.5.4.1-1: Measurement period TL1-RSRP_Measurement_Period_SSB for FR1

| Configuration | TL1-RSRP_Measurement_Period_SSB (ms) |
| --- | --- |
| non-DRX | max(TReport, ceil(M*P)*TSSB) |
| DRX cycle ≤ 320ms | max(TReport, ceil(K *M*P)*max(TDRX,TSSB)) |
| DRX cycle > 320ms | ceil(M*P)*TDRX |
| Note 1: TSSB = ssb-periodicityServingCell is the periodicity of the SSB-Index configured for L1-RSRP measurement. TDRX is the DRX cycle length. TReport is configured periodicity for reporting. Note 2: K = 1 when TSSB ≤ 40 ms and highSpeedMeasFlag-r16 or highSpeedMeasCA-Scell-r17 are configured; otherwise K = 1.5. Note 3: When highSpeedMeasFlag-r16 is configured, the requirements apply only to UE supporting either measurementEnhancement-r16 or intraNR-MeasurementEnhancement-r16. or measurementEnhancementCA-r17 |  |

Table 9.5.4.1-2: Measurement period TL1-RSRP_Measurement_Period_SSB for FR2

| Configuration | TL1-RSRP_Measurement_Period_SSB (ms) |
| --- | --- |
| non-DRX | max(TReport, ceil(M*P*N)*TSSB) |
| DRX cycle ≤ 320ms | max(TReport, ceil(1.5*M*P*N)*max(TDRX,TSSB)) |
| DRX cycle > 320ms | ceil(1.5*M*P*N)*TDRX |
| Note: TSSB = ssb-periodicityServingCell is the periodicity of the SSB-Index configured for L1-RSRP measurement. TDRX is the DRX cycle length. TReport is configured periodicity for reporting. |  |

Table 9.5.4.1-2A: Measurement period TL1-RSRP_Measurement_Period_SSB configured with groupBasedBeamReporting-r17 for FR2-1

| Configuration | TL1-RSRP_Measurement_Period_SSB (ms) |
| --- | --- |
| non-DRX | max(TReport, ceil(M*P*N)*TSSB) |
| DRX cycle ≤ 320ms | max(TReport, ceil(1.5*M*P*N)*max(TDRX,TSSB)) |
| DRX cycle > 320ms | ceil(1.5*M*P*N)*TDRX |
| Note1: TSSB = ssb-periodicityServingCell is the periodicity of the SSB-Index configured for L1-RSRP measurement. TDRX is the DRX cycle length. TReport is configured periodicity for reporting. |  |

For other requirements, e.g. high-speed channels, the same dependency is observed.

Other L1-RSRP measurement dependent procedures

Other L1-RSRP measurement dependent procedures are also impacted from SSB periodicity adaptation, as listed below.

- Measurement restriction for CSI-RS and SSB for L1-RSRP measurement (clause 9.5.5)

- The UE is required to perform the SSB and CSI-RS measurements with measurement restrictions. If CSI-RS and SSB are in the same symbol, there is an overlap of SSB and CSI-RS reference symbols.

- Scheduling availability of UE during L1-RSRP measurement (clause 9.5.6)

- The UE is performing L1-RSRP measurement on serving cell and receiving PDCCH/PDSCH from serving cell and/or cell(s) with different PCI.

Although SSB periodicity adaptation will impact both measurement restriction and scheduling availability performance due to the SSB frequency, there is no specification impact.

L1-SINR measurements

For SSB based L1-SINR measurements, no evaluation period requirements exist in TS 38.133, but requirements related to measurement restriction (clause 9.8.5) in case of overlap of SSB and CSI-RS reference symbols and related to scheduling availability of UE during L1-SINR measurement (clause 9.8.6), similar as for L1-RSRP measurement. Thus, as for L1-RSRP measurement, although SSB periodicity adaptation will impact both measurement restriction and scheduling availability performance due to the SSB frequency, there is no specification impact.

TCI state switching delay

- Active downlink TCI state switching delay (e.g. clause 8.10.3)

  - The main impact due to SSB adaptation to DL TCI state switch delay is identified due to the parameter Tfirst-SSB being the time to first SSB transmission after MAC-CE / RRC based TCI state switch command has been received and decoded by the UE. This time will be extended or shortened due to longer or shorter SSB periodicity, respectively, provided the RS is SSB.

- Active uplink TCI state switching delay (e.g. clause 8.16.3)

  - The main impact from SSB adaptation to UL TCI switch delay is for unknown target TCI state due to L1-RSRP measurement. In particular, the UE shall be able to transmit uplink signal with the target TCI state within the TCI state switching delay, which depends for FR2 on the L1-RSRP measurement time for Rx beam refinement, where L1-RSRP may be based on SSB.

Proposal 4: 	RAN4 to consider the L1 measurement impact from SSB periodicity adaptation both to L1-RSRP measurement delay for active SCells for BFD and CBD, and to TCI state switching delay.

  - Transition requirements

RAN4#112bis discussed the following open issue.

Issue 1-2-5: L3 requirements for “transition period” - Proposals - Option 1: For the measurement requirement during SSB periodicity transitions period, the legacy transition requirement can be applied. (CMCC, CTC) - Option 2: For SSB periodicity adaptation, RAN4 further check whether existing transition period requirements for L3 measurement can be re-used for BFD and L1/L3 measurements. (Vivo) - Option 2a: RAN4 to investigate whether a transition period with no or relaxed RRM requirements can serve as starting point when changing SSB burst periodicity for defining the RRM impact. (Nokia) - Option 3: When the UE transitions when SSB periodicity changes, the L1/L3 measurement requirements apply based on the longer delay before or after the transition. (MTK) - Option 4: In L3 measurement, RAN4 to wait other WGs’ input on whether new SMTC after SSB adaptation will be applied. (Ericsson) - Option 5: (OPPO) - don’t support L3 measurement after the SCell has been activated without relying on SMTC - RAN4 not to introduce new SMTC due to SSB adaptation - Option 5a: For L3 measurement in clause 9, UE is still expected to perform measurement based on configured SMTC. (Huawei) - Option 6: Deprioritize the minimum requirement at transitions for L3 measurement until RAN1 has further progress on SMTC configuration. (CATT) - Option 7: When defining the requirement with adaptation SSB, RAN4 needs only consider the case in which there is no any SSB adaptation period changed within one reporting interval. (Xiaomi) - Option 8: The impact caused by the SSB adaptation in time domain is to identify the relevant L1/L3 measurement requirements based on the adapted SSB periodicity. (ZTE) - Moderator: - Based on the proposals above, the first question needs to be discussed is whether L3 measurement is still based on configured SMTC with SSB adaptation. - Recommended WF: - Discuss following question: - Q1: Whether L3 measurement is still based on configured SMTC with SSB adaptation. - Q1-1: If the answer is “yes”, whether RAN4 needs to discuss new SMTC for SSB adaptation? - Q1-2: If the answer is “NO”, what is the detailed UE measurement behaviour?

Due to the change in SSB burst periodicity, SSB measurements consisting of a number of measurement samples will be faster or slower for shorter or longer SSB burst periodicity, respectively. Thus, there is an impact on L3 measurements due to SSB burst periodicity adaptation, e.g. whether or not to combine measurement samples stemming from different SSB burst periodicities to generate a L3 measurement. Thus, RAN4 needs to investigate whether a transition period is needed when changing SSB burst periodicity, where no or relaxed RRM requirements are specified. Assuming such transition period can be a starting point for defining the RRM impact.

Proposal 5: 	RAN4 to investigate whether a transition period with no or relaxed RRM requirements can serve as starting point when changing SSB burst periodicity for defining the impact to L3 measurements.

- At RAN4#112bis, RAN4 sent LS to RAN2 [5] asking whether it is foreseen to define a new SMTC configuration for the purpose of SSB adaptation. Whilst RAN2 is in charge of responding on this matter, we see a benefit in defining one additional SSB periodicity in the same SMTC configuration, which can be indicated to UE. In this case, either RRC or MAC/CE or DCI based signalling can be used to activate SSB reception with new SSB periodicity at UE side taking into account the SSB adaptation delay as discussed in the next section.

  - SSB adaptation delay requirements

RAN4 #112bis did not discuss the following issue [3].

Issue 1-2-7: Other requirements impacts by SSB adaptation - Proposals - Option 1: RAN4 to consider the impact of processing time for decoding the updated SSB burst periodicity. (Nokia) - Recommended WF: - Discuss above proposal

In fact, the UE needs to process the signaling message, depending on the signaling method, which will be defined by RAN1 and RAN2 (e.g., based on DCI, MAC-CE or RRC). Hence some processing time needs to be defined, until the UE is ready to receive SSBs with the adjusted SSB burst periodicity, which can identify the SSB adaptation delay.

Proposal 6: 	RAN4 to consider the impact of processing time for decoding the updated SSB burst periodicity to be ready to receive the SSBs with the adjusted SSB burst periodicity, identified as the SSB adaptation delay.

- Conclusion

In this contribution, as part of the objective on “adaptation of common signal/channel transmissions” of the Rel-19 WID [1] to further enhance network energy savings for NR, we discussed scenarios for investigating the RRM impact from SSB periodicity adaptation in time domain based on based on RAN1/2 progress and RAN4#112 agreements and presented our views on the potential RRM impact.

Following observations are made.

Observation #1: Options 1 and 2 of the RAN1#117 agreement are discussing the alternating between different SSB periodicities and the operation of up to two SSB configurations, respectively.

Observation #2: For Option 1, only one SSB pattern is active at a time, and this may have different SSB burst periodicities.

Observation #3: Option 4 of the RAN1#116 agreement is intended for Case-2 with always-ON periodic SSB transmission in SCell, where the NW triggers on-demand denser SSB transmission with shorter SSB periodicity from time instance A, i.e. when NW sends SCell activation command, and later NW starts sending SSB with longer periodicity after time instance B, i.e. when NW has received UE measurement report.

And following proposals are made.

Proposal 1: 	RAN4 to start investigation of RRM impacts for SSB adaptation in time domain for a single active SSB pattern, due to usage of different SSB burst periodicities. Further scenarios can be considered based on further RAN1 agreements.

Proposal 2: 	RAN4 to maintain the specified range of SSB burst periodicity (5ms to 160ms) for SSB periodicity adaptation.

Proposal 3: 	RAN4 to consider the L3 measurement impact from SSB periodicity adaptation to NR intra-/inter-frequency cell detection delay and time index delay as well as to NR intra-/inter-frequency measurement period, i.e. the impact to configured SMTC period in case of active SCell.

Proposal 4: 	RAN4 to consider the L1 measurement impact from SSB periodicity adaptation both to L1-RSRP measurement delay for active SCells for BFD and CBD, and to TCI state switching delay.

Proposal 5:	RAN4 to investigate whether a transition period with no or relaxed RRM requirements can serve as starting point when changing SSB burst periodicity for defining the impact to L3 measurements.

Proposal 6: 	RAN4 to consider the impact of processing time for decoding the updated SSB burst periodicity to be ready to receive the SSBs with the adjusted SSB burst periodicity, identified as the SSB adaptation delay.

- References

[1] 	RP-242354, “Revised WID: Enhancements of network energy savings for NR”, Ericsson

[2]	R4-2416860, “WF on RRM requirements for Netw_Energy_NR_enh_Part2”, Huawei

[3]	R4-2415663, “Topic summary for [112bis][216] Netw_Energy_NR_enh_Part2”, Moderator (Huawei)

[4]	TR 38.864, “Study on network energy savings for NR“, V18.1.0

[5]	R4-2416911, “LS on SSB adaptation”, RAN4, RAN4#112bis