3GPP TSG-RAN WG4 Meeting #113	R4-2419327

Orlando, Florida, USA, November 18 – 22, 2024

Source:	Nokia

Title:	Discussion on XR ph3 RRM requirements

Agenda item:	7.25.2

Document for:	Discussion

Introduction

In this paper we present Nokia’s views on RRM requirements for XR enhancements [1]. As part of this discussion, we highlight the deployment scenarios that we expect to be important for XR enhancements, which types of measurements to be considered and the potential benefits of UAI. The discussion on the Reply LS to RAN1 is treated in [7].

RAN4 #112bis further discussed measurement and timing related aspects for XR enhancements based on the topic summary [2], achieved agreements in the WF [3] and identified open issues, which are treated in the following subsections [3].

Discussion highlights

The main contributions of this paper are:

- We demonstrated that UAI related to channel occasions cannot be determined by the UE in a reliable manner for non-serving cell measurements, and made a proposal based on s-MeasureConfig.

- We demonstrated that DCI-based approach provides better capacity gains, which increase with small time offset, and proposed to follow RAN1 decision on Alt 1 with a small time offset for increased performance.

- We made proposals to help focus the work, e.g. focus only on L3 measurements with gaps, and discuss based on Type 1 gaps before defining requirements for other types of measurement gaps.

- We considered the RRM impact due to skipping of measurement gap/scheduling restriction in case of CA configuration.

Scenarios for measurement skipping

Deployment scenarios

RAN4 #112bis achieved following agreements in the WF related to deployment scenarios [3].

Topic #1: Scenarios for measurement skipping ## Issue 1-1: Deployment scenarios < Agreement>: - Frequency ranges - Agree on FR1 and FR2-1 - Scenarios - Start the discussion for - NR-SA - Carrier aggregation

In addition to the agreement, the following issue was left for discussion in the agreed WF [3]:

## Issue 1-1: Deployment scenarios <Way forward>: - Further discussion needed for - FR2-2 - NR-DC, EN-DC, NE-DC

RAN4 #112bis agreed to consider measurement skipping for XR for FR1 and FR2-1. As already discussed in [4], for FR2-2, the beam sweeping scaling factor is extended to 12 instead of 8, and there is an additional scaling factor of KFR=2 and KFR=3 for 480 kHz and 960 kHz, respectively. As a result, the measurement delay in FR2-2 is 1.5x, 3x and 4.5x times larger than the delay for FR2-1 for 120 kHz, 480 kHz and 960 kHz SCS respectively. Additionally, FR2-2 has shorter slot durations and larger bandwidth, and as a result, measurement skipping feature is not as needed as in other scenarios, since there are more transmission opportunities within the packet delay budget of XR services than longer slot durations. Considering the long measurement delays for FR2-2 we believe it is best to avoid further extensions that would be resulting from measurement skipping.

FR2-2 measurement delay requirements are 1.5x, 3x and 4.5x longer than FR2-1 measurements for 120 kHz, 480 kHz and 960 kHz SCS.

RAN4 to not apply measurement skipping for FR2-2 measurements.

RAN4 #112bis agreed to consider SA and CA deployment scenarios for XR enhancements in RRC_CONNECTED state. The support of dual connectivity (NR-DC, EN-DC, NE-DC) deployment scenarios firstly is not stated in the WID [1], and furthermore would increase complexity of the RRM measurement effort during XR operation, hence XR service quality likely is negatively impacted. Given the Rel-19 timeline and the small size of 0.5 TU for the XR enhancements work in RAN4, RAN4 should refrain from considering dual connectivity deployment scenarios for XR enhancements.

RAN4 to not consider dual connectivity deployment scenarios for XR enhancements.

Types of measurements

In the RAN4 #112bis meeting, the following issue was left for discussion in the agreed WF [3]:

## Issue 1-2: Types of measurements to be considered <Way forward>: - Option 1: Agree at least on the following and further discuss other scenario - Intra-frequency - Inter-Frequency - LTE inter-RAT - Option 2: Agree at least on the following and further discuss other scenario - Intra-frequency - Inter-Frequency - LTE inter-RAT - XR requirement do not apply for UE configured to measure UTRAN FDD for SRVCC or 2G/3G for EN-DC/NE-DC - Option 3: include all related scenarios

TS 38.133 specifies in clause 9.4.1 the following:

### 9.4.1 Introduction The requirements in this clause are specified for NR−E-UTRAN FDD and NR−E-UTRAN TDD measurements and are applicable without an explicit E-UTRAN neighbour cell list containing physical layer cell identities, for a UE: - in RRC_CONNECTED state, and - configured - with SA or NR-DC operation mode or configured in NE-DC operation mode by PCell with NR-E-UTRAN FDD or TDD measurement (RSRP, RSRQ, RS-SINR, RSTD, or E-CID RSRP and RSRQ) on E-UTRA non-serving frequency carrier, or - with SA operation mode on NR carrier frequencies with CCA by PCell with NR-E-UTRAN FDD or TDD measurement (RSRP, RSRQ, RS-SINR) on E-UTRA non-serving frequency carrier, and - configured with an appropriate measurement gap pattern according to Table 9.1.2-3.

Thus, according to TS 38.133, inter-RAT E-UTRAN measurements are configurable in SA deployments and hence applicable to enhanced XR operation. On the other hand, inter-RAT 3G and 2G measurements are only applicable for SRVCC or EN-DC/NE-DC scenarios, respectively, which both are not in the deployment scenario focus of XR enhancements.

RAN4 to consider following RRM measurement types for XR enhancements

  - Intra-frequency

  - Inter-frequency

  - LTE inter-RAT

  - RRM requirements for XR enhancements do not apply for UE configured to measure UTRAN FDD for SRVCC or 2G/3G for EN-DC/NE-DC.

L3 Measurements without gaps with scheduling restrictions

In the RAN4 #112bis meeting, the following issue was left for discussion in the agreed WF [3]:

## Issue 1-3: L3 Measurements without gaps with scheduling restrictions <Way forward>: - Option 1: Support skipping of measurements without gaps - Option 2: Measurement skipping does not apply for measurements without gaps

As for measurements without gaps, they may cause scheduling restrictions in some situations. One typical situation for scheduling restrictions of measurements without gaps is when performing measurements in FR2. In that case the UE is expected to beam sweep during L3 measurements, and the beams will in most cases not coincide with beams for data reception. For this reason, scheduling restrictions are needed for FR2 measurements without gaps.

For measurements without gaps, another aspect to consider is whether interruptions are needed or not. Inter-frequency measurements without gaps can typically be performed using a spare RF chain present in the UE. When this RF chain is turned on, or retuned, it may cause some internal UE interference that causes data loss in the main RF chains used for data reception/transmission. This is referred to as interruptions in RAN4 RRM requirements.

Interruptions will cause the UE to lose some data/control before and after SMTC occasions to be measured. This will typically be more problematic for XR traffic than for normal eMBB traffic. The first reason for interruptions being more damaging to XR traffic is that losing data/DCI scheduling data will cause extra delay which can be large in relation to the total delay budget available for the UE.

Another reason why interruptions need to be avoided for XR traffic is that they may cause the UE to miss commands for measurement skipping. If DCI command containing that command is missed, that network could still think that the UE is available during the SMTC occasions, and many data transmissions during that time would be missed. That would further increase the XR data reception latency by the duration of the SMTC occasion. In particular, only the missing HARQ feedback can help the network in detecting the DCI command loss due to interruption. In TDD configuration, the detection may happen beyond the delay budget for the XR data transmission.

Some measurements without gaps scenarios cause interruptions. Interruptions will cause scheduling problems for latency sensitive traffic.

RAN4 has defined RRM requirements for interruptions for measurements without gaps in Rel-18 NR_MG_enh. During this work item, it was defined that interruptions are not restricted to a location. That means that although interruptions are expected to happen close to a SMTC to measure, there is no requirement of how close they should be. During this discussion, some companies pointed out that it was not clear whether Rel-16 UEs supporting NeedForGaps-r16 are causing interruptions.

If measurement skipping requirements are to be defined for measurements without gaps with interruptions, it is important that some method is defined in order to limit the interruptions over XR traffic. As an example a window where interruption is avoided could be defined. This would avoid the problems described above.

Requirements for measurements without gaps with interruptions using NeedForInterruptions-r18 do not limit interruption location in time.

Measurements without gaps based on NeedForGaps-r16 do not have interruption requirements and restrictions.

Measurement skipping does not apply for measurements without gaps.

L1 measurements and L1-L2 triggered mobility measurements

In the RAN4 #112bis meeting, the following issue was left for discussion in the agreed WF [3]:

## Issue 1-4: L1 measurements with scheduling restrictions and procedures <Way forward>: - Option 1: Support skipping of L1 measurements and procedures such as RLM, BFD, L1-RSRP etc - Option 2: Measurement skipping does not apply for L1 measurements and procedures ## Issue 1-5: L1/L2 triggered mobility measurements and procedures <Way forward>: - Option 1: support skipping of L1 measurements and procedures for LTM within and outside gap - Option 2: measurement skipping do not apply for L1 measurements and procedures for LTM

L1 measurements, such as L1-RSRP and L1-SINR, are for serving cell only and refer to RLM, BFD and CBD procedures. Skipping of these measurements will increase the measurement delay and hence may yield a degradation of these procedures. The operation of these procedures should be agnostic of the XR operation. Thus, these L1 measurements should not be subject to measurement skipping during XR operation.

Measurement skipping does not apply for L1 measurements and procedures.

As for L1 measurements above, the operation of the LTM procedure should be agnostic of the XR operation. Thus, these L1/L2 triggered mobility measurements should not be subject to measurement skipping during XR operation.

Measurement skipping does not apply for L1 measurements and procedures for LTM.

Measurements with gaps

In the RAN4 #112bis meeting, the following issue was left for discussion in the agreed WF [3]:

## Topic #2: Measurements with gaps ## Issue 2-1: General approach for defining requirements for measurements with gaps <Way forward>: - Option 1: RAN4 to deprioritize the discussion on applicable type of MGs at early stage of the WI, and focus on the solutions/impacts on RRM requirements. Unless, it is found that a particular type of MG cannot be supported or need special treatment, RAN4 can discuss whether the feature can apply to this MG type. - Option 2: As a baseline RAN4 to define requirements due to measurement skipping based on Type 1 measurement gap. Other gaps are FFS.

As part of the RAN4 #112 and RAN4 #112bis discussions, it was clear that for many companies, Type 1 MG can be treated with higher priority in this work item.

From our perspective, RAN4 can start by agreeing on studying the requirement impact for Type 1 MG, i.e. Rel-16 MGs. This is the clearest and most general case for the measurement skipping scenario. Additionally, once the requirement impact for Type 1 MG is identified, we can discuss how simple it is to extend and apply the same principles for other types of measurement gaps.

As a baseline RAN4 to define requirements due to measurement skipping based on Type 1 measurement gap. Other gaps are FFS.

Impact of measurement skipping

How to capture the impact of measurement skipping

In the RAN4 #112bis meeting, the following issue was left for discussion in the agreed WF [3]:

Topic #3: XR skipping requirements impact ## Issue 3-1: How to capture impact of measurement skipping <Way forward>: - Option 1: - As minimum requirements, cell detection, index reading, and measurement period is extended. - FFS how to extend - Reuse existing measurement accuracy requirements. - Option 2: - For the cases where measurement impact is needed - As minimum requirements, cell detection, index reading, and measurement period for NR cells is extended. - FFS how to extend. - Reuse existing measurement accuracy requirements. - For the cases where measurement impact is not needed - FFS the feasible conditions and necessary UE assistance information.

As part of the Rel 19 discussion, RAN4 has to conclude on the impact that measurement skipping might have on RRM requirements. In order to analyze the impact of measurement skipping we are going to compare it with what RAN4 has already specified in previous releases.

The effect of measurement skipping can be compared to the effect of CCA failures in Rel-16 NR_unlic. In unlicensed bands all devices transmitting need to perform a listen-before-talk (LBT) procedure, or clear channel assessment (CCA). That is also applicable for gNB transmitting SSB. As a result, it is possible that the channel is not available when an SSB occasion is to be transmitted. The impact of the gNB not transmitting SSB was captured in NR_unlic by extending the measurement delay, i.e. the UE is expected to identify that the SSB sample is missing and update the L3 filter only when the next valid SSB sample is available. In the case of NR_unlic the CCA failure is out of network control, since it is caused by the environment. On the other hand, the XR measurement skipping is under network control, and known to both UE and network by the DCI signaling. However, from RRM requirement perspective there is little difference on the measurement impact due to CCA failure or DCI-based skipping. Therefore, we propose that this is captured as an extension of the measurement delay, and that no changes in existing accuracy requirements is needed.

In Rel-16, RAN4 defined requirements for NR_unlic, where RRM measurement delay is extended due to CCA/LBT failures without impact to measurement accuracy.

As minimum requirements, cell detection, index reading, and measurement period is extended due to measurement skipping.

  - FFS how to extend

  - Reuse existing measurement accuracy requirements.

Additionally, we also have to consider the case where multiple frequency layers are measured. When multiple layers are configured, the current RAN4 assumption is that the UE is using a single searcher for measurements with gaps, and a single frequency layer is measured at every given measurement occasion. In some network deployments, the importance of certain frequencies is no equal in relation to others. Therefore, it is desirable that the measurement skipping is not affecting equally the measurement delay for all the frequency layers. This can be achieved by the UE prioritizing higher priority frequency layers to be measured first in case on measurement gap is skipped, while low priority gaps may be measured on remaining gap occasions and experience longer measurement delays.

RAN4 to define RRM requirements such that the network can configure which frequency layers are measured with higher priority.

Principles for measurement delay calculation

In the RAN4 #112bis meeting, the following issue was left for discussion in the agreed WF [3]:

## Issue 3-2: Principles for measurement delay calculation <Way forward>: - Option 1: Follow the scaling factor (Ntotal/Navailable) from Concurrent MG spec to extend the total measurement delay. - Option 2: Follow NR-U like approach to extend the delays by the number of MGs skipped during the measurement period. - Other options are not precluded

In our view, option 1 is clearer than option 2 and is appropriate during XR operation. For measurement delay calculation, the number of available measurement gaps after skipping have to be compared against the number of total gaps before skipping.

Follow the scaling factor (Ntotal/Navailable) from Concurrent MG spec to extend the total measurement delay.

Conditions for measurement skipping

In the RAN4 #112bis meeting, the following issue was left for discussion in the agreed WF [3]:

## Issue 3-3: Conditions for measurement skipping <Way forward>: - Proposal 1: Define a max number of occasions cancelled during a measurement period. - Proposal 2: RAN4 to discuss which conditions the measurement gap configurations need to meet to be relevant for the UE XR operation, e.g., - minimum MGL, - maximum MGL, - maximum MGRP, etc. - Proposal 3: RAN4 to discuss the impact on RRM samples and how they are combined during the UE XR operation, e.g., maximum acceptable time separation between two measurement occasions available for RRM measurements. - Proposal 4: Other conditions.

In this issue few companies proposed that a limitation condition where measurement skipping may be used. These included maximum number of occasions to be skipped or measurement gap configurations.

Regarding proposal 1 of issue 3-3, we understand that some companies have concerns on potential impacts if measurement skipping exceeds a certain limit. However, it is not very clear how to define a limit to measurement gap skipping that is general enough to account for all possible scenarios.

- Example 1: 2 different networks configure the UE to monitor different number of frequency layers:

  - Network 1: UE only monitors 1 frequency layers, and CSSF = 1

  - Network 2: the UE monitors 4 frequency layers, and CSSF = 4

  - Measurement period calculation Max(200ms, Ceil(8 * Kgap) × Max(MGRP , SMTC period)) × CSSFinter with Kgap = 1

| RRM parameter | Network 1 | Network 2 |
| --- | --- | --- |
| T_SSB | 40 | 40 |
| MGRP | 40 | 40 |
| CSSF inter | 1 | 4 |
| Measurement period | 320 | 1280 |

  - In this example the impact of measurement skipping is not the same for Network 1 and Network 2.

    - If a UE in network 1 skips 75% of the gaps, it should be capable of achieving the same measurement delay as the UE in network 2.

Therefore, we think that it is natural that in the network deployment there is a limitation on the maximum number of skipping. However, this is an optimization that can be determined case by case, for example considering the typical mobility of XR UEs in a given location, the ISD, or layers to monitor. If we define a limit on the specification we will simply limit available options on the deployment and the benefit of the feature will be impacted.

When considering proposal 2, it is not clear what is the relationship between the measurement gap length and the impact to measurement skipping. We understand that the measurement skipping will have a direct impact on measurement delay requirements, however those requirements do not depend on the measurement gap length.

Limiting the maximum number of occasions that can be skipped will have an impact on the network deployment flexibility.

No conditions are needed for measurement skipping regarding maximum number of occasions or MG configuration.

Time offset for measurement skipping

In the RAN4 #112bis meeting, the following issue was left for discussion in the agreed WF [3]:

## Topic#4: Time offset for measurement skipping processing ## Issue 4-1: Time offset requirement for measurement gap skipping <Way forward>: - Option 1: 7 ms - Option 2: 5 ms - Option 3: DCI decoding time defined in TS 38.214 + [5]ms - Option 4: assume subclause 5.3.1 of TS38.214 as a baseline for X offset. - Option 5: Fraction of a slot - Option 6: besides the DCI decoding delay, no additional processing delay is needed. - Option 7: N2 symbols, which is to follow the same approach for Rel-17 positioning about the time offset between UE determination of the presence of DL channels and DL PRS processing window (N2 is about UE PUSCH preparation procedure time, which is defined in Clause 6.4, TS38.214)

As mentioned, RAN1 has agreed to adopt Alt 1-1 solution (i.e., explicit dynamic skipping indication using DCI signalling) where the gNB sent command(s) to the UE to skip RRM measurements occasions for the sake of having the UE prioritize PDCCH/PDSCH decoding and/or PUCCH/PUSCH transmissions. Alt 1-1 relies on the gNB sending a skipping indication command with DCI on PDCCH. For this solution, the UE will naturally need time for decoding the DCI with skipping indication command, and secondly to act on the command to actually skip the RRM measurement. The current 3GPP specs already include requirements for a UE to decode a DCI in 3GPP TS 38.214. For cases where the DCI include a scheduling grant (as is also being the default assumption in RAN1 for carrying skipping indication), the current specs define processing times that include both DCI and decoding PDCSH, or PUSCH preparation. We suggest relying on these existing processing values also for a UE to decode a DCI that carries skipping indication commands, i.e., the UE processing time for decoding a DCI with RRM measurement skipping indication should be the same as the currently defined processing requirements in 3GPP TS 38.214 for decoding of DCI and PDSCH decoding, or PUSCH preparation.

Once the UE has realized that it has received a RRM measurement skipping indication command, it needs time to react on this. This time could be rather short as the UE in principle only needs to continue normal operation (i.e. listening to DCI and receiving PDSCH / preparing PUSCH transmissions as per the received scheduling commands). Thus, it does, for instance, not involve any RF retuning or alike actions. In setting the UE requirements for acting on reception of a RRM measurement skipping indication command, we could get inspiration from alike UE behaviours of other commands carried by DCI that instruct the UE to take actions. Among others, those include a power control command received on DCI, a command for a UE to cancel an ongoing PUSCH transmission (aka as cancellation indication). Common for those cases is that the UE react rather fast and implement the action received on the DCI in less than a slot time.

Once the UE has realized that it has received a RRM measurement skipping indication command, the UE only needs to continue normal operation without executing complex RF retuning and gap activation procedures.

Additionally, we have provided simulation results in annex A of this contribution that show the benefits of reduced time between DCI indication and gap to be skipped. When comparing the results of Tsrt=1 ms and Tsrt=5 ms maximum interval between DCI and gap to be skipped, the results with Tsrt=1 ms could achieve improved XR capacity. In one example when comparing (Tp= 6ms;Tsrt= 5ms) and (Tp=4 ms; Tsrt=1 ms) the same skipping ratio is observed. However, even though the skipping ratio is the same for those examples, the configuration with Tsrt=1 ms provided 1 additional user in relation to Tsrt=5 ms. That indicates that a shorter timeline will enable better decisions on which gaps to skip, which will translate in overall better user experience. This also shows that smaller Tsrt enables reduced skipping ratio for the same XR capacity.

Shorter time offset between DCI skipping command to measurement gap enables better XR capacity for the same skipping ratio.

Larger timing offset requires increased skipping ratio to provide the same user happiness which could result in mobility problems.

We therefore suggest the following:

The UE shall be capable of skipping a gap starting at least 1ms after the DCI with RRM measurement skipping indication is received.

Need/feasibility of UAI

UAI related to measurement occasions

In the RAN4 #112bis meeting, the following issue was left for discussion in the agreed WF [3]:

## Topic#5: Need/feasibility of UE assistance information ## Issue 4-1: General on UAI <Way forward>: - Companies are tasked to discuss the need and feasibility of UAI, based on the following options - Option 1: UAI is specified - Companies supporting this option are encouraged to bring contribution with exact proposals on what information is to be included in UAI. - Option 2: UAI is not specified

RAN1 sent LS to RAN4, requesting RAN4 to consider whether or not to introduce UE assistance information related to measurement occasions [5]. As part of this LS, the RAN1 agreement included FFS points related to measurement occasions, channel conditions, traffic, and UE mobility.

From RAN1 LS, several other FFS points are included related to UE assistance information related to measurement occasions. Those include:

- FFS: UE assistance information related to measurement occasions: - FFS: The number of needed measurement gaps/SMTC with restrictions within a time period; - FFS: The maximum number or ratio of MGs/SMTC with restrictions that can be skipped within a time period; - FFS: The number of required SSBs within a time period; - FFS: The number of consecutive RRM measurements that can be skipped; - FFS: The maximum interval between two consecutively reserved gap/restriction occasions for RRM measurements; - FFS: The patterns of gap(s)/restriction(s) where skipping is feasible or acceptable;

Additionally, it is also included in the RAN1 agreement the following related to gNB behaviour:

Note: From specification point of view, there is no mandated gNB behavior in response to any of the UE assistance information.

The benefit of UAI related to measurement occasions can be potentially useful provided that measurement skipping can be performed without extension of measurement delay. As an example, if the UE indicates that X SSBs are required in  T ms, and the gNB sends skipping commands leaving at least X SSBs in the window of T ms, we would expect that this operation would not cause any extension of the measurement delay.

As part of the FFS points, one many bullets relate to the number of SSBs that need to be available in a time window. The existing measurement delay and accuracy requirements are defined considering a minimum SNR. From that SNR a minimum number of samples for averaging is derived, which result on the accuracy and measurement delay. The measurement requirements also include delay regarding detection of new cells. The UE might know the SNR of cells that were previously measured, and assume a smaller number of samples is needed for those based on previous information. However, it is impossible for the UE to estimate the SNR for cells that are unknown (i.e., never sampled in the past) and not yet detected.

In Figure 1 we show one example where the UE can detect 2 different cells at 2 different time instants. In Figure 1 (a) a blocker is placed such that neighbour 1 is not visible for the UE, while Neighbour 2 can be detected. In the same scenario, if either the UE moves or the environment changes (e.g. the blocker moves) neighbour 1 can be detected while neighbour 2 cannot be detected. If we relate this example to the argument that some companies made that the UE can determine the number of samples based on the SNR, how can the UE determine how many samples it would be needed to detect neighbour 1 between time instants (a) and (b)? In the current specification, the UE is required to be able to identify at least 4 cells for inter-frequency measurements, 8 cells for intra-frequency in FR1, and 6 cells for intra-frequency in FR2.

The UE cannot determine SNR and number of samples needed for a cell that is not yet detected.

| (a) | (b) |
| --- | --- |

Figure 1 Example of UE monitoring serving cell and neighbour cells, where (a) neighbour 1 is not detectable for the UE and neighbour 2 can be detected, and (b) neighbour 1 can be detected and neighbour 2 is not detectable.

Also considering the case where the UE determines the SNR and reports UAI, the UE needs some time for performing measurements before it is possible to determine the SNR. That would mean that if the network configures a gap and UAI at the same time for XR traffic, and if it has to wait the UAI report before starting measurement skipping, there will be a degradation on the user experience for the start of the XR until the UAI report is received.

In another proposal, some companies would like the UE to report which measurement occasions are not used and might be used for measurement skipping. The argument of those companies is that the UE is not always performing measurements on every measurement occasion. Therefore, the occasions that are not measured may be used for measurement skipping with reduced impact to the measurement requirements. One challenge with that approach is that if you have latency sensitive traffic, it is not possible to align that traffic to the measurement occasions that the UE would be reporting using UAI. Figure 2 shows one example where there is an XR traffic with a given pattern that would result in 3 out of 9 gaps to require skipping. If a UE reports that it prefers to skip one of every 3 gaps, the resulting desired skipping pattern is only matching 1 of the gaps that require skipping. Therefore, the information cannot be used by the network in a reliable way to improve the XR used experience.

There is no benefit in the UE indicating preference for measurement skipping that is not matching with the XR traffic.

Figure 2 Example of UE reporting preference for measurement occasions for skipping which are not aligned with XR traffic

Therefore, for the UAI information related to measurement occasions, RAN4 has to provide analysis that UAI is beneficial and feasible. Moreover, UAI is optional and network-controlled behavior is required, as the network has best knowledge of both mobility performance and XR traffic quality.

Do not define UAI related to measurement occasions.

Configuration of threshold s-MeasureConfig

In relation to channel conditions, the RAN1 agreement includes an FFS point on s-MeasureConfig. The network can configure the UE with a search threshold (s-MeasureConfig, see below text from TS 38.331) for a UE in connected mode to enable reduction of the measurement effort.

s-MeasureConfig Threshold for NR SpCell RSRP measurement controlling when the UE is required to perform measurements on non-serving cells. Choice of ssb-RSRP corresponds to cell RSRP based on SS/PBCH block and choice of csi-RSRP corresponds to cell RSRP of CSI-RS.

If the network has configured the UE with the s-MeasureConfig condition for conducting RRM measurements, it is important that the network knows if this condition has been fulfilled. This is needed for the network to know if the UE perform neighbour cell measurements (and hence is subject to corresponding scheduling restrictions), or not. Once the s-MeasureConfig condition is fulfilled for a UE, it is suggested that the UE informs the network as it would then need to take corresponding scheduling restrictions into account. In this condition measurement skipping could be increased.

Similarly, for a UE that no longer has its s-MeasureConfig condition fulfilled, and as a consequence may stop conducting RRM measurements, we also suggest that the UE informs the network. This is important for the network to know as it no longer needs to take scheduling restrictions from RRM measurements into accounts (as the UE has stopped such measurements). This leads to:

For UEs configured with search threshold (s-MeasureConfig),

  - the UE shall inform the network that it is stopping doing non-serving cell measurements because the s-MeasureConfig condition is no longer fulfilled.

The basic principle of the solution is as follows and shown in Figure 2. The UE is first configured with the s-MeasureConfig conditions, and asked to report back to the gNB when this condition is fulfilled (e.g. SS-RSRP< s-MeasureConfig) and it starts performing RRM measurements that causes scheduling restrictions. Similarly, when the condition is no longer fulfilled, and the UE is not required to do RRM measurements on neighbouring cells, it shall also report that to the gNB. Thereby, the gNB always knows when the UE is conducting RRM measurements, and hence knows when it can freely schedule the UE, and when it needs to respect certain scheduling restrictions.

Figure 3 Illustration of case where the UE is configured to report when it starts, or stops, conducting RRM measurements as per the s-MeasureConfig condition that results in scheduling restrictions.

We note that the proposals related to UE to network reporting of s-MeasureConfig conditions, has some relations to the already defined RRC Events A1 (Serving cell becomes better than threshold) and A2 (Serving cell becomes worse than threshold). However, while A1/A2 events solely links to serving cell RSRP power levels as compared to a threshold, what we suggest in the listed Proposals links also to UE behaviour of performing RRM measurements (and is related to scheduling restrictions).

Additional skipping schemes

In the RAN4 #112bis meeting, the following issue was left for discussion in the agreed WF [3]:

## Topic#6: Additional skipping schemes ## Issue 6-1: Semi-static schemes <Way forward>: - Option 1: In addition to the dynamic indication scheme to deactivate MGs as agreed by RAN1, support semi-static schemes such as RRC and MAC-CE based signaling as well. Send LS to RAN2 and RAN1. - Option 2: RAN4 also suggest RAN1/2 to introduce the semi-static approach (Alt.3) that NW indicate the indicate occasions where to skip gaps/restrictions, and UE can have a stable measurement pattern/period. - Option 3: To guarantee mobility performance, one potential improvement on dynamic indication by DCI is configuring gaps to be skipped during a time period by RRC in advance and gaps indicated by DCI should be subset of gaps configured by RRC - Option 4: Follow RAN1 working assumption and consider DCI only

In our view, measurement gap skipping should be fully under network control. The network has all information available on mobility performance of the UE as well as on the scheduled data traffic on DL and UL. Hence Alt 1-1 selected by RAN1 based on DCI scheduling is appropriate for this.

The network has information on mobility performance of the UE as well as on the scheduled data traffic on DL and UL.

Semi-static schemes depend on configured measurement objects, i.e. SMTC windows for configured target frequencies and may need to be changed if the UE approaches cell edge. Overall, the performance of semi-static schemes for XR is judged inferior versus the DCI based scheme, as shown in our companion paper R4-2418890.

As shown in Figure 4 and 5 of R4-2418890, the capacity gain of the semi static scheme (Alt-3) in relation to measurements with gaps is very minor in comparison to the dynamic (DCI-based) scheme. This is also reasoned by the fact that Alt-3 is highly depending on prediction (of configured gaps subject to be skipped) and hence generates useless skipping, whilst XR traffic is highly dynamic, has jitter and hence cannot be matched with a predicted traffic pattern. This figure shows that Alt 1-1 approaches the ideal performance with no gaps and no scheduling restrictions.

Our simulations showed for the same skipping ratio only 25% capacity gain for semi-static scheme (Alt 3), whereas 50% to 75% capacity gain was achieved for dynamic schemes (Alt 1-1)

Another problem related to Alt-3 is that it relies on good traffic pattern prediction before the skipping pattern can be configured. Figure 3 shows one example of what happens during the call start of a XR traffic stream. If Alt 3 approach is used as in Figure 3 a), the network needs a training phase in order to determine the XR traffic pattern. This prediction should include parameters such as traffic periodicity, and jitter. Once the prediction is ready at time t1, the network can determine the skipping pattern to be used that matches the XR traffic, and send an RRC command configuring the UE to use that pattern at t2. Therefore, before t2, the UE would experience worst performance related to the XR traffic. On the other hand, Figure 3 b) shows the example of using dynamic approach, as decided by RAN1. When using this approach, the traffic prediction training is not a mandatory step, and the network can start skipping measurement gaps from the start of the XR traffic in t0.

Semi-static schemes are dependent on accurate XR traffic prediction, which may depend on a training stage at XR traffic start, and delay the start of using skipping for XR traffic.

a)

b)

Figure 4 Comparison of XR traffic quality variation over time when using a) semi-static approach (Alt 3) and b) dynamic approach as the decided by RAN1 working assumption (Alt 1-1)

Furthermore, RAN1 has already made a down-selection between DCI and RRC schemes

RAN1 already made down selection between DCI and RRC schemes

Follow RAN1 working assumption and consider DCI scheme only.

CA scheduling restrictions

In RAN1#118bis, RAN1 made following agreements with regard to support of carrier aggregation configuration when XR is operated:

Agreement At least for self-scheduling case, for explicit indication by DCI to skip a particular gap/restriction, one bit indication is included as part of DCI formats 0_1/1_1 and 0_2/1_2. - FFS: Cross carrier scheduling. - FFS: DCI formats 0_3/1_3. Explicit indication can be configured for each DCI format individually by higher layer. Agreement In case of cross carrier scheduling, for explicit indication by DCI to skip a particular gap/restriction, one bit indication included as a part of DCI formats 0_1/1_1 and 0_2/1_2 corresponds to a scheduled cell.

Effectively, the afore RAN1 decision means that the skipping indication carried by the DCI applies to gap/restriction on the scheduled cell i.e. where PDSCH/PUSCH is scheduled, so that UE can conduct reception/transmission on the cell, as illustrated in Figure 3 when carrier aggregation (CA) with two cells has been configured with Component Carriers (CC) A and B.

(a)

(b)

Figure 3. Illustration of DCI based skipping indication behaviour for (a) self-scheduling and (b) cross-carrier scheduling.

Following this RAN1 agreement, RAN4 should investigate scheduling restrictions due to both self-scheduling and cross-carrier DCI scheduling.

RAN4 to investigate scheduling restrictions for CA if DCI skipping indication is sent on one of the CCs.

In context of TS 38.133, the measurement gap can be configured to be one of the three types; per-UE, per-FR1 or per-FR2 (based on UE capability to support). This determines whether the gap applies to cells for a given frequency range (e.g. per-FR1) or to all cells on all frequencies (per-UE). That is, in which cells UE is not required to conduct reception/transmission during the gap based on the measurement gap configuration. From UE RF perspective this can be considered to indicate the dependency/independence of the reception/transmission operation on different carriers/CCs. That is, whether UE receiver shares some components or operations between the different carriers/CCs, e.g. frequency synthesis etc.

Correspondingly for scheduling restrictions within SMTC, there needs to be also consideration by RAN4 for the applicability over different cells based on the frequency domain relation. In short, the scheduling restrictions allowed for the UE apply on all serving cells within same band during the symbols that fully or partially overlap with the restricted symbols. The assumption here is that UE baseband processing, e.g. FFT etc., can be shared over the carriers in the same band so that scheduling restrictions shall be allowed on all ‘intra-band’ cells, which are belonging to the applicable range of carriers.

In RAN1#118bis the implications in context of skipping were discussed, aiming to clarify expected UE behavior when UE is configured with CA and a particular gap/restriction is skipped. As noted above, this could be interpreted that gap/restriction on all cells that fall within the applicable range of carriers, cannot be used by the UE to do measurements (due to assumed sharing of RX components/processes). Effectively, with this assumption, impact on UE measurement requirements should be considered for all cells in the applicable range of carriers, e.g. intra-band cells for scheduling restrictions within SMTC.

If it is assumed that for cells within the applicable range of carriers, UE can share (parts of) receiver over different cells, measurement requirement impact due to skipping of gap/restriction should be accounted for in all those cells.

Furthermore, if afore observation is valid, it should be clarified whether UE is able to conduct reception/transmission in all cells within the applicable range of carriers or only in the scheduled cell to which the indication is given. As noted above, the range of carriers/cells where the gap/restrictions are applied/allowed relates fundamentally to the UE receiver design. For example, UE can be configured with CA covering two cells on two component carriers, CC A and CC B, as illustrated in Figure 4, together with measurement gap configuration so that both cells/carriers fall within the range of applicable carriers where gap/restriction is applied/allowed.

Figure 4. Illustration of DCI skipping indication with multiple cells within applied range of gap/restrictions.

In this scenario, if skip is indicated on a serving cell in CC A, it will result that UE cannot do measurements on CC B, due to dependency in the receiver operation on these carriers/cells. Thus, it should be discussed and clarified whether UE can conduct reception/transmission also on all the carriers/cells that fall within the applicable range of carriers during the skipped gap/restriction.

While measurements are skipped on CC A, UE cannot perform concurrent gap-assisted measurements on CC B due to dependency on common receiver components for a range of carriers.

RAN4 to discuss and clarify whether UE can conduct reception/transmission also on other cells that fall within the applicable range of carriers during the skipped gap/restriction, in addition to the cell to which the skipping was indicated.

Conclusion

In this paper we have presented Nokia’s views on the RAN4 RRM requirements related XR ph3. As part of this discussion, the following Observations and Proposals were made:

Scenarios for measurement skipping

- FR2-2 measurement delay requirements are 1.5x, 3x and 4.5x longer than FR2-1 measurements for 120 kHz, 480 kHz and 960 kHz SCS.

- RAN4 to not apply measurement skipping for FR2-2 measurements.

RAN4 to not consider dual connectivity deployment scenarios for XR enhancements.

RAN4 to consider following RRM measurement types for XR enhancements

  - Intra-frequency

  - Inter-frequency

  - LTE inter-RAT

  - RRM requirements for XR enhancements do not apply for UE configured to measure UTRAN FDD for SRVCC or 2G/3G for EN-DC/NE-DC.

Some measurements without gaps scenarios cause interruptions. Interruptions will cause scheduling problems for latency sensitive traffic.

Requirements for measurements without gaps with interruptions using NeedForInterruptions-r18 do not limit interruption location in time.

Measurements without gaps based on NeedForGaps-r16 do not have interruption requirements and restrictions.

Measurement skipping does not apply for measurements without gaps.

Measurement skipping does not apply for L1 measurements and procedures.

Measurement skipping does not apply for L1 measurements and procedures for LTM.

Measurements with gaps

As a baseline RAN4 to define requirements due to measurement skipping based on Type 1 measurement gap. Other gaps are FFS.

Impact of measurement skipping

In Rel-16, RAN4 defined requirements for NR_unlic, where RRM measurement delay is extended due to CCA/LBT failures without impact to measurement accuracy.

As minimum requirements, cell detection, index reading, and measurement period is extended due to measurement skipping.

- a.	FFS how to extend

- b.	Reuse existing measurement accuracy requirements.

RAN4 to define RRM requirements such that the network can configure which frequency layers are measured with higher priority.

Follow the scaling factor (Ntotal/Navailable) from Concurrent MG spec to extend the total measurement delay.

Limiting the maximum number of occasions that can be skipped will have an impact on the network deployment flexibility.

No conditions are needed for measurement skipping regarding maximum number of occasions or MG configuration.

Time offset for measurement skipping

Once the UE has realized that it has received a RRM measurement skipping indication command, the UE only needs to continue normal operation without executing complex RF retuning and gap activation procedures.

Shorter time offset between DCI skipping command to measurement gap enables better XR capacity for the same skipping ratio.

Larger timing offset requires increased skipping ratio to provide the same user happiness which could result in mobility problems.

The UE shall be capable of skipping a gap starting at least 1ms after the DCI with RRM measurement skipping indication is received.

Need/feasibility of UAI

The UE cannot determine SNR and number of samples needed for a cell that is not yet detected.

There is no benefit in the UE indicating preference for measurement skipping that is not matching with the XR traffic.

Do not define UAI related to measurement occasions.

For UEs configured with search threshold (s-MeasureConfig),

  - the UE shall inform the network that it is stopping doing non-serving cell measurements because the s-MeasureConfig condition is no longer fulfilled.

Additional skipping schemes

The network has information on mobility performance of the UE as well as on the scheduled data traffic on DL and UL.

Our simulations showed for the same skipping ratio only 25% capacity gain for semi-static scheme (Alt 3), whereas 50% to 75% capacity gain was achieved for dynamic schemes (Alt 1-1)

Semi-static schemes are dependent on accurate XR traffic prediction, which may depend on a training stage at XR traffic start, and delay the start of using skipping for XR traffic.

RAN1 already made down selection between DCI and RRC schemes.

Follow RAN1 working assumption and consider DCI scheme only.

CA scheduling restrictions

RAN4 to investigate scheduling restrictions for CA if DCI skipping indication is sent on one of the CCs.

If it is assumed that for cells within the applicable range of carriers, UE can share (parts of) receiver over different cells, measurement requirement impact due to skipping of gap/restriction should be accounted for in all those cells.

While measurements are skipped on CC A, UE cannot perform concurrent gap-assisted measurements on CC B due to dependency on common receiver components for a range of carriers.

RAN4 to discuss and clarify whether UE can conduct reception/transmission also on other cells that fall within the applicable range of carriers during the skipped gap/restriction, in addition to the cell to which the skipping was indicated.

- References

- RP-241771, Revised WID: XR (eXtended Reality) for NR Phase 3, RAN #105, Melbourne, Australia, September 9-12, 2024

- R4-2415667, Topic summary for [112bis][220] NR_XR_Ph3, Nokia, RAN4 #112bis, Hefei, China, October 14 – 18, 2024

- R4-2416864, WF on RRM requirements for XR_Ph3, Nokia, RAN4 #112bis, Hefei, China, October 14 – 18, 2024

- R4-2416468, Discussion on XR ph3 RRM requirements, Nokia, RAN4 #112bis, Hefei, China, October 14 – 18, 2024

- R1-2405736, LS on UE assistance information, RAN4 #112, Maastricht, Netherlands, August 19 – 23, 2024

- R1-2407561, LS on gaps/restrictions that are caused by RRM measurements, RAN4 #112-bis, Hefei, China, October 14 – 18, 2024

- R4-2419326, General aspects for XR enhancements, RAN4 #113, Orlando, USA, November 18-22, 2024

- R4-2418890, XR simulation results, Nokia, RAN4 #113, Orlando, US, Nov 18th – 22nd, 2024