3GPP TSG-RAN4 Meeting # 113 	R4-2419334

# Orlando, US, 18th - 22nd November

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.101-4 | CR | 0696 | rev | - | Current version: | 18.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | CR to 38.101-4: Clean up for the PDCCH requirements for DSS enhancement |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_DSS_enh-Perf |  |  |  |  |  | Date: |  |  | 2024-11-18 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | The CCE to REG mapping type is incorrect in the test parameter table for TDD. There are some typos that need to be fixed. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Correct the CCE to REG mapping type - Correct the typos |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The PDCCH requirements for DSS enhancement is not complete |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.3.2.2, |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | x |  | Test specifications |  |  |  | TS 38.521-4 |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

-------------------------------------------------------------Change 1----------------------------------------------------------------------

#### 5.3.2.2	TDD

The parameters specified in Table 5.3.2.2-1 are valid for all TDD tests unless otherwise stated.

Table 5.3.2.2-1: Test Parameters

| Parameter | Unit | 1 Tx Antenna |  |  | 2 Tx Antenna |
| --- | --- | --- | --- | --- | --- |
| TDD UL-DL pattern |  | FR1.30-1 |  |  |  |
| CCE to REG mapping type |  | Test 3: non-interleaved Other tests: interleaved |  | Test 1 in Table 5.3.2.2.6-2: non-interleaved Other tests: interleaved |  |
| Interleaver size |  | 3 |  |  |  |
| REG bundle size |  | Test 3: 6 Other tests: 2 | Test 1 in Table 5.3.2.2.2-1: 6 Other tests: 2 |  |  |
| Shift Index |  | 0 |  |  |  |

-------------------------------------------------------------End of change 1----------------------------------------------------------------

-------------------------------------------------------------Begin of change 2-------------------------------------------------------------

##### 5.3.2.2.6	Minimum requirements with PDCCH overlapping with LTE CRS

The parameters specified in Table 5.3.2.2.6-1 are additional parameters for requirements with PDCCH overlapping with LTE CRS.

Table 5.3.2.2.6-1: Test Parameters

| Parameter |  | Unit | Value |
| --- | --- | --- | --- |
| TDD UL-DL pattern |  |  | FR1.15-1 |
| NR UL transmission with a 7.5 kHz shift to the LTE raster |  |  | true |
| CCE to REG mapping type |  |  | Non-interleaved |
| REG bundle size |  |  | 6 |
| Shift Index |  |  | 0 |
| PDCCH Configuration | Symbols with PDCCH |  | Symbol#1 and #2 |
| CRS for rate matching (Note 1) | LTE carrier centre subcarrier location |  | Same as NR carrier centre subcarrier location |
|  | LTE carrier BW | MHz | 10 |
|  | Number of antenna ports |  | 4 |
|  | v-shift |  | 0 |
| Note 1: No MBSFN is configured on LTE carrier. Note 2: LTE carrier is configured with Uplink-downlink configuration 2 Table 4.2-2, TS 36.211[15] and Special subframe configuration 7 Table 4.2-1, TS 36.211[15]. The start of transmission of LTE frame is delayed by 2 LTE subframes with respect to the start of transmission of NR frame. Note 3: NR PDCCH data REs and DMRS REs overlapped with LTE CRS are punctured at the transmitter side |  |  |  |

-------------------------------------------------------------End of change 2----------------------------------------------------------------

-------------------------------------------------------------Begin of change 3-------------------------------------------------------------

##### 5.3.3.2.5	Minimum requirement for PDCCH overlapping with LTE CRS

The parameters specified in Table 5.3.3.2.5-1 are additional parameters for requirements with PDCCH overlapping with LTE CRS.

Table 5.3.3.2.5-1: Test Parameters

| Parameter |  | Unit | Value |
| --- | --- | --- | --- |
| TDD UL-DL pattern |  |  | FR1.15-1 |
| NR UL transmission with a 7.5 kHz shift to the LTE raster |  |  | true |
| CCE to REG mapping type |  |  | Non-interleaved |
| REG bundle size |  |  | 6 |
| Shift Index |  |  | 0 |
| PDCCH Configuration | Symbols with PDCCH |  | Symbol#1 and #2 |
| CRS for rate matching (Note 1) | LTE carrier centre subcarrier location |  | Same as NR carrier centre subcarrier location |
|  | LTE carrier BW | MHz | 10 |
|  | Number of antenna ports |  | 4 |
|  | v-shift |  | 0 |
| Note 1: No MBSFN is configured on LTE carrier. Note 2: LTE carrier is configured with Uplink-downlink configuration 2 Table 4.2-2, TS 36.211[15] and Special subframe configuration 7 Table 4.2-1, TS 36.211[15]. The start of transmission of LTE frame is delayed by 2 LTE subframes with respect to the start of transmission of NR frame. Note 3: NR PDCCH data REs and DMRS REs overlapped with LTE CRS are punctured at the transmitter side |  |  |  |

-------------------------------------------------------------End of change 3----------------------------------------------------------------