# 3GPP TSG-RAN WG4 Meeting #113	R4-2419337

# Orlando, US, 18th – 22nd November, 2024

Source:	Sony

Title:	System parameters for 3MHz BW in NTN

Agenda item:	7.8.3.1

Document for:	Discussion

# Background

In RAN#103, a new WID: enhanced requirements and test methodology for NR and IoT NTN has been approved [1]. From core requirement-wise, the target of this WID is to support HPUE for both NR and IoT NTN, and also introduce the 3MHz channel BW into the NTN system.

In this contribution, we discuss the remaining issues in the system parameters.

# Optionality of 3MHz channel BW

Conventionally, a new BW is introduced in release X is optional for the this release but mandatory from release X+1. The main consideration is to ensure that the UE is not mandated to support a feature that may require a change in the implementation. For 3MHz channel BW, considering the UE needs to support the punctured SSB design, it is proposed that the same principle can be applied here as well to avoid any risk on mandating any change of UE design in Rel-19.

Observation 1: Convetionally, a new BW is introduced in release X is optional for the this release but mandatory from release X+1.

Proposal 1: introduce 3MHz to NTN system from Rel-17 as Release independent feature while keep it optional up to  Rel-19 and mandatory from Rel-20.

# Supporting 3MHz for NTN Redcap and eRedcap

The analysis on the importance, spec impact and implementation impact of supporting 3MHz has been provided in appendix. IN general, we dont forsee any impact on RAN1/2 (except possible capability signalling) and device implementations. for The main concern on supporting 3Mhz of Redcap NTN in Rel-19 is that such a feature seems out of the scope of the WI. Therefore, a revision of the WI is necccesery for RAN4 to contninue the work. Therefore, along with this line, we propose that at least RAN4 can discuss and create a common understanding if 3MHz would be feasible for Redcap NTN in Rel-19, and then a revision of WI can be proposed in RAN.

Observation 2: no spec impact other than the capability signalling in RAN2 can be identified to support Redcap and eRedcap with 3MHz CBW in NTN

Proposal 1: It is proposed that RAN4 to discuss and create a common understanding if 3MHz CBW is feasible to be supported for Redcap NTN in Rel-19, a revision of WI can be proposed in RAN if there is a consensus in RAN4.

# Conclusion

In this contribution, we make the following observations and conclusions:

Observation 1: Convetionally, a new BW is introduced in release X is optional for the this release but mandatory from release X+1.

Observation 2: no spec impact other than the capability signalling in RAN2 can be identified to support Redcap and eRedcap with 3MHz CBW in NTN

Proposal 1: introduce 3MHz to NTN system from Rel-17 as Release independent feature while keep it optional up to  Rel-19 and mandatory from Rel-20.

Proposal 1: It is proposed that RAN4 to discuss and create a common understanding if 3MHz CBW is feasible to be supported for Redcap NTN in Rel-19, a revision of WI can be proposed in RAN if there is a consensus in RAN4.

# References

- RP-240857 New WID: Enhanced requirements and test methodology for NR and IoT NTN

- R4-2416532	Topic summary for [112bis][311] NR_IoT_NTN_less_than_5MHz_UERF	Moderator (Xiaomi

- R4-2416562	Way Forward for [112bis][311] NR_IoT_NTN_less_than_5MHz_UERF	Xiaomi

## Appendix

## A1  Why 3Mhz is needed for Redcap NTN

Supporting 3MHz channel BW for Redcap/eRedcap NTN was briefly discussed in RAN4#112 [2][3], and the major focus point is whether such a proposal is in the scope of WI.

3MHz channel BW in NR has been specified for TN operation since Rel-18 to support fragmented spectrums, especially for some re-farmed bands. In Rel-19, it is proposed to add this 3MHz channel BW for NR NTN operation as well to facilitate more efficient usage of spectrum for NTN bands, with the detailed motivation as below:

In Rel-18, the feature to support less than 5MHz for NR is defined for TN to cater for use-cases where there are spectrum limitation/constraints. Satellite deployments in L-Band and S-Band face the same constraints and would benefit from the work done in the Rel-18 WID “NR support for dedicated spectrum less than 5 MHz for FR1” by extending support for NTN.

This feature will enable more efficient deployment of the FR1 NTN bands where parts of the spectrum for satellite use is limited. This feature should be built on the work done in the Rel-18, i.e., WI of NR support for dedicated spectrum less than 5 MHz for FR, as much as possible by extending support for NTN.

In Rel-19, in addition to normal NR NTN devices, Redcap NTN devices are also introduced and are expected to operate under the same network as normal NR NTN devices. Therefore, it may become problematic for the network to utilize the 3MHz channel BW to efficiently use the spectrum if the Redcap NTN device can’t be supported.

Observation A1: as Redcap NTN and NR NTN can operate under the same network, it will be a challenge for the network to efficiently use the spectrum if 3MHz channel BW is not supported by Redcap NTN.

One concern from the RAN4#112 discussion is that the Redcap feature is not included for 3MHz work in the TN case. We admit the fact here, but per our understanding that the Redcap was precluded from 3MHz TN work in Rel-18 is mainly due to the fact that additional work in RAN4 would be needed to specify the RF requirement for the 3MHz channel BW, and thus, it is precluded from the Rel-18 work. In addition, the 3MHz channel was motivated by a particular deployment scenario in TN, where the Redcap support may not be very relevant. However, the current work for NTN is for a more general purpose, as we understand, and thus Redcap shall not be precluded.

Observation A2: Different from the original motivation to introduce the 3MHz in the TN case, the current work for NTN is for a more general purpose, and thus Redcap shall not be precluded.

After RAN4#112 and RAN4#111bis, the majority of RF requirements for Redcap NTN have been agreed with only HD-FDD REFSENS left and some ambiguity regarding GNSS operation. Therefore, it should be feasible to include the Redcap NTN in the scope of the work.

Observation A3: The major part of the core requirements for Redcap NTN UE has been settled.

## A2 Spec impact

From a technical perspective, the major changes due to the 3MHz channel BW is illustrated as below:

- A new channel BW of 3 MHz was introduced with a maximum number of usable resource blocks (RB) of 15 RBs

- a new dedicated synchronization raster for the 3 MHz channel BW was introduced

- 15-RB CORSET#0

we understand that the major changes in the UE implementation required for 3MHz channel BW is on the baseband, where it should be able to decode the punctured SSB. However, very little change is needed in terms of RF implementation.

Observation A4: From the RF perspective, it is feasible for Redcap NTN UE to support a smaller BW.

Per our understanding, Redcap UE share the same initial access procedure as normal NR UEs. Therefoer, since the procedure for NR UE with 3MHz channel BW has been well specified for normal NR UE, we don’t expect any additional impact in RAN1 to work.

Observation A5: Since Redcap UE share the same initial access procedure as normal NR UE, no RAN1 impact is expected to add RAN4