3GPP TSG-RAN WG4 Meeting #113	R4-2419339

Orlando, USA, November 18 – 22, 2024

Source:	Nokia

Title:	NR Demodulation Performance Phase 5: BS demodulation performance requirements for MMSE-IRC

Agenda item:	7.16.3

Document for:	Discussion

Introduction

In RAN4#112bis meeting, the discussions on Rel-19 NR demodulation performance Phase 5 BS Demodulation MMSE-IRC have led to several agreements. They are documented in the way forward (WF) in [1] which also captures the remaining open issues to be further discussed in RAN4#113.

In this contribution, we provide our views on the remaining open issues in [1] to be addressed in RAN4#113.

Discussion

The WF [1] has classified the remaining open issues into 3 subtopics, namely, General, Interference modelling, and Test parameters and simulation. Our views on the remaining open issues are provided in each of the respective subsections.

General

The remaining open issue in this subtopic is about the introduction of applicability rule as highlighted below:

Test scenarios - On HomNet and HetNet scenarios: - Cover HomNet and HetNet scenarios. - FFS whether to introduce additional applicability rule. - On scenario with unaligned TDD pattern: - For NR_demod_Ph5 WI, only consider interference model with aligned TDD pattern between cells.

In the WF [1], as shown above, the proposed applicability rule is not listed in detail. Nonetheless, it could be traced back that it came from [2], which has proposed the following:

Consider both HomNet and HetNet deployment requirements and introduce applicability rules based on BS class (i.e. D.2 in 38.141-1 Table 4.6).

The early discussion on such a proposal has been captured in RAN4 BDaT Chair’s meeting notes in [3], in which some companies would like to postpone the discussion on the proposal. Hence, it is still listed as FFS in the WF [1].

There are still related open issues that have not been agreed by companies, for example, the necessity of having system level simulation and the corresponding interference profiles for HomNet and HetNet due to HPUEs. Hence, it is better to keep the proposal on applicability rule as FFS, and the discussion on it can be brought up again once other open issues have been fully agreed in RAN4.

The issue on applicability rule can be kept as FFS and the discussion on it can be reinitiated after other open issues have been agreed.

Interference modelling

Interference Profile

The open issue to be addressed in this subtopic is as below:

Interference profile - On how to account for HPUE in existing LTE profiles: - Reuse the LTE interference power level as a start point, and check performance gain of MMSE-IRC over baseline MMSE. - It is not precluded for companies to bring system level simulation on other profiles. - Proposals on how to account for HPUE in existing LTE profiles. - Proposal 1: Perform new system-level simulations with the inclusion of HPUEs - Proposal 2: Consider artificially increasing DIP/INR with additional 3 or 7 dB

In RAN4#112bis, we have already provided our views on those issues above in [4]. The fact that HPUEs make the interference in NR more severe than in LTE has been used as one of the justifications of this WI as documented in [5]. Hence, the proper way to work on this WI is to conduct system-level simulation mainly for deriving the proper DIP in NR due to the HPUEs, which at the same time could be used to study the performance gain of MMSE-IRC over MMSE. While using either system-level or link-level simulation one could study the performance gain of MMSE-IRC over MMSE, nonetheless, system-level simulation provides more complete study especially in inter-cell interference scenario, as it includes the interactions among many cells. Hence, it brings more realistic and holistic observations on what could be expected in real deployments.

System-level simulation could provide more realistic DIP values for NR by taking into account interference signals from HPUEs.

Having the proper DIP in NR from system-level simulation, which includes interference from HPUEs, would be the proper way to work on this WI and validate the justifications of this WI.

In evaluating the performance gain of MMSE-IRC over MMSE in multicell scenario with inter-cell interference signals, a system-level simulation is more appropriate to be used than link-level simulation.

RAN4 to consider DIP values from new system-level simulations with the inclusion of HPUEs in NR.

In case system-level simulation is not possible, RAN4 to reuse the DIP from LTE (TR 36.884).

In relation to this open issue, we have another contribution in [11] that needs to be treated in different Agenda Item, as it is more generic and might lead to a discussion in RAN Plenary to revise the WID.

Number of Interferers

The next issue is related to the number of interferers:

Number of interferers - For 4Rx and 8Rx, consider 2 interferers. - Check the performance gain of 1 or 2 interferers for 2Rx and make decision based on simulation result.

The justification of this WI is that the introduction of HPUEs in NR would make the interference in NR to be more severe than in LTE. Hence, with such a justification, there should be one interference signal that represent a HPUE and another interference signal that represent normal UE.

RAN4 to consider only 2 interference signals, in which one signal represent HPUE and one signal represent a normal UE.

As stated in 36.884 [6], there is no significant gain of MMSE-IRC to the baseline MMSE receiver in the case of 2RX when the explicit interferer number is increased from 1 to 2. Hence, 2 explicit interferers are modeled for only 4Rx and 8Rx cases, while 2Rx only has 1 interferer.

One key point that needs to be raised here is that in 36.884 [6], the gain of MMSE-IRC over MMSE in the HetNet case is higher than HomNet. Most importantly, the least gain observed is 3,1 dB, which is the gain of 2Rx in HomNet, as highlighted below [6]:

## 7.3 Summary of link level evaluation According to the above simulation campaign, the link performance gain of MMSE-IRC over MMSE can be summarized as follows: - With respect to homogeneous network, for 1x2 cases the gain is about 3,1 dB; for 1x4 cases the gain is 4,4 dB ~4,7 dB; for 1x8 cases the gain is 4,9 dB ~5,0 dB. - With respect to heterogeneous network, for 1x2 cases the gain is 5,5 dB~5,9 dB; for 1x4 cases the gain is 7,5 dB~8,3 dB; for 1x8 cases the gain is 8,4 dB~8,9 dB. Therefore, it is concluded that the MMSE-IRC receiver can bring in the significant gain over MMSE receiver.

In RAN4#112bis, some companies have provided their simulation results which have been summarized in [8]. From [8] one can observe that in the case of 2Rx, there is SNR gain of MMSE-IRC over MMSE. Nonetheless, if one makes a further observation, most of the simulation results in HomNet scenario, the gain is (much) less than 3 dB, regardless of the number of interferers.

In RAN4, it is common when defining requirements to add impairments margin on top of the ideal simulation. The margin can be around 3 dB. Such additional impairment margin itself can already offset the gain that can be observed from the simulation results in RAN4#112bis in [8]. Hence, the final requirement itself cannot separate MMSE-IRC and MMSE.

The simulation results from companies in RAN4#112bis show that the gain of MMSE-IRC compared to MMSE in the 2Rx case is less than 3 dB in the case of HomNet regardless of the number of interferer signals.

In RAN4, it is common to add impairment margin, which can be around 3 dB, to the ideal simulation results. Such impairment margin can already offset the gain of MMSE-IRC over MMSE.

In this WID, the objective is to define the requirements for both HomNet and HetNet. Hence, the gain of MMSE-IRC should be sufficiently high enough for both HomNet and HetNet.

RAN4 do not need to specify requirements for 2Rx since the gain of MMSE-IRC over MMSE is less than 3 dB, especially in the case of HomNet.

For the 4Rx and 8Rx cases, RAN4 shall ensure that the gain of MMSE-IRC over MMSE is more than 3 dB.

In 36.766 [7], 8Rx BS is not covered in the evaluation, and one of the reasons is due to a large number of channel faders are needed for potential 8Rx IC (interference cancellation) test. It is noted here that in [7], the study is on intracell interference cancellation, in addition to the intercell interference.

In TR 36.884, with 2 explicit interferers, only 4Rx and 8Rx are considered.

In TR 36.766, only 2Rx and 4Rx are considered.

Based on previous studies documented in TR 36.884 and TR 36.766, RAN4 shall prioritize 4Rx case with 2 explicit interferers.

Test parameters and simulation assumptions

Channel Bandwidth

The remaining open issue in this subtopic is as highlighted below:

Channel bandwidth - Proposals for CP-OFDM: - Option 1: - For 15kHz SCS, cover 5MHz, 10MHz, 20MHz - For 30kHz SCS, cover 10MHz, 20MHz, 40MHz, 100MHz - Option 2: 10MHz/15kHz and 40MHz/30kHz for initial simulation purpose, additional channel bandwidth can be considered for requirement definition - Option 3: Minimum channel bandwidth for each SCS, i.e., 5MHz for 15kHz SCS and 10MHz for 30kHz - Option 4: Minimum/Maximum channel bandwidth for each SCS, i.e., 5/50MHz for 15kHz SCS and 10/100MHz for 30kHz - Option 5: 5/20MHz BW with 15KHz SCS and 10/100MHz BW with 30KHz - Candidate options for DFT-s-OFDM: - Option 1: - For 15kHz SCS, 5MHz - For 30kHz SCS, 10MHz - Other options are not precluded - Agreement: - For CP-OFDM: - For performance evaluation purpose, Minimum channel bandwidth for each SCS, i.e., 5MHz for 15kHz SCS and 10MHz for 30kHz. - Further discuss the CHBW for requirement definition

In RAN4#112bis, we have addressed the issue on channel bandwidth in [4]. The lowest and highest bandwidth sizes shall be considered for the requirements for each SCS. If RAN4 decides only 1 bandwidth sizes for each SCS, the lowest bandwidth size shall be considered.

Re-using FRCs currently available in 38.104 as much as possible would make the specification to be compact and easier to maintain. The Bandwidth sizes considered in 38.104 are 5/10/20MHz for 15KHz subcarrier spacing and 10/20/40/100MHz for 30KHz.

RAN4 to prioritize 5/50MHz BW with 15KHz SCS and 10/100MHz BW with 30KHz SCS to cover both extremes and have all DUTs testable regardless of BW support declaration. If RAN4 would reuse FRCs from 38.104, 5/20MHz BW with 15KHz SCS and 10/100MHz BW with 30KHz shall be prioritized.

RAN4 to consider the minimum BW if only one BW is chosen per SCS, namely, 5MHz BW with 15KHz SCS and 10MHz BW with 30KHz SCS.

Propagation Condition

The issue on propagation condition is as follows:

Propagation condition - Candidate options: - For serving cell: - Option 1: TDLA30-10 and TDLC300-100 for HetNet and HomNet respectively - Option 2: TDLA30-10 and TDLB100-100 for HetNet and HomNet respectively - Option 3: Cover TDLA30-10, TDLB100-400, TDLC300-100 - Option 4: Use TDLB100-400 - For interference cell: - Option 1: Introduce new propagation channel for testing with desired delay spread 1000ns for the interference PUSCH, i.e., TDLC1000 - Option 2: Same propagation condition for serving and interference PUSCH - Option 3: Use TDLC300-100

In LTE case as shown in [6], Table 7.1-2: Cases for link level evaluations, the studies on the gain of MMSE-IRC over MMSE were performed using the same propagation condition of intended cell and interfering cell, namely, (EPA5, EPA5) and (EVA70, EVA70). While in the definition of requirements in [6][9], a different propagation of intended cell and interfering cell was used, namely, (EPA5, ETU5) and (EVA70, ETU70).

Ideally, the propagation conditions that brings the least gain of MMSE-IRC over MMSE shall be chosen. Nonetheless, this will lead to exhaustive simulation effort from companies to simulate multiple combinations of propagation conditions. To simplify it, it is better to choose one fix propagation condition for the serving cell and study 2 propagation condition candidates for the interfering cell.

In LTE, to study the MMSE-IRC gain, same propagation conditions were used for both the serving cell and the interfering cell. Nonetheless, the requirements were defined using different propagation conditions.

RAN4 to consider TDLA30-10 for the serving cell.

RAN4 to consider TDLA30-10 and TDLC300-100 as candidates for interfering cells in the study and choose one of them for the requirements.

MCS, Waveform, DMRS Configuration and Test Requirement Definition

MCS - Candidate options: - Option 1: Cover QPSK/16QAM/64QAM - Option 1A: Use MCS 5(QPSK, R=379/1024), MCS 13(16QAM, R=490/1024) and MCS 24(64QAM, R=772/1024) - Option 1B: MCS index 2, 12, 20 in MCS table 1 - Option 1C: Cover MCS 2, 16, 20 - Option 2: Choose only one MCS configuration - Option 3: Consider MCS 5(QPSK, R=379/1024) and MCS 13(16QAM, R=490/1024) and FFS 64QAM (ZTE) Waveform - Agreement - Cover CP-OFDM and FFS DFT-s-OFDM DMRS configuration - Candidate options on DMRS type: - Option 1: Cover DMRS configuration type 1 and type 2 - Option 2: DMRS configuration type 1 - Candidate options on DMRS sequence generation: - Option 1: - NID0=0, nSCID =0 for CP-OFDM - NID0=0, group hopping and sequence hopping are disabled for DFT-s-OFDM - Option 2: - nSCID =0 for all cells - Different NID0=0 for serving and interference cells Test requirement definition - Proposals: - Option 1: Using SINR based requirement definition - Option 2: SNR based requirement definition - Proposals on SINR definition: - Option 1: - For BS type 1-C and BS type 1-H: The SINR used in this clause is specified based on a single carrier and defined as: SINR = S / N’ Where: S is the total signal energy of one intra-cell UE in the slot on a single antenna connector (for BS type 1-C) or on a single TAB connector (for BS type 1-H). N’ is the summation of the received energy of the strongest inter-cell interfering signals explicitly defined in a test procedure plus the white noise energy N in a bandwidth corresponding to the transmission bandwidth over the same duration where signal energy exists on a single antenna connector (for BS type 1-C) or on a single TAB connector (for BS type 1-H). The respective energy of each inter-cell interfering signal relative to N is defined by its associated INR value.. - For BS type 1-O The SINR used in this clause is specified based on a single carrier and defined as: SINR = S / N’ Where: S is the total signal energy of one intra-cell UE in a slot on a RIB. N’ is the summation of the received energy of the strongest inter-cell interfering signals explicitly defined in a test procedure plus the white noise energy N in a bandwidth corresponding to the transmission bandwidth over the same duration where signal energy exists on a RIB. The respective energy of each inter-cell interfering signal relative to N is defined by its associated INR value..

- 38.104 [10] has already captured test parameters and FRCs for different scenarios. Such test parameters and FRCs can be reused as well for defining requirements for BS MMSE-IRC. Reusing as many as possible the test parameters and FRCs already available in 38.104 will also bring advantages such as easier to maintain and the compactness of the specification.

The FRCs in 38.104 for the corresponding bandwidth sizes can be used for the intended signals.

As synchronous deployments (with same UL:DL configurations for TDD) among cells are considered, the interference signals could also use the same FRC as the intended signal.

RAN4 to reuse the FRCs available in 38.104 for the considered bandwidth sizes.

RAN4 to consider only CP-OFDM.

RAN4 to consider DMRS configuration type 1.

RAN4 to consider nSCID=0 for all cells and different NID0=0 for serving and interference cells

RAN4 to consider the same MCS for both intended signal and interference signal. Furthermore, as the DIP in LTE is generated with 16QAM, the intended signal is also using 16QAM.

RAN4 to consider requirements based on SNR.

Conclusion

In this contribution, we provide our views on Rel-19 NR Demodulation Performance Phase 5 UL BS MMSE-IRC. The following Observations and Proposals were made:

Proposal 1: The issue on applicability rule can be kept as FFS and the discussion on it can be reinitiated after other open issues have been agreed.

Observation 1: System-level simulation could provide more realistic DIP values for NR by taking into account interference signals from HPUEs.

Observation 2: Having the proper DIP in NR from system-level simulation, which includes interference from HPUEs, would be the proper way to work on this WI and validate the justifications of this WI.

Observation 3: In evaluating the performance gain of MMSE-IRC over MMSE in multicell scenario with inter-cell interference signals, a system-level simulation is more appropriate to be used than link-level simulation.

Proposal 2: RAN4 to consider DIP values from new system-level simulations with the inclusion of HPUEs in NR.

Proposal 3: In case system-level simulation is not possible, RAN4 to reuse the DIP from LTE (TR 36.884).

Proposal 4: RAN4 to consider only 2 interference signals, in which one signal represent HPUE and one signal represent a normal UE.

Observation 4: The simulation results from companies in RAN4#112bis show that the gain of MMSE-IRC compared to MMSE in the 2Rx case is less than 3 dB in the case of HomNet regardless of the number of interferer signals.

Observation 5: In RAN4, it is common to add impairment margin, which can be around 3 dB, to the ideal simulation results. Such impairment margin can already offset the gain of MMSE-IRC over MMSE.

Observation 6: In this WID, the objective is to define the requirements for both HomNet and HetNet. Hence, the gain of MMSE-IRC should be sufficiently high enough for both HomNet and HetNet.

Proposal 5: RAN4 do not need to specify requirements for 2Rx since the gain of MMSE-IRC over MMSE is less than 3 dB, especially in the case of HomNet.

Proposal 6: For the 4Rx and 8Rx cases, RAN4 shall ensure that the gain of MMSE-IRC over MMSE is more than 3 dB.

Observation 7: In TR 36.884, with 2 explicit interferers, only 4Rx and 8Rx are considered.

Observation 8: In TR 36.766, only 2Rx and 4Rx are considered.

Proposal 7: Based on previous studies documented in TR 36.884 and TR 36.766, RAN4 shall prioritize 4Rx case with 2 explicit interferers.

Observation 9: Re-using FRCs currently available in 38.104 as much as possible would make the specification to be compact and easier to maintain. The Bandwidth sizes considered in 38.104 are 5/10/20MHz for 15KHz subcarrier spacing and 10/20/40/100MHz for 30KHz.

Proposal 8: RAN4 to prioritize 5/50MHz BW with 15KHz SCS and 10/100MHz BW with 30KHz SCS to cover both extremes and have all DUTs testable regardless of BW support declaration. If RAN4 would reuse FRCs from 38.104, 5/20MHz BW with 15KHz SCS and 10/100MHz BW with 30KHz shall be prioritized.

Proposal 9: RAN4 to consider the minimum BW if only one BW is chosen per SCS, namely, 5MHz BW with 15KHz SCS and 10MHz BW with 30KHz SCS.

Observation 10: In LTE, to study the MMSE-IRC gain, same propagation conditions were used for both the serving cell and the interfering cell. Nonetheless, the requirements were defined using different propagation conditions.

Proposal 10: RAN4 to consider TDLA30-10 for the serving cell.

Proposal 11: RAN4 to consider TDLA30-10 and TDLC300-100 as candidates for interfering cells in the study and choose one of them for the requirements.

38.104 [10] has already captured test parameters and FRCs for different scenarios. Such test parameters and FRCs can be reused as well for defining requirements for BS MMSE-IRC. Reusing as many as possible the test parameters and FRCs already available in 38.104 will also bring advantages such as easier to maintain and the compactness of the specification.

Observation 11: The FRCs in 38.104 for the corresponding bandwidth sizes can be used for the intended signals.

Observation 12: As synchronous deployments (with same UL:DL configurations for TDD) among cells are considered, the interference signals could also use the same FRC as the intended signal.

Proposal 12: RAN4 to reuse the FRCs available in 38.104 for the considered bandwidth sizes.

Proposal 13: RAN4 to consider only CP-OFDM.

Proposal 14: RAN4 to consider DMRS configuration type 1.

Proposal 15: RAN4 to consider nSCID=0 for all cells and different NID0=0 for serving and interference cells

Proposal 16: RAN4 to consider the same MCS for both intended signal and interference signal. Furthermore, as the DIP in LTE is generated with 16QAM, the intended signal is also using 16QAM.

Proposal 17: RAN4 to consider requirements based on SNR.

- References

- R4-2416548, WF on [112bis][319] NR_demod_Ph5_Part1_General_BS, 3GPP RAN4#112bis, China Telcom, Hefei, China, October 14-18, 2024

- R4-2416272, On general issues for MMSE-IRC receiver under interference, 3GPP RAN4#112bis, Ericsson, Hefei, China, October 14-18, 2024

- R4-xxxxxxx, RAN4_112bis_BDaT_Session_report_v7_PostMeeting, 3GPP RAN4#112bis, Hefei, China, October 14-18, 2024

- R4-2416442, NR Demodulation Performance Phase 5: BS demodulation performance requirements for MMSE-IRC, 3GPP RAN4#112bis, Nokia, Hefei, China, October 14-18, 2024

- RP-241297, Revised WID on NR demodulation performance: Phase 5, 3GPP RAN#104, China Telecom, NTT DOCOMO, Shanghai, China, June 17-20, 2024

- 3GPP TR 36.884, Performance requirements of MMSE-IRC receiver for LTE BS, v13.1.0, 09-2016

- 3GPP TR 36.766, Study on interference cancellation receiver for LTE BS, v15.0.0, 07-2017

- R4-2415994, Simulation results summary for BS requirement with MMSE-IRC receiver, 3GPP RAN4#112bis, Samsung, Hefei, China, October 14-18, 2024

- 3GPP TS 36.104, Evolved Universal Terrestrial Radio Access (E-UTRA) - Base Station (BS) radio transmission and reception, v18.5.0, 03-2024

- 3GPP TS 38.104, NR - Base Station (BS) radio transmission and reception, V18.6.0, 06-2024

- R4-2419340, HPUE justification in WID and interference profile, 3GPP RAN4#113, Nokia, Orlando, USA, November 18-22, 2024