3GPP TSG-RAN WG4 Meeting # 113 	    R4-2419359

Orlando, US, 18  – 22 November 2024

Agenda Item:	5.22.2.6

Source: 	Ericsson

Title:	Accuracy requirements for additional PRS configurations for CPP

Document for:	Approval

Introduction

- In the last meeting accuracy requirements for CPP for additional PRS configurations were discussed. In this contribution simulation results for RSCPD measurements for the following configurations are therefore presented:

- SINR values:

  - (-6,-13,-13)dB

  - (-3, -6, -6)dB

- Propagation conditions:

  - AWGN

  - Two-tap channel model

- PRS configuration FR1:

  - 30 kHz: PRS BW 132

  - 60 kHz: PRS BWs: 64 and 132

- PRS configuration FR2:

  - 120 kHz, PRS BW 128

Simulation results

Table 1. AWGN channel for (-6, -13, -13)dB.

| SCS | PRS BW | Error (degree) |
| --- | --- | --- |
| 30 kHz | 132 PRBs | 4.93 |
| 60 kHz | 64 PRBs | 6.78 |
| 60 kHz | 132 PRBs | 4.66 |
| 120 kHz | 128 PRBs | 5.53 |

Table 2. Two-tap channel for (-6, -13, -13)dB.

| SCS | PRS BW | Error (degree) |
| --- | --- | --- |
| 30 kHz | 132 PRBs | 6.57 |
| 60 kHz | 64 PRBs | 9.40 |
| 60 kHz | 132 PRBs | 6.18 |
| 120 kHz | 128 PRBs | 7.06 |

Table 3. AWGN channel for (-3, -6, -6)dB.

| SCS | PRS BW | Error (degree) |
| --- | --- | --- |
| 30 kHz | 132 PRBs | 2.32 |
| 60 kHz | 64 PRBs | 3.60 |
| 60 kHz | 132 PRBs | 2.42 |
| 120 kHz | 128 PRBs | 3.05 |

Table 4. Two-tap channel for (-3, -6, -6)dB.

| SCS | PRS BW | Error (degree) |
| --- | --- | --- |
| 30 kHz | 132 PRBs | 3.53 |
| 60 kHz | 64 PRBs | 5.13 |
| 60 kHz | 132 PRBs | 3.45 |
| 120 kHz | 128 PRBs | 4.59 |

Proposal 1: Consider simulation results in Table 1, Table 2, Table 3, and Table 4 while deriving accuracy requirements for RSCPD and relative RSCP measurements under AWGN and Two-tap propagation conditions for side conditions (-6, -13, -13)dB and (-3, -6, -6)dB.

Summary

Simulation results for RSCPD measurements for additional PRS configurations are presented in the contribution and the following is proposed.

Proposal 1: Consider simulation results in Table 1, Table 2, Table 3, and Table 4 while deriving accuracy requirements for RSCPD and relative RSCP measurements under AWGN and Two-tap propagation conditions for side conditions (-6, -13, -13)dB and (-3, -6, -6)dB.

References

- R4-2416870, WF on RRM requirements for NR_pos_enh2_Part2, CATT.